3GPP TSG-WG RAN4 Meeting #104-bis-e	R4-2217125

# Online, 10th – 19th October, 2022

Source: 				Nokia, Nokia Shanghai Bell, OPPO

Title: 				TP on System parameters for APT600

Agenda Item: 	5.27.2

Document for:	Approval

# Introduction

A new WI to introduce a 600 MHz band for APT region was approved in RAN#96 [1]. The feasibility study of the band has been already completed as reported in [2]. The band plan to specify is based on option B1 with a single FDD duplexer 2x40 MHz for 663-703 MHz uplink and 612-652 MHz.

In this contribution a TP to introduce a band definition and system parameters are proposed according to the agreed WF [3] in RAN4#104-e.

# Discussion

Band number n105 is available for APT600 after it was confirmed not used anymore by WI LTE_TDD_1670_1675MHz. NR operating band for APT600 band can be defined as presented in Table 1.

Table 1: NR operating band for APT600 band

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

As agreed in WF [3], the channel bandwidth and subcarrier spacing are according to the WID scope [1] as presented in Table 2. No more bandwidth (such as 40 MHz) is not considered for now.

Table 2: Channel bandwidth and SCS parameters for APT600 band

| SCS (kHz) | channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |
| 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |

Channel raster has been agreed to be based on 100 kHz. ARFCN range is defined according to Table 3.

Table 3: Applicable NR-ARFCN for APT600 band

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n105 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |

Then, for sync raster, SSB SCS is agreed to be 15kHz and pattern A [3] as presented in Table 4. The GSCN range is 1535-<1>-1624.

Table 4: Applicable SS raster entries for APT600 band

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n105 | 15 kHz | Case A | 1535 – <1> – 1624 |

# Conclusion

A text proposal on the system parameter to the TR is attached in Annex.

# Reference

[1] RP-221778 New WID on APT 600 MHz NR band

[2] TR 38.860 Study on extended 600MHz NR band

[3] R4-2214433 WF on system parameters for APT 600 MHz, Huawei, HiSilicon

[4] R4-2214434 WF on BS requirements for APT 600 MHz, Nokia, Nokia Shanghai Bell

[5] R4-2214435 WF on UE requirements for APT 600 MHz, Skyworks Solutions Inc

[6] R4-2215090 draft CR to TS38.104 the introduction of APT600MHz, ZTE Corporation

[7] R4-2215102 Introduction of APT 600 MHz band, Qualcomm Incorporated

[8] TR 38.892 v0.0.1

ANNEX: Text proposal to TR 38.892

# 5	Frequency band arrangement

## 5.1	Operating band, channel bandwidth and channel arrangement

NR band APT600 is designed to operate in the operating band defined in Table 5.1-1.

Table 5.1-1: NR operating band for APT600 band

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

The requirements in the TR apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.1-2.

Table 5.1-2: Channel bandwidth and SCS parameters for APT600 band

| SCS (kHz) | channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |
| 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |

The channel raster of APT600 band is 100 kHz based (the same as band 71/n71). The applicable NR-ARFCN entries for band n105 are defined in Table 5.1-3.

Table 5.1-3: Applicable NR-ARFCN for APT600 band

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n105 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |

The synchronization raster for APT600 band is give in Table 5.1-4.

Table 5.1-4: Applicable SS raster entries for APT600 band

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n105 | 15 kHz | Case A | 1535 – <1> – 1624 |