3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2217131

Electronic Meeting, 10 Oct. – 19 Oct. 2022

Agenda Item:	5.28.3

Source:	Charter Communications, Inc.

Title:	WF on NR-U PC3 requirements and MPR/A-MPR

Document for:	Approval

# 1	Introduction of Power Class 3

## 1.1	Sub-topic 1-1 on PC3 MPR

Issue 1-1-1: ACLR requirement

- Proposals

  - Option 1: Define 30dBc ACLR

  - Option 2: Define 27dBc ACLR

  - Option 3: No ACLR requirement shall be defined

- Agreement:

  - Define 27dBc ACLR requirement.

Issue 1-1-2: PC3 MPR

- Proposals

  - Option 1: Define MPR according to R4-2215783 (Proposal for 30dBc ACLR)

  - Option 2: Define MPR according to R4-2215651 (Proposals for 27dBc and 30dBc ACLR)

  - Option 3: Postpone discussion until ACLR requirements are fully agreed

  - Option 4: Other (please specify)

- Recommended WF

  - Further check and confirm the following tentative agreement in the next meeting.

  - Define MPR based on 27dBc ACLR

Issue 1-1-3: Antenna Isolation

- Proposals

  - Option 1: For 2Tx, antenna isolation >16dB is used to minimize the difference between 1Tx and 2Tx MPR which is already 1dB without RIMD contribution.

  - Option 2: Further evaluate for 10dB and 16dB isolation as one company observed no difference in power back-off need.

  - Option 3: Other (please specify)

- Tentative agreements:

  - The background for antenna isolation assumption was discussed and it was determined that the practical design considerations and UE form factor are the driving factors for the value. As one company did not observe major differences between 10dB and 16dB antenna isolation it was proposed to further study the significance of the two proposed isolation values.

- Recommended WF

  - Further evaluate for 10dB and 16dB isolation by providing the measurements and simulation results in future meetings.

## 1.2	Sub-topic 1-2: A-MPR

Issue 1-2-1: US LPI 1Tx (NS_53)

- Proposals

  - Option 1: Define A-MPR according to R4-2215651

  - Option 2: Other (please specify)

- Tentative agreements:

  - The A-MPR table from R4-2215651 seems acceptable for 1Tx by most companies.

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | QPSK | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 16 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 64 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 256 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
| CP-OFDM | QPSK | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 16 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 64 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 256 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [6.5] | ≤ [8.5] | ≤ [6.5] | ≤ [7.5] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - Evaluate A-MPR for 2Tx to understand if one or two A-MPR tables are needed to be specified.

    - Merging two non-equal tables are not precluded.

Issue 1-2-2: US SP 1Tx (NS_54)

- Proposals

  - Option 1: Define A-MPR according to R4-2215351

  - Option 2: Define A-MPR according to R4-2215651

- Tentative agreements:

  - It seems to lean to take Option 1 as starting point for further discussion.

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | QPSK |  | ≤ [3.0] | ≤ [5.0] |
|  | 16 QAM |  | ≤ [3.5] | ≤ [5.0] |
|  | 64 QAM |  | ≤ [3.5] | ≤ [5.0] |
|  | 256 QAM |  | ≤ [5.0] | ≤ [6.0] |
| CP-OFDM | QPSK |  | ≤ [4.5] | ≤ [6.0] |
|  | 16 QAM |  | ≤ [5.0] | ≤ [6.0] |
|  | 64 QAM |  | ≤ [5.5] | ≤ [6.0] |
|  | 256 QAM |  | ≤ [6.5] | ≤ [7.0] |
| NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz], and 100 MHz channels centered at the nearest NR-ARFCN corresponding to [5995 MHz]. |  |  |  |  |

- Recommended WF

  - Discuss what values to add for PI/2 BPSK.

  - Discuss if value for CP-OFDM-64QAM should be 5.0, as the average from the two papers.

  - Discuss why (any) backoff is needed for allocations away from the edges.

  - Evaluate and discuss if a dedicated 2Tx A-MPR tables is needed.

    - Merging two non-equal tables are not precluded.

Issue 1-2-3: Reuse NS_60 for A-MPR of NR-U PC3 based LPI in South Korea

- Proposals

  - Option 1: Yes

  - Option 2: Other (please specify)

- Agreement:

  - Yes

Issue 2-2-4: South Korea LPI 1Tx (NS_60)

- Proposals

  - Option 1: Define A-MPR according to R4-2215783

  - Option 2: Define A-MPR according to R4-2215651

- Tentative agreements:

  - Proposal of Merging the tables for 1Tx. The average value from the two proposals, if the number is not .5 dB or .0 dB, it is round up to the closest of them.

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ [9.5] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [2.5] | ≤ [5.0] |
|  | QPSK | ≤ [9.5] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [2.5] | ≤ [5.0] |
|  | 16 QAM | ≤ [9.5] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [2.5] | ≤ [5.0] |
|  | 64 QAM | ≤ [9.5] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [2.5] | ≤ [5.5] |
|  | 256 QAM | ≤ [9.5] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [4.5] | ≤ [6.0] | ≤ [4.5] | ≤ [5.5] |
| CP-OFDM | QPSK | ≤ [9.5] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [4.0] | ≤ [6.0] | ≤ [3.5] | ≤ [5.0] |
|  | 16 QAM | ≤ [9.5] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [4.0] | ≤ [6.0] | ≤ [3.5] | ≤ [5.0] |
|  | 64 QAM | ≤ [9.5] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [4.0] | ≤ [6.0] | ≤ [4.0] | ≤ [5.0] |
|  | 256 QAM | ≤ [9.5] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [5.5] | ≤ [6.5] | ≤ [5.5] | ≤ [6.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - Further discuss to merge the tables by averaging the proposals, and if difference e.g. 0.5dB, should the agreement be to take the lower or the higher value.

Issue 2-2-5: South Korea LPI 2Tx (NS_60)

- Proposals

  - Option 1: Define A-MPR according to R4-2215783

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [3.0] | ≤ [5.5] |
|  | QPSK | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [3.0] | ≤ [5.5] |
|  | 16 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [3.0] | ≤ [5.5] |
|  | 64 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [3.0] | ≤ [5.5] |
|  | 256 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [4.0] | ≤ [6.0] | ≤ [4.0] | ≤ [5.5] |
| CP-OFDM | QPSK | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [5.0] | ≤ [7.0] | ≤ [4.5] | ≤ [6.0] | ≤ [4.5] | ≤ [5.5] |
|  | 16 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [5.0] | ≤ [7.0] | ≤ [4.5] | ≤ [6.0] | ≤ [4.5] | ≤ [5.5] |
|  | 64 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [5.0] | ≤ [7.0] | ≤ [4.5] | ≤ [6.0] | ≤ [4.5] | ≤ [5.5] |
|  | 256 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [5.0] | ≤ [7.0] | ≤ [5.5] | ≤ [6.0] | ≤ [5.5] | ≤ [5.5] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - See if more companies may bring simulation results for next meeting.

  - Further discuss 2Tx A-MPR in relation to 1Tx A-MPR.

    - The two tables differ for the bandwidths’ 80 and 100MHz. Discuss if a dedicated needed for 2Tx A-MPR?

    - Merging two non-equal tables are not precluded.

## 1.3 	Sub-topic 1-3: NR-U power class fallback

Issue 1-3-1: Introduce power class fallback for NR-U

- Proposals

  - Option 1: Introduce NR-U power class fallback similar to licenced operation

    - Option 1a: Network should be made aware of any fallback behaviour

  - Option 2: Power class fallback is not needed for unlicenced access

    - Option 2a: Unlicenced access does not have SAR issue

- Recommended WF

  - Further discuss the need and justification for unlicensed power class fallback.

    - If fallback is introduced, discuss option 1a.

# 2	Reference

[1] R4-2216957, Email Discussion Summary for [104-bis-e][120] NR_unlic_enh, Apple