3GPP TSG-RAN WG4 Meeting # 104bis-e												R4- 2217133

Electronic Meeting, October 10 – October 19 2022

Agenda item:			4.3.8

Source:	Moderator (Qualcomm)

Title:	Email discussion summary for [104-e][204] NR_ext_to_71GHz_RRM_2

Document for:	Information

# Introduction

This email discussion document focuses on RRM performance requirements for NR operation extension to 71GHz including test configurations, impact of LBT on RRM performance requirements etc.

Draft CRs are treated in this document.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round

  - Test configurations

  - CCA aspects

  - Draft CRs

  - Test-cases and work-split

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei, HiSilicon | Zhongyi Shen | shenzhongyi3@huawei.com |
| Ericsson | Ming Li | ming.l.li@ericsson.com |
| Nokia | Rafael Paiva | Rafael.paiva@nokia.com |
| CATT | Gao Lingyu | gaolingyu@catt.cn |
| Qualcomm | Prashant Sharma | prasshar@qti.qualcomm.com |
| Apple | Steven Chen | steven.x.chen AT apple.com |
| Nokia | Erika Almeida | erika.almeida@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: General

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215417 | CATT | Proposal 1: In order to test the behavior of UE more thoroughly, it is suggested to use a fixed sequential mode to shift the SSB index. For example, the unavailable SSB in the first 12 SSBs could be the first SSB, the unavailable SSB in the second 12 SSBs could be the second SSB, and so on. |
| R4-2216259 | Nokia | Observation 1: Timing requirements defined for FR2-2 are related to the subcarrier spacing, with more accurate requirements applied for larger SCS. Observation 2: SCS have little impact on the RRSI measurement accuracy. Therefore it was agreed in RAN4 that the UE only needs to be tested for one of the supported test configurations. Observation 3: The same conclusions from RSSI do not apply for other test cases, where the SCS have strong impact on the requirements. Proposal 1: The test configurations in which the UE is required to be tested must be discussed for each test case. Proposal 2: UE is required to be tested with the largest supported SCS for UL transmit timing test cases. Proposal 3: UE is required to be tested with the largest supported SCS for timing advance accuracy test cases |
| R4-2216267 | Huawei | Proposal 1: Define CCA model as follows: - Prior to each SSB/SMTC group which is consist of 12 SSB/SMTC, the test equipment shall determine whether the CCA attempt is successful based on probability PCCA_DL. - If the CCA attempt is determined to be successful, then the test equipment shall transmit remaining transmissions for the SSB/SMTC group. - If the CCA attempt is determined to be unsuccessful, one of the SSB shall not be transmitted by the test equipment. The SSB within the SSB/SMTC group shall be randomly chosen from all SSBs within the group. The test equipment shall transmit rest transmissions for the SSB/SMTC group. Proposal 2: Define PCCA_DL = 0.9 in each test case, which is the probability that all SSBs are available within one SSB/SMTC group. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Test configurations

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-1-1: Test configurations - General

- Proposal 1 (Nokia): The test configurations in which the UE is required to be tested must be discussed for each test case

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We agree with proposal 1. As also commented in last meeting, 480/960 kHz are optionally supported by UE, if UE can pass test for 120 kHz, it doesn’t not sufficient to verify that the performance of 480/960kHz can be fulfilled. |
| Ericsson | Proposal 1 is rational. Keeping current stuff also is acceptable. |
| Nokia | We agree with proposal 1. We need to differentiate which test cases are more important to be tested with some of SCS declared to be supported by the UE. |
| CATT | Agree with proposal 1, and the same with Huawei. |
| MTK | Fine with proposal 1 for tests for 480/960 kHz |
| Qualcomm | Proposal 1 is too generic. Proponents are expected to clarify the configurations and test cases. |
| Apple | Good test coverage and reasonable testing overhead are always the goal of testing. We wonder if this is still the case with FR2-2. Perhaps some clarification on proposal is needed. |

Issue 1-1-2: Test configurations – Timing

- Proposal 1 (Nokia): UE is required to be tested with the largest supported SCS for UL transmit timing test cases

- Proposal 2 (Nokia): UE is required to be tested with the largest supported SCS for timing advance accuracy test cases

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We are fine with p1and p2. |
| Ericsson | We’re fine with the two proposals. |
| Nokia | We agree with proposal 1 and proposal 2. We think that the UE should be tested for timing with the largest SCS that it declares support. |
| CATT | Ok with proposal 1 and proposal 2. |
| MTK | Ok with proposal 1 and proposal 2. |
| Qualcomm | We don’t have such test requirements for legacy FR2. We understand that 480/960 have tighter requirements, but it should be sufficient to pass one of the test cases |
| Apple | To clarify, if a UE supports 120/480/960kHz SCS, does it mean only 960kHz SCS is tested? |
| vivo | Fine with P1 and P2. |
| Qualcomm | Based on our comment during the GTW, we propose the following: Proposal 3 (Qualcomm): For UL transmit timing and timing advance accuracy test cases: - Test configurations are defined for all SCS – 120kHz, 480kHz and 960kHz - UE that supports 120kHz SCS only is required to pass the test with 120kHz SCS configuration - UE that supports 120kHz and 480kHz SCS only is required to pass the test with 480kHz SCS configuration - UE that supports 120kHz and 960kHz SCS only is required to pass the test with 960kHz SCS configuration - UE that supports all SCS(120kHz, 480kHz and 960kHz) is required to pass the test with one of the configurations from 480kHz and 960kHz SCS |

### Sub-topic 1-2: CCA aspects

Sub-topic description: The following was agreed in the last meeting:

Agreement: For CCA model in test cases, an unavailable SSB/SMTC group can be modelled as that there is exactly one SSB not transmitted by TE in N consecutive SSB/SMTC occasions

•	Shift SSB index in each N consecutive SSB/SMTC occasions rather than keeping one fixed SSB index

•	FFS: Exact shifting pattern

Open issues and candidate options before e-meeting:

Issue 1-5-1: CCA modelling in test cases

- Proposal 1 (Huawei): Define CCA model as follows:

  - Prior to each SSB/SMTC group which is consist of 12 SSB/SMTC, the test equipment shall determine whether the CCA attempt is successful based on probability PCCA_DL.

  - If the CCA attempt is determined to be successful, then the test equipment shall transmit remaining transmissions for the SSB/SMTC group.

  - If the CCA attempt is determined to be unsuccessful, one of the SSB shall not be transmitted by the test equipment. The SSB within the SSB/SMTC group shall be randomly chosen from all SSBs within the group. The test equipment shall transmit rest transmissions for the SSB/SMTC group

- Proposal 1b (new): Define CCA model as follows:

  - Prior to each SSB/SMTC group which is consist of 12 SSB/SMTC, the test equipment shall determine whether the CCA attempt is successful based on probabilities PCCA_DL1 and PCCA_DL2.

  - If the CCA attempt is determined to be successful, then the test equipment shall transmit remaining transmissions for the SSB/SMTC group.

  - If the CCA attempt is determined to be unsuccessful, none of the SSB/SMTC occasions shall be transmitted by the test equipment during the SSB/SMTC group for that SSB index.

- Proposal 2 (Huawei): Define PCCA_DL = 0.9 in each test case, which is the probability that all SSBs are available within one SSB/SMTC group

- Proposal 2b (new): Define PCCA_DL1 = PCCA_DL2 = 0.75 in each test case, which is the probability that all SSBs are available within one SSB/SMTC group

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We support p1 and p2, which is also related to issue 1-5-2 about the SSB index shifting. In P1, the SSB is randomly chosen within 12 SSBs, while in proposal 1 in issue 1-5-2, a fixed pattern is used. Either way is fine to us. |
| Nokia | We agree with P1b (new) and P2b (new) For the P2, we think the success probability is too large. We should use P=0.75 as in NRU, otherwise the change of a single LBT failure during a test run is really small. P1a is proposed because it was unclear to us the relationship between SSB occasions and SSB indexes for a given SSB group. |
| CATT | We prefer the wording of the third bullet in Proposal 1b, and we are not sure about the success probability P, but prefer the value in Proposal 2b. |
| MTK | Prefer to P1b for the simplicity. |
| Qualcomm | Prefer proposal 1b. We support proposal 2 since FR2-2 will be implemented with highly directional beams and CCA failure probability is extremely low. Furthermore, very few devices are expected to operate in FR2-2 band as compared to FR1 Unlicensed band which is crowded with other technologies. |
| Apple | We support proposal 1 as it is more aligned with the core requirement. We also support proposal 2 as the CCA failure rate is not expected to be high. |
| Huawei2 | For P1b, it is not very clear to UE the meaning of PCCA_DL1 and PCCA_DL2? Is it the probability of the 1st and 2nd SSB index within one SSB burst as defined for NR-U? In Rel-16 NR-U, it is to test whether UE can monitor two candidate SSB index since there could be more QCL-ed candidate SSB index. However, for FR2=2, there could be at most two QCL-ed SSB index (Q=32). Besides, according to previous agreements, the unavailable SSB group is modelled as exactly one SSB is not available. It means for this particular SSB burst, all SSB index are not transmitted. Do we need to have independent CCA probability for SSB index within the SSB burst? |
| vivo | Probabilities PCCA_DL1 and PCCA_DL2 needs further clarification. |
| Nokia2 | After the GTW, we think that there are some concepts that might be unclear. We try to clarify them using the previous agreements in this topic. In RAN4 #102, it was agreed: - Use “FR2-2” term for all CCA related requirements in the spec since currently there is no CCA operation in FR2-1 - The step of FR2-2 RRM requirements extension due to missed SMTC/SSB occasions is equal to N SMTC/SSB occasions, where N is RX beam sweeping scaling factor - The RRM requirements are extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC occasions group consists of N consecutive SSB/SMTC occasions. An SSB/SMTC occasions group is not available, when at least one SSB/SMTC occasion in the group is not transmitted by the gNB. - The definition of SSB/SMTC occasion follows Rel-16 NR-U definition. From TS 38.133, we have that: In the requirements of clause 9.2A, the term SMTC occasion not available at the UE refers to when the SMTC contains SSBs configured by gNB in a cell on a carrier frequency subject to CCA, but NSSB candidate SSB positions for the same SSB index within the discovery burst transmission window are not available at the UE due to DL CCA failures at gNB during the corresponding period, where: - For the cell detection procedure: NSSB is at least one candidate SSB position (NOTE: the one candidate SSB position for the cell detection shall not be impacted by the set of candidate SSB positions which are already being measured by the UE within the current measurement period of the on-going measurements), and - For other procedures in clause 9.2A: NSSB are the first two successive candidate SSB positions when two or more candidate SSB positions are configured for this SSB index in one discovery burst transmission window, otherwise NSSB is one candidate SSB position; otherwise the SMTC occasion is considered as available at the UE.. Therefore, in our view, when the agreement from last meeting (copied in the description of this issue) mentions that “there is exactly 1 SSB not transmitted by TE…” it means that both candidate positions for a given SSB index failed. Therefore, our proposal to consider two probabilities, just as in NR-U. After the TE determines that both candidate positions of a SSB index failed, then we agree that this SSB index is not transmitted in N consecutive SSB/SMTC occasions, following the agreement from last meeting. We hope this clarifies our comment. |
| Qualcomm | Thanks to Nokia for clarifying the scenario. We think the following approach can be used: 1. TE picks one SSB out of a group of each 12 consecutive SSBs 2. For this SSB, TE determines with probabilities PCCA_DL1 and PCCA_DL2 whether to transmit this SSB or not. Note that other 11 SSBs shall be transmitted by the gNB 3. If the SSB is not transmitted, the SSB occasion group is considered as unavailable to the UE 4. TE repeats the procedure for next 12 consecutive SSBs The only question here is how TE picks one SSB out of 12 SSBs. One way to do it would be to pick a random SSB out of the 12 and the other way would be that the SSBs are picked in a sequential manner. |
| Huawei3 | Thanks to Nokia for the clarification. We think we are all on the same page. If an SSB (or SSB burst if more than 1 SSB index is configured in the test) is determined to be unavailable, none of the SSB index (including candidate positions) will be transmitted. Then, for other SSB/SSB burst in 12 consecutive SSBs/SSB bursts, all SSB index are transmitted. Thus, it seems the status of SSB index within an SSB burst are same. For instance, SSB index 0 is configured with candidate position 32. If the SSB is determined to be unavailable, then (SSB 0, SSB 32) are all muted. For other 11 SSB bursts, SSB 0 is transmitted. It means, it seems there is no need to have independent P for 1st/2nd candidate positions. Thus we slightly modified the proposal from QC as follows: 1. TE picks one SSB out of a group of each 12 consecutive SSBs 2. For this SSB, TE determines with probabilities PCCA_DL whether to transmit this SSB or not. Note that other 11 SSBs shall be transmitted by the gNB 3. If the SSB is not transmitted, the SSB occasion group is considered as unavailable to the UE 4. TE repeats the procedure for next 12 consecutive SSBs Otherwise, if the CCA outcome of two candidate positions are independent, it could possible that if the SSB is chosen as an unavailable one, but the 2nd position is transmitted. Then it is an available SSB instead. If companies prefer to keep PCCA_DL1 and PCCA_DL2, we can further state that for the chosen SSB out of 12 consecutive SSB, the CCA result of two candidate positions are same. |

Issue 1-5-2: SSB index shift

- Proposal 1 (CATT): In order to test the behaviour of UE more thoroughly, it is suggested to use a fixed sequential mode to shift the SSB index.

  - For example, the unavailable SSB in the first 12 SSBs could be the first SSB, the unavailable SSB in the second 12 SSBs could be the second SSB, and so on.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | See our comments in issue 1-5-1 |
| Ericsson | Agree on Proposal 1, sequential index is enough. |
| Nokia | Ok with proposal 1. |
| CATT | Agree with proposal 1. |
| Qualcomm | Fine with the proposal |
| Apple | Proposal 1 is OK, but better avoid using SSB index, which means one SSB in the SSB burst set. Here our understanding is each SSB means an SSB burst set. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 Test configurations | Issue 1-1-1: Test configurations - General Companies’ views: Some companies agree with the proposal, others think that the proposal is very generic and that the focus should be on exact configurations, as in Issue 1-1-2 Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: No further discussion in needed in the second round. Companies are encouraged to comment directly on Issue 1-1-2 Issue 1-1-2: Test configurations – Timing Companies’ views: Some companies agree with the proposal, others think that further clarification is needed. Another option was proposed by one of the companies Tentative agreements: No agreement Candidate options: Option 1: For UL transmit timing and timing advance accuracy test cases: - UE is required to be tested with the largest supported SCS Option 2: For UL transmit timing and timing advance accuracy test cases: - Test configurations are defined for all SCS – 120kHz, 480kHz and 960kHz - UE that supports 120kHz SCS only is required to pass the test with 120kHz SCS configuration - UE that supports 120kHz and 480kHz SCS only is required to pass the test with 480kHz SCS configuration - UE that supports 120kHz and 960kHz SCS only is required to pass the test with 960kHz SCS configuration - UE that supports all SCS (120kHz, 480kHz and 960kHz) is required to pass the test with one of the configurations from 480kHz and 960kHz SCS Recommendations for 2nd round: Continue discussion in the second round |
| Sub-topic #1-2 CCA Aspects | Issue 1-5-1: CCA modelling in test cases Companies’ views: Companies had different opinions on this issue and further discussion is needed Tentative agreements: No agreement Candidate options: Option 1: 1. TE picks one SSB out of a group of each 12 consecutive SSBs 2. For this SSB, TE determines with probabilities PCCA_DL1 and PCCA_DL2 whether to transmit this SSB or not. Note that other 11 SSBs shall be transmitted by the gNB 3. If the SSB is not transmitted, the SSB occasion group is considered as unavailable to the UE 4. TE repeats the procedure for next 12 consecutive SSBs Option 2: 1. TE picks one SSB out of a group of each 12 consecutive SSBs 2. For this SSB, TE determines with probability PCCA_DL whether to transmit this SSB or not. Note that other 11 SSBs shall be transmitted by the gNB 3. If the SSB is not transmitted, the SSB occasion group is considered as unavailable to the UE 4. TE repeats the procedure for next 12 consecutive SSBs Recommendations for 2nd round: Continue discussion in the second round and see if one of the options are acceptable. Further suggestions are encouraged. Also discuss the probability values. Issue 1-5-2: SSB index shift Companies’ views: This issue depends on the previous issue and can be discussed together Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: No further discussion is needed. Comment directly on Issue 1-5-1. |

## Discussion on 2nd round (if applicable)

# Topic #2: Draft CRs for test cases

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215418 | CATT | Draft CR on test cases for SA RRC Re-establishment for extending NR operation to 71GHz |
| R4-2215419 | CATT | Draft CR on test cases for Beam failure detection and link recovery for extending NR operation to 71GHz |
| R4-2215863 | Vivo | Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2 |
| R4-2216258 | Nokia | Draft CR random access test cases in FR2-2 |
| R4-2216260 | Nokia | Draft CR introducing BFD and TCI state switch test cases in FR2-2 |
| R4-2216268 | Huawei | CR on test cases for HO for FR2-2 |
| R4-2216501 | Ericsson | draft CR on Test Cases on RLM for SCell activation to 71GHz |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2215418 | Company A |
|  | Company B |
| R4-2215419 | Company A |
|  | Company B |
|  | Huawei: “UE is only required to be tested in one of …” seems missing. |
| R4-2215863 |  |
| R4-2216258 |  |
| R4-2216260 |  |
| R4-2216268 |  |
| R4-2216501 | Huawei: We have one question about the spec structures. Per previous agreements, test cases without CCA will be included in A.7. However, some test cases were introduced in a now dedicated section A.15 in h70 version. Now the situation is that, in the formal spec, most TC without CCA are captured in A.7, some TC without CCA are captured in A.15. We would like to hear more views from companies on how to handle this. |
|  | Nokia: We agree with Huawei that this should be clarified. We suppose this agreement (RAN4 #104-e – R4-2214478) is valid for operation without CCA. Agreement: For operation in FR2-2 bands without CCA, create dedicated test cases for FR2-2 in existing sections (A.7) - Replace “FR2” in existing FR2 test cases by FR2-1. For operation with CCA, our view is that this agreement is valid (RAN4 #103-e R4-2210590): Agreement: Specification structure for test cases - Define test cases for operation in FR2-2 in dedicated sections - Create in TS 38.133 the following new top-level sections for NR-test cases with CCA in FR2-2 - A.14 NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR2-2 - A.15 NR Standalone Tests with NR Scell under CCA and All Other Cells in FR2-2. Can companies confirm? |

### CRs/TPs

Although some of the draftCRs didn’t receive any comments, but it appears that comments on other CRs regarding section numbers are applicable in general. Companies are encouraged to check the section numbers carefully for each draftCR.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2215418 | To be revised |
| R4-2215419 | To be revised |
| R4-2215863 | To be revised |
| R4-2216258 | To be revised |
| R4-2216260 | To be revised |
| R4-2216268 | To be revised |
| R4-2216501 | To be revised |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: Test cases & Work Split

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions

### Sub-topic 3-1: Test cases for RRM performance requirements without CCA

Sub-topic description: The table below lists the agreed set of test cases. Companies are encouraged to volunteer for test cases and provide comments, if any, in the box below the table.

| Group of requirements | Test cases | Sub-test | 120 kHz SCS | 480/960 kHz SCS | Company |
| --- | --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 | intra-frequency | Yes | Yes | Ericsson |
|  |  | inter-frequency | Yes | yes | Ericsson |
| HO (delay and interruptions) | FR2-2->FR2-2 | intra-frequency, unknown target cell | Yes | Yes | Huawei |
|  |  | inter-frequency, unknown target cell | Yes | Low priority | Huawei |
|  | FR1->FR2-2 | unknown target cell | Yes | Yes | Huawei |
| RRC Re-establishment | FR2-2->FR2-2 | intra frequency | Yes | Low priority | CATT |
|  |  | inter frequency | Yes | Yes | CATT |
|  |  | intra frequency without serving cell timing | Yes | Yes | CATT |
| Random access | 4-step RACH | Contention based | Reuse FR2-1 | Yes | Nokia |
|  |  | Non-contention based | Reuse FR2-1 | Yes | Nokia |
| RRC Connection Release with Redirection | FR2-2->FR2-2 |  | Yes | Low priority | Qualcomm |
| Timing | UE Transmit Timing |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes | Huawei |
|  | Timing advance adjustment accuracy |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes | Huawei |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2->FR2-2 | Reuse FR2-1 | Yes | Huawei |
|  |  | FR1->FR2-2 | Reuse FR2-1 | Low priority |  |
|  |  | One FR2-2 cell | Reuse FR2-1 | Low priority |  |
|  | RRC-based active BWP switch |  | Reuse FR2-1 | Yes | Huawei |
|  | SCell dormancy switch | FR2-2->FR2-2 | Reuse FR2-1 | Low priority | Huawei |
|  |  | FR1->FR2-2 | Reuse FR2-1 | Low priority |  |
|  | RRC-based Active BWP Switch on multiple CCs |  | Reuse FR2-1 | Yes | Huawei |
| Radio link monitoring | FR2-2 PCell, SSB-based | Out-of-sync, in non-DRX | Yes | Yes | Ericsson |
|  |  | In-sync, in non-DRX mode | Yes | Low priority | Ericsson |
|  |  | Out-of-sync, in DRX mode | Yes | Low priority | Ericsson |
|  |  | In-sync, in DRX mode | Yes | Yes | Ericsson |
|  | FR2-2 PCell, CSI-RS-based | Out-of-sync, in non-DRX | Yes | Low priority | Ericsson |
|  |  | In-sync, in non-DRX mode | Yes | Yes | Ericsson |
|  |  | Out-of-sync, in DRX mode | Yes | Yes | Ericsson |
|  |  | In-sync, in DRX mode | Yes | Low priority | Ericsson |
|  | Scheduling restrictions |  | Yes | Yes | Ericsson |
| Beam failure detection and link recovery | FR2-2 PCell | SSB-based in non-DRX mode | Yes | Yes | Nokia |
|  |  | SSB-based in non-DRX mode | Yes | Low priority | Nokia |
|  |  | CSI-RS-based in non-DRX mode | Yes | Low priority | CATT |
|  |  | CSI-RS -based in non-DRX mode | Yes | Yes | CATT |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Yes | Yes | CATT |
| SCell activation/deactivation delay | SCell in FR2 | intra-band in non-DRX | Yes | Yes | Ericsson |
|  |  | inter-band in non-DRX | Yes | Low priority | Ericsson |
|  | FR1+FR2 inter-band with target SCell in FR2 |  | Yes | Yes | Ericsson |
| PSCell addition and release delay | Known PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | No | Qualcomm |
|  | Unknown PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes | Qualcomm |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 | Reuse FR2-1 | Yes | Nokia |
|  | RRC based | PCell in FR2-2 | Reuse FR2-1 | Low priority | Nokia |
| Interruptions | during measurements on deactivated NR SCC in FR2-2 |  | Reuse FR2-1 | Yes |  |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Yes | Yes | vivo |
|  |  | under DRX | Yes | Low priority | vivo |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Yes | Low priority | vivo |
|  |  | under DRX | Yes | Yes | vivo |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Yes | Yes | vivo |
|  |  | PCell in FR2-2, DRX is used | Yes | Low Priority | vivo |
|  |  | PCell in FR1, DRX is not used | Yes | Low Priority | vivo |
|  |  | PCell in FR1, DRX is used | Yes | Yes | vivo |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Yes | Low Priority | vivo |
|  |  | PCell in FR2-2, DRX is used | Yes | Yes | vivo |
|  |  | PCell in FR1, DRX is not used | Yes | Yes | vivo |
|  |  | PCell in FR1, DRX is used | Yes | Low Priority | vivo |
| L1-RSRP measurement for beam reporting | SSB based | DRX not used | Yes | Low Priority | Qualcomm |
|  |  | DRX is used | Yes | Yes | Qualcomm |
|  | CSI-RS based | DRX not used | Yes | Yes | Qualcomm |
|  |  | DRX is used | Yes | Low Priority | Qualcomm |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR2-2 serving and target cell | Yes | Yes | Qualcomm |
|  |  | Inter frequency FR2-2 serving and target cell | Yes | Low Priority | Qualcomm |
|  |  | Inter frequency FR1 serving and FR2-2 target cell | Yes | Yes | Qualcomm |
|  | SS-RSRQ | Intra-frequency FR2-2 serving and target cell | Yes | Low Priority | Qualcomm |
|  |  | Inter frequency FR2-2 serving and target cell | Yes | Yes | Qualcomm |
|  | SS-SINR | Intra-frequency FR2-2 serving and target cell | Yes | Yes | Qualcomm |
|  |  | Inter frequency FR2-2 serving and target cell | Yes | Low Priority | Qualcomm |
|  | L1-RSRP | SSB based | Yes | Yes | Qualcomm |
|  |  | CSI-RS based | Yes | Yes | Qualcomm |

| Company | Comments |
| --- | --- |
| XXX |  |

### Sub-topic 3-2: Test cases for RRM performance requirements with CCA

Sub-topic description: The table below lists the agreed set of test cases. Companies are encouraged to volunteer for test cases and provide comments, if any, in the box below the table.

| Group of requirements | Test cases | Sub-test | Needed / Not needed | Company |
| --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 (CCA) | intra-frequency | Yes, at least with 120 kHz and 480 kHz |  |
|  |  | inter-frequency | Yes, at least with 120 kHz and 480 kHz |  |
| HO (delay and interruptions) | FR2-2->FR2-2 (CCA) | intra-frequency, unknown target cell | Yes | Huawei |
|  |  | inter-frequency, unknown target cell | Low priority |  |
|  | FR1->FR2-2 (CCA) | unknown target cell | Yes | Huawei |
| RRC Re-establishment | FR2-2->FR2-2 (CCA) | intra frequency | Low priority |  |
|  |  | inter frequency | Yes | Nokia |
|  |  | intra frequency without serving cell timing | Yes | Nokia |
| Random access | 4-step RACH | Contention based | Yes | Nokia |
|  |  | Non-contention based | Yes | Nokia |
| RRC Connection Release with Redirection | FR2-2->FR2-2 (CCA) |  | Low priority |  |
| Timing | UE Transmit Timing |  | Yes | Huawei |
|  | Timing advance adjustment accuracy |  | Yes | Huawei |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2 ->FR2-2 (CCA) | Yes, but this test case can be same as the one defined for operation without CCA |  |
|  |  | FR1->FR2-2 (CCA) | Low priority |  |
|  |  | One FR2-2 (CCA) cell | Low priority |  |
|  | RRC-based active BWP switch |  | Yes, but this test case can be same as the one defined for operation without CCA |  |
|  | UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | FR2-2 (CCA) | Yes |  |
|  |  | FR1->FR2-2 | Not needed |  |
| Radio link monitoring | FR2-2 Pcell (CCA), SSB-based | Out-of-sync, in non-DRX | Yes |  |
|  |  | In-sync, in non-DRX mode | Low priority |  |
|  |  | Out-of-sync, in DRX mode | Low priority |  |
|  |  | In-sync, in DRX mode | Yes |  |
|  | Scheduling restrictions |  | Yes, but this test case can be same as the one defined for operation without CCA |  |
| Beam failure detection and link recovery | FR2-2 PCell (CCA) | SSB-based in non-DRX mode | Yes |  |
|  |  | SSB-based in non-DRX mode | Low priority |  |
|  |  | CSI-RS-based in non-DRX mode | Low priority |  |
|  |  | CSI-RS -based in non-DRX mode | Yes |  |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Yes, but this test case can be same as the one defined for operation without CCA |  |
| SCell activation/deactivation delay | SCell in FR2 (CCA) | intra-band in non-DRX | Yes |  |
|  |  | inter-band in non-DRX | Low priority |  |
|  | FR1+FR2 (CCA) inter-band with target SCell in FR2 |  | Yes |  |
| PSCell addition and release delay | Known PSCell |  | No |  |
|  | Unknown PSCell |  | Yes | Huawei |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 (CCA) | Yes | Huawei |
|  | RRC based | PCell in FR2-2 (CCA) | Low priority | Huawei |
| Interruptions | during measurements on deactivated NR SCC in FR2-2 |  | Yes |  |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Yes |  |
|  |  | under DRX | Low priority |  |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Low priority |  |
|  |  | under DRX | Yes |  |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Yes |  |
|  |  | PCell in FR2-2, DRX is used | Low Priority |  |
|  |  | PCell in FR1, DRX is not used | Low Priority |  |
|  |  | PCell in FR1, DRX is used | Yes |  |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Low Priority |  |
|  |  | PCell in FR2-2, DRX is used | Yes |  |
|  |  | PCell in FR1, DRX is not used | Yes |  |
|  |  | PCell in FR1, DRX is used | Low Priority |  |
| L1-RSRP measurement for beam reporting | SSB based | DRX not used | Low Priority | Qualcomm |
|  |  | DRX is used | Yes | Qualcomm |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  |  | Inter frequency FR1 serving and FR2-2 target cell | covered by the requirements without CCA |  |
|  | SS-RSRQ | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  | SS-SINR | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  | L1-RSRP | SSB based | covered by the requirements without CCA |  |
|  |  | CSI-RS based | covered by the requirements without CCA |  |
|  | RSSI | Intra-frequency | Needed in all SCS | Qualcomm |
|  |  | Inter-frequency | Needed in all SCS | Qualcomm |

| Company | Comments |
| --- | --- |
| XXX |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on NR extension to 71 GHz – RRM - 1 | Qualcomm | All agreements to be captured in this document |
|  | Test cases and work split for NR extension to 71 GHz RRM performance requirements | Qualcomm | Captures the test cases and work split |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2215418 |  | Draft CR on test cases for SA RRC Re-establishment for extending NR operation to 71GHz | CATT | Revised |  |
| R4-2215419 |  | Draft CR on test cases for Beam failure detection and link recovery for extending NR operation to 71GHz | CATT | Revised |  |
| R4-2215863 |  | Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2 | Vivo | Revised |  |
| R4-2216258 |  | Draft CR random access test cases in FR2-2 | Nokia | Revised |  |
| R4-2216260 |  | Draft CR introducing BFD and TCI state switch test cases in FR2-2 | Nokia | Revised |  |
| R4-2216268 |  | CR on test cases for HO for FR2-2 | Huawei | Revised |  |
| R4-2216501 |  | draft CR on Test Cases on RLM for SCell activation to 71GHz | Ericsson | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents