3GPP TSG-RAN WG4 Meeting # 104-bis-e										R4-2217186

Electronic Meeting, 10– 19 October 2022

Title: 	WF on core requirements maintenance for FR2-2

Agenda Item:	4.3.5

Source: 	Huawei, HiSilicon

Document for:	Approval

- Topic #1: General requirements

Issue 1-1-1: Inter-frequency QCL-ed assumption for RSSI

Agreement:

  - Wait for the RAN1 reply LS in case there are no serving cells in FR2-2, and remove the “if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier” in the description of the case no TCI information.

FFS:

  - Option 1: If no TCI state is provided in the RMTC configuration UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier in FR2-2 having the [lowest or highest] cell index among the serving cells for which corresponding TCI state ID is provided in another RMTC configuration.

  - Option 2: if no TCI state is provided in the RMTC configuration, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of a carrier in FR2-2.

Issue 1-2-1: Cell detection relaxation

FFS:

  - Introduce UE capability on relaxed requirements for cell detection.

Issue 1-3-1: Beam sweeping factor for SCell activation

Agreement:

  - Update the SCell activation requirements for FR2-2 with beam sweeping scaling factor of 12

- Topic #2: LBT impact on RRM requirements

Issue 2-1-1: Maximum separation between two consecutive measurements

FFS:

  - Option 1: The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known.

  - Option 2: The requirement only applies when

    - Within the set of measurements any two measurements shall not be separated in time by more than 2 seconds when no DRX is configured, and

    - Within the set of measurements any two measurements shall not be separated in time by more than max(1 DRX occasion group duration, 2 seconds) when DRX is configured.

Issue 2-1-1: LBT assumptions on neighbour cell measurement

Agreement:

  - The requirements with CCA apply if the UE is signalled that the neighbour cells are configured with CCA, and requirements without CCA apply otherwise.

Issue 2-3-1: Impact of 480/960 kHz on RSSI/CO measurement

Agreement：

  - For RSSI/CO measurement with SCS of 480 kHz and 960 kHz, the RSSI/CO measurement results shall be derived based on symbols configured by RMTC except for the first symbol and the last symbol in RMTC, and there is no requirements when measDurationSymbols is configured as 1

- Reference

R4-2217136	Email discussion summary for [104-bis-e][203] NR_ext_to_71GHz_RRM_1