3GPP TSG-RAN WG4 Meeting # 104bis-e										R4-2217253

Electronic Meeting, October 10 – October 19, 2022

Title: 	WF on inter-RAT measurement without gap

Agenda Item:	6.10.3.2

Source: 	Intel Corporation

Document for:	Approval

# Topic #2: inter-RAT measurement without gap(AI 6.10.3.2)

## Sub-topic 2-1: Using scenarios

[Moderator notes:

Up to this meeting, all proposed possible using scenarios for inter-RAT NR/LTE measurements without gap can summarized as:

- the inter-RAT NR measurements without gap in Rel18 includes the two scenarios below.

  - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

  - Case a-2: NR reference signal to be measured are fully contained within UE’s LTE channel bandwidth

- the inter-RAT LTE measurements without gap in Rel18 includes the two scenarios below.

  - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

  - Case b-2: LTE CRS are fully contained within UE’s active BWP

We can use these index below to aviod any confustion.

]

### Issue 2-1-1: extend to MR-DC cases

< Way forward/Agreement >:

  - Define capability and/or requirements to support inter-RAT NR measurement without gap case a-1 in SA only if case a-1 is agreed as one of inter-RAT NR target scenario (issue 2-1-3)

  - Define capability and/or requirements to support inter-RAT E-UTRAN measurement without gap case b-1 in SA only if case b-1 is agreed as one of inter-RAT LTE target scenario (issue 2-1-4)

  - Define capability and/or requirements to support inter-RAT NR measurement without gap case a-2 in SA if case a-2 shall be considered in as one of inter-RAT NR target scenario (issue 2-1-3).

    - FFS on MR-DC

  - Define capability and/or requirements to inter-RAT E-UTRAN measurement without gap case b-2 in SA if case b-2 is agreed as one of inter-RAT LTE target scenario (issue 2-1-4)

    - FFS on MR-DC

### Issue 2-1-2 Numerology

< Agreement >:

  - For inter-RAT measurement without MG, including both inter-RAT NR measurement and inter-RAT LTE measurement, the mixed numerology needs to be supported.

    - FFS on whether the additional UE capability is needed

### Issue 2-1-3: inter-RAT NR target scenario

< Way forward/Agreement >:

  - Case a-1 (Another spare RF chain is available for UE) can be considered as the inter-RAT NR target scenario.

  - FFS on Case a-2

### Issue 2-1-4: inter-RAT LTE target scenario

< Way forward/Agreement >:

  - Case b-2(The target CRS to be measured is with UE’s active BWP) shall be considered as the inter-RAT LTE scenario under objective 2 of NR_MG_ehn2-core WI

  - FFS on Case b-1

## Sub-topic 2-2: UE capabilities

### Issue 2-2-1: On top of which UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain available  (Case a-1)

< Way forward/Agreement >:

  - On top of “interRAT-NeedForGapsNR-r16” UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-1)

    - The exact value to be reported can be FFS.

  - FFS on top of NCSG to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-1)

### Issue 2-2-2: On top of which UE capability to support the inter-RAT NR measurement without gap when the measurement reference signal can be contained within UE’s LTE channel bandwidth (Case a-2)

< Way forward >:

  - FFS on:

    - On top of “interRAT-NeedForGapsNR-r16” UE capability to support the inter-RAT NR measurement without gap when the measurement reference signal can be contained within UE’s LTE channel bandwidth (Case a-2)

### Issue 2-2-3: On top of which UE capability to support the inter-RAT LTE measurement requirements when UE has vacant RF chain available(Case b-1)

< Way forward/Agreement >:

  - Reuse the existing capability.

    - UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoEUTRAN for the inter-RAT LTE measurement

    - FFS on: UE indicates via NeedForGap.

### Issue 2-2-4: On top of which UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2)

< Way forward >:

  - Option 1:

    - the capability of NeedForGaps for inter-frequency measurement wo gap in Rel16 can be taken as the baseline

  - Option 1a:

    - extend the capability of NeedForGaps

  - Option 2:

    - A new UE capability should be defined to indicate whether UE supports MG-less measurement in this scenario

## Sub-topic 2-3: Measurement requirements

### Issue 2-3-1: General principle to define the requirements

< Way forward >:

  - FFS on the general principle to define the requirements for inter-RAT NR measurements

    - Proposal 1:

      - For the inter-RAT NR measurements without gap, the  requirements frameworks can be based on NR inter-frequency measurement without gap

    - Others are not precluded

  - FFS on the general principle to define the requirements for inter-RAT LTE measurements

    - Proposal 2:

      - For the inter-RAT LTE gap-less measurement when LTE CRS to be measured is contained in UE’s active BWP, the requirements can be based on the measurement requirements for LTE inter-frequency measurement.

    - Proposal 3: Ericsson

      - RAN4 needs to further discuss where to perform the inter-RAT E-UTRAN measurement without gaps to reduce the performance degradation to legacy NR intra-frequency measurement without gap and L1-RSRP measurement.

      - Both NW and UE shall have the same understanding on the measurement occasions for Inter-RAT E-UTRAN measurement without gap.

    - Others are not precluded

### Issue 2-3-2: Scheduling restriction

< Way forward >:

  - FFS on the scheduling restriction

    - Proposal 1:

      - The existing scheduling availability specified for intra-frequency measurements in TS 38.133 section 9.2.5.3 can also be applied to the inter-RAT measurement without measurement gaps as a start point.

    - Proposal 2:

      - When the target inter-RAT E-UTRAN frequency layers belong to an inter-band with the serving cells, no scheduling restriction is expected.

      - When the target inter-RAT E-UTRAN frequency layers belong to an inter-band with the serving cells, scheduling restriction is expected, such as UE performing measurements in TDD bands or with different SCS.

      - RAN4 to discuss how to apply the scheduling restriction based on LTE measurement RSs.

    - Others are not precluded

### Issue 2-3-3: Searcher limitation

< Way forward >:

  - FFS on the searcher limitation

    - Proposal 1:

      - Inter-RAT measurement without gap can be performed in parallel with NR measurement without searcher limitation.

    - Others are not precluded

### Issue 2-3-4: CCSF

< Way forward >:

  - The updates of CSSF requirements when these inter-RAT measurements without gap introduced (e.g. CSSF_outside_gap) is needed

  - FFS on:

    - The scaling factor for inter-RAT E-UTRAN measurement without gap equals to the total number of frequency layers for E-UTRAN measurement without gap.

### Issue 2-3-5: Interruption requirements for inter-RAT measurement without gap

< Way forward >:

  - FFS on whether interruption is allowed for inter-RAT NR measurements without MG

### Issue 2-3-6: Effective measurement window

< Way forward >:

  - FFS on:

    - RAN4 needs to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap.

      - The effective measurement window can be defined based on measurement duration, measurement periodicity and offset.

    - Others are not precluded

### Issue 2-3-7: Release independent requirements

< Way forward >:

  - FFS on whether to introduce the inter-RAT LTE measurements without gap case b-2 as release independent from Rel-17

  - Others are not precluded

# Reference

[1] RP-222337, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation