3GPP TSG-RAN WG4 Meeting#104-bis-e	R4-2217286

Online, 10th Oct – 19th Oct, 2022

Source: 	Qualcomm Incorporated

Title: 	Simulation Results on Multi-TRP PDCCH Tests

Agenda item:	4.5.3.1.2

Document for:	Discussion

- Introduction

As presented below, the following simulation assumptions were agreed to define requirements for the multi-TRP PDCCH repetition scheme during RAN4#103-e [1]. In this paper, we share our simulation results for this scheme.

- Simulation Results for m-TRP PDCCH Repetition Scheme

Based on the agreed simulation assumptions, we share our alignment and impairment results both for FDD 15KHz and TDD 30KHz scenarios with 10MHz and 40MHz BW, respectively.

Table 1: Simulation assumptions for multi-TRP FDM PDCCH transmission

| Parameters | Values |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| CORESET RB | 24 | 48 |
| Timing/Freq offsets | -0.5us/200HZ | -0.25us/300Hz |
| CORESET duration | 2 |  |
| Aggregation level | 2/4 |  |
| Propagation Condition | TDLA30-10 |  |
| Antenna configuration | 2x2, 2x4 |  |
| CCE to REG mapping type | nonInterleaved |  |
| REG bundle size | 6 |  |
| Payload bits(without CRC) | 39 | 41 |
| Test metric | 1% of Pm-dsg (%) |  |

Table 2: Alignment results for multi-TRP FDM PDCCH transmission

| Duplex Mode/SCS/BW | 2x2 | 2x4 |  |
| --- | --- | --- | --- |
|  | Alignment SNR (dB) | Alignment SNR (dB) |  |
|  | AL2 |  | AL2 |
| FDD 15KHz/10MHz | 0.32 |  | -3.52 |
| TDD 30KHz/40MHz | 1.48 |  | -2.95 |

| Duplex Mode/SCS/BW | 2x2 | 2x4 |
| --- | --- | --- |
|  | Impairment SNR (dB) | Impairment SNR (dB) |
|  | AL2 | AL2 |
| FDD 15KHz/10MHz | 3.32 | -0.52 |
| TDD 30KHz/40MHz | 4.48 | 0.05 |

Table 3: Impairment results for multi-TRP FDM PDCCH transmission

- Conclusions

In this paper, we share our simulation results for multi-TRP PDCCH repetition scheme.

# 4. References

[1] R4-2210670, “WF on demodulation requirement for Enhancement on Multi-TRP,” 3GPP TSG RAN WG4 Meeting #103-e, Online, May 09 – May 20, 2022.