3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2217292

Electronic Meeting, October 10 – 19, 2022

Source:	NEC

Title:	TP to 38.115-1: EVM requirement

Agenda item:	4.1.5.2.1

Document for:	Approval

Introduction

This contribution provides text proposal to TS 38.115-1 for EVM requirement.

Discussion

In the previous RAN4 meeting, TP on EVM requirements was agreed [1].

In the agreed text, DL EVM is defined as the difference between the symbols provided at the input of the repeater and the measured symbols at the output of the repeater. With this definition, conformance to the requirement does not guarantee the performance. EVM definition shall be corrected to be the difference between the ideal symbols and the measured symbols at the output of the repeater.

The text proposal is provided to TS 38.115-1 [2].

References

[1] R4-2214867, “TP to TS 38.115-1 clause 6.6 EVM - conducted”, Nokia, Nokia Shanghai Bell

[2] TS 38.115-1 v0.1.0

TEXT PROPOSAL:

--------------Start of change-------------

6.6	Error Vector Magnitude

6.6.1	Downlink Error vector magnitude

6.6.1.1	General

The Error Vector Magnitude (EVM) is a measure of the difference between the ideal symbols and the measured signal symbols at the output of the repeater after the equalization by the measurement equipment. This difference is called the error vector. Details about how the EVM is determined are specified in TS 38.104 Annex B for FR1. The EVM result is defined as the square root of the ratio of the mean error vector power to the mean reference power expressed in percent.

The EVM requirement is applicable for a repeater operating at an input power in the range from what is required to reach the maximum output power to the minimum power level in table 6.6.1.1-1.

Table 6.6.1.1-1: Minimum input power for EVM

| Repeater DL class | Minimum input power spectral density (dBm/MHz) |  |
| --- | --- | --- |
|  | QPSK, 16 QAM, 64QAM | 256QAM1 |
| WA | -82 | -75 |
| MR | -77 | -70 |
| LA | -74 | -67 |
| Note 1: support of 256QAM is based on the declaration |  |  |

--------------End of change-------------