3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2217325

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.181 – Clauses 4.10 and 4.11

Agenda item:	4.2.3.1.3

Document for:	Approval

Introduction

RAN4 is working on conformance specification TS 38.181 for Satellite Access Node (SAN). During last RAN4#104-e meeting work split for drafting text proposals between interesting companies was agreed to facilitate efforts. Work split is included in agreed Way forward [1].

This contribution provides text proposals to TS 38.181 for clause 4.10 Co-location requirements, and clause 4.11 Reference coordinate system.

Text proposal

<Start of TP to TS 38.181>

## 4.10	Co-location requirements

- 4.10.1	General

Co-location requirements are requirements which are based on assuming the SAN type 1-O is co-located with another SAN of the same base station class. They ensure that both co-located systems can operate with minimal degradation to each other.

The co-location requirements in table 4.10.1-1 rely on a co-location reference antenna used to mimic a base station to base station co-location scenario.

Table 4.10.1-1: Co-location requirements

| Clause number | Requirement | Co-location reference antenna operation | Type |
| --- | --- | --- | --- |
| 9.7.5.3 | OTA spurious emission: Protection of the SAN receiver of own or different SAN | Measure emission | Optional based on declaration |

The co-location reference antenna is defined in TS 38.108 [2].

### 4.10.2	Co-location test antenna

#### 4.10.2.1	General

Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of a co-location test antenna (CLTA).

A CLTA is a practical antenna which can be used to test conformance to the co-location requirements.

#### 4.10.2.2	Co-location test antenna characteristics

A co-location test antenna is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. A CLTA shall comply with the requirements specified in table 4.10.2.2-1.

Translation of the requirements to other test antennas are not precluded but suitable translations between the co-location reference antenna and test antenna must be provided to demonstrate that the method is within the specified MU.

NOTE:	The currently defined CLTAs are suitable for testing SAN type 1-O implemented with a planar antenna array. The method for testing SAN with other antenna array implementations is not covered by the present release of this specification.

Table 4.10.2.2-1: CLTA characteristics

| Parameter | In-band CLTA | Out-of-band CLTAs |
| --- | --- | --- |
| Vertical radiating dimension (h) | Test object vertical radiating length ±30% | Test object vertical radiating length ±30% (Note 2) |
| Horizontal beam width | 65° ± 10° | 65° ± 10° |
| Vertical beam width | N/A | The half-power vertical beam width of the CLTA equals the narrowest declared (D.3) vertical beamwidth ±3° (Note 2) |
| Polarization (Note 3) | Match (Note4) | Match to in-band (Note 4) |
| Conducted interface return loss | > 10 dB | > 10 dB |
| NOTE 1: If a multi-column or multi-band antenna is used the column closest to the NR SAN shall be selected while other columns are terminated during testing. NOTE 2: The vertical radiating dimension definition shall be used instead of the vertical beam width definition when the test chamber dimensions limit the use of vertical beam width definition. Otherwise the vertical beam width definition shall be used. NOTE 3: For SAN type 1-O with dual polarization the CLTA has two conducted interfaces each representing one polarization. NOTE 4: Matched to the polarization of EUT antenna. |  |  |

#### 4.10.2.3	Co-location test antenna alignment

The alignment between the NR SAN under test and the co-location test antenna is described in table 4.10.2.3-1 and figure 4.10.2.3-1. The same physical alignment applies to in-band and out-of-band co-location requirements.

Table 4.10.2.3-1: CLTA alignment tolerances

| Parameter |  |
| --- | --- |
| Edge-to-edge separation between the NR SAN and the CLTA, d | 0.1 m ± 0.01 m |
| Vertical alignment | Centre ± 0.01 m |
| Front alignment | Random front ± 0.01 m |

NR SAN type 1-O

CLTA

Horizontal View

Vertical View

d

M

echanical

bore

-

sight

direction

M

echanical

bore

-

sight

direction

Side View

Back

sid

e

Front

sid

e

Figure 4.10.2.3-1: Alignment of NR SAN and CLTA

4.11	Reference coordinate systemRadiated requirements are stated in terms of electromagnetic characteristics (e.g. EIRP and EIS) at certain angles with respect to the base station. To be able to declare radiated characteristics part of radiated requirements a reference coordinate system is required. The reference coordinate system is should be associated to an identifiable physical feature on the base station enclosure. The location of the origin and the orientation of the reference coordinate system are for the base station manufacturer to declare.

The reference coordinate system is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () as showed in figure 4.11-1.

Figure 4.11-1: Reference coordinate system

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°, inclusive. Note that  is defined as positive along the down-tilt angle.

<End of TP to TS 38.181>

- References

- R4- 2214371, WF on NTN SAN RF conformance, Ericsson