3GPP TSG-RAN WG4 Meeting # 104-bis-e					R4-2217344

Electronic Meeting, 10– 19 October 2022

Agenda item:			4.2.8

Source:	Qualcomm Incorporated

Title:	WF for NTN demodulation requirements - general and PDSCH

Document for:	Approve

# WF on general aspects

Issue 1-1-1: Channel model for NTN-TDLC (LOS)

- Proposals

  - Option 1 (Ericsson, Apple, Nokia, Qualcomm): DS=5ns, Doppler=200Hz. K-factor=8.05dB.

  - Option 2 (Huawei, Nokia): DS=3.5ns, Doppler=200Hz, K-factor=8.05dB.

- Agreement:

  - option 1 is agreed

Issue 1-1-2: Delay resolution

- Proposals

  - Option 1 (Ericsson, Qualcomm, Nokia, Apple): Adopt 5 ns as the delay resolution

  - Option 2 (Huawei): Reduce the delay resolution to 2 ns for NTN TDL channel model simplification.

- Agreement:

  - Option 1 is agreed

Issue 1-1-3: Delay profile for NTN-TDLA

- Agreement on delay profile for NTN-TDLA

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.7 | Rayleigh |
| 3 | 285 | -6.5 | Rayleigh |

Issue 1-1-4: Delay profile for NTN-TDLC

- Proposals: To further confirm option 1 or option 2 (only difference is Power for Tap#2)

  - Option 1 (Apple, Huawei):

| Tap # | Delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |

  - Option 2 (Qualcomm, MTK):

| Tap # | Delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.7 | Rayleigh |

- Agreement

  - Option 1 is agreed

# WF NTN PDSCH demodulation requirements

Issue 2-1: K_offset value

- Proposals

  - Option 1 (Nokia, Huawei, Qualcomm, Apple, Ericsson): Confirm K_offset equal to 8 slots for 15kHz SCS for all the HARQ configurations

  - Option 2: Specify other option if any

- Agreement

  - Option 1 is agreed

Issue 2-2: Doppler shift for LOS Path in the NTN TDL-C Channel

- Proposals

  - Option 1 The same scaling factor as the TN channel models, i.e., (0.7*fD) with AoA of 45 degree is assumed for UE side in the simulation. No need to explicitly reflect this agreement in the TS 38101-5.

- Agreement

  - Option 1 is agreed

Issue 2-3: Modulation order

- Agreement:

  - Do not define PDSCH performance requirements for 64QAM

    - It’s not precluded to discuss and specify requirements for 64QAM in future releases.

Issue 2-4: SCS/CBW set

- Proposals

  - Option 1: De-prioritize requirements with 30KHz /20MHz SCS/CBW set in Rel-17 and propose to revisit in later release based on demand and deployment plans.

- Agreement

  - Option 1 is agreed

Issue 2-5: HARQ configurations

Agreement: Define PDSCH test cases for Disabled HARQ, 16 HARQ proc and 32 HARQ processes and further discuss the detailed test cases as below options.

- Proposals

  - Option 1 (Apple, Nokia):

| Prop. Channel | MCS | HARQ Config |
| --- | --- | --- |
| NTN-TDLA100-200 | MCS4 | Disabled HARQ |
|  | MCS13 | 16 HARQ Proc |
| NTN-TDLC5-200 | MCS4 | 32 HARQ Proc |
|  | MCS13 | 16 HARQ Proc |

  - Option 2 (Apple, Nokia, Ericsson, Qualcomm): If the UE supports disabled HARQ, then UE will be tested with 12(enabled)+4(disabled) HARQ processes for MCS4 with NTN-TDLA100-200, otherwise UE will be tested with 16 HARQ processes for MCS4 with NTN-TDLA100-200. This way we don’t need to define an additional test.

| Prop. Channel | MCS | HARQ Config |
| --- | --- | --- |
| NTN-TDLA100-200 | MCS4 | Disabled HARQ |
|  | MCS4 | 16 HARQ Proc |
| NTN-TDLC5-200 | MCS4 | 32 HARQ Proc |
|  | MCS13 | 16 HARQ Proc |

  - Option 3 (Huawei): If UE support 32HARQ process, then MCS13 will be tested.

| Prop. Channel | MCS | HARQ Config |
| --- | --- | --- |
| NTN-TDLA100-200 | MCS4 | Disabled HARQ |
|  | MCS4 | 16 HARQ Proc |
| NTN-TDLC5-200 | MCS13 | 32 HARQ Proc |
|  | MCS13 | 16 HARQ Proc |

- Recommended WF

  - TBA

Issue 2-6: Testability of the disabled HARQ scenario

- Proposals

  - Option 1(Ericsson, Qualcomm, Apple, Nokia): 4 disabled, 12 enabled in 16 HARQ processes. The peak throughput with scaling 3/4 can be used as the metric for this test.

  - Option 2 (Huawei): Keep previous agreements that all HARQ feedback are disabled. How to perform test will up to RAN5 discussion.

- Recommended WF

  - TBA

Issue 2-7: Simulation assumption for PDSCH performance with disabled HARQ process

- Proposals

  - Option 1(Ericsson, Qualcomm, Apple, Nokia): set the number of HARQ Processes as: 4 disabled, 12 enabled in 16 HARQ processes

  - Option 2 (Huawei): Keep previous agreements that all HARQ feedback are disabled.

- Recommended WF

  - TBA

# TS 38.101-5 CR work split

The following CR work split is agreed:

| Section | Title | Company |
| --- | --- | --- |
| 8 | Conducted performance requirements |  |
| 8.1 | General | Huawei |
| 8.2 | Demodulation performance requirements |  |
| 8.2.1 | General | Huawei |
| 8.2.1.1 | Applicability of requirements |  |
| 8.2.1.1.1 | General |  |
| 8.2.1.1.2 | Applicability of requirements for different number of RX antenna ports |  |
| 8.2.1.1.3 | Applicability of requirements for optional UE features |  |
| 8.2.1.1.4 | Applicability of requirements for mandatory UE features with capability signalling |  |
| 8.2.1.2 | PDSCH demodulation requirements | Qualcomm |
| 8.2.1.2.1 | 1RX requirements (void) |  |
| 8.2.1.2.2 | 2RX requirements |  |
| 8.2.1.2.2.1 | FDD |  |
| 8.2.1.2.2.1.1 | Minimum requirements for PDSCH with Mapping Type A |  |
| A.1 | General | Ericsson |
| A.1.1 | Throughput definition |  |
| A.3 | DL reference measurement channels |  |
| Annex B (normative) | Propagation conditions | Apple |
| B.1 | Static propagation condition (void) |  |
| B.2 | Multi-path fading propagation conditions |  |
| Annex C (normative) | Downlink physical channels | Apple |
| C.1 | General |  |
| C.2 | Setup (Conducted) |  |
| C.3 | Connection (Conducted) |  |
| Annex E (normative) | Environmental conditions | Apple |
| E.1 | General |  |
| E.2 | Environmental (Conducted) |  |