3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2217348

# Electronic Meeting, 10th – 19th Oct., 2022

Title:	WF on NTN SAN demodulation requirements

Source:	Huawei, HiSilicon

Agenda item:			4.2.8

Document for:	Approval

Introduction

This WF capture all agreements and open issues for the following topics discussed in [104-bis-e][318] NR_NTN_Demod_Part2].

- Satellite Access Node demodulation requirements

- CR work splitting

- Simulation assumption

Satellite Access Node demodulation requirements

Normal PUSCH requirement

Antenna configuration for PUSCH requirements

Agreements

- Keep previous agreement that consider both 1Rx and 2Rx for SAN PUSCH demodulation requirements

Test applicability rule for different antenna configuration for PUSCH requirements

Agreements

- Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

PUCCH requirements

Simulation alignment for PUCCH format 0

Agreements

- More simulation results are needed to derive the final performance requirements.

- Companies are encouraged to submit results for next meeting.

Simulation alignment for PUCCH format 2

Agreements

- More simulation results are needed to derive the final performance requirements.

- Companies are encouraged to submit results for next meeting

- For 1T1R cases for ACK missed detection, further alignment is needed considering the larger span.

Antennal configuration for PUCCH requirements

Candidate options

- Option 1: Consider SAN PUCCH format 0 and format 2 requirements for only 2Rx configuration with NLOS channel. Corresponding manufacture declarations and applicability rules should be further discussed.

- Option 2: Introduce LOS channel for SAN PUCCH format 0 and 2 demodulation requirements with 1Rx configuration.

- Option 3: Discuss a general rule that if the final derived simulation result is larger than a certain value, such as [10dB], then the corresponding cases can be not defined.

- Option 4: Consider both 1Rx and 2Rx for SAN PUCCH format 0 and 2 requirements with NLOS channel with the same test applicability rule for different antenna configurations as for PUSCH.

- Option 5: Option 1 + Option 2, i.e. Consider 1Rx SAN PUCCH format 0 and 2 requirements with LOS channel and 2Rx SAN PUCCH format 0 and 2 requirements with NLOS channel, with the same test applicability rule for different antenna configurations as for PUSCH.

SNR values for performance requirements

Agreements

- Keep TBD for this meeting and decide the SNR values in next meeting based on more submitted simulation results from interesting companies.

CR work spliting

Table 3-1: CR splitting for TS 38.108

| 38.108 Satellite Access Node radio transmission and reception |  |  |
| --- | --- | --- |
| bigCR for TS 38.108 |  | Huawei (see agreed WF R4-2214387 from RAN4#104-e) |
| 8 Conducted performance requirements |  |  |
| 8.1 | General | THALES |
| 8.2 | Performance requirements for PUSCH | Huawei (see agreed WF R4-2214387 from RAN4#104-e) |
| 8.3 | Performance requirements for PUCCH | Ericsson (see agreed WF R4-2214387 from RAN4#104-e) |
| 8.4 | Performance requirements for PRACH | Huawei |
| 11 Radiated performance requirements |  |  |
| 11.1 | General | THALES |
| 11.2 | Performance requirements PUSCH | Huawei (see agreed WF R4-2214387 from RAN4#104-e) |
| 11.3 | Performance requirements for PUCCH | Ericsson (see agreed WF R4-2214387 from RAN4#104-e) |
| 11.4 | Performance requirements for PRACH | Huawei |
| Annex Reference measurement channels |  |  |
| A.3 | Fixed Reference Channels for performance requirements (QPSK R=308/1024) | Ericsson |
| A.3A | Fixed Reference Channels for performance requirements (QPSK R=99/1024) |  |
| A.4 | PRACH Test preambles | Huawei |
| Annex [D] (Normative): Propagation conditions |  |  |
| [D.1] | Static propagation condition | Huawei |
| [D.2] | Multi-path fading propagation conditions | Huawei |
| [D.3] | Moving propagation conditions | Huawei |

Table 3-2: CR splitting for TS 38.181

| 38.181 Satellite Access Node conformance testing |  |  |
| --- | --- | --- |
| Big CR for TS 38.181 |  | Ericsson (see agreed WF R4-2214387 from RAN4#104-e) |
| 4 General test conditions and declarations |  |  |
| 4.6 | Manufacturer declarations | (THALES) |
| 8 Conducted performance requirements |  |  |
| 8.1 | General | THALES |
| 8.2 | Performance requirements for PUSCH | Ericsson (see agreed WF R4-2214387 from RAN4#104-e) |
| 8.3 | Performance requirements for PUCCH | Huawei (see agreed WF R4-2214387 from RAN4#104-e) |
| 8.4 | Performance requirements for PRACH | Ericsson (see agreed WF R4-2214387 from RAN4#104-e) |
| 11 Radiated performance requirements |  |  |
| 11.1 | General | THALES |
| 11.2 | Performance requirements PUSCH | Huawei |
| 11.3 | Performance requirements for PUCCH | Ericsson |
| 11.4 | Performance requirements for PRACH | Huawei (see agreed WF R4-2214387 from RAN4#104-e) |
| Annex A |  |  |
| A.3 | Fixed Reference Channels for performance requirements | Huawei |
| A.4 | PRACH test parameters | Huawei |
| Annex C Test tolerances and derivation of test requirements |  |  |
| C.3 | Measurement of performance requirements | Ericsson |
| Annex D Measurement system set-up |  |  |
| [D.5] | SAN type 1-H performance requirements | Ericsson |
| [D.6] | SAN type 1-O performance requirements | Ericsson |
| Annex G Propagation conditions |  |  |
| G.1 | Static propagation condition | Ericsson |
| G.2 | Multi-path fading propagation conditions | Ericsson |
| G.3 | Moving propagation conditions | Ericsson |

Simulation assumption (For information)

PUSCH

NTN normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled, Enabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 group hopping and sequence hopping are disabled for transform precoding enabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low, NTN-TDLC5-200 Low (K_factor = 8.05) |
| Test metric |  | SNR@70%maximum throughput |

NTN UL timing adjustment

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 12 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 12 |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | CSRS = 5, BSRS =0, for 20 RB |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low |
| Test metric |  | SNR@70%maximum throughput |
| NOTE 1. The transmission of SRS is optional. The transmission comb is configured as KTC = 2. The SRS periodic is configured as TSRS = 10 for 15kHz SCS and 20 for 30kHz SCS respectively. |  |  |

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: NTN-TDLA100-200 Low |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 0.04 S-1 for 15kHz SCS and 0.08 S-1 for 30kHz SCS |

NTN PUSCH repetition type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 5 in Table 3 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low |
| Test metric |  | SNR@70%maximum throughput |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

PUCCH

NTN PUCCH format 0

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Number of OFDM symbols | 2 |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | ACK missed detection probability <1% |

NTN PUCCH format 1

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

NTN PUCCH format 2 (ACK missed detection)

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

NTN PUCCH format 2 (UCI BLER)

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

NTN PUCCH format 3

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DM-RS, Additional DM-RS |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

NTN PUCCH format 4

| Parameter | Value |
| --- | --- |
| PUCCH format | 4 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DMRS, Additional DM-RS |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

NTN PUCCH format 1 (multi-slot)

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

PRACH

| Parameter | Value |
| --- | --- |
| Preamble format | 0/B4/C2/2 |
| SCS | 1.25kHz for long preamble format 15kHz and 30kHz SCS for short preamble format (LRA=139) |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | AWGN, NTN-TDLA100-200 Low |
| Time error tolerance | 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel |
| Frequency offset | 0Hz for AWGN, 200Hz for fading channel |
| Test metric | false alarm probability<0.1%, detection probability>99% |

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| 2 | 1.25 | 13 | 22 | 32 |
| B4,C2 | 15 | 23 | 0 | 0 |
|  | 30 | 46 | 0 | 0 |

Reference

- R4-22xxxxx, Email discussion summary for [104-bis-e][318] NR_NTN_Demod_Part2], RAN4#104bis-e, Moderator (Huawei)

- R4-2214387, WF on NTN SAN demodulation requirements, RAN4#104-e, Huawei, HiSilicon

- R4-2210662, WF on NTN SAN demodulation requirements, RAN4#103-e, Huawei, HiSilicon

- R4-2207198, WF on NTN SAN demodulation requirements, RAN4#102-e, Huawei, HiSilicon

- R4-2203043, WF on NTN SAN demodulation requirements, RAN4#101bis-e, Huawei, HiSilicon