3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216021

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Simulation results on FR2-2 PUSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.3.7.3.1

Document for:	Discussion

Background

As per [1], in this paper, we provide our simulation results for FR2-2 PUSCH

Simulation results

The common simulation assumptions are summarized in Table 2-1:

Table 2-1: Common simulation results for PUSCH requirements

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled,Enabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 for rank1,{0,1} for rank2 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| Phase noise model |  | Set 2 in TR 38.808 |
| Phase noise compensation |  | CPE compensation for 16QAM and 64QAM. No compensation for QPSK |
| PT-RS | Frequency density (KPT-RS) | 2 for 16QAM and 64QAM, Disabled for QPSK |
| configuration | Time density (LPT-RS) | 1 for 16QAM and 64QAM, Disabled for QPSK |

CP-OFDM

The simulation results for CP-OFDM are summarized in Table 2-1:

Table 2-1: Simulation results for CP-OFDM

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | -2.0 |
|  |  |  |  | 2x2 Low | 2.6 |
| 120 | 100 | 16 | TDLA30-650 | 1x2 Low | 9.8 |
|  |  |  |  | 2x2 Low | 19.6 |
| 120 | 100 | 16 | TDLD30-650 | 2x2 Low | 11.5 |
| 120 | 100 | 20 | TDLA30-200 | 1x2 Low | 10.9 |
|  |  |  |  | 2x2 Low | 15.2 |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | -3.5 |
|  |  |  |  | 2x2 Low | 0.2 |
| 480 | 400 | 16 | TDLA10-650 | 1x2 Low | 7.3 |
|  |  |  |  | 2x2 Low | 11.3 |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low | 10.1 |
|  |  |  |  | 2x2 Low | 15.1 |

DFT-S-OFDM

The simulation results for CP-OFDM are summarized in Table 2-2

Table 2-2 Simulation results for CP-OFDM

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | -1.8 |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | -2.3 |

Conclusion

In this paper we provide our simulation results for FR2-2 PUSCH requirements.

Reference

[1]   R4-2214388_WF_PUSCH_FR2-2. Nokia, Nokia Shanghai Bell.