3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216025

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Discussions on remain issues on FR2-2 PRACH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.3.7.3.3

Document for:	Discussion

Background

As per [1], RAN4 left some open issues for FR2-2 PRACH requirements. In this paper we provide our  corresponding views.

Discussions

SCS for PRACH requirements

- Option 1: Define requirements for PRACH using 120 kHz, 480 kHz and 960 kHz SCS - Option 2: Define requirements for PRACH using 120 kHz and 480 kHz SCS - Option 3: Define requirements for PRACH using 120 kHz, 480 kHz and 960 kHz SCS - Define that 960 kHz is optional

Similar to other PHY channel, we suggest to not consider 960kHz SCS. Firstly, 960kHz SCS brings much trouble to testability as analysed in previous meetings. Secondly, 960kHz needs too many continuous UL slots which exceeds maximum number of supported HARQ processes. For option 3, we don’t understand the intention since all features are based on declaration.

Proposal 1: RAN 4 to define the PRACH requirements with 120kHz SCS and 480kHz SCS.

Time error tolerance

In last meeting, RAN 4 agreed to define time error tolerance for PRACH requirements as follows:

| SCS | Channel model | Time error tolerance (ns) |
| --- | --- | --- |
| 120 | AWGN | 70 |
|  | TDLA10-650Note 1 | [120]Note 1 |
| 480 | AWGN | 18 |
|  | TDLA10-650 | [68] Note 1 |
| 960 | AWGN | 9 |
|  | TDLA10-650 | [59] Note 1 |
| NOTE 1: To be revised after definition of channel taps used for TDLA 10. |  |  |

In [2], RAN4 agreed to define following simplified channel for TDLA10:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

We can observe that the power of last tap is too small which can be ignored. Hence we propose to use 50ns as maximum delay for fading channel. Therefore, we can confirm to use following configurations as time error tolerance:

| SCS | Channel model | Time error tolerance (ns) |
| --- | --- | --- |
| 120 | AWGN | 70 |
|  | TDLA10-650 | 120 |
| 480 | AWGN | 18 |
|  | TDLA10-650 | 68 |

Proposal 2: Use following configurations as time error tolerance:

| SCS | Channel model | Time error tolerance (ns) |
| --- | --- | --- |
| 120 | AWGN | 70 |
|  | TDLA10-650 | 120 |
| 480 | AWGN | 18 |
|  | TDLA10-650 | 68 |

Conclusion

In this paper we provide our views on open issues on performance requirements for PRACH. The proposals are:

Proposal 1: RAN 4 to define the PRACH requirements with 120kHz SCS and 480kHz SCS.

Proposal 2: Use following configurations as time error tolerance:

| SCS | Channel model | Time error tolerance (ns) |
| --- | --- | --- |
| 120 | AWGN | 70 |
|  | TDLA10-650 | 120 |
| 480 | AWGN | 18 |
|  | TDLA10-650 | 68 |

Reference

[1]   R4-2214389 WF on PRACH demodulation requirements for FR2-2. Samsung

[2]   R4-2214655 Way Forward on general aspects for FR2-2 BS demodulation requirements. Huawei