3GPP TSG-RAN WG4 Meeting #104-bis-e	R4- 2215918

# Electronic Meeting,  Ocotber 10 – Ocotber 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on PDSCH requirements for 52.6 - 71 GHz band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2022-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Define PDSCH requirements for 52.6 – 71 GHz band |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Define PDSCH requirements for FR2-2 single carrier |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDSCH for NR extension to 71 GHz is not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1 ------------------------------------------------------------

## 7.2	PDSCH demodulation requirements

The parameters specified in Table 7.2-1 are valid for all PDSCH demodulation tests unless otherwise stated.

Table 7.2-1: Common Test Parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| PTRS epre-Ratio |  |  |  | 0 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 60 or 120 or 480 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-2 [7] for tested channel bandwidth and subcarrier spacing |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot for 120 KHz SCS (Xs, Ys) = (4, 1) for 480 KHz SCS |
|  | Symbols with PDCCH |  |  | 0 |
|  | Number of PRBs in CORESET |  |  | Table 7.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI state |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | Single Panel Type I, Random per slot with equal probability of each applicable i1, i2 combination, and with REG bundling granularity for number of Tx larger than 1 |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 6 for CSI-RS resource 1 and 3 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 for CSI-RS resource 1,2,3,4 120 kHz SCS: 160 for CSI-RS resource 1,2,3,4 480 kHz SCS: [640] for CSI-RS resource 1, 2, 3, 4 |
|  | CSI-RS offset |  | Slots | 60 kHz SCS: 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 120 kHz SCS: 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 480 kHz SCS: 320 for CSI-RS resource 1 and 2 321 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 12 |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 120 kHz SCS: 160 480 kHz SCS: 640 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 120 kHz SCS: 160 480 kHz SCS: 640 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |
| CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 for CSI-RS resource 1,2 120 kHz SCS: 160 for CSI-RS resource 1,2 480 kHz SCS: 640 for CSI-RS resource 1,2 |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | Repetition |  |  | ON |
|  | QCL info |  |  | TCI state #1 |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| PTRS configuration | Frequency density (KPT-RS) |  |  | 2 |
|  | Time density (LPT-RS) |  |  | 1 |
|  | Resource Element Offset |  |  | 2 |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 120 KHz SCS: 4 480 KHz SCS: 16 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, andwith Wideband granularity |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 for FR2-1 tests OP.1 TDD as defined in Annex A.5.2.1 for FR2-1 tests No symbols on unused REs for FR2-2 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-2 [7] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |

Table 7.2-2: Number of PRBs in CORESET

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 30 | 66 | 132 | 264 |
| 480 | N.A | N.A | N.A | 66 |

------------------------------------------------------------ End of change 1 ------------------------------------------------------------

----------------------------------------------------- Beginning of Change 2 ------------------------------------------------------------

### 7.2.2	2RX requirements

#### 7.2.2.1	FDD

#### (Void)

#### 7.2.2.2	TDD

##### 7.2.2.2.1	Minimum requirements for PDSCH Mapping Type-A

For PDSCH Type-A scheduling, the requirements are specified in Table 7.2.2.2.1-3, 7.2.2.2.1-4, 7.2.2.2.1-5, and 7.2.2.2.1-6 with the addition of the parameters in Table 7.2.2.2.1-2 and the downlink physical channel setup according to Annex C.5.1. The purpose is to verify the performance of PDSCH Type-A scheduling.

The test purposes are specified in Table 7.2.2.1.1-1.

Table 7.2.2.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance in FR2-1 under 2 receive antenna conditions and with different channel models, MCSs andnumber of MIMO layers in FR2-1. | 1-1, 1-3, 1-4, 2-1, 2-2, 2-3, 2-4, 2-5, 2-6 |
| Verify the PDSCH mapping Type A HARQ soft combining performance in FR2-1 under 2 receive antenna conditions. | 1-2 |
| Verify the PDSCH mapping Type A performance requirements for Enhanced Receiver Type 1 in FR2-1 under 2 receive antenna conditions. | 3-1 |
| Verify the PDSCH mapping Type A normal performance in FR2-2 under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 4-1, 4-2, 4-3, 4-4, 4-5, 4-6 |

Table 7.2.2.2.1-2: Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | For Test 1-1 and 1-2: 3 for CSI-RS resource 1 and 3 7 for CSI-RS resource 2 and 4 |
|  | CSI-RS offset | Slots | For Test 1-2: 82 for CSI-RS resource 1 and 2 83 for CSI-RS resource 3 and 4 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL4 for Test 1-4 and 2-3 1/AL8 for other tests |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband for Test 1-1, 2 for other tests |
|  | Resource allocation type |  | Test 2-1: Type 1 with start RB = 30, LRBs = 6 Test 4-4: TBD (Center) Test 4-7: TBD (Center) Other tests: Type 0 |
|  | RBG size |  | Test 2-1: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 for Test 1-1, 1-3, 1-4, 2-2, 2-4, 4-1, 4-2, 4-4 10 for Test 2-1, 2-3, 2-5, 2-6, 3-1 16 for Test 1-2, 4-3, 4-5, 4-6 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

Table 7.2.2.2.1-3: Minimum performance for Rank 1 (FRC) in FR2-1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 1-1 | R.PDSCH.5-1.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-1A | TDLC60-300 | 2x2 ULA Low | 70 | -0.4 |
| 1-2 | R.PDSCH.5-2.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 30 | 1.7 |
| 1-3 | R.PDSCH.5-3.1 TDD | 100 / 120 | 64QAM, 0.46 | FR2.120-1 | TDLA30-300 | 2x2 XPL Medium | 70 | 12.4 |
| 1-4 | R.PDSCH.5-10.1 TDD | 50 / 120 | 256QAM 0.67 | FR2.120-1 | TDLD30-75 | 2x2 ULA Low | 70 | 20.2 |

Table 7.2.2.2.1-4: Minimum performance for Rank 2 (FRC) in FR2-1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 2-1 | R.PDSCH.5-4.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 4.1 |
| 2-2 | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.4 |
| 2-3 | R.PDSCH.5-5.2 TDD | 50 / 120 | 16QAM,0.48 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 14.0 |
| 2-4 | R.PDSCH.5-2.3 TDD | 200 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.2 |
| 2-5 | R.PDSCH.4-1.1 TDD | 50 / 60 | 16QAM, 0.48 | FR2.60-1 | TDLA30-75 | 2x2 ULA Low | 70 | 14.3 |
| 2-6 | R.PDSCH.5-6.1 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 18.6 |

Table 7.2.2.2.1-5: Minimum performance for Rank 2 (FRC) for Enhanced Receiver Type 1 in FR2-1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 3-1 | R.PDSCH.5-5.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-2 | TDLA30-75 | 2x2 ULA Medium | 70 | 19.0 |

Table 7.2.2.2.1-6: Minimum performance for Rank 1 (FRC) in FR2-2

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of max through-put (%) | SNRBB (dB) |
| 4-1 | R.PDSCH.5-1.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 70 | TBD |
| 4-2 | R.PDSCH.5-2.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-200 | 2x2 ULA Low | 70 | TBD |
| 4-3 | R.PDSCH.5-2.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | [TDLA30-650] | 2x2 ULA Low | 30 | TBD |
| 4-4 | R.PDSCH.5-3.2 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | 70 | TBD |
| 4-5 | R.PDSCH.7-1.1 TDD | 400 / 480 | QPSK, 0.30 | FR2.480-1 | TDLA10-200 | 2x2 ULA Low | 70 | TBD |
| 4-6 | R.PDSCH.7-2.1 TDD | 400 / 480 | 16QAM 0.48 | FR2.480-1 | TDLD10-200 | 2x2 ULA Low | 70 | TBD |

------------------------------------------------------------ End of change 2 ------------------------------------------------------------