3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2217429

Electronic Meeting, 10 ‒ 19 October 2022

Title: 	WF on RedCap UE demodulation and CQI reporting requirements

Agenda Item:	4.6.6

Source: 	Ericsson

Document for:	Approval

# Topic #1: UE demodulation requirements

## Sub-topic 1-1: Applied requirements for HD-FDD

Agreement: Whether to apply the same requirements for both 2Rx HD-FDD and 2Rx FD-FDD

- Confirm the agreement in RAN4#104: the requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements

## Final requirements

- RAN4 sets the final PDSCH/PDCCH/PBCH demodulation requirements according to the simulation results summary R4-2216234.

# Topic #2: CSI reporting requirements

## Sub-topic 2-1: CQI reporting requirements

Agreement: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD

- Set the following configuration for CQI reporting test in static/fading condition for FR1 FDD 15kHz

  - CSI-RS periodicity and offset: 10/5

  - CSI-Report periodicity and offset: 10/9

  - CQI/RI/PMI delay: 10ms

Agreement: Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests

- Set the static channel matrix in the frequency domain as H1RX=11.

- Set SNR test point X=3dB lower than 2Rx test case, that is, SNR=5/6dB and SNR=11/12dB.

## Sub-topic 2-2: PMI reporting requirements

Agreement: MIMO correlation matrix for 1Rx UE PMI reporting tests

- Define the MIMO correlation matrix for 4x1 ULA high as follows:

Rhigh=1.00000.98830.95420.89990.98331.00000.98830.95420.95420.98831.00000.98830.89990.95420.98831.0000

## Sub-topic 2-3: RI reporting requirements

Agreement: Test configuration for RI reporting tests

- Use the following configuration for RI reporting test

  - FR1 FDD

    - CQI table 1

    - Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.1-1

  - FR1 TDD

    - CQI table 1

    - CBW/SCS: 20MHz/30kHz

    - Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.2-1

  - FR2

    - Reuse the requirements

Agreement: CQI/RI/PMI reporting delay for RI reporting tests for FDD 15kHz

- Set the following configuration for RI reporting test for FR1 FDD 15kHz

  - CSI-RS periodicity and offset: 10/5

  - CSI-Report periodicity and offset: 10/9

  - CQI/RI/PMI delay: 10ms