3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216027

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Simulation results on remain issues on RedCap PDSCH

Source: 	Huawei, HiSilicon

Agenda item:       4.6.5.1.1

Document for:	Discussion

Background

As per [1], RAN 4 agreed to introduce 256QAM performance requirements for 1RX RedCap UE:

Agreement: Define 256QAM demodulation requirements for 1Rx RedCap UE in FR1 or not. - Define 256QAM demodulation requirements for 1Rx RedCap UE in FR1 - Set MCS20

In this paper, we provide our corresponding simulation results.

Simulation results

The simulation results are captured in Table 2-1:

Table 2-1: Simulation results for 1Rx RedCap UE for 256QAM

| 1Rx UE or 2Rx UE | CBW / SCS | Duplex mode | MCS and rank | Propagation condition | Antenna configuration | Metric | Ideal SNR(dB)@ 70% of max TP | Impairment SNR(dB)@ 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | FDD | MCS20,Rank1 | TDLA30-10 | 2x1 ULA low | 70% of peak rate | 20.5 | 22.0 |
| 1Rx UE | 20MHz/30kHz | TDD | MCS20,Rank1 | TDLA30-10 | 2x1 ULA low | 70% of peak rate | 21.9 | 23.4 |

One issues is whether to define the same requirements for 2RX FD-FDD and HD-FDD. Considering we have agreed to define the same requirements for 1RX, we don’t think it can be changed for 2RX.

Proposal 1: Define the same requirements for 1RX HD-FDD and FD-FDD

Conclusion

In this paper, we provide our simulation results for 1RX RedCap UE for 256QAM. The proposals are:

Proposal 1: Define the same requirements for 1RX HD-FDD and FD-FDD

Reference

[1]   R4-2214394 WF on Redcap UE demodulation and CQI reporting requirements. Ericsson