3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216032

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Discussion on remaining issues for RedCap RI requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.6.5.2.2

Document for:	Discussion

Background

As per [1], RAN4 agreed to define RI requirements:

Agreement: Whether to define RI reporting requirements for RedCap 2Rx UEs - Define RI reporting requirements for RedCap 2Rx UEs - Apply Test 2 only - Replace fading CQI test for 2 Rx UE (high SNR point) by RI test case (Test 2)

But the detailed simulation assumptions and SNR point are not discussed. In this paper, we provide our views.

Discussions

RAN 4 has agreed to apply Test 2 in Table 6.2.2.1-1 for RedCap RI test. However, RAN 4 also agreed to define same CSI pattern for FDD and HD-FDD. Therefore we should define the new CSI pattern for FDD RI test since the existing pattern in Test 2 can’t be applied for HD-FDD(The CSI is reporting periodicity and offset is 5/0 which means the CSI is transmitted in DL slot). To be simplified, we suggest to apply the CSI pattern of CQI test to RI test.I.e. CSI-RS periodicity and offset is 10/1, CSI reporting periodicity and offset is 10/9, CSI delay is 14ms.

Proposal 1: Use following configuration for RI test:

- FR1

- FDD:

- CSI-RS periodicity and offset is 10/1,

- CSI reporting periodicity and offset is 10/9,

- CSI delay is 14ms.

- CQI table 1

- Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.1-1

- TDD:

- CQI table 1

- CBW/SCS: 20MHz/30kHz

- Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.2-1

- FR2: Reuse the requirements

Simulation results

Based on our proposed simulation assumptions, we provide our simulation results in Table 3-1:

Table 3-1: Simulation results for FR1 RI test

| SNR(dB) | TP ratio: Follow RI/Fixed RI=1 |  |
| --- | --- | --- |
|  | FDD/HD-FDD | TDD |
| 16 | 1.36 | 1.50 |
| 17 | 1.30 | 1.43 |
| 18 | 1.33 | 1.56 |
| 19 | 1.41 | 1.58 |
| 20 | 1.46 | 1.63 |

We can observe that when SNR=20dB, the TP gain of follow RI/Fixed RI is maximum, we suggest to use SNR=20dB for RI test.

Proposal 2: Use SNR=20dB for RI test.

Conclusion

In this paper we provide our views on remaining issues for RedCap RI requirements. The proposals are:

Proposal 1: Use following configuration for RI test:

- FR1

- FDD:

- CSI-RS periodicity and offset is 10/1,

- CSI reporting periodicity and offset is 10/9,

- CSI delay is 14ms.

- CQI table 1

- Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.1-1

- TDD:

- CQI table 1

- CBW/SCS: 20MHz/30kHz

- Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.2-1

- FR2: Reuse the requirements

Proposal 2: Use SNR=20dB for RI test.

Reference

[1]   WF on RedCap demodulation and CQI reporting requirements. Ericsson