3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2217454

Electronic Meeting, 10 ‒ 19 Oct, 2022

Title: 	WF on ATG BS RF requirements

Agenda Item:	     6.13.4

Source: 	ZTE

Document for:	Approval

Topic #1 ATG BS RF requirements

Issue 1-1: Typical vertical altitude range [TBD km] for ATG scenarios with a ground BS to air UE

- Proposals

  - Option 1: 9.5-12km (CATT)

  - Option 2: 7-12km (CMCC)

  - Option 3: other, please specify

- Agreement:

  - Typical vertical altitude as [3/7] -12 km

  - Further check the feasibility of supporting 3km based on co-existence study

Issue 1-2: Whether or not to include BS type 1-C?

- Proposal:

  - Option 1: Yes (CATT, CMCC, Huawei)

  - Option 2: No (Ericsson, ZTE)

- Agreement: Include BS type 1-C with the assumption that no additional work on co-existence study

Issue 1-3-1: Whether or not to support 256QAM?

- Proposal:

  - Option 1: Yes (CMCC, Ericsson, ZTE)

  - Option 2: No, please provide reasons.

Issue 1-3-2: EVM value for 256QAM

- Proposal:

  - Option 1: 3.5% (CMCC, ZTE)

  - Option 2: No, please provide reasons.

- Agreement: 256QAM is supported and EVM value for 256QAM equals to 3.5%.

Issue 1-4-1: Descriptions for the ATG BS class

- Proposals in R4-2216400:

  - Proposal 1		Confirm the following description for the ATG BS class: “ATG Base Stations are characterized by requirements derived from ATG scenarios with a ground BS to air UE with typical vertical altitude range [TBD km]”

  - Proposal 2		Consider adding a note after the class definition that ATG BS requirements are, unless otherwise stated, the same as WA BS requirements.

- Agreement:

  - ATG Base Stations are characterized by requirements derived from ATG scenarios with a ground BS to air UE with typical vertical altitude range [TBD km]

    - Note: Further refinement on the text proposal to TS related to altitude range not precluded

Issue 1-4-2: Whether or not to remove Rx IM requirement?

- Proposal:

  - Option 1: Yes

  - Option 2: No, please provide reasons.

- Agreement: FFS whether Rx IM requirements needed or not; if introduced, baseline assumption is existing WA BS class requirements can be reused.