3GPP TSG-RAN WG4 Meeting #104-bis-e									                 R4-2217464

Electronic Meeting, 10– 19 October 2022

Agenda item:			6.17.4

Source:	Samsung

Title:	WF on SBFD feasibility study and RF impact: BS aspects

Document for:	Approval

# Introduction

This document provides way forwards SBFD feasibility study and RF impact from BS aspects, based on the outcomes of email discussion summary for [104-bis-e][310] FS_NR_duplex_evo_Part1.

# Way Forward on Self-Interference Model

## BS class and feasibility for self-interference modelling

Agreement (based on GTW on Oct. 14th):

- Considering BS classes for deriving the different value ranges of the RSIC and corresponding RF feasibility study

  - FR1: Different power limits, selectivity level associated with BS classes

  - FR2: The assumption of values for output power (TRP level) with the candidate range {30 ~40 dBm}

    - Others values out of above candidate range not precluded

    - Note 1: Companies are encouraged to provide the detailed assumption with corresponding proposed upper limit value

    - Note 2: Further discuss the power output per antenna element in 2nd round

## 1dB Desense Target and the threshold for RSIC

Agreement (based on GTW on Oct. 14th):

- Criteria on gNB UL receiver sensitivity degradation due to self-interference:

- Taking 1dB sensitivity degradation due to self-interference of DL transmission as baseline target for system level evaluation and feasibility study at current stage from RAN4 perspective

  - Final values used in co-existence evaluation shall be aligned with feasibility analysis conclusion.

  - RAN4 can use 1dB sensitivity degradation as criteria in feasibility study

  - FFS whether other values can be considered for some special cases

  - Above conclusion intended for RAN4 only and other WGs can make conclusion based on their own analysis.

## SBFD configuration assumption for RSIC

Agreement:

- For RSIC analysis, to be provided by each company:

  - SBFD configuration assumption should be reported.

## Guardband assumption for RSIC

Agreement:

- For RSIC analysis, to be provided by each company:

  - Guardband assumption (whether guard-band assumed and the values of guard-band if any) should be reported.

## Necessity/feasibility on RB level scaling

Agreement (based on GTW on Oct. 14th):

- gNB self-interference can be modelled as frequency flat.

## RSI dependency on blocking and AGC

Agreement:

- RAN4 further study on LNA saturation/non-linearity:

  - Option 1: -43 dBm can be assumed as the maximum blocking level to ensure the receiver of UL sub-band is not blocked and maintain an acceptable reference sensitivity, for FR1 WA BS.

    - FFS -43dBm could be enough for FR1 WA BS;

    - FFS whether RMS/peak value should be used.

    - FFS an acceptable reference sensitivity:

      - It is assumed that the receiver performs sufficiently better than 3GPP minimum requirements that the sensitivity degradation is <=0.5-1dB when a -43dBm signal is applied instead of the RAN4 requirement of 6dB degradation.

      - Other assumption is not precluded.

    - FFS other BS classes in next meeting.

  - Option 2: Further Evaluation should be based on IIP3 performance with respect to RAN4 receiver IM requirement

    - FFS how/whether LNA IIP3 value can be aligned.

- RAN4 further study on ADC dynamic range:

  - FFS ADC dynamic range is overloaded or not, especially considering different BS classes.

- RAN4 further study on AGC impact:

  - The below AGC model is one proposal in RAN4#104-bis-e:

    - the effect of non-linearities at the gNB receiver suffered can be modelled as a linear increase (with slope SL1 and SL2) of the base station noise figure as a function of the RF peak input power at each Rx chain once such peak input power exceeds a first and a second threshold a and b.

  - Other AGC model is not precluded.

Figure. Behaviour of noise figure as a function of Peak input power.

Table: Example parametrization of proposed model

| Snf | Small signal noise figure | 5 | dB |
| --- | --- | --- | --- |
| a | Peak input power threshold 1 | -40 to -25 | dBm |
| b | Peak input power threshold 2 | -20 to -10 | dBm |
| SL1 | Noise figure slope 1 | 0.1 to 1 |  |
| SL2 | Noise figure slope 2 | 1 to 2 |  |

NF = Snf for Peak input power < a NF = Snf – a * SL1 + SL1* Peak input power for: a < Peak input power < b NF = Snf – a*SL2 + b*(SL1 – SL2) + SL2*( Peak input power) for Peak input power > b

## Refinement of RSIC value/range

Agreement:

- RAN4 further study the detailed values for RSIC analysis:

  - In addition to below already listed parameters and resultant overall RSIC capability, including

    - Spatial isolation

    - Frequency isolation

    - Beam nulling /isolation

    - Digital IC

    - Overall RSIC capability

  - additional implementation details should be included in each companies’ analysis:

    - SBFD configuration

    - Guardband assumption

    - Sub-band filtering assumption (if exist)

    - Other details.

  - The following template on detailed isolation contributions and other factors is suggested to be used in the following RAN4 analysis:

    - The below table is taken as an example for FR1 WA BS, and similar table for other FR1 BS type and FR2 BS.

| FR1 Wide-Area BS |  | Company-A | Company-B |
| --- | --- | --- | --- |
| Component capability | Spatial isolation | xxx dBc | xxx dBc |
|  | Frequency isolation | xxx dBc | xxx dBc |
|  | Beam nulling /isolation | xxx dBc | xxx dBc |
|  | Digital IC | xxx dBc | xxx dBc |
| Overall RSIC capability |  | xxx dBc | xxx dBc |
| additional implementation details | SBFD configuration |  |  |
|  | Guardband assumption (if exist) |  |  |
|  | Sub-band filtering assumption (if exist) |  |  |
|  | bandwidth over which suppression is achieved |  |  |
|  | Others |  |  |

# Way Forward on Co-channel Inter-subband gNB-gNB CLI Model

## Co-channel co-site inter-sector gNB-gNB CLI modelling

Agreement:

On the feasibility and how to model co-site inter-sector gNB-gNB CLI modelling: similar modelling as for self-interference (RSI) can be applied but may with different parameters especially on antenna isolation

- FFS on possibility to apply digital IC for this case

- For co-channel co-site inter-sector gNB-gNB CLI modelling, it is encouraged to provide the numerical value for:

  - The achievable coupling loss in the case of co-site inter-sector gNB-gNB

  - Compared to self-interference, FFS the antenna isolation (with the achievable coupling loss).

    - Practical issues to achieve antenna isolation can be considered: e.g. increasing sector separation, mounting isolating materials on the site and the physical characteristics of such materials (size, weight etc.)

- Clarification on the value discussed here:

  - the co-channel co-site inter-sector gNB-gNB CLI discussed here is for the sum contributions from all co-site sectors.

## Co-channel inter-site gNB-gNB CLI modelling

Agreement:

On feasibility and how to model inter-site gNB-gNB CLI modelling considering unwanted emission and receiver selectivity:

- Proposal: Same Transmitter leakage and receiver impairment model as used for investigating gNB self-interference, but antenna isolation is replaced with inter-site isolation.

  - TX leakage baseline: gNB ACLR

  - Receiver impairment baseline: FFS

  - RAN4 will further study the possibility of improved receiver impairment performance compared to gNB ACS.

# Way Forward on Adjacent-channel gNB-gNB CLI model

## Adjacent-channel co-site inter-sector gNB-gNB CLI modelling

Agreement:

- For adjacent-channel co-site inter-sector gNB-gNB CLI modelling, it is encouraged to provide the numerical value for:

  - The achievable coupling loss in the case of co-site inter-sector gNB-gNB

  - Compared to self-interference, FFS the antenna isolation (with the achievable coupling loss).

    - Practical issues to achieve antenna isolation can be considered: e.g. increasing sector separation, mounting isolating materials on the site and the physical characteristics of such materials (size, weight etc.)

- Clarification on the value discussed here:

  - the adjacent-channel co-site inter-sector gNB-gNB CLI discussed here is for the sum contributions from all co-site gNBs.

# Way Forward on Other Considerations for Feasibility Study

## BS self-interference related to timing and SCS

Agreement:

- For the BS self-interference issue related to timing and SCS of D and U for both BS and UE:

  - RAN4 understanding is that this issue will be studied in RAN1.

  - RAN4 has not evaluated the timing and SCS impacts to BS SI. RAN4 for now assumes they are negligible and do not impact RAN4 requirement work.

  - RAN4 will not consider this issue in the BS SI feasibility study if no request from RAN1.

## Receiver phase noise reciprocal mixing

Agreement:

- Receiver phase noise reciprocal mixing:

  - For FR1, the impact due to phase noise reciprocal mixing is negligible when the blocking level is -43 dBm.

  - For FR2, phase noise reciprocal mixing should be examined further, in particular for FR2 BS with 40dBm output power

# Way Forward on RF Requirement Impact

## Potentially impacted RF requirement for SBFD capable gNB

Agreement:

- RAN4 further study on the necessity of new RF requirements for SBFD operation with candidates as below:

  - In-channel adjacent subblock leakage ratio (new)

  - In-channel adjacent subblock Blocking (new)

  - Receiver OTA REFSENS (FFS)

  - Receiver intermodulation (FFS)

  - Transmitter intermodulation (for FR2 only)

  - Other proposals on new RF requirement(s) not precluded

## Existing requirement without impact for SBFD capable gNB

Agreement:

- RAN4 further study on:

  - FFS that existing RF requirements with respect to wanted signal as below are still applicable for gNB capable of SBFD

| Conducted RF requirement | Radiated RF requirement |
| --- | --- |
| BS output power Output power dynamics Transmit ON/OFF power Transmitted signal quality Occupied bandwidth Dynamic range | Radiated transmit power OTA base station output power OTA output power dynamics OTA transmitted signal quality OTA occupied bandwidth OTA sensitivity OTA dynamic range |

  - FFS that receiver out-of-band blocking and receiver spurious emission requirement in TS38.104 are still applicable gNB capable of SBFD.

  - With new requirement for SBFD operation (to be discussed in the previous issue), FFS that below requirement would be remained unchanged with respect to SBFD operation.

| Conducted RF requirement | Radiated RF requirement |
| --- | --- |
| Operating band unwanted emissions Transmitter spurious emissions Transmitter intermodulation In-channel selectivity | OTA out-of-band emission OTA transmitter spurious emission OTA transmitter intermodulation for BS type 1-O OTA in-channel selectivity |

## RF requirement for SBFD capable gNB based on coexistence study

Agreement:

- For gNB capable of SBFD:

  - ACLR and ACS are pending on RAN4 adjacent channel co-existence study