3GPP TSG-RAN WG4 Meeting #104-bis-e									                 R4-2217465

Electronic Meeting, 10– 19 October 2022

Agenda item:			6.17.4

Source:	Qualcomm Inc

Title:	WF on SBFD feasibility study and RF impact: UE aspects

Document for:	Approval

# Scope

This WF discusses UE feasibility aspects from 310

# UE TX modelling aspects

This WF discusses UE feasibility aspects from 310

## Adjacent channel model

### UE TX aggressor toward adjacent channel victim (FR1)

An ACLR type model was previously  agreed as a starting point, FFS for ACLR2, and FFS on the granularity in frequency

What base value for ACLR1 in TX model for FR1 power class 3?

Option 1: 30 dB

Option 2: 31 dB

Proposed WF: Option 1

| Company | Comment |
| --- | --- |
| QCOM | We are ok with either option |
| vivo | We suggest to firstly clarify the aggressor bandwidth and the victim bandwidth and SBFD configuration. If the aggressor bandwidth and victim bandwidth are the same and adjacent, Option 1 is the baseline according to current UE spec. |
| Ericsson | Agree with vivo comment; at least when the aggressor and victim bandwidth are the same we can use the UE spec. |
| Huawei | Ok with Option 1 |
| MediaTek | If the legacy NR ACLR for FR1 is 30dB why would we select something more stringent? Option 1. |
| Samsung | If PC3 is assumed, Option 1 is straightforward. |

Moderator: The victim BW is not part of the issue here but I understand the confusion about allocation and where the distortion is in relation to the channel boundaries. The proposal below simplifies.

Agreement:

- 30 dB is the total distortion power on either side of a fully allocated uplink sub-band. The ACLR1 distortion PSD is modeled as flat over that range (From the agreement below)

- FFS whether we need to consider whether we need to model allocations that are less than fully  allocated uplink sub-bands

What base value value should the model use for FR1 PC3 ACLR2?

Option 1: 33 Db

Option 2: Don’t use ACLR2 at all.

Option 3: Something else

Proposed WF: Further discuss the 3 options

| Company | Comment |
| --- | --- |
| QCOM | Option After some consideration our view has changed on this aspect. ACLR |
| vivo | We suggest to firstly clarify the aggressor bandwidth and the victim bandwidth and SBFD configuration. ACLR2 may be needed based on specific adjacent channel co-existence scenario. |
| Ericsson | Not a strong view but it may be preferable to assume a single ACLR and see if there is any issue with UE-UE CLI. If there is an issue we could revisit, but most likely it does not make a difference. So slight preference for option 2. |
| Huawei | No strong view. We can start with option 2. |
| Samsung | We are okay with ACLR2, while seems till now, companies may still need to discussion the details of ACLR2, which may not a easy job to be aligned. |

Agreement:

- Follow Ericsson suggestion and evaluate the effect of UE-UE CLI with ACLR1 only.

- Revisit the discussion on ACLR2 if UE-UE CLI becomes significant

Do we need to model TX power classes other than FR1 PC3?

Option 1: power class 3 only

Option 2: other power classes needed

Proposed WF: Further discuss the two options

| Company | Comment |
| --- | --- |
| QCOM | Likely the answer is option 1, and we base that on the expectation that the UEs in the system simulation will all be PC3 UEs. |
| vivo | We prefer Option 1. |
| Ericsson | Option 1 is fine |
| Huawei | Ok with option 1 |
| Verizon | We agree Option 1 first, and then others |
| Nokia, Nokia Shanghai Bell | Ok with option 1. |
| MediaTek | Option 1 |
| Samsung | We are okay with Option 1. |

Agreement: Option 1 power class 3 only

What is the frequency resolution (granularity) of the model?

Option 1  - Distortion is modeled per RB across the frequency range of the distortion

Option 2 – Distortion is modeled as a flat power spectral densitity across the frequency range of the distortion

Proposed WF: Further discuss the two options

| Company | Comment |
| --- | --- |
| QCOM | We prefer option 2 for simplicity. Note that RAN1 seems to have some confusion as how to apply option2 distortion in the case where the victim sub-band is narrower than the aggressor sub-band. We should make it clear to them to scale the total interferer power down with the victim sub-band BW. For example if the victim SB BW is half the aggressor SB BW, scale the interferer down 3 dB. We would be OK with option 1 also if the group goes toward that direction. Either option can work. |
| vivo | Option 2 is aligned with our understanding since ACLR model is a frequency flat model. The distortion can be seen as flat per RB in the victim bandwidth. |
| Ericsson | Agree with option 2 with some scaling if needed. There is no need for a PRB resolution |
| Huawei | Ok with option 2 |
| Verizon | We are ok for Option 2 |
| Nokia, Nokia Shanghai Bell | Ok with option 2 |
| Samsung | As commented by QC, we see the RAN1 discussion, and RAN4 can further provide the clarification based on the above common understanding of Option 2. |

Agreement: Option 2 – Distortion is modeled as a flat power spectral densitity across the frequency range of the distortion

Should the ACLR-based interference be scaled with backoff?

Moderator: In R4-2216794 a backoff-dependent approach was proposed for UE TX, applicable for both adjacent channel and in-channel. We can discuss this aspect here in adjacent channel. The 2c option got some support in co-channel let’s discuss that one here

  - Option 2: Use ACLR-type model:

    - Option 2c: Model C: Use an ACLR1-type interference model that improves 1 dB per dB of power reduction from maximum, with at most 8 dB of ACLR improvement. In additional apply a -50 dBm floor to the interference to reflect the limit in 38.101-1.

Options

Option 1: Yes with option 2c for ACLR1 and some similar method if we use ACLR2

Option 2: Yes with some FFS method for ACRL1 and a similar method for ACLR2

Option 3: No, do not model improved ACLR with backoff

Proposed WF: Further discuss the 3 options

| Company | Comment |
| --- | --- |
| QCOM | We prefer option 1. Option 2 is also acceptable. For Option 3 we can compromise as for outdoor deployments preliminary results show we expect UE-UE CLI to be insignificant. We want study if this holds for indoor deployments. |
| vivo | In the adjacent channel co-existence modelling, do we need to consider UE output power less than UE power class? |
| Ericsson | If we take the specification as a baseline then it is difficult to assume anything other than ACLR. Our analysis shows that the UE-UE CLI will not be an issue even if we assume ACLR even at lower power. But not a strong view. |
| Huawei | No strong view. |
| MediaTek | We should take existing spec as baseline – option 3. |
| Samsung | Option 1 is okay to us. |
| Company |  |

Moderator to vivo: My understanding based on inital results I have seen is that the UE is often quite backed off.

Agreement:

- Option 3 – Do not model improved ACLR with backoff

- Revisit the discussion on backoff-dependent ACLR if UE-UE CLI becomes significant

### UE TX aggressor toward adjacent channel victim (FR2-1)

Should the model use ACLR or OBW as the base value?

Option 1: Use ACLR as basis (17 dB)

Option 2: Use OBW as basis (23 dB)

Proposed WF1:  Option 2

| Company | Comment |
| --- | --- |
| QCOM | Agree Option 2. The UE will always comply with the OBW spec. We don’t understand why companies would use 17 dB when it is clear that the UE will meet 23 dB. The fact that OBW is the limiting spec and ACLR is no is a common understanding in RAN4. |
| vivo | We prefer Option 1. |
| Huawei | Option 2 |
| Samsung | Option 2 |
| Company |  |

Agreement: For FR2-1 use OBW as basis (23 dB)

Should the FR2-1 model include an ACLR2-type aspect, similar to FR1?

Option 1: Follow the decision on FR1 about ACLR2. If you support this what would you propose for value?

Option 2: Exclude ACLR2-type aspect even if FR1 implements it

Proposed WF1:  Option 2

| Company | Comment |
| --- | --- |
| QCOM | Our preference is Option 2 however we are flexible on this aspect. if most other companies prefer Option 1 |
| vivo | We prefer Option 2. |
| Ericsson | Similarly to FR1, take option 2; can revisit if a problem is seen for UE-UE CLI, but we do not expect that. |

Agreement: ACLR-2 model aspect is precluded for FR2-1

## Co-channel model

### UE TX aggressor toward co-channel victim (FR1)

ACLR or IBE based model

The options are downselected to the two that received the majority of the support

- Options

  - Option 1: Use the same ACLR-based method as in adjacent channel

  - Option 2: Use the same ACLR-based method as in adjacent channel but only if the scaling with backoff is agreed

  - Option 3 : Use  IBE-based model

Proposed WF: Discuss between the 3 options

| Company | Comment |
| --- | --- |
| QCOM | We prefer option 2 We would be OK with option 3 Option 1 is too pessimistic and will result in more interference than will really be seen. |
| vivo | This is for co-channel case, Option 3 is used. At current stage, it is better to clarify firstly how can ACLR be used to subband level considering legacy UE architecture before we agree on using ACLR-type model for Tx modelling. |
| CATT | Option 3 is still our understanding. |
| Ericsson | We’re OK for option 3 |
| Huawei | Ok with Option 3 |
| MediaTek | Option 3 |
| Samsung | Okay with option 2 and 3. |

  - Agreement  - Option 3 : Use  IBE-based model for co-channel

IF AN IBE-BASED MODEL IS CHOSEN Frequency-domain model  granularity

Option 1: If an IBE-based model is chosen the model granularity is 1 RB.

Option 2: If an IBE-based model is chosen, the model granularity is something wider than 1 RB

Proposed WF: Option 1

| Company | Comment (commenting does not necessarily mean you are in favor of IBE-based) |
| --- | --- |
| QCOM | Option 1 is our preference. This is best for handling guard band in model. |
| Vivo | The UE IBE definition implies Option 1. UE IBE=the UE output power in a non–allocated RBthe UE output power in an allocated RB |
| CATT | The similar with current specification may be ok. So option 1. |
| Ericsson | Option 1 is fine to calculate the interference level. To simplify in simulaitons, companies might want to calculate the whole interference over the DL part, based on IBE. Maybe no need to agree whether to do that or not though. |
| MediaTek | Option 1 is fine |
| Samsung | The proposed WF (option 1 if IBE-based) is okay for us. |

Agreement: Option 1: IBE-based model granularity is 1 RB.

IBE-based model inclusion of image and LO location assumption

Option 1: If an IBE-based model is chosen it should Include the image aspect of IBE and assume the LO is in the middle of the channel to allow for correct placement of the image frequency.

Option 2: If an IBE-based model is chosen  Exclude the image for simplicity

Proposed WF: Option 1

| Company | Comment (commenting does not necessarily mean you are in favor of IBE-based) |
| --- | --- |
| QCOM | Option 1 is our preference. Image is important as is the LO location. |
| Vivo | We prefer Option 1. |
| Ericsson | Option 1 is OK |
| MediaTek | Option 1 is ok, but probably can highlight what happens in other configs qualitatively |
| Samsung | Option 1 is okay |

Agreement: The IBE-based model should Include the image aspect of IBE and assume the LO is in the middle of the channel to allow for correct placement of the image frequency.

### UE TX aggressor  toward co-channel victim (FR2-1)

Proposal 1 ; Use the same approach as in adjacent channel aggressor model for FR2-1

Proposed WF: Agree proposal 1

| Company | Comment |
| --- | --- |
| QCOM | Agree with the WF. There is no need to use something else. |
| Vivo | Proposed WF is OK. |

Agreement: Use the same approach as in adjacent channel aggressor model for FR2-1

# UE RX modeling aspects

## Adjacent channel model

### UE RX victim from adjacent channel aggressor (FR1)

ACS value as one performance point in the model

Proposed WF: agree 33 Db value as performance point in the model (based on comments in round 1)

| Company | Comment |
| --- | --- |
| QCOM | Agree WF |
| vivo | The proposed WF is OK. |
| Ericsson | Agree |
| Huawei | Agree |
| Samsung | Agree with the proposed WF. |

Agreement: agree 33 dB value (33 dB comes from ACS) as performance point in the RX model

RX model with adjacent channel blocker over the RX dynamic range

- Proposals:

  - Proposal 1 (Samsung): To align with legacy CLI and co-existence study, no receiver check point needed for adjacent channel UE-UE CLI model.

  - Proposal 2 (Apple): For UE-UE adjacent-channel inter-subband CLI modelling of RX selectivity/blocking, use the following model:

    - If the blocker is higher than -25dBm, it is assumed it will result large receiver degradation and hence the RX will not correctly decode the data (100% packet loss)

    - For the blocker that is smaller than -25dBm, use the ACS values to calculate the resulting interference

    - In addition, consider a 5dB SNR degradation due to receiver gain backoff

    - Per RB granularity is not considered.

- Proposed WF

  - Agree proposal 2.1

  - Further discuss the rest of proposal 2 as a vehicle to facilitate discussion

| Company | Comment |
| --- | --- |
| QCOM | We can agree with proposal 2.1. We need to develop a model for the dynamic range which includes - Behavior for interferer power from no power to interferer the ACS interferer power level - Behavior from the ACS interferer power level to the maximum input power (-25) - Behavior above maximum input power (Apple Proposal 2.1) We are not sure we fully understand proposals 2.2 and 2.3 in detail and how they would be applied. From what we can see they are likely too conservative for a model. On the granularity of the model, 2.4, we understand this to mean that the interferer RB allocation does not matter in the model. Is that the meaning? |
| vivo | The proposed WF is OK. Proposal 2.1 2.2 2.4 are OK for us. |
| CATT | For adjacent channel model, does this mean the conclusion will be used in co-exist simulation? Our understanding is that current simulation only considers ACIR. If NF needs to be taken into account, then NF+ACS is the correct approach. NF may not be only two step value, i.e. 9 dB and (9+5) dB. |
| Ericsson | We are OK with 2.1-2.4. One clarification for 2.3 though; is this 5dB gain backoff applied only when the blocker power is near -25dBm, or for any blocker power level ? |
| Huawei | We are ok with 2.1 and 2.4, further discuss on 2.2 and 2.3. |
| Samsung | Proposal 2.1 and 2.2 seems okay. |
| Apple | To QC: Proposal 2.2 says if the blocker power P_blocker is no greater than -25dBm, the interference to the victim UE due to the blocker is P_blocker – ACS. Proposal 2.3 means that in addition to the interference calculated following proposal 2.2, the UE receiver noise figure will increase as a result of RX gain backoff (or reduction). To account for this effect, we assume the previously calculated SNR is further reduced by 5dB. We are open to discussing if 5dB is agreeable. To Ericsson: usually gain backoff increases when the blocker power increases. We propose 5dB as an average value, meaning it applies to any blocker level. |

Agreement: proposal 2.1 If the blocker is higher than -25dBm, it is assumed it will result large receiver degradation and hence the RX will not correctly decode the data (100% packet loss)

### UE RX victim from adjacent channel aggressor (FR2-1)

ACS value as one performance point in the model

Proposed WF: agree 23 dB value as performance point in the model (based on comments in round 1)

| Company | Comment |
| --- | --- |
| QCOM | Agree WF |
| Company | The proposed WF is OK. |
| Ericsson | OK |
| Huawei | Ok |
| Samsung | Okay |

Agreement: agree 23 dB value (from ACS) as performance point in the FR2-1 model

RX model with adjacent channel blocker over the RX dynamic range

Proposed WF : Use the same method as in FR1, with changes being related to the parameters of ACS value, REFSENS, and maximum input power level

| Company | Comment |
| --- | --- |
| QCOM | Agree WF |
| Company | The proposed WF is OK. |
| Ericsson | OK |
| Huawei | Ok |

Agreement: For FR2-1 Use the same method as in FR1, with changes being related to the parameters of ACS value, REFSENS, and maximum input power level

## Co-channel model

### UE RX victim from co-channel aggressor (FR1)

Receiver sub-band selectivity

How much sub-band filtering/ selectivity should be in the RX model?

Option 1: 0 dB

Option 2: Something based on 33 dB FR1 ACS but the details are not clear

Option 3: Typical performance model

Proposed WF: Option 1

| Company | Comment |
| --- | --- |
| QCOM | Option 1: 0 dB Legacy UEs are the assumption and there is no sub-band filtering existing in those. Option 2 does not make sense. ACS is based on channel selectivity which does not exist for a sub-band. |
| vivo | If the subband is configured as UE CHBW, then ACS can be considered instead of 0dB selectivity. |
| CATT | As discussed in our contribution R4-2215384, if Rx filter is wide, then big FFT size will be used. But here still some noise will go to the wanted signal because of image degradataion. That’s why we think -25 dBc is a reasonable performance can be assumed. 61.44 MHz FFT size 122.88 MHz FFT size ACS ICS 50MHz Rx digital filter 50MHz Rx digital filter Small FFT implementation (b) Big FFT implementation For option 1: 0 dB, how to use it need to be clarified. Does it mean no rejection? The ACS rejection understanding is that (interference level) – (ACS level) = (the noise level fall in the wanted signal spectrum.) So here 0 dB means no power falls to the wanted signal BW or all of the power fall into? To our understanding, neither is correct. So the question is vague, sub-filter selectivity is not always equal to the inter-subband selectivity according to the different implementations. |
| Ericsson | Similar to CATT, we don’t fully understand the definition of the 0dB. ACS refers to the ratio of the RX power in the adjacent channel to the power in the wanted channel. So here would this mean that whatever power there is in the UL part would be experienced by the victim UE in the DL part ? That seems very pessimistic ? |
| Huawei | ICS can be used for in-band case. |
| MediaTek | Agree with Qualcomm, and feel that FFT rejection would be the only level of rejection that is valid, but needs to consider non-orthogonality. Please can somebody define what they mean by ICS if not that? |
| Samsung | Agree with CATT’s analysis to have 25dBc as assumption here for future feasibility checking. |
| Apple | Our understanding of option 1 is there is no rejection of the interference coming from the adjacent sub-band. In other words, if a UE transmits in the adjacent sub-band with power P, the victim will see interference I (=P). It is reasonable to assume there is no sub-band filtering for UE. |

Configuring the UE channel bandwidth to be equal to a sub-band for selectivity

Can the UE channel bandwidth be configured to equal the sub-band BW for SBFD operation?

Option 1: No

Option 2: Yes

Proposed WF: Option 1

| Company | Comment |
| --- | --- |
| QCOM | The WF Option 1. Option 2 is not feasible for a legacy UE. For Option 2 First a legacy UE would not know how to do this. Second this would erase the latency improvement motivating SBFD due to having to reconfigure the UE BW. Option 2 does not make sense to us. |
| vivo | From the current spec, UE channel bandwidth can be configured as less than BS channel bandwidth as long as it is within the BS CHBW. Therefore, it can be configured to equal the sub-band BW for SBFD operation from the spec perspective. It is better to be clarified by network vendors whether this configuration is possible in current network. |
| mediaTek | Option 2 is not feasible for legacy UE, as it would mean 3 cells per BS channel BW, and multiple SSBs. |
| Samsung | Need to discussion more on this issue, and seems it is not a case UE can’t be configured. Whether or not UE should be configured as this is another story. |

Effect of power contained in uplink sub-band on receiver model (blocker)

Proposal 1: This model is proposed

  - x axis is total power in the channel at the receiver input, so signal + any uplink jammer or blocker power. Let’s call it Pin.

  - y axis is the ratio of total input channel power to noise, so it is Pin/noise power

  - The receiver performance breaks down above the maximum input power level, so the receiver would not be able to demodulate the signal at all in this regime

Proposed WF: Use this model as a working assumption

Option 1: Yes

Option 2: No and possible alternatives

Proposed WF: Option 1

| Company | Comment |
| --- | --- |
| QCOM | The WF Option 1. If some simplification is desired it might be convenient to replace the sawtooth portion showing the gain state changes to some flat line at say 40 dB. |
| vivo | Option 2. For now, we are only considering maximum input level for Rx model. |
| CATT | Option 1 may be a good starting point. One clarification question, what’s the NF for the different ranges? It seems 10 dB increase for each range, then finally 40 dB increase for R+76 level, It’s different with Apple’s proposal: all 5 dB increase for signal level below -25 dBm |
| MediaTek | I wonder where the values came from. These seem very implementation specific, so not sure how we would conclude on those. Although we agree with the concept of the model itself. Also I would appreciate an answer on how interference is affecting the SNR when added. |
| Samsung | Thanks for QC’s carination, as below, option 1 is okay to be used as working assumption. |
| Apple | The proposed model needs further clarification. Is it based one particular AGC implementation or based on typical AGC implementation that most UEs use? |

### UE RX victim from co-channel aggressor (FR2-1)

Proposal 1 ; Use the same method as in adjacent channel victim for FR1

Proposed WF: Agree proposal 1

| Company | Comment |
| --- | --- |
| QCOM | Agree with WF |
| vivo | similar method as co-channel Rx victim for FR1. |
| Ericsson | Agree |
| Samsung | Agree |

Agreement: For FR2-1 use the same method as in co-channel RX victim for FR1. Note that the co-channel RX victim method for FR1 has not been agreed yet.