3GPP TSG-RAN WG4 Meeting # 104-bis-e												    R4-2217467

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			6.22.5

Source:	Moderator (THALES)

Title:	Way Forward on NR NTN Enhancements Part1

Document for:	Agreement

# Introduction

This proposed way forward is based on the outcomes of “Email discussion summary for [104-bis-e][312] NTN_enh_Part1”, see R4-2217496 (revision of R4-2216896). Moreover, as suggested by “RAN4#104-bis-e E-meeting Arrangements and Guidelines”, this Way Forward uses WORD document rather than POWERPOINT (.PPTX) in order to facilitate others to comment and easily track the changes.

As described in latest Tdoc version R4-2217496, the Email discussion summary for [104-bis-e][312] NTN_enh_Part1 identified the following topics and issues:

- Topic #1: NTN system parameters

Sub-topic 1-1: Terminal types

  - Issue 1-1-1: VSAT and/or ESIM (see Huawei/R4-2216076/P1, Ericsson/R4-2216515/P2 & P3)

  - Issue 1-1-2: Power class (see Xiaomi/R4-2216148/P2)

Sub-topic 1-2: Orbit types

- Issue 1-2-1: GSO, NGSO (see Huawei/R4-2216076/P2, Ericsson/R4-2216515/P1, ZTE/R4-2216558/P2)

Sub-topic 1-3: Frequency range

- Issue 1-3-1: Downlink frequency range (see Huawei/R4-2216076/P3, Nokia/R4-2216372/P3, ZTE/R4-2216556/P1, Ericsson/R4-2216515/P4)

- Issue 1-3-2: Uplink frequency range (see Huawei/R4-2216076/P3, ZTE/R4-2216556/P1, Ericsson/R4-2216515/P4, Verizon/R4-2215775/P2&P3&P4&P5&P6)

- Issue 1-3-3: handling of between 10 and 24.250 GHz (see Nokia/R4-2216372/P1, Qualcomm/R4-2216651/P1)

Sub-topic 1-4: Waveform parameters

- Issue 1-4-1: channel bandwidth (see Ericsson/R4-2216516/P1 & P3, ZTE/R4-2216556/P2 & P3, Qualcomm/R4-2216651/P2 & P3)

- Issue 1-4-2: sub-carrier spacing (see Ericsson/R4-2216516/P2, P4 & P5, ZTE/R4-2216556/P2)

- Issue 1-4-3: Spectral utilization requirement (see ZTE/R4-2216556/P4)

- Issue 1-4-4: Channel raster (see ZTE/R4-2216556/P5, Qualcomm/R4-2216651/P4)

- Issue 1-4-5: Tx-Rx separation values (see Qualcomm/R4-2216651/P5)

Sub-topic 1-5: band definition

- Issue 1-5-1: band numbering (see Xiaomi/R4-2216148/P3 and Nokia/R4-2216372/P2)

- Issue 1-5-2: specific recommendations (see Verizon/R4-2215775/P7&P8)

  - Issue 1-5-3: Handling of Region/country specific regulatory requirements (see Ericsson/R4-2216515/P5, Verizon/R4-2215775/P1&P6)

- Topic #2: SAN RF aspects

Sub-topic 2-1: SAN RF Requirements

- Issue 2-1-1: Radiated (see Xiaomi/R4-2216148/P1)

- Issue 2-1-2: type 2-O (see ZTE/R4-2216558/P1, CATT/R4-2215415/O2)

- Issue 2-1-3: SAN RF parameters (see ZTE/R4-2216558/P3, CATT/R4-2215415/O3)

Sub-topic 2-2: NR NTN enhancement workplan

  - Issue 2-2-1: Work plan

# Agreements

1. Topic #1: NTN system parameters

Sub-topic 1-1: Terminal types

Issue 1-1-1: VSAT and/or ESIM

Issue 1-1-2: Power class

Sub-topic 1-2: Orbit types

Issue 1-2-1: GSO, NGSO

The following proposals are agreed.

Proposal 1-2-1-1: RAN4 shall prioritize GSO, NGSO (LEO-600 km and LEO-1200 km) orbit.

Proposal 1-2-1-2: RAN4 should consider MEO and HEO if time allows.

Sub-topic 1-3: Frequency range

[see GTW Agreement 18/10/2022]

Following options can be considered as starting point for future RAN4 meetings for above 10GHz Band introduction

- Option 1: the downlink frequency range 17.7-20.2 GHz, the uplink frequency range 27.5-30.0 GHz for NTN Ka-band definition (i.e. spectrum allocated by ITU to FSS).

- Option 2: the downlink frequency range 19.7-20.2 GHz, the uplink frequency range 29.5-30.0 GHz for NTN Ka-band definition (i.e. spectrum allocated by ITU to MSS and FSS).

- Option 3: the downlink frequency range 17.3-17.7 GHz, the uplink frequency range 29.5-30.0 GHz for NTN Ka-band definition (the DL part is out of frequency range of current harmonized Ka band)

- Option 4: define multi bands in this frequency range i.e. one band as option 1 and another band as option 2; and/or another band as option 3

- Other options not precluded

Sub-topic 1-4: Waveform parameters

Issue 1-4-1: channel bandwidth

The following proposal is agreed.  Optional higher channel bandwidths and asymmetric channel bandwidths between DL and UL require further discussion.

Proposal 1-4-1-1: For NTN operation above 10 GHz, at least the channel bandwidths 50, 100 and 200 MHz are supported.

Note: 400 MHz can be considered as Optional or FFS.

Proposal 1-4-1-2: RAN4 to further discuss whether:

          - 1) higher channel bandwidths (e.g. 400 MHz and beyond) are supported (with 120 kHz SCS)

          - 2) Asymmetric channel bandwidths between DL and UL of 50+100, 50+200, and 50+400 MHz are considered for the Ka-band.

Issue 1-4-2: sub-carrier spacing

The following proposal is agreed.

Proposal 1-4-2-1: For NTN operation above 10 GHz, 60 kHz and 120 kHz Sub-Carrier Spacing (SCS) are supported.

Issue 1-4-3: Spectral utilization requirement

Further discussion is needed.

Issue 1-4-4: Channel numbering

The following proposals are agreed.

Proposal 1-4-4-1: For NTN operation in above 10 GHz, use the same channel raster for NTN Ka-band and the NR-ARFCN in the Table 2.4.1 and GSCN in Table 2.4.2.

Table 2.4.1. NR-ARFCN for NTN Ka-band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
|  | 60 | TBD – <1> – TBD |
|  | 120 | TBD – <2> – TBD |
| Note: Table adopted for FDD |  |  |

Table 2.4.2. GSCN for NTN Ka-band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
|  | 120 kHz | Case D | TBD- <1> - TBD |
|  | 240 kHz | Case E | TBD - <2> - TBD |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. |  |  |  |

Note: Addressable range of ARFCN and GSCN to be discussed e.g. if extended.

Proposal 1-4-4-2: For NTN operation in above 10 GHz, instead of [n1024] numbering use [n512] as starting number for the first introduced band.

Note: It is understood that the NTN band number in above 10 GHz will decrease with new NTN band introductions (as previously done for e.g. n256 and n255 in FR1).

Issue 1-4-5: Tx-Rx separation values

Further discussion is needed

Sub-topic 1-5: band definition

Issue 1-5-1: band numbering

Further discussion is needed.

Issue 1-5-2: specific recommendations

The following proposals are agreed.

Proposal 1-5-2-1: RAN4 to follow for NTN above 10 GHz bands the usual 3GPP process adopted for adjacent band coexistence.

Proposal 1-5-2-2: The recommendations from ITU, WRC and regional entities (FCC, CEPT, etc) have to be considered, as it was done for example for NTN FR1 or any other RAN4 band definition.

Issue 1-5-3: Handling of Region/country specific regulatory requirements

Further discussion is needed.

2. Topic #2: SAN RF aspects

Sub-topic 2-1: SAN RF Requirements

Issue 2-1-1: Radiated

The following proposal is agreed (see GTW Agreement 18/10/2022).

Proposal 2-1-1-1: As starting point, at least SAN radiated requirements shall be defined for NTN Ka-band in Rel-18. FFS whether conductive requirements shall be defined or not

Issue 2-1-2: type 2-O

Further discussion is needed.

Issue 2-1-3: SAN RF parameters

The following parameters can be considered as starting point for Tx and Rx Requirements.

Proposal 2-1-3-1: Further discussion is needed on whether all of the following requirements need to be specified.

- SAN output power

- Output power dynamics

- RE power control dynamic range

- Total power dynamic range

- Transmit ON/OFF power

- Transmitted signal quality

- Frequency error

- Modulation quality

- Time alignment error

- Unwanted emissions

- Occupied bandwidth

- Adjacent Channel Leakage Power Ratio

- Operating band unwanted emissions

- Transmitter spurious emissions

- Transmitter intermodulation

Proposal 2-1-3-2: Further discussion is needed on whether all of the following requirements need to be specified.

- Reference sensitivity level

- Dynamic range

- ACS

- In-band blocking

- Out of band blocking

- Receiver spurious emission

- Receiver intermodulation

- In-channel selectivity