3GPP TSG-RAN WG4 Meeting # 104-bis-e 									    		         R4-2217469

Electronic Meeting, 10 - 19 October, 2022

Agenda item:			6.22.2

Source:	Samsung

Title:	Simulation assumptions for NTN co-existence study in bands above 10GHz

Document for:	Approval

- Introduction

This document captures initial simulation assumptions for the NTN coexistence study in frequency bands above 10 GHz.

Remaining issues for further discussion are highlighted in yellow mark with square brackets.

- Discussion

Co-existence simulation scenarios

[It is proposed to have a phase-by-phase approach to conduct co-existence study considering scenarios.]

The proposed scenarios for coexistence study are in the following table.

Table 2.1-1 Scenarios for NTN-NTN/TN co-existence

| 30GHz(UL)/20GHz(DL) |  |  | Set 1 |  |  | Set 22 |  |  | HAPS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GEO | LEO 600km | LEO 1200km5 | GEO | LEO 600km | LEO 1200km |  |
| NR | Rural |  | [TBD] | [TBD] | [TBD] | N/A | N/A | N/A | N/A |
|  | Urban macro |  | X | X | X | N/A | N/A | N/A | N/A |
|  | Dense Urban6 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NTN1,4 | GEO3 | Set 1 | [TBD] | [TBD] | [TBD] | N/A | N/A | N/A | N/A |
|  | LEO 1200km |  | [TBD] | [TBD] | [TBD] | N/A | N/A | N/A | N/A |
|  | LEO 600km |  | [TBD] | [TBD] | [TBD] | N/A | N/A | N/A | N/A |
|  | GEO | Set 2 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | LEO 1200km |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | LEO 600km |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| HAPS |  |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Note 1: Start with Earth Fixed beam first, Earth Moving Beams could be further discussed Note 3: GEO and LEO only operate at adjacent channel. [Note 4: Use GEO and LEO@600km when TN is victim. Note 5: Further check the possibility to remove LEO 1200km cases in future RAN4 meetings.] Note 6: Dense Urban is not considered as it is expected NTN UE will connect to terrestrial networks rather than satellite networks in such scenario. |  |  |  |  |  |  |  |  |  |

The aggressor and victim combination is list in Table 2.1-2.

Table 2.1-2 Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| [1 | TN with NTN | NTN UL | TN UL | Applicable for n257, n258 and n261 |  |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for n257, n258 and n261 |  |
| 3 | TN with NTN | NTN UL | TN DL | Applicable for n257, n258 and n261 |  |
| 4 | TN with NTN | TN DL | NTN UL | Applicable for n257, n258 and n261 |  |
| 5 | TN with NTN | TN DL | NTN DL | FFS |  |
| 6 | TN with NTN | NTN DL | TN DL | FFS |  |
| 7 | TN with NTN | NTN DL | TN UL |  |  |
| 8 | TN with NTN | TN UL | NTN DL |  |  |
| 7 | NTN with NTN1 | NTN DL | NTN DL | LEO-LEO |  |
|  |  |  |  | GEO-GEO |  |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS |  |
|  |  | NTN UL | NTN UL | LEO-LEO |  |
|  |  |  |  | GEO-GEO |  |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS |  |
| Note 1: With respect to the case 7 (for the NTN-NTN scenario), at least for the Satellite Access Node use case the corresponding coexistence simulations have not yet been confirmed by all companies.] |  |  |  |  |  |

The proposed frequency and bandwidth are listed as table 2.1-3.

Table 2.1-3.  Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Urban macro | 30GHz | 200MHz, [TBD] | TDD | 1 |
| [TN Rural macro] | 30GHz | 200MHz, [TBD] | TDD | 1 |
| GEO | 30GHz(UL)/20GHz(DL) | 200MHz, [TBD] | FDD | 1 |
| LEO | 30GHz(UL)/20GHz(DL | 200MHz, [TBD] | FDD | 1 |

Network layout model

Co-existence between NTN and TN

Cellular cell structure is considered for both NTN and TN network layout.

Coordination System

Referring to TR 38.811 Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN beams direction and location on the earth surface. It means the NTN beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z).

Simulation Methodology

Following simulation steps can be used for NTN-TN co-existence study.

- Generate aggressor and victim networks.

- [NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams. NTN FRFs higher than 1 need to be considered. Assume one NTN aggressor as default.

- Deployment of TN network (19 cells with wraparound) refers to Annex 2.]

- UE associations

- TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

- [Deployment of NTN UE refers to Annex 2.]

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering ACI as below:

- ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

For TN-NTN SINR calculation, the satellite receiver off angle should be considered in the satellite receiver gain calculation when calculating SINR. Note that such angle is not considered in TR 38.821 section 6.1.3 equations. Thus those equations should be used for SINR calculation.

- Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

To simplify the simulation of interference from TN to NTN UL in Case 2 and 6, following method can be used. Consider the active TN cells from central NTN beam for the ACI evaluation from TN to NTN UL. The scaling factor is to be discussed and determined if any in next meeting. There is a view that simplifying such coexistence simulation work for Case 2 may even not be required.

- Step 1: to drop NTN UE per beamprint randomly;

- Step 2: to drop N clusters consisting of 57 sectors per beamprint randomly

- Step 3: to calculate the total ACI per beam to NTN UL by following scaling factor:

where:

active_TN = active_factor*round (the area per beam/the area of 57 sectors)

active_factor = 20% (or lower, particularly for urban scenarios)

- Step 4: to calculate the total ACI from all beams (e.g. M=7) for NTN:

Co-existence between NTN and NTN

[The following 2 cases are considered as candidate options and to be further discussed.

- One satellite carries two neighbour carriers, where the footprints of the 2 carriers are the same and coordinated see Figure 2.2-1.

- Two satellites (GEO and LEO) operate on two neighbour carriers but at different height, see Figure 2.2-2. The number of LEO satellite and footprints are FFS.

Figure 2.2-1 Layout for coexistence between NTN systems

Figure 2.2-2 Layout for coexistence between NTN systems (different height satellites)

]

Simulation parameters

NTN parameters

Satellite parameters

Set-1 satellite parameters are shown in Table 2.3.1-2 according to TR 38.821.

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 2.3.1-1 NRB configuration per BandWidth size and SCS

| Configuration for Above 10GHz bands | NRB ([TBD]MHz BW) | NRB ([TBD]MHz BW) | NRB ([TBD]MHz BW) |
| --- | --- | --- | --- |
| SCS [TBD] kHz | [TBD] | [TBD] | [TBD] |
| SCS [TBD] kHz | [TBD] | [TBD] | [TBD] |

Table 2.3.1-2 Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO |  |  |  | LEO-1200 |  |  |  | LEO-600 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  |  |  | 1200 km |  |  |  | 600 km |  |  |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | Ka-band (i.e. 20 GHz for DL) | 40 dBW/MHz |  |  |  | 10 dBW/MHz |  |  |  | 4 dBW/MHz |  |  |  |
| Satellite max TX power in dBm | BW (MHz) |  | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
|  | SCS [TBD]kHz |  | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
|  | SCS [TBD]kHz |  | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Satellite Tx max Gain |  |  | 58.5 dBi |  |  |  | 38.5 dBi |  |  |  | 38.5 dBi |  |  |  |
| Channel bandwidth |  |  | [TBD] |  |  |  | [TBD] |  |  |  | [TBD] |  |  |  |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.1765 deg |  |  |  | 1.7647 deg |  |  |  | 1.7647 deg |  |  |  |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | [TBD] |  |  |  | [TBD] |  |  |  | [TBD] |  |  |  |
| Satellite beam diameter |  |  | 110 km |  |  |  | 40 km |  |  |  | 20 km |  |  |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G/T |  | Ka-band (i.e. 30 GHz for UL) | 28 dB K-1 |  |  |  | 13 dB K-1 |  |  |  | 13 dB K-1 |  |  |  |
| Satellite Rx max Gain |  |  | 58.5 dBi |  |  |  | 38.5 dBi |  |  |  | 38.5 dBi |  |  |  |

Table 2.3-2 Other parameters for NTN

| Parameters | NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 30GHz(UL)/20GHz(DL) |  |
| The number of active UE (UL) | [9 UEs and 2RBs per UE for GEO and LEO1] |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 2.6.2 |  |
| NTN satellite Noise figure in dB | See Table 2.3.1-3 |  |
| Handover margin | 3dB |  |
| Note 1: UEs are equally splitted inside the channel bandwidth into ACIR 3 regions. Scheduled PRB position for UE1 per satellite beam should be also fully aligned to simulate the worst case for co-channel interference and this is also aligned with full buffer case. |  |  |

Table 2.3.1-3 NTN satellite Noise figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 28 | 13 | 13 |
| G_Rx (dBi) | 58.5 | 38.5 | 38.5 |
| NF (dB) | [5.9] | [0.9] | [0.9] |

UE parameters

UE parameters are shown in Table 2.3.1-4

[Table 2.3.1-4 UE characteristics for system level simulations

| Characteristics | VSAT (Note 2) | Other (Note 1) |
| --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear: +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | TBD dBi per element |
| Antenna temperature | 150 K | TBD K |
| Noise figure | 1.2 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | TBD dBi per element |
| UE height | TBD | TBD |

]

TN parameters

Table 2.3.2-1 Simulation assumptions of TN NR

| Carrier frequency in GHz | 30 |  |
| --- | --- | --- |
| Size of each nominal channel BW in MHz | [TBD] |  |
| Transmission bandwidth in MHz | [TBD] |  |
| Environment | Urban macro | [Rural macro] |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance in meter | 200m |  |
| Number of micro BSs per macro cell |  |  |
| System loading and activity | See Annex 2 |  |
| Network location | See Annex 2 |  |
| DL subcarrier spacing | 120kHz |  |
| UL | OFDMA |  |
| DL power control | No |  |
| UL power control | See Section 2.7.1, 2.7.2 |  |
| Frequency reuse | 1 |  |
| Number of scheduled UE per cell (DL) | Same as the number of BS beam |  |
| Number of scheduled UE per cell (UL) | Same as the number of BS beam |  |
| UE antenna height in meter | Same as 3D-UMi in TR 36.873 |  |
| UE TX power in dBm | -40 to 23 |  |
| UE antenna gain in dBi | 0 |  |
| Building penetration loss | In path loss model, TR 38.901 |  |
| Cell selection margin in dB | 3 |  |
| BS-MS min distance in meters | 35 |  |
| BS noise figure in dB | 11 |  |
| UE noise figure in dB | 9 |  |
| BS-UE path-loss model | TR 38.901 |  |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |  |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |  |
| Link-level performance model | See Section 2.10 |  |
| UE distribution | Uniform |  |
| Evaluation metrics | See Section 2.9 |  |

Table 2.3.2-2 Deployment-related parameters of TN (30GHz)

|  | Urban Macro | [Rural macro] | Remarks |  |
| --- | --- | --- | --- | --- |
| ISD in meters | 200 | [FFS] |  |  |
| BS Antenna height in meters | 25 | [FFS] |  |  |
| UE Outdoor/indoor | 100% Outdoor | [FFS] |  |
| UE height in meter | 1.5 | [FFS] |  |  |

Table 2.3.2-3 ACLR/ACS for TN (30GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 28dB |
|  | ACS | 24dB |
| UE | ACLR | 17dB |
|  | ACS | 22dB |

Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811.

Satellite antenna pattern

[The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered

1

where J1(x) is the Bessel function of the first kind and first order with argument x, a is the radius of the antenna's circular aperture, k = 2f/c is the wave number, f is the frequency of operation, c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam. Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

The antenna patterns for LEO 600km, 1200km and GEO are show in Figure 2.4.1-1 and 2.4.1-2.

Figure 2.4.1-1:  Antenna pattern for LEO 600KM and 1200KM (4.4127 deg for 3dB beamwidth)

Figure 2.4.1-2 Antenna pattern for antenna aperture of GEO (0.4011 deg for 3dB beamwidth)]

Satellite and UE beam forming pattern

The following table is agreed for the beam layout definition for a single satellite simulation in S-Band.

Table 2.4.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |  |  |
| --- | --- | --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |  |  |
| UV plane illustration (extracted from [19]) |  |  |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |  |  |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |  |  |
| Central beam bore sight direction definition | Baseline: [Case 1: Central beam center is considered at nadir point Case 2: 45° for GEO and LEO] Interested companies can bring analysis and results for other values. |  |  |
|  | Option 1: FRF=1 | Option 2: FRF=3 | Option 3: FRF=2 |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |  |  |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |  |  |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |  |  |
| UE configuration | Fixed and mobile VSAT for GSO Fixed VSAT for NGSO |  |  |
| UE orientation | Ideal Tracking serving beam |  |  |
| UE attachment | RSRP |  |  |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). |  |  |  |

TN BS and UE antenna and beam forming pattern modelling

BS antenna

Refer to Table 2.4.2-1 with parameters in TR 38.803 as baseline.

Table 2.4.2-1 BS antenna modelling for Urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | [8/5.5] dBi |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Antenna downtilt | 10˚ |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

UE antenna

Refer to TR 38.803.

Table 2.4.2-2 Antenna parameters for 30GHz

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

ACIR model

Refer to TR 38.803 Clause 5.2.6.

Propagation model

Propagation model between NTN SAN and UE

Propagation model between NTN and UE could be referred to section 6.6 in TR 38.811.

- Additional losses between NTN SAN and UE need to be considered as following:

| Atmospheric absorption | Figure 6 of ITU-R P.676 (XX) shows the corresponding zenith attenuation for frequencies between 1 and 350 GHz. For an elevation angle α, the corresponding attenuation is given by: |
| --- | --- |
| Rain and cloud attenuation | For system-level simulations, the baseline is to consider clear sky conditions only. No rain and cloud attenuation should then be considered. |
| Scintillation | The ionospheric scintillation corresponds to rapid fluctuations of the received signal amplitude and phase. Ionosphere propagation shall only be considered for frequencies below 6 GHz. This is not applicable then for coexistence system simulations for the NTN Ka-band. The tropospheric scintillation corresponds to rapid fluctuations of the received signal amplitude and phase. Tropospheric propagation shall only be considered for frequencies above 6 GHz. Table 6.6.6.2.1-1 from TR 38.821 could be used as starting point. |

Propagation model between TN BS and UE

Propagation model between TN BS and UE could be referred to section 7.4 in TR 38.901.

Propagation model between NTN SAN and TN BS

Propagation model between NTN BS and TN BS should reference to TS 38.811 which is used for DL-UL cross link interference for S band.

Propagation model between NTN UE and TN UE

[Propagation model between NTN UE and TN UE should reference to the following propagation model,

- UE-to-UE: Umi (h_BS=1.5 m ~ 22.5 m)

+ penetration loss see TR 38.803]

Transmission power control model

TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

NTN UL TPC

For calibration purpose, reuse the same TN TPC for NTN with SNR target 15dB.

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted but needs to revise CLx-ile to align with UE UL power control parameters used in TR38.821. The CLx-ile value should be adapted for rural scenario.

DL TPC

For downlink scenario, no power control scheme is applied.

Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

Performance metric

For NR,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

For NTN,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics

- Conclusion

It is proposed to use the simulation assumptions in this paper for NTN co-existence study in bands above 10GHz.

- Reference

[1] R4-22xxxx Summary_319_2nd round

Appendix 1. Interference Chart

Coexistence scenarios for NTN-TN scenarios are shown in Figure A.1.

[

Figure A.1: NTN-TN adjacent band coexistence scenarios with TN in FDD mode]

Appendix 2. Consideration of Network and UE depolyment

[Note 1: The NTN UE(s) shall be dropped at the edge of the “central 19 TN cells (cluster)”.] No isolation distance is considered for initial simulation.

[Note 2: Only the TN stations in the “central 19 TN cells (cluster)”, but not the TN stations in ‘wrap-around TN cells’ are to be evaluated.]

[Editor’s Note: the table below needs further discussion.

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | All active TN clusters which has the NTN UE(s) at its edge. |

| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | Consider an active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 3 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values) NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam |  |

| 4 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters | TN randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |
| --- | --- | --- | --- | --- | --- | --- |

| 5 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 6 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

]