3GPP TSG-RAN WG4 Meeting # 104bis-e												R4-2217486

Electronic Meeting, 10– 19 Oct 2022

Agenda item:			4.5.3

Source:	Moderator (Samsung)

Title:	Email discussion summary for [104-bis-e][322] NR_FeMIMO_Demod

Document for:	Information

# Introduction

Rel-17 NR FeMIMO WI is a RAN1 leading WI with below major enhancement in RAN1 area

- Enhancement on multi-beam operation

- Enhancement on multi-TRP

- Enhancement on SRS

- Enhancement on CSI reporting

In the last meeting, the test setup of performance requirement of NR FeMIMO was under discussion.

In practical, the scope of this email discussion is indicated as follows agenda:

- UE Demodulation and CSI requirements (4.5.3)

- Demodulation requirement (4.5.3.1)

- Enhancement on HST-SFN scenario (4.5.3.1.1)

- Enhancement on Multi-TRP (4.5.3.1.2)

- CSI requirement (4.5.3.2)

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Finalize requirement of UE demodulation and CSI parts test cases and related CR updated

- 2nd round: Finalize requirement of UE demodulation and CSI parts test cases and related CR.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Samsung (Moderator) | Yunchuan Yang | yc0301.yang@samsung.com |
| Ericsson | Jiakai Shi | Jiakai.shi@ericsson.com |
| Samsung | Lili Wang | lili008.wang@samsung.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: Demodulation requirement for Multi-TRP enhancement

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215538 | Nokia, NSB | Draft CR for 38.101-4 FeMIMO Applicability of Requirements |
| R4-2215597 | Apple | Proposal 1 Define the requirements with an extra margin based on the span in results due to different receiver assumption. |
| R4-2215874 | Qualcomm | Simulation results on multi-TRP PDCCH tests |
| R4-2215876 | Qualcomm | Observation 1: The performance gap owing to different receiver assumptions is around 1.6dB. Observation 2: Usually 0.5 dB margin is added to the impairment results, but there is still more than 1dB performance gap. Proposal 1: Option 2 ([1] dB) |
| R4-2215939 | MTK | Proposal 1: We support to define PDCCH requirement for multi-TRP repetition transmission scheme with high enough additional margin to support all UE implementation assumptions. |
| R4-2216005 | Huawei, HiSilicon | Proposal 1: The decision of how much margin to be added should be based on the latest simulation results provided by companies in this meeting. No addition margin should be considered if the simulation results are well aligned. |
| R4-2216006 | Huawei, HiSilicon | Simulation results on PDCCH demodulation mTRP |
| R4-2216391 | Ericsson | Observation 1: For FDD, 2Rx, the performance gap so far between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.33dB. Observation 2: For FDD, 4Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.17dB. Observation 3: For TDD, 2Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.75dB. Observation 4: For TDD, 4Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.33dB. Proposal 1: More results from different companies are needed. Consider an additional 1dB margin to be added on top of alignment results if the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ is larger than 3dB but less than 4dB. |
| R4-2216392 | Ericsson | Simulation results for PDCCH enhancement for Multi-TRP |
| R4-2216393 | Ericsson | Draft CR to 38.101-4: PDCCH requirement with inter-slot repetition |
| R4-2216790 | Qualcomm | Draft CR on reference measurement channels for multi-TRP PDCCH performance |

## Open issues summary

Last RAN4 meeting agreements in the WF R4-2214397

List of open issues

- Sub-topic 1-1: PDCCH requirement for Enhancement on Multi-TRP

  - Issue 1-1-1: additional margin for PDCCH requirement for multi-TRP repetition transmission

### Sub-topic 1-1: PDCCH requirement for Enhancement on Multi-TRP

Issue 1-1-1: additional margin requirement for multi-TRP repetition transmission

- Observations

  - Observation 1 (Qualcomm):

    - The performance gap owing to different receiver assumptions is around 1.6dB

    - Usually, 0.5 dB margin is added to the impairment results, but there is still more than 1dB performance gap.

  - Observation 2(Ericsson)

    - For FDD, 2Rx, the performance gap so far between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.33dB.

    - For FDD, 4Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.17dB.

    - For TDD, 2Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.75dB.

    - For TDD, 4Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.33dB.

- Proposals

  - Option 1 (Apple):

    - Define the requirements with an extra margin based on the span in results due to different receiver assumption.

  - Option 2 (Qualcomm, Ericsson, MTK, Apple): 1 dB

    - Ericsson: More results from different companies are needed. Consider an additional 1dB margin to be added on top of alignment results if the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ is larger than 3dB but less than 4dB.

    - MTK: We support to define PDCCH requirement for multi-TRP repetition transmission scheme with high enough additional margin to support all UE implementation assumptions.

  - Option 3 (Huawei):

    - The decision of how much margin to be added should be based on the latest simulation results provided by companies in this meeting. No addition margin should be considered if the simulation results are well aligned.

- Recommended WF

  - Agreement in the last meeting

    - Specify requirements without specific receiver assumption, additional margin will be included:

    - Additional margin

      - Option 1: [0.8] dB

      - Option 2: [1] dB

      - Other options not precluded

  - Simulation results summary

    - Companies are encouraged to check the simulation results, where scaling factor for PDCCH is considered?

    - Results from four companies with soft-combining are well aligned within 2dB gap for soft-combining

    - Results from three companies without soft-combining are aligned within 2.5dB gap for soft-combining

    - Around 3.5dB performance gap between the best result (with soft-combing) and worst results (without soft-combining), additional 1 dB margin can be considered on top of 0.5dB margin for impairment results derivation

    - Based on simulation results, encourage companies can check whether option 2 can be agreeable to move forward

    - Method 1: Check the available results across with and without soft-combing

| Duplex | Tx/RX | HW （w） | MTK wo | MTK w | Apple (w) | QC(wo) | E/// (w) | E/// (wo) | Gap (w) | Gap (w0) | Gap(w wo) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 2x2 | -1.71 | 1.03 | -0.59 | -1.4 | 0.32 | -2.3 | -0.8 | 1.71 | 1.83 | -3.33 |
| FDD | 2x4 | -5.78 | -2.93 | -4.50 | -5.2 | -3.52 | -6.1 | -4.6 | 1.6 | 1.67 | -3.17 |
| TDD | 2x2 | -1.37 | 1.15 | -0.43 | -1.2 | 1.48 | -2.3 | -0.9 | 1.87 | 2.38 | -3.78 |
| TDD | 2x4 | -5.73 | -2.89 | -4.40 | -4.8 | -2.95 | -6.2 | -5.0 | 1.8 | 2.11 | -3.31 |

    - Method 2: Check results without soft-combing if companies provide results for both with and without soft-combing

| Qualcomm | Huawei | Ericsson | MTK | Apple | Average | Span | STD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.32 | -1.71 | -0.80 | 1.03 | -1.40 | -0.51 | 2.74 | 1.04 |
| -3.52 | -5.78 | -4.60 | -2.93 | -5.20 | -4.41 | 2.85 | 1.05 |
| 1.48 | -1.37 | -0.90 | 1.15 | -1.20 | -0.17 | 2.85 | 1.22 |
| -2.95 | -5.73 | -5.00 | -2.89 | -4.80 | -4.27 | 2.84 | 1.15 |

    - Additional margin

      - Option 1 (MTK, Samsung, Ericsson, Apple?): [1] dB based on method 1

      - Option 2 (Huawei, Nokia): [0.5] dB based on method 2

      - Option 3 (QC): [2] dB

  - Recommended WF:

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1 |
| Ericsson | We are fine with the recommended WF. |
| Samsung | Issue1-1-1 We are fine with the recommended WF. Btw, about the simulation summary table above, for case TDD 2x4 of QC(wo), it seems -0.22 should be corrected to 0.22. |
| Apple | Based on last meetings simulation alignment, we had a < 3dB span in results across both with and without soft combining. Now we have a much larger span. Could companies please check if they have considered 1/sqrt(2) scaling per TRP in their simulations? In our opinion, we should look at the span in results across all cases (w/ and w/o soft combining) and decide the additional margin. |
| Qualcomm | Issue1-1-1 Given the span of latest results (hovering around 3dB), which seems much larger than that of the previous meeting (was around 1.6 dB), in our view more than 1dB additional margin should be considered here. Since the decision was to define requirement with receive agnostic implementation, we think sufficient additional margin should be added to compensate for the implementation specific performance gap. We suggest adding 2dB additional margin. To all: We have updated our results, which is captured in the above table under the recommended WF. Apologies for the confusion. |
| Mediatek | Issue 1-1-1 We had accidentally included additional 1/sqrt(2) scaling in our results. This error is now fixed in the latest simulation results collection. With the fixed results from MTK and QC the span seems to be between 3.2 to 3.8 dB. As we have agreed to define requirements to support all UE implementation assumptions, we need to select high enough additional margin. Ideal additional margin would be close to half of the span. However, we support Option 2 with 1dB additional margin, that we believe is just enough, and we consider a good compromise. |
| Huawei | Issue 1-1-1 We have agreed to define performance requirements with receiver agnostic and interesting companies can provide results based on their preference without receiver clarification in last meeting, we are curious which results are their preference for companies that submitted both results. The main concern from some companies are to define the requirements with soft-combing that may put strict requirement for some UEs, so we can use the results without soft-combing if companies provide results for both with and without soft-combing, then we can get the following results: Qualcomm; Huawei; Ericsson; MTK; Apple; Average; Span; STD. 0.32; -1.71; -0.80; 1.03; -1.40; -0.51; 2.74; 1.04. -3.52; -5.78; -4.60; -2.93; -5.20; -4.41; 2.85; 1.05. 1.48; -1.37; -0.90; 1.15; -1.20; -0.17; 2.85; 1.22. -2.95; -5.73; -5.00; -2.89; -4.80; -4.27; 2.84; 1.15. The agreed span to derive final performance requirements from Rel-15 is 2.5dB, now the span is a little larger than 2.5dB, we think that additional 0.5dB margin is enough. |
| Nokia, Nokia Shanghai Bell | Issue 1-1-1 We agree with Huawei's proposal. It is a reasonable compromise to use the w/o soft combining results, in cases where both w/o and w results are available, and in exchange use an additional margin of 0.5dB. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216393 (Ericsson, draft CR to 38.101-4: PDCCH requirement with inter-slot repetition) | Ericsson: The reference channel for TDD need to be aligned with R4-2216790. |
|  | Apple: Suggest adding all the relevant parameters for mTRP transmission in the test parameters table instead of referencing another test parameter table. Also, some indication is needed that PDCCH is transmitted from both TRP – Modify note 2 of performance requirements table as – Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Suggest removing Note 2 in header row of table. |
|  | Nokia: Some elements in the tables does not use the correct styles (TAC/TAH). Neither are the headings. Following should be changed if to fully follow the MCC guidelines: - New sections should be named with .X at the end and table number should be updated accordingly. |
|  | Huawei: It should be intra-slot PDCCH repetition cases and we are defining FDM cases. |
| R4-2216790 (Qualcomm, Draft CR on reference measurement channels for multi-TRP PDCCH performance) | Nokia: Some elements in the tables does not use the correct styles (TAC/TAH). Following should be changed if to fully follow the MCC guidelines: - New table should be named A.3.3.2.2-X. - New reference channel should be named R.PDCCH.2-X.1 TDD For all above we do not see the need to ask for a revision if no other issues are found. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: Issue 1-1-1: additional margin requirement for multi-TRP repetition transmission GTW agreement - Choose the results to derive the requirements based on the companies’ declaration of which results need to be taken into account when multiple results submitted from companies - The additional margin can be further discussed based the collected results Recommendations for 2nd round: - Simulation results collection based on companies’ declaration for requirement derived. - The span for ideal results for PDCCH is 2.5dB - Additional margin can be considered as the [average span -2.5dB] = [0.9]. Encourage companies to check whether [0.9] dB is agreeable? Qualcomm (WO)-; Huawei (W); Ericsson; MTK; Apple; Span; STD. 0.32; -1.71; -2.3; 1.03; -1.40; 3.33; 1.27. -3.52; -5.78; -6.1; -2.93; -5.20; 3.17; 1.26. 1.48; -1.37; -2.3; 1.15; -1.20; 3.78; 1.49. -2.95; -5.73; -6.2; -2.89; -4.80; 3.31; 1.38. Average: 3.4. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Issue 1-1-1: additional margin requirement for multi-TRP repetition transmission

GTW agreement

  - Choose the results to derive the requirements based on the companies’ declaration of which results need to be taken into account when multiple results submitted from companies

  - The additional margin can be further discussed based the collected results

Recommendations for 2nd round:

  - Simulation results collection based on companies’ declaration for requirement derived.

  - The span for ideal results for PDCCH is 2.5dB

  - Additional margin can be calculated as the [average span -2.5dB] = [0.9]. Encourage companies to check whether [0.9] dB is agreeable?

| Qualcomm (WO)- | Huawei (W) | Ericsson (W) | MTK (WO) | Apple(W) | Span | STD |
| --- | --- | --- | --- | --- | --- | --- |
| 0.32 | -1.71 | -2.3 | 1.03 | -1.40 | 3.33 | 1.27 |
| -3.52 | -5.78 | -6.1 | -2.93 | -5.20 | 3.17 | 1.26 |
| 1.48 | -1.37 | -2.3 | 1.15 | -1.20 | 3.78 | 1.49 |
| -2.95 | -5.73 | -6.2 | -2.89 | -4.80 | 3.31 | 1.38 |
|  |  |  |  |  | Average: 3.4 |  |

| Company | Comments |
| --- | --- |
| Apple | Issue 1-1-1 We are fine with the moderator’s suggestion to have total margin of 1.4 dB for deriving requirements |
| Qualcomm | Is the span consideration for PDCCH same as PDSCH? We are aware that the span for PDSCH is 2.5 dB but not sure whether it is same for PDCCH. Can moderator confirm? We would like to note that UE will have to pass each test, i.e., FDD/TDD, 2Rx/4Rx separately. Therefore, we don’t think adding margin based on the average span is justified. Having noted this, we would like to put forward following options (assuming 2.5 dB span): Option 1: Add margin based on the worst-case span, i.e., 3.78-2.5dB=1.28 dB, for all tests. Option 2: Add margin for each individual test based on the span for that given test. For the given span of [3.33, 3.17, 3.78, 3.31], the set of additional margins would be [0.83, 0.67, 1.28, 0.81]. We prefer option 1 to be fair to the companies who favor w/o soft combining to define requirement so as to compensate for the performance gap with respect to soft-combining |
| Moderator | To QC: Regarding the additional margin, in our understanding, it should be a common value, to indicate the gap between with soft combining and without soft combing. In our view, the implemention of them should be agnostic with antenna configuration. As proposed the value based on average span can full this test and also is large than te individual value proposed by QC, excepting for TDD 2Rx case. Meanwhile, based on companies results, the gap for other configuration is similar. Given that, I would like to encourage companies to further the simulation results, for TDD 2Rx case. Beside, 0.5dB margin will be added on top of average impairment, my understanding it have already provide the flexiblity and enough margin for UE implementation |
| Mediatek | Issue 1-1-1 We are fine with the moderator’s suggestion to have total margin of 1.4 dB for deriving requirements |
| Samsung | Issue 1-1-1 We are fine with the moderator’s suggestion to have total margin of 1.4 dB for deriving requirements. |
| Ericsson | We are fine with the recommendation for 2nd round above. |
| Nokia, Nokia Shanghai Bell | We agree with the intent of the moderator to calculate the excess span using [average span -2.5dB] and then compensating for it by increasing the TT margin using the result. This should sufficiently cover requirements to be passable without soft combining according to our prior alignment threshold. Hence we agree with Moderators suggestion on the total margin. |
| Huawei | We think it is more reasonable to consider the margin case by case when we use receiver agnostic method to derive the performance requirements. However, we are also fine with the recommendation for 2nd round above. |
| Qualcomm2 | We agreed to define requirement for this feature with the premise that sufficient additional margin will be added such that companies that prefer w/o soft-combining are not put at a disadvantage, which is a mandated algorithm for this feature. As Huawei commented above, we also think it is reasonable to consider individual margin on test-by-test basis since we have different receiver assumptions here (unlike other typical requirements). For the sake of progress, we can compromise going with individual margin, i.e., option 2 in our proposal. |
| Mod2 | To QC In general, we have 0.5 dB margin on top of average impairment is to allow UE different implemenetation For this issue, the purpose of adding additional margin is to consider the difference of receiver with and without soft-combinning. In our understanding, the implementation is agnostic with different antenna configuration. As shown in results, MTK and Ericsson provide the results with and without soft-combining with following span, The span is very close and also agnostic with antenna configuration. Therefore, we think it is reasonable to have a common margin based on averaged span Ericsson; MTK. 1.50; 1.62. 1.50; 1.57. 1.40; 1.58. 1.20; 1.51. As mentioned, based on current results, the gap of 2Rx case for QC is large, compared with other companies results, it is necessary to double check results. |
| QC3 | To moderator: If we look at the above results for TDD 2Rx case w/o soft-combining, the delta between Ericsson and MTK is 0.25 dB and that of between QC and MTK it is 0.33 dB. While Ericsson has the best results, I think the delta between UE vendors, i.e., between MTK and QC is very reasonable (0.33 dB), which is less than 0.5 dB. If moderator is concerned with this small spread, then I think QC is right to be concerned with large span that we see between w/ and w/o soft-combining for this case, which is 3.78 dB. I think we should allocate proper additional margin to compensate for the performance gap between w/ and w/o soft-combining, rather than focusing on minor deltas between companies who assumed same receiver implementation. On the other hand, we have many instances in the above table where deltas among companies who assumed soft-combining are quite large, e.g., for this case it is >1 dB. |
| Mod3 | To QC In general, considering the different implementation, in case the span of simulaion results is within 2.0 dB or 2.5dB, we think the results are well aligned and reasonable, if you check the results with soft-combining, the span is within 2.0dB, similar for the results without soft-combining. Therefore, large >1 dB for alignment results should be reasonable. For this specical case with TDD 2Rx, as you mentioned, it can be impact due to different BW for FDD and TDD. while, the gap for 4Rx, the gap is minor. As observed in provided results, the gap for same antenna configuration with different duplex is minor, no matter the assumption with or with out soft-combining. Therefore, I do encourage company can double check results, either 2Rx FDD or TDD Considering the target of this meeting is to decide the value of additional margin and derive the PDCCH requirement, my suggestion is to add [0.8] dB for additional margin based on average span, excepting for 2Rx TDD case. The margin of can be proceeded in case manner, with [1.28] dB, the value can be updated in the next meeting . Hope it can address your concern |

| QC4 | To Moderator: Thanks for your comments. We are okay with your proposal of adding 0.8dB margin to all cases except TDD 2Rx case, for which 1.28dB margin would be added. |
| --- | --- |

# Topic #2: Demodulation requirement for Enhancement on HST-SFN scenario

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215538 | Nokia, NSB | Draft CR for 38.101-4 FeMIMO Applicability of Requirements |
| R4-2215595 | Apple | Observation 1: With MCS17 in HST Scheme-A max throughput is not achieved for FDD and TDD. Proposal 1: Define requirements with MCS13 for HST Scheme-A. |
| R4-2215596 | Apple | Draft CR on PDSCH requirement for HST-SFN scheme A |
| R4-2215713 | CMCC | Updated simulation results for HST-SFN |
| R4-2215726 | Samsung | Observation1: for HST-SFN scheme A, both FDD and TDD, MCS13 related cases could achieve peak throughput, while MCS17 related cases could not achieve peak throughput. Observation2: for HST-SFN scheme A, both FDD and TDD, MCS17 related cases could get better throughput performance than 70% of peak throughput. For FDD MCS17 case could reach 98% of peak throughput, for TDD MCS17 case could reach 88% of peak throughput. Proposal 1: for HST-SFN scheme A, MCS17 and rank2 should be used for FDD PDSCH requirement setup, while MCS13 and rank2 should be used for TDD PDSCH requirement setup. Proposal 2: considering the impairment margin, SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme A with single carrier Case Number; Duplex; MCS level; Maximum Doppler(Hz); Antenna Configuration; SNR(dB). 1; FDD; 17; 870; 2x2; 16.7. 2; FDD; 17; 870; 2x4; 13.5. 3; TDD; 13; 1667; 2x2; 12.0. 4; TDD; 13; 1667; 2x4; 9.1. Observation3: for HST-SFN scheme B, both FDD and TDD, MCS13 related cases could achieve peak throughput, while MCS17 related cases could not achieve peak throughput. Observation4: for HST-SFN scheme B, both FDD and TDD, MCS17 related cases could get better throughput performance than 70% of peak throughput. For FDD MCS17 case could reach 98% of peak throughput, for TDD MCS17 case could reach 90% of peak throughput. Proposal 3: for HST-SFN scheme B, MCS17 and rank2 should be used for FDD PDSCH requirement setup, while MCS13 and rank2 should be used for TDD PDSCH requirement setup. Proposal 4: considering the impairment margin, SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme B with single carrier Case Number; Duplex; MCS level; Maximum Doppler(Hz); Antenna Configuration; SNR(dB). 1; FDD; 17; 870; 2x2; 15.3. 2; FDD; 17; 870; 2x4; 12.4. 3; TDD; 13; 1667; 2x2; 11.4. 4; TDD; 13; 1667; 2x4; 8.8. |
| R4-2215728 | Samsung | Simulation results summary |
| R4-2215882 | Qualcomm | Observation 1: Peak throughput can’t be achieved for MCS17 with Rank2 Proposal 1: Option 1 (MCS 13 with Rank 2) |
| R4-2215886 | Qualcomm | Simulation results on HST-SFN Scheme A |
| R4-2215887 | Qualcomm | Simulation results on HST-SFN Scheme B |
| R4-2215938 | MTK | Observation 1: UE has problems to achieve relative throughput of higher than 90% with Rank2 MCS17. Proposal 1: We support MCS 13 and Rank 2. |
| R4-2216002 | Huawei, HiSilicon | Proposal 1: For the test applicability rule, test different UE feature with different SCS, i.e. test HST-SFN scheme A for 15kHz and HST-SFN scheme B for 30kHz if UE support both HST SFN schemes. Proposal 2: Select MCS 17 and rank 2 for HST SFN scheme A performance requirements definition. |
| R4-2216003 | Huawei, HiSilicon | Simulation results on UE FeMIMO demod HST-SFN |
| R4-2216004 | Huawei, HiSilicon | Draft CR on PDSCH requirement for HST-SFN scheme B |
| R4-2216387 | Ericsson | Observation 1: For MCS13, the max throughput can be reached. The SNR at 70% max throughput for each case is within a reasonable range. Observation 2: For MCS17, the max throughput cannot be reached. Proposal 1: Set MCS13 for SFN Scheme A requirement |
| R4-2216388 | Ericsson | Simulation results for PDSCH requirement for HST-SFN |

## Open issues summary

Last RAN4 meeting agreements in the WF R4-2214397

List of open issues

- Sub-topic 2-1 PDSCH requirement for HST-SFN scheme A and scheme B

  - Issue 2-1-1: MCS for HST-SFN scheme A

  - Issue 2-1-2: MCS for HST-SFN scheme B

  - Issue 2-1-3 Test applicability rule for scheme A and scheme B

### Sub-topic 2-1: PDSCH requirement for HST-SFN scheme A and scheme B

Issue 2-1-1: MCS for HST-SFN scheme A

- Observations

  - Observation 1 (Apple)

    - With MCS17 in HST Scheme-A max throughput is not achieved for FDD and TDD.

  - Observation 2 (Qualcomm)

    - Peak throughput can’t be achieved for MCS17 with Rank2

  - Observation 3(MTK)

    - UE has problems to achieve relative throughput of higher than 90% with Rank2 MCS17.

  - Observation 4 (Samsung)

    - For HST-SFN scheme A, both FDD and TDD, MCS13 related cases could achieve peak throughput, while MCS17 related cases could not achieve peak throughput.

    - For HST-SFN scheme A, both FDD and TDD, MCS17 related cases could get better throughput performance than 70% of peak throughput. For FDD MCS17 case could reach 98% of peak throughput, for TDD MCS17 case could reach 88% of peak throughput.

  - Observation 5 (Ericsson)

    - For MCS13, the max throughput can be reached. The SNR at 70% max throughput for each case is within a reasonable range.

    - For MCS17, the max throughput cannot be reached.

- Proposals

  - Option 1 (MTK, Qualcomm, Apple, Ericsson, Samsung): MCS 13 for both FDD and TDD

  - Option 2 (Samsung)

    - TDD:  MCS 13

    - FDD:  MCS 17

  - Option 3 (Huawei): MCS 17 for both FDD and TDD

- Recommended WF

  - Agreement in the last meeting

    - MCS 13 with Rank 2

    - MCS 17 with Rank 2

    - It’s encouraged companies can provide results for both options under FDD and TDD modes.

    - RAN4 will decide based on the alignment outcome among companies’ results and whether peak TP can be achieved or not.

  - Simulation results summary

    - Large 2 dB gap among companies results, companies are encouraged to further check the simulation results

    - Based on Marjory companies’ observation, MCS 17 is not feasible to achieve maximum throughput. Encourage company can check whether MCS 13 can be agreeable?

| Duplex & Antenna | MCS | Ericsson | Samsung | Huawei | Apple | MTK | Qualcomm | CMCC | Gap |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SNR @ 70% max Tput | SNR @ 70% max Tput | SNR @ 70% max Tput | SNR @ 70% max Tput | SNR @ 70% max Tput | SNR @ 70% max Tput | SNR @ 70% max Tput |  |
| FDD, 2x2 | 13 | 8.6 | 9.3 | 10.74 | 10.1 | 8.5 | 8.99 | 8.8 | 2.24 |
| FDD, 2x4 |  | 5.7 | 5.8 | 8.14 | 8.3 | 6.4 | 6.42 | 6.2 | 2.6 |
| TDD, 2x2 |  | 10.1 | 9.5 | 10.83 | 11.1 | 11.0 | 9.75 | 10.2 | 1.6 |
| TDD, 2x4 |  | 7.6 | 6.6 | 8.27 | 9.4 | 8.6 | 7.51 | 7.4 | 2.8 |
| FDD, 2x2 | 17 | Max TP can’t be reached | 14.2 | 14.86 | 14.6 | 17.1 | N. A | 13.2 |  |
| FDD, 2x4 |  |  | 11.0 | 12.35 | 13.0 | 13.3 | N.A | 10.4 |  |
| TDD, 2x2 |  |  | 16.5 | 14.80 | 15.1 | 19.3 | N.A | TBA |  |
| TDD, 2x4 |  |  | 13.7 | 12.32 | 13.6 | 15.0 | N.A | TBA |  |

  - Recommended WF:

    - Introduce HST-SFN scheme A with MCS 13 for both FDD and TDD

Issue 2-1-2: MCS for HST-SFN scheme B

- Observations

  - Observation 1 (Samsung)

    - For HST-SFN scheme B, both FDD and TDD, MCS13 related cases could achieve peak throughput, while MCS17 related cases could not achieve peak throughput.

- Proposals

  - Option 1 (MTK, Apple, QC, Samsung): same as HST-SFN scheme A with MCS 13

  - Option 2 (Samsung)

    - TDD:  MCS 13

    - FDD:  MCS 17

  - Option 3 (Ericsson, Huawei, DCM): MCS 17

- Recommended WF

  - Simulation results summary

    - Large 2 dB gap among companies results, companies are encouraged to further check the simulation results

  - Follow the same test setup with HST-SFN A?

| Duplex & Antenna | MCS | Ericsson | Samsung | Huawei | Apple | MTK | Qualcomm | CMCC | Gap |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SNR @ 70% max Tput | SNR @ 70% max Tput | SNR @ 70% max Tput | SNR @ 70% max Tput | SNR @ 70% max Tput | SNR @ 70% max Tput | SNR @ 70% max Tput |  |
| FDD, 2x2 | 13 |  | 8.7 |  |  | 7.8 | 8.98 |  | 1.18 |
| FDD, 2x4 |  |  | 5.4 |  |  | 5.2 | 6.4 |  | 1.2 |
| TDD, 2x2 |  |  | 8.9 |  |  | 8.4 | 9.17 |  | 0.77 |
| TDD, 2x4 |  |  | 6.3 |  |  | 5.8 | 6.91 |  | 1.11 |
| FDD, 2x2 | 17 | 11.2 | 12.8 | 13.22 |  | 11.4 | N.A |  | 2.02 |
| FDD, 2x4 |  | 8.6 | 9.9 | 10.42 |  | 8.8 | N.A |  | 1.62 |
| TDD, 2x2 |  | 12.2 | 14.5 | 13.07 |  | 11.8 | N.A |  | 2.7 |
| TDD, 2x4 |  | 9.3 | 12.1 | 10.30 |  | 9.6 | N.A |  | 2.8 |

Issue 2-1-3: Test applicability rule for scheme A and scheme B

- Proposals

  - Option 1 (Agreement in the last meeting, Samsung, QC, MTK, DCM, CMCC, Apple, HW (compromised)): [If UE support HST-SFN scheme A and pass the test of HST SFN scheme A, it can skip the test of HST SFN scheme B]

  - Option 2 (Huawei):

    - For the test applicability rule, test different UE feature with different SCS, i.e. test HST-SFN scheme A for 15kHz and HST-SFN scheme B for 30kHz if UE support both HST SFN schemes.

- Recommended WF

  - Follow the agreement in the last meeting?

  - Recommended WF:

    - Define the test applicability rule for HST-SFN scheme A and HST-SFN scheme B: If UE support HST-SFN scheme A and pass the test of HST SFN scheme A, it can skip the test of HST SFN scheme B

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1 Issue 2-1-2 Issue 2-1-3 |
| Ericsson | Issue 2-1-1 From our observation, most of companies’ results showed that the performance of MCS17 can not reach the max throughout. Given that situation, we support option 1 to only consider MCS13 for defining requirements. Issue 2-1-2 We recall that we had an agreement on MCS17 for scheme B in RAN4 #102-e: Issue 2-3-4: MCS and Rank - MCS 17 with Rank 2 as a baseline. Meanwhile, from our simulation results, the max Tput can be achieved with MCS17 and the SNR point at 70% max Tput is within a reasonable range. Thus, we propose to use MCS17 for both FDD and TDD. |
| Samsung | Issue 2-1-1 MCS13 is acceptable from our side. Issue 2-1-2 It seems simulation results from Samsung is not correctly captured. I updated them inline, please check. It’s fine to follow the same MCS level with HST-SFN scheme A. Issue 2-1-3 We still prefer option 1. |
| Apple | Issue 2-1-1: MCS for HST-SFN scheme A Based on the results we think MCS17 is not feasible to define requirements and we agree to define requirements with MCS13 for HST Scheme A. Issue 2-1-2: MCS for HST-SFN scheme B We can use the same set up as HST scheme A and use MCS 13 for both FDD and TDD. Issue 2-1-3: Test applicability rule for scheme A and scheme B We prefer option 1 to confirm tentative agreement in last meeting. |
| Docomo | Issue 2-1-2: MCS for HST-SFN scheme B We don't have strong motivation, but it seems that there are simulation results that the scheme B reaches 70% max T-put at lower SNR than scheme A. So, we slightly prefer MCS17 for both FDD and TDD. Issue 2-1-3: Test applicability rule for scheme A and scheme B We prefer Option 1. |
| CMCC | Issue 2-1-3: Test applicability rule for scheme A and scheme B Both Option 1 and Option 2 are OK for us. Option 2 is more preferred, since both schemes are tested without increasing test number. |
| Qualcomm | Issue 2-1-1: MCS for HST-SFN scheme A Sine peak throughput can’t be achieved with MCS17, we concur with moderator’s suggestion to define requirements with MCS13 for HST Scheme A. Issue 2-1-2: MCS for HST-SFN scheme B For Scheme B, agree with moderators’ suggestion to use the same set up as HST scheme A and also use MCS 13 for both FDD and TDD. Issue 2-1-3: Test applicability rule for scheme A and scheme B We support option 1. |
| MediaTek | Issue 2-1-1: MCS for HST-SFN scheme A We support Option 1 to use MCS13 for both FDD and TDD. Issue 2-1-2: MCS for HST-SFN scheme B We support to use MCS13 for both FDD and TDD as in HST-SFN scheme A. Issue 2-1-3: Test applicability rule for scheme A and scheme B We support Option 1. We would like to remind that adding HST-SFN scheme B test with applicability rule was already a compromise to some companies in the previous meeting. |
| Huawei | Issue 2-1-1: MCS for HST-SFN scheme A We still prefer MCS17 for both FDD and TDD. Based on our simulation results, MCS 17 is feasible for Scheme A. If MCS13 is selected, it is meaningless to define the performance requirements since it is same as Rel-16 normal SFN, no any benefit for those enhanced functionality. Considering that some companies think maximum throughput cannot be achieved for MCS 17, we propose to use MCS 14, 15, or 16 for HST-SFN scheme A performance definition. Issue 2-1-2: MCS for HST-SFN scheme B We still prefer MCS17 for both FDD and TDD. Based on our simulation results, MCS 17 is feasible for Scheme B. Similar as Issue 2-1-1, other MCS that is higher than MCS 13 can be considered for the performance requirements definition. Issue 2-1-3: Test applicability rule for scheme A and scheme B We still think SFN scheme A and scheme B are different UE processing and there may be test coverage hole for Option 1. However, for the sake of progress, we are fine to compromise to Option 1. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2215538 (Nokia, Draft CR for 38_101-4 FeMIMO Applicability of Requirements) | Ericsson: mTRP-PUCCH-InterSlot-r17 should be mTRP-PDCCH-InterSlot-r17. We also wonder if there is a need to start a new table for these applicability rules since these are all for FeMIMO feature rather than HST feature. |
|  | Samsung: For FR1 TDD of “Support for SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH (sfn-SchemeA-r17)” and “Support for SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH (sfn-SchemeB-r17)” in 5.1.1.3, Clause 5.2.2.1.X2 should be corrected to Clause 5.2.2.2.X2. For FR1 FDD of “Support for PDCCH with inter-slot repetition (mTRP-PUCCH-InterSlot-r17)” in 5.1.1.3, Clause 5.3.2.2.X1 should be Clause 5.3.3.1.X1, while for FR1 TDD, Clause 5.3.3.1.X1 should be Clause 5.3.2.2.X1. For the second additional row in 5.1.1.5, it seems Clause 5.2.2.1.X1 and Clause 5.2.2.1.X2 should be updated to Clause 5.2.2.2.X1 and Clause 5.2.2.2.X2. Apple: Same comments as Ericsson. Suggest to have new applicability table for “Enhanced SFN Transmission schemes” Same comments as Samsung on section numbers. |
|  | Nokia: @Ericsson: Thanks for the comments. We will correct to mTRP-PDCCH-InterSlot-r17. Regarding starting a new table, we can make this change as it is also requested by Apple. @Samsung: Thanks for highlighting the incorrect clause references. We will make the proposed corrections. |
|  | Huawei: “mTRP-PUCCH-InterSlot-r17” should be mTRP-PDCCH-Repetition-r17. It should be intra-slot PDCCH repetition cases and we are defining FDM cases. There is no IE named “mTRP-PDCCH-InterSlot-r17”. |
| R4-2215596 (Apple, Draft CR on PDSCH requirement for HST-SFN scheme A) | Moderator: RAN4#104b-e meeting information should be Oct instead of Aug |
|  | Ericsson: R.PDSCH.1-8.3 assumes one TRS configuration, which is assuming 6 resource elements in one slot. For multi-TRP, there should be two TRS configurations. |
|  | Samsung: For all “HST-Scheme A”, maybe it is more suitable to use “HST-SFN Scheme A”. About the value of TCI state #3 “CSI-RS resource 6 from 'CSI-RS for tracking Resource set #3' configuration”, I guess maybe this 6 should be 9. About formula B.3.X.5 in B.3.X, It seems this formula and the figure below (B.3.X-6) is not corresponding with each other, maybe one negative sign is needed. Apple: Thanks for the comments, we will revise the CR. We will align the formula for Doppler shift to use fc and v like in HST-SFN Scheme B and current HST channel models. |
|  | Huawei: 1. TCI state number should be from 0 to 5. 2. CSI-RS resource 6 in TCI state 3 should be CSI-RS resource 9. 3. In Figure B.3.X-2, The maximum power should be at the position 700m rather than around 700m. |
| R4-2216004 (Huawei, HiSilicon, Draft CR on PDSCH requirement for HST-SFN scheme B) | Samsung: About the value of TCI state #2 “CSI-RS resource 9 from 'CSI-RS for tracking Resource set #2' configuration”, I guess maybe this set #2 should be set #3. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
|  | Issue 2-1-1: MCS for HST-SFN scheme A GTW agreement Introduce HST-SFN scheme A with MCS 13 for both FDD and TDD Issue 2-1-2: MCS for HST-SFN scheme B GTW agreement Introduce HST-SFN scheme B with MCS 13 for both FDD and TDD Issue 2-1-3: Test applicability rule for scheme A and scheme B GTW agreement If UE support HST-SFN scheme A and pass the test of HST SFN scheme A, it can skip the test of HST SFN scheme B |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Issue 2-1-1: SNR value for PDSCH requirement with HST-SFN A

- Simulation results summary (R4-2215728)

  - Ideal results are aligned well within 2.5dB among companies for HST-SFN A with MCS 13, excepting for FDD 2x4 case

  - Companies are encouraged to further check simulation results and provide the impairment results

    - Ideal results

|  | Cases |  | Samsung | Qualcomm | Huawei | Ericsson | MTK | Apple | CMCC | Average | Span | STD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | schemeA | 2a-1 (FDD 2x2 MCS13) | 9.30 | 8.99 | 10.74 | 8.60 | 8.50 | 10.10 | 8.80 | 9.3 | 2.24 | 0.773582 |
|  |  | 2a-3 (FDD 2x4 MCS13) | 5.80 | 6.42 | 8.14 | 5.70 | 6.40 | 8.30 | 6.20 | 6.7 | 2.6 | 0.990057 |
|  |  | 2a-5 (TDD 2x2 MCS13) | 9.50 | 9.86 | 10.83 | 10.10 | 11.00 | 11.10 | 10.20 | 10.4 | 1.6 | 0.56823 |
|  |  | 2a-7 (TDD 2x4 MCS13) | 6.60 | 7.65 | 8.27 | 7.60 | 8.60 | 9.40 | 7.40 | 7.9 | 2.8 | 0.841978 |

- Proposals

  - Option 1:   Apply same method in Rel-15 for SNR derivation

    - SNR = average impairment results + extra margin (0.5) dB

Issue 2-1-2: SNR value for PDSCH requirement with HST-SFN B

- Simulation results summary (R4-2215728)

  - Ideal results are aligned well within 2.5dB among companies for HST-SFN B with MCS 13

  - Companies are encouraged to provide the impairment results

    - Ideal results

|  | Cases |  | Samsung | Qualcomm | Huawei | Ericsson | MTK | Apple | CMCC | Average | Span | STD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | schemeB | 2a-1 (FDD 2x2 MCS13) | 8.70 | 8.98 |  |  | 7.80 |  |  | 8.5 | 1.18 | 0.773582 |
|  |  | 2a-3 (FDD 2x4 MCS13) | 5.40 | 6.40 |  |  | 5.20 |  |  | 5.7 | 1.2 | 0.990057 |
|  |  | 2a-5 (TDD 2x2 MCS13) | 8.90 | 9.31 |  |  | 8.40 |  |  | 8.9 | 0.91 | 0.56823 |
|  |  | 2a-7 (TDD 2x4 MCS13) | 6.30 | 7.03 |  |  | 5.80 |  |  | 6.4 | 1.23 | 0.841978 |

- Proposals

  - Option 1:   Apply same method in Rel-15 for SNR derivation

    - SNR = average impairment results + extra margin (0.5) dB

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1 Issue 2-1-2 |
| Apple | Issue 2-1-1 We are fine with Moderator’s proposal Issue 2-1-2 We are fine with Moderator’s proposal |
| Qualcomm | Issue 2-1-1 We are okay with moderator’s proposal. We suggest to keep the final requirements within the square brackets. Issue 2-1-2 We are fine with moderator’s proposal. We suggest to keep the final requirements within the square brackets. |
| Mediatek | Issue 2-1-1 We are fine with moderator’s proposal. Issue 2-1-2 We are fine with moderator’s proposal |
| Samsung | Issue 2-1-1 We are fine with moderator’s proposal. Issue 2-1-2 We are fine with moderator’s proposal. |
| Ericsson | Issue 2-1-1 We are fine with moderator’s proposal. Issue 2-1-2 We are fine with moderator’s proposal. BTW: we wonder if it is ok to submit results for the scheme B in the next meeting, since we only provided results with MCS17 which is based on the previous agreement. |
| Huawei | Issue 2-1-1 We are fine with moderator’s proposal. Issue 2-1-2 We are fine with moderator’s proposal. |

# Topic #3: CSI reporting requirment for multi-TRP

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215727 | Samsung | Draft CR on PMI requirement for multi-TRP |
| R4-2215728 | Samsung | Simulation results summary |
| R4-2216007 | Huawei, HiSilicon | Proposal 1: Confirm to use the gamma value 1.6 for 8 ports. |
| R4-2216008 | Huawei, HiSilicon | Simulation results on UE FeMIMO CSI mTRP |
| R4-2216389 | Ericsson | Proposal 1: Consider following gamma values: TP ratio (following Type I PMI /random Type I PMI); Proposed Gamma value. FDD, 2Rx; 2.36; 1.6. FDD, 4Rx; 2.82; 2.1. TDD, 2Rx; 2.92; 2.2. TDD, 4Rx; 3.12; 2.4. |
| R4-2216390 | Ericsson | Simulation result for PMI reporting tests for multi-TRP |

## Open issues summary

Last RAN4 meeting agreeements in the WF R4-2214397

List of open issues

- Sub-topic 3-1: CSI reporting requirement for m-TRP transmission

  - Issue 3-1-1: Gamma value

### Sub-topic 3-1: CSI reporting requirement for m-TRP transmission

Issue 3-1-1: Gamma value

- Proposals

  - Option 1 (Agreement in the last meeting, Huawei, Ericsson, QC, MTK, Samsung): [1.6]

  - Option 2 (Ericsson):

|  | TP ratio (following Type I PMI /random Type I PMI) | Proposed Gamma value |
| --- | --- | --- |
| FDD, 2Rx | 2.36 | 1.6 |
| FDD, 4Rx | 2.82 | 2.1 |
| TDD, 2Rx | 2.92 | 2.2 |
| TDD, 4Rx | 3.12 | 2.4 |

- Recommended WF

  - Simulation results summary

|  | Samsung | Huawei | Ericsson |
| --- | --- | --- | --- |
| SNR point(dB)/TP ratio (following Type I PMI /random Type I PMI) | XP High 8x2 (90%) | XP High 8x2 (90%) | XP High 8x2 (90%) |
| FDD 2RX | 8.7 dB/2.6 | 2.06 | 2.36 |
| FDD 4RX | 5.1 dB/3.27 | 2.39 | 2.82 |
| TDD 2RX | 7.6 dB/2.96 | 2.06 | 2.92 |
| TDD 4RX | 4.9 dB/3.38 | 2.38 | 3.12 |

  - Based on the simulation results summary, [1.6] should be feasible considering implementation margin. Encourage companies can check whether gamma value as 1.6 is agreeable?

  - Recommended WF:

    - Capture gamma value as [1.6] for CSI reporting requirement for m-TRP transmission

    - The gamma value can be updated pending on updated simulation results before end of this meeting if any

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1-1 |
| Ericsson | We are fine with the recommended WF. |
| Samsung | Issue 3-1-1 We are ok to use 1.6 as gamma value. |
| Qualcomm | Issue 3-1-1 We suggest keeping the gamma value within the square bracket so that if and when remaining companies submit results, the gamma value can be updated considering the new results. |
| Mediatek | Issue 3-1-1 We are fine with gamma value 1.6 with or without square brackets. |
| Huawei | Issue 3-1-1 We are OK with the recommended WF. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2215727 (Samsung, Draft CR on PMI requirement for multi-TRP) | Ericsson: Configuring Resource ID of CSI-RS as #1 and #2 may cause confusion since resource 1 to 8 have been defined at the beginning of this table. R.PDSCH 1-6.2 can not be used since our test requires 4 TRS resources per slot. |
|  | Nokia: The definition of SCS and CBW in the added “Test parameters” tables are missing. We see this is most likely due to copying from PDSCH mDCI without including the additional table (PDSCH mDCI section does not include the parameters in the table as it is in the minimum requirement tables). Perhaps other parameters are missing due to that, so we suggest to re-check. The test parameter tables are quite long, as a single configuration change w.r.t. the common parameters result in the whole parameter set to be repeated. While this is in line with what has been done in the spec until now, we would like to get other companies input concerning abandoning this practice and only defining the exact changed configuration in the tables. |
|  | Huawei: There is conflicting for two NZP CSI-RS resource. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: Issue 3-1-1: Gamma value GTW agreement Keep the gamma value with [1.6] into draftCR. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on UE demodulation and CSI requirement for FeMIMO | Samsung | Capturing the agreement |
|  | Big CR on UE demodulation and CSI requirement for FeMIMO | Samsung | Big CR for Email approve |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2216393 |  | draft CR to 38.101-4: PDCCH requirement with inter-slot repetition | Ericsson | Revised |  |
| R4-2216790 |  | Draft CR on reference measurement channels for multi-TRP PDCCH performance | QC | Revised |  |
| R4-2215538 |  | Draft CR for 38_101-4 FeMIMO Applicability of Requirements | Nokia | Revised |  |
| R4-2215596 |  | Draft CR on PDSCH requirement for HST-SFN scheme A | Apple | Revised |  |
| R4-2216004 |  | Draft CR on PDSCH requirement for HST-SFN scheme B | Huawei | Revised |  |
| R4-2215727 |  | Draft CR on PMI requirement for multi-TRP | Samsung | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2217440 |  | WF on UE demodulation and CSI requirement for FeMIMO | Samsung | Agreeable | Need to be confirmed 1.1.1 Sub-topic 1-1-1: |
| R4-2215728 |  | Simulation results summary for FeMIMO demodulation and CSI requirement | Samsung | Note |  |
| R4-2217435 |  | Draft CR on PDSCH requirement for HST-SFN scheme A | Apple | Agreeable | Need to check channel profile in appendix for Figure B.3.x-1: Deployment of HST-SFN Scheme A Updated SNR value based on latest simulation summary |
| R4-2217434 |  | Draft CR for 38_101-4 FeMIMO Applicability of Requirements | Nokia | Agreeable |  |
| R4-2217436 |  | Draft CR on PDSCH requirement for HST-SFN scheme B | Huawei | Agreeable | Need to check channel profile in appendix for Figure B.3.x-1: Deployment of HST-SFN Scheme B Updated SNR value based on latest simulation summary |
| R4-2217437 |  | Draft CR on PMI requirement for multi-TRP | Samsung | Agreeable |  |
| R4-2217438 |  | draft CR to 38.101-4: PDCCH requirement with inter-slot repetition | Ericsson | Agreeable |  |
| R4-2217439 |  | Draft CR on reference measurement channels for multi-TRP PDCCH performance | Qualcomm | Agreeable | FRC table |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents