3GPP TSG-RAN WG4 Meeting # 104-bis-e											         R4-2217497

Electronic Meeting, 10– 19 October 2022

Agenda item:			6.22.2

Source:	Moderator (Samsung)

Title:	Email discussion summary for [104-bis-e][313] NR_NTN_enh_Part2

Document for:	Information

Introduction

This document captures issues related to the coexistence aspect of NR NTN enhancement work item in Rel-18. It contains a summary of the contributions under Agenda Item 6.22.2 at TSG-RAN WG4 #104-bis-e, together with identified key open issues, and recommends topics/questions to be handled via email discussions.

The goal of this document is to provide recommendations on prioritization of discussion and to facilitate aforesaid discussions to reach consensus on assumptions of coexistence studies as much as possible.

A total of 5 TDOCs have been received for this agenda (See Annex 1) and 4 topics are listed as below to cover proposals and contents in these documents as appropriate.

- Topic #1: Co-existence scenarios

- Topic #2: Network layout model

- Topic #3: System parameters

- Topic #4: Evaluation methodology

Moreover, it is worth mentioning that the draft work plan for the Rel-18 NR-NTN-enh work item has been proposed in R4-2215709 in which relative timeline on co-existence studies has been set.

To progress the discussion, it is proposed that the meeting could:

- in 1st round: focus on Topic #1, #2 & #4 and try to reach consensus as much as possible, and;

- in 2nd round: continue discussion on Topic #1, #2 & #4 and discuss Topic #3.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator | Yiran JIN | yiran.jin@samsung.com |
| Ericsson | Dominique Everaere | dominique.everaere@ericsson.com |
| Huawei | Peng (Henry) Zhang | zhangpeng169@huawei.com |
| Qualcomm | Bin Han | binhan@qti.qualcomm.com |
| Eutelsat | Keith Edwards | kedwards-ext@eutelsat.com |
| Lockheed | Robert Olesen | robert.l.olesen@lmco.com |
| Nokia, Nokia Shanghai Bell | Bartlomiej Golebiowski | Bartlomiej.golebiowski@nokia.com |
| ZTE | Fei Xue | Xue.fei25@zte.com.cn |
| THALES | Dorin Panaitopol |  |
| Hughes/EchoStar | Munira Jaffar | munirajaffar@echostar.com |
| Verizon | Zheng Zhao | zheng.zhao@verizonwireless.com |
| Samsung | Sunlin Zhu | sunlin.zhu@samsung.com |
| CATT | Qiuge Guo | guoqiuge@catt.cn |
| Hispasat | Jorge Garcia | jgarcia@hispasat.es |

Topic #1: Coexistence scenario

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215352 | Thales et al. | Observation 1: Based on the current range, some part of the satellite service allocated frequency bands for both GEO and NGSO fall outside of the FR1 and FR2 range. As shown in the diagram below, these include: - Ka-Band DL - Ku-Band UL and DL Observation 2: The Harmonized Satellite Ka-band refers to [17.7 – 20.2 GHz] on the downlink and [27.5 – 30.0 GHz] on the uplink. Proposal 1: RAN4 shall consider FDD harmonized Satellite Ka-band (17.7 – 20.2 GHz on the downlink, and 27.5 – 30.0 GHz on the uplink) for coexistence analysis. Proposal 2: RAN4 shall focus on FDD UL 27.5 – 30.0 GHz NTN coexistence analysis with TDD TN n258 (24.250 - 27.5 GHz). Proposal 3: Leveraging the work in FR1, RAN4 shall focus on Ka-band coexistence analysis using urban and rural macro scenarios. Proposal 4: Leveraging the work in FR1, FR2 studies related to NTN-NTN coexistence should be deprioritized. Proposal 5: RAN4 shall focus on Ka-band coexistence analysis with LEO 600km and GEO constellations. Proposal 6: RAN4 shall consider the following table for co-existence studies in Ka-band: Table x - TN-NTN coexistence scenarios in adjacent bands for Ka-band No.; NTN Frequency; TN Frequency; TN scenario; NTN. 1; 27.5 GHz-30 GHz; NR n258 (24.250 - 27.5 GHz); Urban macro; GEO. 2; 27.5 GHz-30 GHz; NR n258 (24.250 - 27.5 GHz); Urban macro; LEO@600km. 3; 27.5 GHz-30 GHz; NR n258 (24.250 - 27.5 GHz); Rural macro; GEO. 4; 27.5 GHz-30 GHz; NR n258 (24.250 - 27.5 GHz); Rural macro; LEO@600km. Proposal 7: RAN4 shall consider the following figure for co-existence studies in Ka-band: Figure x. Different interference scenarios in Ka adjacent bands between NTN 5G NR and TN 5G NR Observation 3: It can be noted that for FR2, coexistence scenarios will be much simpler than for FR1, because the NTN-TN scenarios for FR2 are limited only to i1 (DL TN in UL NTN), i2 (UL NTN in DL TN), i3 (UL NTN in UL TN). |
| R4-2215777 | Samsung | Proposal 1: The proposed scenarios for coexistence study are in the Table 2.1-1. Proposal 2: The aggressor and victim combinations are listed in Table 2.1-2. Proposal 3: For uplink of NTN system, we propose to use 30GHz, for downlink we propose to use 20GHz. And other assumptions are listed as table 2.1-3 based on the TR 38.803 and TR 38.821. |
| R4-2216517 | Ericsson | Proposal 1: Further discuss the dense urban deployment scenario. Proposal2: Consider only GSO type of satellite for RAN4 coexistence study with ESIM and consider GSO and NGSO types of satellite for fixed VSAT, pending on further clarification for fixed VSAT scenarios. Observation1: ITU Resolution COM5/6 has specified a minimum distance (M-ESIM) and a maximum pfd (A-ESIM ) for those 2 types of ESIM to operate. Proposal3: RAN4 should only consider L-ESIM type of NTN device for its coexistence studies, A-ESIM might be considered as second priority. Proposal4: RAN4 shall consider both NTN UL and NTN DL cases when doing its coexistence studies. Proposal5: RAN4 should use 20 GHz as frequency reference for NTN DL and 30 GHz for NTN UL. Proposal7: Adopt the assumptions in Annexes 1 and 2 for the coexistence simulations in NTN Ka-band. (Table 6.1-1 & 6.1-2 for scenarios) |
| R4-2216557 | ZTE | Proposal 1: to consider the coexistence scenario in Table 2.1-1 and proposed frequency and bandwidth configuration in Table 2.1-2 for Ka band NTN coexistence study in Rel-18. |

Open issues summary

Sub-topic 1-1

Issue 1-1: NTN DL simulation

- Proposals

  - Option 1 (Thales, et al.): RAN4 shall focus on FDD UL NTN coexistence analysis with TDD TN

  - Option 2 (Ericsson):  RAN4 shall consider both NTN UL and NTN DL cases when doing its coexistence studies.

  - Option 3 (Samsung, ZTE): Further discuss NTN DL cases.

- Recommended WF

  - TBA

Issue 1-2: Exemplary bands for NTN in above 10GHz bands

- Proposals

  - Option 1 (Thales, et al.): 27.5-30GHz

  - Option 2 (Samsung, Ericsson, ZTE): 30GHz for UL and 20GHz for DL

- Recommended WF

  - For and just for the purpose of coexistence study, use 30GHz as UL and 20GHz as DL, but whether NTN DL cases need to be considered depends on the decision of Issue 1-1.

Issue 1-3: Bandwidth of TN & NTN

- Proposals

  - Option 1 (Samsung): TN 200MHz and NTN 400MHz

  - Option 2 (Ericsson): TN 100MHz and NTN 100MHz

  - Option 3 (ZTE): TN 400MHz/200MHz and NTN 400MHz/200MHz

- Recommended WF

  - TBA

Issue 1-4: NTN Frequency Reuse Factor

- Proposals

  - Option 1: Only use FRF=1 for co-existence study.

- Recommended WF

  - Agree on Option 1.

Sub-topic 1-2

Issue 1-5: NTN SAN Type

- Proposals

  - Option 1 (Thales, et al.): GEO & LEO@600km

  - Option 2 (Samsung): GEO, LEO@600km & LEO@1200km

  - Option 3 (ZTE): GEO, LEO & FFS MEO

- Recommended WF

  - TBA

Issue 1-6: NTN UE Type

- Proposals

  - Option 1 (Thales): both fixed and movable VSAT

  - Option 2 (Ericsson): Fixed VSAT and ESIM for GSO and Fixed VSAT for NGSO

- Recommended WF

  - Fixed and mobile VSAT for GSO and Fixed VSAT for NGSO

Issue 1-7: ESIM type

- Proposals

  - Option 1 (Ericsson): only consider L-ESIM type of NTN device for its coexistence studies, A-ESIM might be considered as second priority.

- Recommended WF

  - TBA

Sub-topic 1-3

Issue 1-8: TN scenarios

- Proposals

  - Option 1 (Thales, et al.): Urban macro & Rural macro

  - Option 2 (Samsung): Urban macro & Dense Urban

  - Option 3 (Ericsson): Urban macro and further discuss Dense Urban scenario

  - Option 4 (ZTE): Urban macro and further discuss Rural macro scenario

- Recommended WF

  - Agree on Urban macro and FFS Rural macro & Dense Urban scenarios

Issue 1-9: Aggressor and victim table

- Proposals

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | Applicable for n257, n258 and n261 | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for n257, n258 and n261 | Phase 1 |
| 3 | TN with NTN | NTN UL | TN DL | Applicable for n257, n258 and n261 | Phase 1 |
| 4 | TN with NTN | TN DL | NTN UL | Applicable for n257, n258 and n261 | Phase 1 |
| 5 | TN with NTN | TN DL | NTN DL | FFS | Phase 2 |
| 6 | TN with NTN | NTN DL | TN DL | FFS | Phase 2 |
| 7 | TN with NTN | NTN DL | TN UL |  |  |
| 8 | TN with NTN | TN UL | NTN DL |  |  |
| 9 | NTN with NTN | NTN DL | NTN DL | FFS |  |
|  |  | NTN UL | NTN UL | FFS |  |
| NOTE 1: For coexistence between Ka band DL and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified. |  |  |  |  |  |

  - Option 1 (ZTE): No. 1,2,3,4,5,6

  - Option 2 (Thales, et al.): No. 1,3,4 with Notes only applicable for n258

  - Option 3 (Ericsson): No. 1,2,3,4,5,6,7,8

  - Option 4 (Samsung): No. 1,2,3,4,5,6,9

- Recommended WF

  - TBA

Issue 1-10: Interference figure for above 10GHz bands

- Proposals

  - Option 1 (Thales, et al.): RAN4 shall consider the following figure for co-existence studies in Ka-band:

Figure x. Different interference scenarios in Ka adjacent bands between NTN 5G NR and TN 5G NR

- Recommended WF

  - Capture the figure in TR 38.863 Section 6.1.

Companies views’ collection for 1st round

Open issues

Issue 1-1: NTN DL simulation

| Company | Comments |
| --- | --- |
| Ericsson | Option 2, the coexistence study is supposed to be band agnostic so that we don’t have to redo any coex study when introducing a new band covered by the representative frequency used for coex. If NTN DL is not considered, would that mean that the NTN DL part of the Ka-band will not be specified? The NTN Ka-band would only be specified as a UL only band? |
| Huawei | Option 3. |
| Qualcomm | Support Option 3 |
| Eutelsat | Option 1 – the focus should be FDD UL NTN coexistence with TDD TN(in line with Thales Tdoc). Open to some further discussion (option 3) if a technical case can be made to perform other studies but any discussion should not prevent or delay option 1 starting. |
| Lockheed | Option 1 – We believe the focus should be on FDD UL NTN, |
| Nokia, Nokia Shanghai Bell | We support option 2. If we are considering defining an NTN Ka band both DL and UL. |
| ZTE | We support the option 3, however we could also fully understand Ericsson’s concern, the key question is how to consider the NTN DL coexistence? Coexist with TN or coexist with NTN? |
| THALES | Option 1 for the reasons presented in the paper. If concerns with respect to DL, companies are encouraged to present relevant use cases. For the time being is not it was not identified what adjacent band to be used for coexistence cases in DL. |
| Hughes/EchoStar | Option 1 – focus on NTN UL with TN TDD Agree with Thales - if DL is a concern, companies are encouraged to present relevant use cases. For the time being is not it was not identified what adjacent band to be used for coexistence cases in DL. |
| Verizon | Option 2. The coexistence study is important part of this work, and the study need to take the national regulation into account. We cannot agree the NTN frequency band proposed in R4-2215352 for this coexistence study as their operations are still held to be within the interference environment in the USA operation and do not consider FCC and US federal regulatory requirements. |
| Samsung | Worse cases should be considered for the coexistence study. RAN4 can have an agreement for the worse case frequency combination and identify it as the first priority. For the other combination, RAN4 can discuss the necessity of the simulation. |
| CATT | Fine with option 1 and option 3 |
| Hispasat | Option 1. Consider FDD UL as a priority and worse case identified, open to further study as proposed in Option 3 considering other relevant interference scenarios in DL, if any. |

Issue 1-2: Exemplary bands for NTN in above 10GHz bands

| Company | Comments |
| --- | --- |
| Ericsson | Agree with the recommended WF from moderator. |
| Huawei | OK with recommended WF. |
| Qualcomm | Agree with recommended WF. |
| Eutelsat | Prefer option 1 but can accept the recommended WF. |
| Lockheed | We prefer option 1. |
| Nokia, Nokia Shanghai Bell | We agree with recommended WF. |
| ZTE | Agree with the recommended WF from moderator. |
| THALES | Option 1 in principle. For the carrier frequency value we could use for instance 28,750 GHz as presented in R4-2215348. This value is actually the center of the proposed (harmonized) Ka band. Moreover, from the point of view of VSAT antenna UE we need to use something realistic that would be applicable for the entire range. VSAT UE Parameters; Tx (Uplink); Rx (Downlink). Polarisation; Circular; Circular. Low Frequency; (MHz); 27 500; 17 700. Centre frequency; 28 750; 18 950. High frequency; 30 000; 20 200. Efficiency; 60%; 57%. On-axis antenna gain at Fc; (dBi); 42,9; 39,0. |
| Hughes/EchoStar | Support Option 1 |
| Verizon | We agree with the recommended WF from moderator |
| Samsung | We agree with the recommended WF from moderator |
| CATT | Agree with recommended WF. |
| Hispasat | We prefer option 1. Consider realistic scenarios applicable to the entire range to study, as proposed by Thales. |

Issue 1-3: Bandwidth of TN & NTN

| Company | Comments |
| --- | --- |
| Ericsson | We would prefer having same channel BW for TN and NTN. |
| Huawei | Option 4: TN 200MHz and NTN 200MHz |
| Qualcomm | Agree with Huawei’s proposal. In TN FR2 co-ex study, we used 200MHz BW. And we prefer to use the same BW for TN and NTN. |
| Eutelsat | No strong preference at present. |
| Lockheed | No strong preference but the same BW for TN and NTN may make more sense. |
| Nokia, Nokia Shanghai Bell | We have preference of option 2 which is narrowest channel bandwidth. |
| ZTE | We also support to have the same channel bandwidth for aggressor and victim. Maybe 200MHz could be good starting point which is also aligned with legacy Rel-14 coexistence study. |
| THALES | Maybe we should consider worst case with same values for the BWs? In FR1 we considered 20 MHz for TN and 20MHz for NTN which was the highest channel bandwidth configuration. Therefore, for FR2/Ka-band coexistence we could assume 400 MHz for TN and 400 MHz channel bandwidth configuration for NTN. |
| Hughes/EchoStar | We should pick one similar BW between NTN and TN, e.g 200 MHz for both |
| Samsung | Different channel BW for TN and NTN would be possible in the realistic deployment, therefore, it’s necessary to consider it. |
| CATT | Prefer Huawei comment. |
| Hispasat | No strong preference, maybe option 3 considering worst case possible. |

Issue 1-4: NTN Frequency Reuse Factor

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Qualcomm | Option 1 |
| Eutelsat | Option 1. |
| Lockheed | Option 1 |
| Nokia, Nokia Shanghai Bell | Option 1. |
| ZTE | Option 1. |
| THALES | If we follow similar FR1 procedures, then FRF=1 for FR2 (Option 1). |
| Hughes/EchoStar | Follow FR1 |
| Samsung | Option 1 |
| CATT | Option 1 |
| Hispasat | Option 1 |

Issue 1-5: NTN SAN Type

| Company | Comments |
| --- | --- |
| Ericsson | Ok with option 2, option 1 could be further discussed. But for option 3 MEO should not be in the scope. |
| Huawei | It’s better to clarify which scenario ESIM is applicable. Only GEO or both GEO and LEO? |
| Qualcomm | Need input from satellite industry. |
| Eutelsat | Option 1 or Option 2 (LEO@1200 is also of interest). |
| Lockheed | Prefer Option 2. |
| Nokia, Nokia Shanghai Bell | Similar comment as Ericsson. |
| ZTE | Ok with option 2, |
| THALES | Option 1. This is based on past experience, please recall that for FR1 we did not see noticeable differences between LEO@600 and LEO@1200. We could therefore simplify.. |
| Hughes/EchoStar | Support Option 3 but OK with option 1 |
| Samsung | Option 2 |
| CATT | Fine with option 1 or option 2. |
| Hispasat | Option 2. |

Issue 1-6: NTN UE Type

| Company | Comments |
| --- | --- |
| Moderator | Propose to align the wording of movable VSAT, rather than ESIM. |
| Ericsson | It would be ok to align the wording, using movable VSAT and not ESIM but we would like then to get clarified that ESIM and movable VSAT have the exact same meaning and definition. |
| Huawei | We have a proposal to align the wording. OK with Ericsson’s proposal. |
| Qualcomm | Is there any difference between the fixed and movable VSAT in the co-ex simulation? |
| Eutelsat | Avoid the term ESIM, agree with moderator. |
| Lockheed | Prefer Option 1 but we can accept the WF. |
| Nokia, Nokia Shanghai Bell | Similar view as Ericsson. |
| ZTE | The question as Qualcomm, if antenna pattern and steering capability is the same for fixed VSAT or ESIM, then we don’t see the difference at least from coexistence perspective. |
| THALES | Not clear why we need to exemplify the naming from the point of view of the coexistence simulations. Both VSAT or ESIM can be steerable, and both can use parabolic antennas or phased array. |
| Hughes/EchoStar | Option 1 |
| Samsung | Agree with Moderator’s proposal. |
| Moderator | The recommended WF is somehow a modification to Option 2 using the wording in Option 1. And I need to apologize that the word “movable” should be “mobile” according to the WID. It is not the intension to discuss the wording here in co-existence study. The purpose is to align the wording with the description in the Rel-18 WID. Any discussion on the definition of the UE should be made in other threads, e.g. [312] or in upper levels. |
| Hispasat | Option 1 is preferred, although first priority can be aligned with moderators WF. |

Issue 1-7: ESIM type

| Company | Comments |
| --- | --- |
| Ericsson | Actually, this is pending on issue 1-6: to our understanding, movable VSAT and ESIM are the same. If a movable VSAT might have different regulatory requirements than an ESIM, then we would need to consider any type of movable VSAT (aeronautical, maritime and land). |
| Huawei | For A-EISM, we have to define the altitude. |
| Eutelsat | See point 1-6 above. |
| Lockheed | Suggest we defer this question. |
| ZTE | This could be further discussed in 312 thread firstly. |
| THALES | It does not seem essential for the coexistence discussion. |
| Hughes/EchoStar | See 1.6 response - both fixed and movable VSAT where applicable |
| Samsung | This issue should be clarified clearly in order to modelling correctly for the simulation. And we can defer the discussion till clear outcomes from [312] and other issues in [313]. |
| Hispasat | Please align with 1-6. |

Issue 1-8: TN scenarios

| Company | Comments |
| --- | --- |
| Ericsson | We are ok with options 2 or 3, but we would like to get other companies view on the need for dense urban scenario. In this scenario, all TN micro BSs will be deployed in a similar area than for TN macro urban, the satellite beam will overlap this area and the NTN UEs will be outside the TN area coverage. It’s not clear to us if this would make any major differences, but welcome other views. We don’t think rural scenario should be considered for TN here. |
| Huawei | Do we have FR2 rural macro parameters and propagation model? The main difference between urban macro and dense urban is BS output power and height. It’s better to consider Urban macro as first priority. |
| Qualcomm | Option 2 or Option 3. In TR 38803 for FR2 co-ex, we considered Urban and Dense Urban. Rural is not simulated for FR2. |
| Eutelsat | Agree with recommended WF |
| Lockheed | Agree with the WF |
| Nokia, Nokia Shanghai Bell | We agree with recommended WF. |
| ZTE | We agree with recommended WF. |
| THALES | Option 1, as we did for FR1. Please see FR2 THALES paper with the justifications. |
| Hughes/EchoStar | Option 1 |
| Samsung | We agree with Ericsson’s analysis, and Dense Urban could be the second priority of the coex-study. However, if the simulation results show that the Macro Base stations would be significantly affected by the NTN system, the interference for Micro Base stations should be evaluated in the next step. And Rural macro is not considered for TN in FR2. |
| CATT | Agree with recommended WF |
| Hispasat | We prefer option 1. |

Issue 1-9: Aggressor and victim table

|  | No.1 | No.2 | No.3 | No.4 | No.5 | No.6 | No.7 | No.8 | No.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Company | Y/N | Y/N | Y/N | Y/N | Y/N | Y/N | Y/N | Y/N | Y/N |
| Ericsson | Y | Y | Y | Y | Y | Y | Y | Y | N |
| THALES | Y |  | Y | Y | (NA) | (NA) | (NA) | (NA) | N |

| Company | Comments |
| --- | --- |
| Moderator | Please check the items your preferred for co-existence study in the table above and leave you comment below. |
| Ericsson | Pending on issue 1-1 We don’t agree with the 2 phases approach here. |
| Huawei | More discussion are needed. The question here is whether the coexistence study is band agnostic? If so, we can remove notes. |
| Qualcomm | The notes for the bands just for the example. Need more discussion pending on the outcome of previous issues. |
| Eutelsat | No preference at this time (agree with Qualcomm). |
| Lockheed | Agree with Qualcomm. |
| ZTE | This still depend on the previous discussions for issue 1-1. |
| THALES | Option 2, but if companies really insist, we can consider also scenario #2. In any case, VSAT UE transmits with higher power and has better antenna gain compared with NTN FR1 UE. Therefore, the FR2 TN UEs will generate less interference in the UL of NTN VSAT UE, as compared to FR1. It is something expected. For DL, no adjacent bands have been currently identified. Therefore we do not see interest for scenarios 5, 6, 7, 8. For Scenario 9, for FR1 there was no NTN-NTN coexistence performed, and we should follow similar approach for FR2. See also Proposal 4 from R4-2215352: Proposal 4: Leveraging the work in FR1, FR2 studies related to NTN-NTN coexistence should be deprioritized. |
| Hughes/EchoStar | Option 2 |
| Samsung | For cases considering NTN DL, we may defer the discussion until the agreement on Issue 1-1 can be made. For case 9 “NTN-NTN”, we have no preference. Although it is not considered in FR1 but we put it here just as a starting point for discussion. |
| CATT | Need further discussion pending on issue 1-1 |
| Hispasat | We prefer option 2. Please clarify that the notes for the bands are just for the example, as pointed by Qualcomm. |

Issue 1-10: Interference figure for above 10GHz bands

| Company | Comments |
| --- | --- |
| Ericsson | To be updated according to Issue 1-1 conclusion. |
| Huawei | To be updated according to Issue 1-1 conclusion. |
| Eutelsat | Agree with WF, accept that it may need to be updated depending upon technical discussion and consensus on 1-1. |
| Lockheed | Agree with the WF. |
| ZTE | Similar understanding as Ericsson. |
| THALES | We could (at least) use this figure as basis. |
| Hughes/EchoStar | OK with WF |
| Verizon | Does this recommended WF intend to use 30GHz as UL and 20GHz as DL same as Issue 1-2 discussion? This needs to be specify. Following FCC orders, we cannot agree NTN frequency range 27.5-28.35 GHz for the US operation as what we discussed in thread [312] unless the necessary national regulations are taken in account. |
| Samsung | To be updated according to Issue 1-1 conclusion. |
| CATT | Same comment as Ericsson |
| Hispasat | Agree with WF. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| N/A | N/A |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Issue 1-1: NTN DL simulation | Tentative agreements: N/A Candidate options: - Option 1: Consider NTN UL cases first and further discuss NTN DL cases Recommendations for 2nd round: Further discuss the new Option 1 |
| Issue 1-2: Exemplary bands for NTN in above 10GHz bands | The moderator would like to remind all that it is the EXEMPLARY band to be determined here. And please kindly be noted with following facts or experience of RAN4 co-existence study: - 30GHz, 40GHz and 70GHz have been used for co-existence studies of NR in FR2 – see TR 38.803. - 2GHz has been used for NTN co-existence study in FR1 to represent S & L bands. – see TR 38.863 Tentative agreements: - For and just for the purpose of coexistence study, use 30GHz as UL and 20GHz as DL. Whether NTN DL is needed depends on Issue 1-1 outcome. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 1-3: Bandwidth of TN & NTN | Tentative agreements: Use the same channel BW for TN and NTN. Candidate options: - Option 1: TN 100MHz & NTN 100MHz - Option 2: TN 200MHz & NTN 200MHz - Option 3: TN 400MHz & NTN 400MHz Recommendations for 2nd round: Further discuss the new options considering which is the worst case |
| Issue 1-4: NTN Frequency Reuse Factor | Tentative agreements: Use FRF=1 for co-existence study Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 1-5: NTN SAN Type | Tentative agreements: GEO, LEO@600km & LEO@1200km. Proponents can provide study results accordingly. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 1-6: NTN UE Type | Tentative agreements: To be in line with the WID: - Fixed and mobile VSAT for GSO - Fixed VSAT for NGSO Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 1-7: ESIM type | Tentative agreements: Defer the discussion to await results of Issue 1-6 and relative discussion in [312] Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 1-8: TN scenarios | Tentative agreements: Urban macro is to be used for TN Candidate options: - Option 1: Rural macro for TN - Option 2: Dense Urban for TN Recommendations for 2nd round: Further discuss two options. |
| Issue 1-9: Aggressor and victim table | Tentative agreements: N/A Candidate options: Existing four options Recommendations for 2nd round: Further discuss each case and it seems case 9 NTN-NTN can be neglected. |
| Issue 1-10: Interference figure for above 10GHz bands | Tentative agreements: In principle, the figure will be captured in TR 38.863 with updates based conclusions of Issue 1-1 and other issues as appropriate. Candidate options: N/A Recommendations for 2nd round: N/A |

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| N/A | N/A |

Discussion on 2nd round (if applicable)

Open issues and view collection

Issue 1-1: NTN DL simulation

- Proposals

  - Option 1: Consider NTN UL cases first and further discuss NTN DL cases

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | This could be acceptable but we should not defer too much the decision. If we can’t decide soon, we are concerned NTN DL would be further discussed until the end of the WI… We should better decide in next meeting the latest. |
| Hughes/EchoStar | Should be done together but may start with UL |
| Hispasat | OK |

Issue 1-3: Bandwidth of TN & NTN

- Proposals

  - Option 1: TN 100MHz & NTN 100MHz

  - Option 2: TN 200MHz & NTN 200MHz

  - Option 3: TN 400MHz & NTN 400MHz

- Recommended WF

  - TBA

| Company | Opt.1(Y/N) | Opt.2(Y/N) | Opt.3(Y/N) | Comments |
| --- | --- | --- | --- | --- |
| Ericsson | Y | Y | TBD | Option 3 is pending on the decision of introducing 400MHz or not for NTN. |
| Samsung | Y | Y | TBD |  |
| Hughes/EchoStar | Y | Y |  |  |
| Hispasat | Y | Y | TBD | Option 3 acceptable as Ericsson’s comment. |
| THALES |  |  |  | No strong view, we just think that Option 3 is the worst case (as we noticed for FR1). |

Issue 1-8: TN scenarios

- Proposals

  - Option 1: Rural macro for TN

  - Option 2: Dense urban for TN

- Recommended WF

  - TBA

| Company | Opt.1 (Y/N) | Opt.2 (Y/N) | Comments |
| --- | --- | --- | --- |
| Ericsson | N | Y? | Rural is definitively not a FR2 scenario. Regarding dense urban, we are open for discussion but we are not sure if this is really needed. We welcome other companies’ view. |
| Samsung | N | Y | Rural macro is not necessary scenario for the FR2 BS. Dense Urban can be the second step of the coexistence study. But it still needs to be one of the options of the coexistence study. |
| Hughes/EchoStar | N | Y | WE believe TN doesn’t deploy FR2 in rural macro |
| Hispasat | N | Y | Rural macro in TN may not be necessary to be considered. |
| THALES | Y? Yes for TN, because NTN rural macro is actually a relevant scenario. | Y? Actually urban but not necessary dense. | In FR1 we considered rural macro and urban macro. Rural TN FR2 it might not be the case for terrestrial, but for NTN there is. So why not to consider it? |

Issue 1-9: Aggressor and victim table

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | Applicable for n257, n258 and n261 | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for n257, n258 and n261 | Phase 1 |
| 3 | TN with NTN | NTN UL | TN DL | Applicable for n257, n258 and n261 | Phase 1 |
| 4 | TN with NTN | TN DL | NTN UL | Applicable for n257, n258 and n261 | Phase 1 |
| 5 | TN with NTN | TN DL | NTN DL | FFS | Phase 2 |
| 6 | TN with NTN | NTN DL | TN DL | FFS | Phase 2 |
| 7 | TN with NTN | NTN DL | TN UL |  |  |
| 8 | TN with NTN | TN UL | NTN DL |  |  |
| 9 | NTN with NTN | NTN DL | NTN DL | FFS |  |
|  |  | NTN UL | NTN UL | FFS |  |
| NOTE 1: For coexistence between Ka band DL and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified. |  |  |  |  |  |

- Proposals

  - Option 1: Do not consider case 9.

  - Option 2: Agree on case 1,2,3,4

  - Option 3: Further discuss case 5,6,7,8 which are related to NTN DL

- Recommended WF

  - Agree on Option 1 & 2.

For Option 1 & 2

| Company | Opt.1 (Y/N) |  | Opt.2 (Y/N) |  | Comments |
| --- | --- | --- | --- | --- | --- |
| Ericsson | Y |  | Y |  | But we don’t agree on this phase approach. |
| Samsung | Y |  | Y |  |  |
| Hughes/EchoStar | Cases 1,2,3,4,5 should only be applicable to n258 |  |  |  |  |
| Hispasat | - | - |  | Please remove band applicability, as they will be further discussed (i.e. 1,2,3,4,5 should only apply to N258). If so, scenarios may be agreed. |  |
| THALES |  |  |  | It is true that we could downscope to n258. |  |

For Option 3

| Company | Comments |
| --- | --- |
| Ericsson | Yes As we highlighted in the 1st round, the coexistence study is supposed to be band agnostic so that we don’t have to redo any coex study when introducing a new band covered by the representative frequency used for the coex simulations. If NTN DL is not considered, would that mean that the NTN DL part of the Ka-band will not be specified? The NTN Ka-band would only be specified as a UL only band? |
| Samsung | Basic on the conclusion of issue 1-1. |

Summary for 2nd round

|  | Status summary |
| --- | --- |
| Issue 1-1: NTN DL simulation | Agreements: Consider NTN UL cases first and further discuss NTN DL cases and preferably to conclude on NTN DL cases in next RAN4 meeting. |
| Issue 1-3: Bandwidth of TN & NTN | GTW Agreement (Oct.18): using option 2 as starting point for initial simulation purpose, other options not precluded. |
| Issue 1-8: TN scenarios | GTW Agreement (Oct.18): In initial evaluation stage, focus on Urban macro scenario for TN only; FFS whether Rural macro need to be considered or not. |
| Issue 1-9: Aggressor and victim table | Agreements: N/A Candidate options: See Table and Options in Issue 1-9 above Recommendations for way forward: FFS |

Topic #2: Network layout model

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215777 | Samsung | Proposal 4: Refers to TR38.863 section 6.2.1 to determine the FR2 TN network layout model. In additional, the deployment of Dense Urban micro base station should follow TR38.803. |
| R4-2216517 | Ericsson | Proposal6: For coexistence simulations in the NTN Ka-band, RAN4 should not consider any isolation area in a first step. Proposal7: Annex 1. 6.2 and Annex 2 |

Open issues summary

Sub-topic 2-1

Issue 2-1: Base line

- Proposals

  - Option 1: Refers to TR38.863 section 6.2.1 to determine the FR2 TN network layout model and refers to TR 38.863 section 6.3.1 to determine the simulation methodology, but with modifications in following issues.

- Recommended WF

  - Agree on Option 1.

Issue 2-2: TN isolation distance

- Proposals

  - Option 1(Ericsson): For coexistence simulations in the NTN Ka-band, RAN4 should not consider any isolation area in a first step.

- Recommended WF

  - TBA

Issue 2-3:  Network and UE deployment

- Proposals

  - Option 1(Samsung): the deployment of Dense Urban micro base station should follow TR38.803.

  - Option 2 (Ericsson): see Annex 2

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Issue 2-1: Base line

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Huawei | If isolation area is not needed, FR2 TN network layout model should be further studied. |
| Qualcomm | Option 1 |
| Nokia, Nokia Shanghai Bell | Agree with recommended WF. |
| ZTE | Fine with option 1 |
| THALES | Sure |
| Hughes/EchoStar | OK |
| Samsung | Option 1 |
| CATT | Fine with recommended WF. |

Issue 2-2: TN isolation distance

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Huawei | More studies are needed. |
| Qualcomm | Need more discussion. VSAT will only support satellite functionality? |
| ZTE | Yes, baseline should be no isolation, further evaluation is needed |
| THALES | All companies seem to have basic understanding that similar FR1 approach should be used. Maybe we should avoid un-necessary discussions. |
| Hughes/EchoStar | Need more discussion. - VSAT uses are directional antennas |
| Samsung | Agree with option 1. Meanwhile, whether the isolation area is valid for protect the TN system, it should be proved. And we think this should be further discussed. |
| CATT | Need more study |

Issue 2-3:  Network and UE deployment

| Company | Comments |
| --- | --- |
| Moderator | Note that Option 1 and Option 2 are not opposite and can be merged. |
| Ericsson | Fine with both options but pending on Issue 1-8 conclusion. |
| Huawei | It’s related to the scenario. Issue is too general. More studies are needed. |
| ZTE | Fine with both options. |
| THALES | We should use Urban Macro and Rural Macro if we use similar approach as for TR 38.863. |
| Hughes/EchoStar | Similar to FR1 - use Urban Macro and Rural Macro |
| Samsung | Agree with Moderator’s proposal. |
| CATT | Need more discussion on TN scenario. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| N/A | N/A |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Issue 2-1: Base line | Tentative agreements: As the baseline, TR38.863 section 6.2.1 will be referred to to determine the FR2 TN network layout model and TR 38.863 section 6.3.1 will be referred to to determine the simulation methodology, but with modifications to but not limited to network layout, TN isolation distance, N/W&UE deployment, etc. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 2-2: TN isolation distance | Tentative agreements: N/A Candidate options: - Option 1: Not to consider isolation area for NTN coexistence study in above 10GHz in a first step. - Option 2: Consider isolation area for NTN coexistence study in above 10GHz Recommendations for 2nd round: Further discuss two options. |
| Issue 2-3: Network and UE deployment | Tentative agreements: N/A Candidate options: Existing options Recommendations for 2nd round: Further discuss two options and focus on Annex 2 table. |

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| N/A | N/A |

Discussion on 2nd round

Open issues and view collection

Issue 2-2: TN isolation distance

- Proposals

  - Option 1: Not to consider isolation area for NTN coexistence study in above 10GHz in a first step.

  - Option 2: Consider isolation area for NTN coexistence study in above 10GHz

- Recommended WF

  - TBA

| Company | Opt.1 (Y/N) | Opt.2 (Y/N) | Comments |
| --- | --- | --- | --- |
| Ericsson | Y | N, as a starting point | This should the assumption to start with and, as we did for FR1, if any isolation is needed, we could introduce it, but not yet at this stage. |
| Samsung | Y | N | In the first step of coexistence study should not consider any limitation for the NTN system in order to evaluate the interference situation |

Issue 2-3:  Network and UE deployment

- Proposals

  - Option 1: Comment on Annex 2.

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Moderator | Some issues are related to this topic, e.g. Issue 3-7. |
| Ericsson | Ok with the annex |
| Samsung | Ok with the annex |

Summary for 2nd round

| Issue 2-2: TN isolation distance | GTW Agreement (Oct.18): Not to consider isolation area for NTN coexistence study in above 10GHz in a first step. |
| --- | --- |
| Issue 2-3: Network and UE deployment | Agreements: N/A Candidate options: Annex2 Recommendations for way forward: FFS |

Topic #3: System parameters

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215348 | Thales | Proposal 1: For the coexistence analysis in Ka band, RAN4 to use the following VSAT UE parameters Table x. VSAT UE Parameters VSAT UE Parameters; Tx (Uplink); Rx (Downlink). Polarisation; Circular; Circular. Low Frequency; (MHz); 27 500; 17 700. Centre frequency; 28 750; 18 950. High frequency; 30 000; 20 200. Efficiency; 60%; 57%. On-axis antenna gain at Fc; (dBi); 42,9; 39,0. Output power; (W); 2. Output power; (dBW); 3,0. Output loss; (dB); -1,0. EIRP; 44,9. Receiver noise figure; (dB); 1,2. Feeder loss; (dB); -0,50. Sky temperature; (K); 30. Ground temperature; (K); 10. Antenna temperature; (K); 40. G/T figure of merit; (dB/K); 16,5. Proposal 2: With respect to VSAT UE secondary lobes and related coexistence analysis, RAN4 to use the recommendation from ITU-R S.465-5: |
| R4-2215777 | Samsung | Proposal 5: The proposed set-1 and set-2 satellite parameters for co-existence study are in the Table 2.3-1, 2.3-2 and 2.3-3. Proposal 6: Propose Table 2.3-4 as the noise figure parameter of NTN system. Proposal 7: Propose Table 2.3-5 as the UE parameter of NTN system. Proposal 8: Horizontal boresight and Vertical tilt assumptions of NTN UEs should be discussed and decided by the meeting. Proposal 9: Propose Table 2.3-6, 2.3-7 and 2.3-8 as the BS parameter of TN system. Proposal 10: Propose Table 2.3-9 as the ACS and ACLR assumptions of TN system. Proposal 11: Refers to TR 38.863 to determine the Satellite and UE Antenna and beam forming pattern modelling. Proposal 12: Propose Table 2.4-1 and 2.4-2 as the antenna pattern assumptions of TN BS and UE. |
| R4-2216517 | Ericsson | Proposal6: For coexistence simulations in the NTN Ka-band, RAN4 should not consider any isolation area in a first step. Proposal7: Annex 1. 6.2 and Annex 2 |
| R4-2216557 | ZTE | Propose to use the simulation assumption in this contribution as baseline for NTN Ka band coexistence study. |

Open issues summary

Sub-topic 3-1

Issue 3-1: Satellite parameter baseline

- Proposals

  - Option 1: Refer to Set-1 satellite table in TR 38.821

  - Option 2: Refer to Set-1 and Set-2 satellite tables in TR 38.821

- Recommended WF

  - TBA

Issue 3-2: NRB configuration per BandWidth size and SCS

- Proposals

  - Option 1(Ericsson):

| Configuration FR2 Ka-band | NRB (100MHz BW) |
| --- | --- |
| SCS 120 kHz | 66 |

- Recommended WF

  - TBA

Issue 3-3: Set-1 NTN satellite Noise figure in dB

- Proposals

  - Option 1(Ericsson, Samsung):

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 28 | 13 | 13 |
| G_Rx (dBi) | 58.5 | 38.5 | 38.5 |
| NF (dB) | 5.9 | 0.9 | 0.9 |

- Recommended WF

  - Agree on Option 1

Issue 3-4: Number of active UE (UL)

- Proposals

  - Option 1(Ericsson): 9 UEs and nRBs per UE for GEO and LEO1

  - Option 2 (Samsung): FFS

- Recommended WF

  - TBA

Issue 3-5: NTN Fixed VSAT UE

- Proposals

| Characteristics | Option 1 (Ericsson, Samsung, ZTE) | Option 2 (Thales) |
| --- | --- | --- |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |  |
| Polarisation | Circular | Circular |
| Efficiency |  | UL 60%, DL 57% |
| Rx Antenna gain | 39.7 dBi | 39 dBi |
| Noise figure | 1.2 dB | 1.2 dB |
| Rx Feeder loss |  | -0.5 dB |
| Antenna temperature | 150 K | 40K |
| Sky temperature |  | 30 K |
| Ground temperature |  | 10 |
| G/T figure of merit |  | 16.5 (dB/K) |
| Rx Feeder loss |  | -0.5 dB |
| Tx transmit power | 2 W (33 dBm) | 2W (3dBW) |
| Tx antenna gain | 43.2 dBi | 42.9 dBi |
| Output loss |  | -1.0 |
| EIRP |  | 44.9 dBm |
|  | NOTE: VSAT terminal characteristics could be implemented with phased array antenna |  |

- Recommended WF

  - TBA

Issue 3-6: NTN movable VSAT UE

- Proposals

  - Option 1(Ericsson, Samsung):

| Characteristics | ESIM (TR 38.821) | ESIM (ITU) |
| --- | --- | --- |
| Antenna type and configuration | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) | Antenna pattern: S.580-6 |
| Polarisation | Linear: +/-45°X-pol |  |
| Rx Antenna gain | TBD dBi per element | 45 dBi |
| Antenna temperature | TBD K |  |
| Noise figure | TBD dB |  |
| Tx transmit power | [TBD W (TBD dBm)] | Transmit power density（dBW/Hz）-46.3 |
| Tx antenna gain | TBD dBi per element | 45 dBi |

  - Option 2 (Thales): With respect to VSAT UE secondary lobes and related coexistence analysis, RAN4 to use the recommendation from ITU-R S.465-5:

- Recommended WF

  - TBA

Issue 3-7: Horizontal boresight and Vertical tilt assumptions of NTN UEs

- Proposals

  - Option 1:

|  | GEO | LEO |
| --- | --- | --- |
| Antenna vertical tilt | 20 degree above horizontal | 30 degree above horizontal |
| Antenna horizontal boresight | FFS | FFS |

- Recommended WF

  - TBA

Sub-topic 3-2

Issue 3-8: TN parameters

- Proposals

|  | Option 1(Ericsson) | Option 2(Samsung) |  | Option 3(ZTE) |  |
| --- | --- | --- | --- | --- | --- |
|  | Urban macro | Urban macro | Dense urban | Urban macro | Dense urban |
| Carrier frequency in GHz | 20 / 30 | 20 / 30 | 20 / 30 | 30 | 30 |
| Size of each nominal channel BW in MHz | 100 | 200 | 200 | 400MHz/200MHz | 400MHz/200MHz |
| Transmission bandwidth in MHz | N/A |  |  |  |  |
| Environment | Deployment scenario related, check Table below | Urban macro | Dense urban | Urban macro | Dense urban |
| Network layout | 19-sites 57 sectors with wrap-around | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | Fixed cluster circle within a macro cell. |  |  |
| Inter-site distance in meter | Deployment scenario related, see Table below | 200m (baseline) 300m (optional) |  |  |  |
| Number of micro BSs per macro cell |  |  | 3 Note: 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |  |  |
| Radius of UE dropping within a micro cell |  |  | < 28.9 m |  |  |
| System loading and activity | See Annex 2 |  |  |  |  |
| Network location | See Annex 2 |  |  |  |  |
| DL subcarrier spacing | 120kHz |  |  |  |  |
| UL | OFDMA |  |  |  |  |
| DL power control | No | No | No |  |  |
| UL power control | 36.942 | Yes | Yes |  |  |
| Frequency reuse | 1 |  |  |  |  |
| Number of scheduled UE per cell (DL) | 1 | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| Number of scheduled UE per cell (UL) | 3 | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| UE antenna height in meter | Same as 3D-UMi in TR 36.873 | Same as 3D-UMi in TR 36.873 | Same as 3D-UMi in TR 36.873 |  |  |
| UE TX power in dBm | -40 to 23 | -40 to 23 | -40 to 23 | -40 to 22.4 | -40 to 22.4 |
| UE antenna gain in dBi | 0 |  |  |  |  |
| Building penetration loss | In pathloss model, TR 38.901 |  |  |  |  |
| Cell selection margin in dB | 3 | 3 | 3 | 3 | 3 |
| BS-MS min distance in meters | 35 | 35 | 3 |  |  |
| BS noise figure in dB | 5 | 11 | 11 | 11 | 11 |
| UE noise figure in dB | 9 | 9 | 9 | 9 | 9 |
| BS-UE path-loss model | TR 38.901 |  |  |  |  |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |  |  |  |  |
| Shadowing correlation | Inter-cell 0.5 Intra-cell 1 | Between cells: 1.0 Between sites: 0.5 | Between cite: 0.5 |  |  |
| Link-level performance model | See Throughput-SINR mapping |  |  |  |  |
| UE distribution | Uniform | Uniform | Uniform |  |  |
| Evaluation metrics | See Throughput loss criteria |  |  |  |  |

|  | Option 1(Ericsson) | Option 2(Samsung) |  | Option 3(ZTE) |  |
| --- | --- | --- | --- | --- | --- |
|  | Urban Macro | Urban macro | Dense urban | Urban macro | Dense urban |
| ISD in meters | 200 |  |  | 200 | 200 |
| BS Antenna height in meters | 25 | 25 | 10 | 25 | 10 |
| UE Outdoor/indoor | 100% outdoor | 80% outdoor 20% indoor | 20% outdoor 80% indoor | 100% Outdoor | 100% Outdoor |
| UE height in meter | 1.5 |  |  | 1.5 | 1.5 |

- Recommended WF

  - TBA

Issue 3-9: ACS and ACLR assumptions of TN system

- Proposals

  - Option 1:

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 28 dB |
|  | ACS | [23.5/24] dB |
| UE | ACLR | 17dB |
|  | ACS | 22dB |

- Recommended WF

  - TBA

Sub-topic 3-3

Issue 3-10: Satellite and UE Antenna and beam forming pattern modelling

- Proposals

  - Option 1: Refer to TR 38.863 with Ka parameters in TR 38.821.

  - Option 2: TBA

- Recommended WF

  - Agree on Option 1 and further check the assumption in 2nd round.

Issue 3-11: TN BS antenna modelling

- Proposals

  - Option 1(Ericsson):

|  | Macro urban |
| --- | --- |
| Base Station Antenna Characteristics |  |
| Antenna pattern | TR 38.921 |
| Element gain GE,max (dBi) | 5.5 |
| Horizontal 3dB /vertical 3dB 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | dh = 0.5  dv = 0.5  |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Base station maximum coverage angle in the horizontal plane (degrees) range | -60 to 60 |
| Base station vertical coverage range (degrees) range | 90-105 |
| Mechanical downtilt (degrees) | 10 |

  - Option 2 (ZTE):

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| (Mg, Ng, M, N, P) | For 30 GHz: (1, 1, 8, 16, 2) Note1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. |  |

  - Option 3 (Samsung):

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Antenna Downtilt | 10 degree |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

- Recommended WF

  - TBA

Issue 3-12: TN UE antenna modelling

- Proposals

  - Option 1(Ericsson, ZTE, Samsung):

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Issue 3-1: Satellite parameter baseline

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Huawei | Option 3, others. It’s better to clarify the satellite type. I think phased array antenna is applicable to Ka band satellite. If so, we should study the antenna configuration firstly instead of reusing TR 38.821. |
| Nokia, Nokia Shanghai Bell | We support option 1. |
| ZTE | We support the Option 1 |
| THALES | Set-1 for FR2, as we did Set-1 for FR1. Set-1 (from TR 38.821) corresponds to higher antenna gain and higher sensitivity which is the worst case, compared to satellite Set-2 (from TR 38.821). We had similar discussions for TR 38.863. We should follow similar process as for FR1 and avoid un-necessary discussions. |
| Hughes/EchoStar | Option 1 is fine |
| Samsung | Set-1 satellite could be the first priority of the coexistence study. But it not means that Set-2 no need to be considered, unless we could prove the interference is same with Set-1 or less then Set-1. However, even its interference is lower then Set-1, how to determine the ACLR should be discussed as well. |
| CATT | Fine with option 1. |

Issue 3-2: NRB configuration per BandWidth size and SCS

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Huawei | Why don’t we consider 60kHz. I think 200MHz is more general for Ka band. |
| Nokia, Nokia Shanghai Bell | We can be OK with option 1 depending on other discussions. |
| ZTE | It should depend on the previous channel bandwidth discussions. |
| THALES | 120 kHz is reasonable in FR2. With respect to channel bandwidth we should maybe use worst case (which is 400 MHz). For FR1 we also considered highest available bandwidth for the respective band configuration. |
| CATT | Same comment as Huawei. |

Issue 3-3: Set-1 NTN satellite Noise figure in dB

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Huawei | Option 2, Others. It seems that the NF in Ka band is stricter than FR1. It’s better to clarify the antenna configuration firstly and then we can get the antenna gain, reevaluate G/T. |
| ZTE | Similar concerns as huawei. |
| THALES | We should follow TR 38.821 for satellite basic assumptions. Basically ok with Option 1 if derived from 38.821. |
| Hughes/EchoStar | Agree with Thales |
| Samsung | Option 1 |
| CATT | Same view as Thales. |

Issue 3-4: Number of active UE (UL)

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 a priori but could be further discussed, of to keep it FFS as well. |
| Huawei | It’s based on the UL budget. Maybe we don’t need so many UEs in Ka band. |
| Qualcomm | Need more study |
| ZTE | More study is needed. |
| THALES | Option 1 seems ok as basic assumption. |
| Hughes/EchoStar | Option 2 |
| Samsung | Satellite vendors can provide clarifications for the UE and RB parameters for the Ka-band. These assumptions might be different with the S-band. |
| CATT | Need more study |

Issue 3-5: NTN Fixed VSAT UE

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 is from TR 38.821, we would like to understand the rationale for the differences in option 2 to agree on it. |
| Huawei | Firstly, we should study the Antenna type and configuration. For Ka band, it’s better to assume phased array antenna. We need to further study the parameters after we have an agreement on Antenna type and configuration. |
| ZTE | Similar comments as Ericsson. |
| THALES | We provided VSAT UE parameters based on the considered harmonized Ka band. Please check THALES paper R4-2215348. |
| Hughes/EchoStar | Need further checking between the two |
| Samsung | Agree with the existing parameters in option1, unless Option 2 can be explained clearly why these assumptions are not same with the TR 38.821 during the Ka-band coexistence study. |
| CATT | Need to further check the value difference between options. |

Issue 3-6: NTN movable VSAT UE

| Company | Comments |
| --- | --- |
| Ericsson | The proposed parameters in option 1 are the ones from TR 38.821 (all are TBD) and from the ITU study on ESIM. We would prefer option 1 then, option 2 would need further discussion. |
| Huawei | Firstly, we should study the Antenna type and configuration. And further check ITU-R recommendation. |
| ZTE | We need to further discuss phase array for VSAT UE similar as in thread 312. |
| THALES | The VSAT UE parameters were not clearly defined in TR 38.821. We provided new assumptions based on the antenna lobes/1st order Bessel assumptions, with respect to considered fc and λ. Basically all VSAT UE antenna parameters have been adapted from TR 38.821 by considering the following parameters adapted to Ka satellite band: - Downlink frequency: 17.7 - 20.2 GHz - Uplink frequency: 27.5 GHz - 30 GHz - 60 cm antenna aperture diameter (2*a=60cm) Please refer to R4-2215348 for details. For the adjacent band interference, we could actually use Option 2. |
| Hughes/EchoStar | Need further checking but agree with the frequency range proposed by Thales |
| Samsung | Option 2 is one of the typical antenna pattern models from ITU-R. However, it should be noted that there is an issue in the ITU-R discussion: the power normalization is large than 1 by using the S.465 and S.580. This issue should be clarified and confirmed to make sure it would not affect the coexistence study in RAN4. |
| CATT | Need further check, It seems that option 2 is more straightforward. |

Issue 3-7: Horizontal boresight and Vertical tilt assumptions of NTN UEs

| Company | Comments |
| --- | --- |
| Ericsson | If we assume such antenna vertical tilt, would that mean we are expecting the satellite is not at nadir point then? |
| Huawei | It should be aligned with satellite beam direction. |
| ZTE | Not in the nadir point for NTN UE? With evelvation angle for it? |
| THALES | Agree with Huawei. FFS |
| Samsung | As a clarification, the values we proposed are ranges for consideration, not a fixed value. For the S-band coexistence study, the VSAT UE antenna pattern is Omni antenna pattern. Vertical tilt and horizontal boresight are not necessary to be considered. However, for the Ka-band VSAT UE, Circular (ITU-R S.465 or S.580) and antenna pattern (3GPP multi element antenna pattern) are mentioned to be used. Therefore, vertical tilt and horizontal boresight are needed. For the GEO satellite: - fixed VSAT UE antenna would be fixed in one direction point (fix vertical and horizontal degrees) to the sky. - Movable VSAT UE beam direction or antenna direction are changing during the movement of the UE. But in this scenario, GEO satellite very far away from the earth, and it is possible to simply the direction as fixed. For the LEO satellite: - The lower a satellite's altitude, the higher its speed is. VSAT UE antenna should trace the satellite in order to maintain the connection with satellite. VSAT UE will reconnect with next suitable satellite, until current connection satellite leaves the lowest connection degree. Therefore, during the coexistence study, VSAT UE vertical and horizontal degrees should be assumed by some methods, e.g. randomly generated with some limit, move with the satellite tracking (modelling the satellite movement), or determine some fixed value, 30 degree, 45 degree etc. More details are needed to be discussed. |

Issue 3-8: TN parameters

| Company | Comments |
| --- | --- |
| Ericsson | Many parameters are common to all proposals. Our initial thinking is that all TN UEs are outdoor to align with FR1 assumptions. We might not want to run simulations with the optional ISD (300m). The channel BW would need further discussion and alignment. |
| Huawei | For FR2 TN UE, does we only assume handheld UE? Or we can consider other UE type? |
| Qualcomm | To follow the parameters defined in TR38803 for TN |
| ZTE | The same understanding as Qualcomm |
| THALES | Actually, proposals do not seem too different. Fine to use TR 38.803. We could also combine Option 2 for instance with higher BW configuration (400 MHz). However, we think is better to consider Urban Macro and Rural Macro as we did for FR1. Please see R4-2215352: Proposal 3: Leveraging the work in FR1, RAN4 shall focus on Ka-band coexistence analysis using urban and rural macro scenarios. |
| Hughes/EchoStar | We believe it is handheld UE for TN FR2 |
| Samsung | We are OK to only consider outdoor UE. And we share the same view with Huawei that other UE types may be considered as well. |
| CATT | TN scenario needs to be agreed firstly. |

Issue 3-9: ACS and ACLR assumptions of TN system

| Company | Comments |
| --- | --- |
| Moderator | Which BS ACS value do you prefer? |
| Ericsson | TN BS ACS should be 24 dBi to align with NR TS, right? |
| Nokia, Nokia Shanghai Bell | We have a preference to align with TN ACS of 24 dB. |
| ZTE | It should be 24dBc which is round up from 23.5dB. |
| THALES | Fine with the assumptions. Also, 0.5 dB for ACS BS is not a huge difference.. In any case, we should be aligned with NR TS values. |
| Samsung | Fine to use 24dB |
| CATT | Same comment as Thales. |

Issue 3-10: Satellite and UE Antenna and beam forming pattern modelling

| Company | Comments |
| --- | --- |
| Ericsson | UE Antenna is also depending on issues 3-5 and 3-6 |
| Huawei | Option 2. Phased array antenna pattern modelling should be considered. |
| ZTE | Fine with recommended WF. |
| THALES | Fine with the WF and assumptions. However, please also note that adaptations have been made with respect to Tx and Rx frequency bands with respect to VSAT UE parameters. |
| Samsung | Fine with the WF. |

Issue 3-11: TN BS antenna modelling

| Company | Comments |
| --- | --- |
| Moderator | No big differences except the Horizontal 3dB /vertical 3dB (90 or 65) and the consideration of 10 degree down tilt. |
| Ericsson | Option 1: the parameters are normalized and aligned with the latest agreed antenna model for 6GHz. We don’t think other parameters are normalized. |
| Huawei | We’d like to know the rational on different beam width. |
| Qualcomm | To reuse the antenna modelling specified in TR38803 |
| ZTE | The same understanding as Qualcomm |
| THALES | We should reuse TR 38.803 assumptions. |
| Samsung | Reuse TR38.803 with the missed antenna assumption 10degree down-tilt. |
| CATT | Support to follow TR38803. |

Issue 3-12: TN UE antenna modelling

| Company | Comments |
| --- | --- |
| Qualcomm | Agree with option 1 which reuses the antenna modelling specified in TR38803 |
| ZTE | Agree with option 1 |
| THALES | We should reuse TR 38.803 assumptions. |
| Samsung | Agree with option 1 |
| CATT | Agree with option 1 |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| N/A | N/A |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Issue 3-1: Satellite parameter baseline | Tentative agreements: - Set 1 satellite table from TR 38.821 can be used as baseline. - Await the conclusion of discussion on AAS antenna in [140] Candidate options: N/A Recommendations for 2nd round: Further discuss AAS antenna matter. |
| Issue 3-2: NRB configuration per BandWidth size and SCS | Tentative agreements: Await relative discussion in [312] Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 3-3: Set-1 NTN satellite Noise figure in dB | Tentative agreements: - Option 1 (derived from TR 38.821) as baseline. - Further discuss the matter after the antenna configuration is settle. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 3-4: Number of active UE (UL) | Tentative agreements: N/A Candidate options: - Option 1: 9 UEs and nRBs per UE for GEO and LEO1 - Option 2: FFS Recommendations for 2nd round: Further discuss other values. Satellite operators and vendors are encouraged to provide a number. |
| Issue 3-5: NTN Fixed VSAT UE | Tentative agreements: N/A Candidate options: - Existing options Recommendations for 2nd round: Further discuss two options. |
| Issue 3-6: NTN movable VSAT UE | Tentative agreements: N/A Candidate options: - Existing options Recommendations for 2nd round: Further discuss two options. |
| Issue 3-7: Horizontal boresight and Vertical tilt assumptions of NTN UEs | Tentative agreements: N/A Candidate options: - Existing Option 1 Recommendations for 2nd round: Further discuss this matter in connection with Issue 2-3. |
| Issue 3-8: TN parameters | Tentative agreements: Follow parameters defined in TR 38.803 with modifications. - 100% outdoor Candidate options: - BW to be further confirmed in Issue 1-3 - Scenarios to be further confirmed in Issue 1-8 Recommendations for 2nd round: N/A |
| Issue 3-9: ACS and ACLR assumptions of TN system. | Tentative agreements: NR. BS; ACLR; 28 dB. ACS; 24dB. UE; ACLR; 17dB. ACS; 22dB. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 3-10: Satellite and UE Antenna and beam forming pattern modelling | Tentative agreements: Await result of other issues. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 3-11: TN BS antenna modelling | Tentative agreements: Follow TR 38.803 assumption with possible modification. Candidate options: Issue 3-11-1 Horizontal 3dB /vertical 3dB 3 dB beam width of single element (degree) - Option1: 65 - Option2: 90 Issue 3-11-2 Antenna Downtilt - Option1: 10 degree - Option2: Others Recommendations for 2nd round: Further discuss candidate options |
| Issue 3-12: TN UE antenna modelling | Tentative agreements: Option 1 from TR 38.803. Candidate options: N/A Recommendations for 2nd round: N/A |

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| N/A | N/A |

Discussion on 2nd round

Open issues and view collection

Issue 3-4: Number of active UE (UL)

- Proposals

  - Option 1: 9 UEs and nRBs per UE for GEO and LEO

  - Option 2: Others

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Moderator | nRBs in Option 1 needs to be figured out. |
| Ericsson | In the 1st round, the majority of companies would prefer further study this, we are ok to come back in next meeting then. |

Issue 3-5: NTN Fixed VSAT UE

- Proposals

| Characteristics | Option 1 (Ericsson, Samsung, ZTE) | Option 2 (Thales) |
| --- | --- | --- |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | [2] with 60 cm equivalent aperture diameter adapted for Ka-band |
| Polarisation | Circular | Circular |
| Efficiency |  | UL 60%, DL 57% |
| Rx Antenna gain | 39.7 dBi | 39 dBi |
| Noise figure | 1.2 dB | 1.2 dB |
| Rx Feeder loss |  | -0.5 dB |
| Antenna temperature | 150 K | 40K |
| Sky temperature |  | 30 K |
| Ground temperature |  | 10 |
| G/T figure of merit |  | 16.5 (dB/K) |
| Rx Feeder loss |  | -0.5 dB |
| Tx transmit power | 2 W (33 dBm) | 2W (3dBW) |
| Tx antenna gain | 43.2 dBi | 42.9 dBi |
| Output loss |  | -1.0 |
| EIRP |  | 44.9 dBm |
|  | NOTE: VSAT terminal characteristics could be implemented with phased array antenna |  |

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Samsung | Option 2 needs more clarification to explain the different with option 1. |
| THALES | Option 2 considers a higher band range, and therefore less antenna gain. Is more realistic considering the provided ranges. Please see THALES proposal. |

Issue 3-6: NTN movable VSAT UE

- Proposals

  - Option 1(Ericsson, Samsung):

| Characteristics | ESIM (TR 38.821) | ESIM (ITU) |
| --- | --- | --- |
| Antenna type and configuration | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) | Antenna pattern: S.580-6 |
| Polarisation | Linear: +/-45°X-pol |  |
| Rx Antenna gain | TBD dBi per element | 45 dBi |
| Antenna temperature | TBD K |  |
| Noise figure | TBD dB |  |
| Tx transmit power | [TBD W (TBD dBm)] | Transmit power density（dBW/Hz）-46.3 |
| Tx antenna gain | TBD dBi per element | 45 dBi |

  - Option 2 (Thales): With respect to VSAT UE secondary lobes and related coexistence analysis, RAN4 to use the recommendation from ITU-R S.465-5:

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Moderator | Answers to questions in 1st round (copied below) are encouraged. Q: Option 2 is one of the typical antenna pattern models from ITU-R. However, it should be noted that there is an issue in the ITU-R discussion: the power normalization is large than 1 by using the S.465 and S.580. This issue should be clarified and confirmed to make sure it would not affect the coexistence study in RAN4. |
| Ericsson | From the other thread, fixed and mobile VSAT will have same technical characteristics. If so, they should have same parameters. But what’s still remain unclear is if VSAT could have an antenna array (as proposed also in TR 38.821…). If so, we need to simulate this case as well. |
| Samsung | Option 1. Option 2 can be further discussed once the question can be answered |
| Hughes/EchoStar | Fixed and Mobile VSAT should have similar characteristics, the difference can be the antennas |
| Hispasat | Antenna implementation should not be considered as a parameter, as both Movable and Fixed VSAT may be implemented by both technologies, and also both Movable and Fixed VSAT may have similar characteristics. That’s way the proposal in the parallel thread is to start with a common UE type. Note. VSAT is just a “de facto” terminology while ESIM is a type of service (implemented by “movable VSATs”, proposal is to unify as Movable/Fixed NTN UE/terminal. |
| THALES | Both VSAT or ESIM can be steerable, and both can use parabolic antennas or phased array. Therefore, it should not be too much differentiation. Same parameters could be therefore considered for both cases in the coexistence analysis. |

Issue 3-7: Horizontal boresight and Vertical tilt assumptions of NTN UEs

- Proposals

  - Option 1:

|  | GEO | LEO |
| --- | --- | --- |
| Antenna vertical tilt | 20 degree above horizontal | 30 degree above horizontal |
| Antenna horizontal boresight | FFS | FFS |

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Moderator | Please refer to the clarification on this parameter made in 1st round. |
| Ericsson | Thanks to Samsung for the clarification, it sounds relevant but that would need further consideration. We propose to come back in next meeting. |
| Samsung | To highlight the issue again, if the antenna pattern of VSAT UE is not Omni antenna pattern, its need vertical and horizontal angles to do simulation. Otherwise, there antenna gain cannot be calculated. Therefore, the modelling of the Antenna vertical tilt and Antenna horizontal boresight are necessary for the coexistence study. |

Issue 3-11: TN BS antenna modelling

Issue 3-11-1 Horizontal 3dB /vertical 3dB 3 dB beam width of single element (degree)

- Proposals

  - Option1: 65

  - Option2: 90

- Recommended WF

  - TBA

| Company | Opt.1 (Y/N) | Opt.2 (Y/N) | Comments |
| --- | --- | --- | --- |
| Moderator |  |  | Question in 1st round: We’d like to know the rational on different beam width. |
| Ericsson | N | Y | This is aligned with TR 38.921, which is the latest model agreed by RAN4. |
| Samsung | Y | Y |  |

Issue 3-11-2 Antenna Downtilt

- Proposals

  - Option1: 10 degree

  - Option2: other values

- Recommended WF

  - TBA

| Company | Opt.1 (Y/N) | Opt.2 (Y/N) | Comments |
| --- | --- | --- | --- |
| Ericsson | Y | N |  |
| Samsung | Y | N | Option 1 is the common assumption for the TN BS antenna down-tilt. |

Summary for 2nd round

|  | Status summary |
| --- | --- |
| Issue 3-4: Number of active UE (UL) | Agreements: N/A Recommendations for way forward: FFS |
| Issue 3-5: NTN Fixed VSAT UE | Agreements: N/A Recommendations for way forward: FFS, as well as UE height |
| Issue 3-6: NTN movable VSAT UE | Agreements: N/A Recommendations for way forward: FFS, as well as UE height |
| Issue 3-7: Horizontal boresight and Vertical tilt assumptions of NTN UEs | Agreements: N/A Recommendations for way forward: FFS |
| Issue 3-11: TN BS antenna modelling | GTW Agreement (Oct.18): follow TR 38.803 assumption as baseline with possible modifications on the parameters listed in issue 3-11-1/3-11-2. |
| Issue 3-11-1 Horizontal 3dB /vertical 3dB 3 dB beam width of single element (degree) | Agreements: N/A Candidate options: - Option1: 65 - Option2: 90 Recommendations for way forward: FFS |
| Issue 3-11-2 Maximum directional gain of an antenna element GE,max | Agreements: N/A Candidate options: - Option1: 5.5 dBi - Option2: 8 dBi |
| Issue 3-11-3 Antenna Downtilt | GTW Agreement (Oct.18): 10 degree as starting point for intial simulation purpose, other options not precluded |

Topic #4: Evaluation methodology

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215348 | Thales | Proposal 3: With respect to link budget analysis for Ka band, RAN4 to use explained methodology from TR 38.821, TR 38.811. |
| R4-2215777 | Samsung | Proposal 13: Refers to TR38.803 section 5.2.6 to determine ACLR and ACS modelling. Proposal 14: Refers to TR38.863 section 6.2.5 and TR38.811 section 6.6 to determine the propagation model. Proposal 15: Refers to TR38.863 section 6.2.6 to determine the transmission power control model. Proposal 16: Refers to TR38.863 section 6.2.7 to determine the reveived power model. Proposal 17: Refers to TR38.863 section 6.2.8 to determine the Performance metric. Proposal 18: Refers to TR38.863 section 6.2.9 to determine throughput to SNR mapping. |
| R4-2216517 | Ericsson | Proposal7: Adopt the assumptions in Annexes 1 and 2 for the coexistence simulations in NTN Ka-band. |

Open issues summary

Sub-topic 4-1

Issue 4-1: ACIR modelling

- Proposals

  - Option 1 (Ericsson, ZTE): ACIR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942 is used as baseline.

  - Option 2 (Samsung): Refers to TR38.803 section 5.2.6 to determine ACLR and ACS modelling.

- Recommended WF

  - TBA

Issue 4-2: Propagation model baseline

- Proposals

  - Option 1: Refers to TR 38.863 Clause 6.2.5.1, 6.2.5.2 and 6.2.5.3

- Recommended WF

  - TBA

Issue 4-3:  Additional losses between NTN SAN and UE

- Proposals

  - Option 1(Ericsson): Following losses should be considered

| Atmospheric absorption | Figure 6 of ITU-R P.676 (XX) shows the corresponding zenith attenuation for frequencies between 1 and 350 GHz. For an elevation angle α, the corresponding attenuation is given by: |
| --- | --- |
| Rain and cloud attenuation | For system-level simulations, the baseline is to consider clear sky conditions only. No rain and cloud attenuation should then be considered. |
| Scintillation | The ionospheric scintillation corresponds to rapid fluctuations of the received signal amplitude and phase. Ionosphere propagation shall only be considered for frequencies below 6 GHz. This is not applicable then for coexistence system simulations for the NTN Ka-band. The tropospheric scintillation corresponds to rapid fluctuations of the received signal amplitude and phase. Tropospheric propagation shall only be considered for frequencies above 6 GHz. Table 6.6.6.2.1-1 from TR 38.821 could be used as starting point. |

- Recommended WF

  - TBA

Issue 4-4:  Propagation model between NTN UE and TN UE

- Proposals

  - Option 1(ZTE): Propagation model between NTN UE and TN UE should reference to the following propagation model,

- UE-to-UE: Umi (h_BS=1.5 m ~ 22.5 m)

+ penetration loss see TR 38.803

- Recommended WF

  - TBA

Issue 4-5:  TN UL TPC

- Proposals

  - Option 1: For uplink scenario, TPC model specified in Section 9.1 TR 36.942[8] is applied for TN with following parameters.

where:

-	Pmax = 23dBm,

-	Rmin = TBD dB,

-	CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

- Recommended WF

  - Option 1

Issue 4-7:  NTN and TN DL TPC

- Proposals

  - Option 1: Not applied

- Recommended WF

  - Option 1

Issue 4-8:  Received power model

- Proposals

  - Option 1: Refers to TR38.863 section 6.2.7 to determine the reveived power model.

- Recommended WF

  - Option 1

Issue 4-9:  Performance metric

- Proposals

  - Option 1:

    - For TN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

    - For NTN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

- Recommended WF

  - Option 1

Issue 4-10:  Throughput ~ SNR mapping

- Proposals

  - Option 1(Samsung): Refer to TR38.863 section 6.2.9 to determine throughput to SNR mapping.

  - Option 2(Ericsson):

Adopt Section 5.2.7 of TR 38.803 as the SINR-Throughput performance metrics, but , SNIRMIN, and SNIRMAX need to be further studied and decided for NR NTN. [Parameters below can be a starting point for α, SNIRMIN, and SNIRMAX, but other options are not precluded. Parameter; DL; UL; Notes. α, attenuation; [0.6]; [0.4]; Represents implementation losses. SNIRMIN, dB; [-10]; [-10]; Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL). SNIRMAX, dB; [30]; [22]; Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL). It’s infeasible to achieve 30dB/22dB DL/UL maximum SNIR for NR NTN, so the following parameters need to be further studied and RAN4 need to check them with RAN1.]

  - Option 3(ZTE):

The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation: Where: S(SNIR) Shannon bound, S(SNIR) =log2(1+SNIR) [bps/Hz] Attenuation factor, representing implementation losses SNIRMIN Minimum SNIR of the code set, dB SNIRMAX Maximum SNIR of the code set, dB The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 2 represent a baseline case, which assumes: - 1:1 antenna configurations - AWGN channel model - Link Adaptation (see table 2 for details of the highest and lowest rate codes) - No HARQ Table 2: Parameters describing baseline Link Level performance for 5G NR Parameter; DL; UL; Notes. α; 0.6; 0.4; Represents implementation losses. SNIRMIN, dB; -10; -10; Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL). SNIRMAX, dB; 30; 22; Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL).

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Issue 4-1: ACIR modelling

| Company | Comments |
| --- | --- |
| Ericsson | Option 2 would also be ok. |
| Huawei | OK with option 2. |
| Qualcomm | OK with option 2 |
| Nokia, Nokia Shanghai Bell | We are fine with option 2. |
| ZTE | Okay with option 2. |
| THALES | Option 2 is fine. In any case, now companies have all similar understanding. |
| Hughes/EchoStar | OK with option 2 |
| Samsung | OK with option 2 |
| CATT | OK with option 2, since TR38.803 is for NR TN coexistence. |

Issue 4-2: Propagation model baseline

| Company | Comments |
| --- | --- |
| Huawei | Ka band TN propagation should consider TR 38.803. |
| ZTE | Okay for it, however the details could be further checked in TR38.821 and 38.811 in case of any difference between 2GHz and Ka-band. |
| THALES | Agree with both Huawei and ZTE. |
| Samsung | Agree to further check the propagation model. |
| CATT | OK |

Issue 4-3:  Additional losses between NTN SAN and UE

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Huawei | It seems that we didn’t consider this in FR1. |
| ZTE | FFS, we need to double check it and come back to 2nd round. |
| THALES | We propose to follow similar process as for FR1. Is ok to neglect what is possible to neglect. |
| Hughes/EchoStar | Leverage work done in FR1 |
| Samsung | Option 1 |
| CATT | Agree with Thales. |

Issue 4-4:  Propagation model between NTN UE and TN UE

| Company | Comments |
| --- | --- |
| Ericsson | Why should we use Umi here? Further clarification would be needed. |
| Huawei | If scenario is Macro, Umi is not reasonable. |
| Qualcomm | Dual slope model is the typical PL model for UE-UE |
| ZTE | Just as clarified by Qualcomm which is for ue to ue propagation model. |
| THALES | Macro only, see FR1 approach. |
| Hughes/EchoStar | Leverage FR1 |
| Samsung | Agree with Qualcomm and ZTE. If the VSAT UE is a lower antenna (10m to 1.5m height), UMi is more suitable the UMa. Cause the Hbs is 10m in the TR 38.901, and this value would 25m for UMa. |
| CATT | We are fine to define Propagation model between NTN UE and TN UE. |

Issue 4-5:  TN UL TPC

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Qualcomm | Option 1 |
| ZTE | Option 1 |
| THALES | Ok for TN UE. |
| Samsung | Option 1 |
| CATT | Option 1 |

Issue 4-7:  NTN and TN DL TPC

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Qualcomm | Option 1 |
| ZTE | Option 1 |
| THALES | Ok for Option 1 |
| Samsung | Option 1 |
| CATT | Option 1 |

Issue 4-8:  Received power model

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| ZTE | Option 1 |
| THALES | Ok for Option 1 |
| Samsung | Option 1 |
| CATT | OK |

Issue 4-9:  Performance metric

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Qualcomm | Option 1 |
| ZTE | Option 1 |
| THALES | Usual process, ok for Option 1. |
| Samsung | Option 1 |
| CATT | OK |

Issue 4-10:  Throughput ~ SNR mapping

| Company | Comments |
| --- | --- |
| Moderator |  |
| Ericsson | All options are similar, right? |
| Qualcomm | Option 2 |
| ZTE | Option 1 and option 2 should be same. |
| THALES | Ok as basic assumptions |
| Samsung | Option 1 |
| CATT | Fine with all the options. Unless specifically stated, reference to TR 38.863 is preferable. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| N/A | N/A |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Issue 4-1: ACIR modelling | Tentative agreements: Refers to TR38.803 section 5.2.6 to determine ACLR and ACS modelling. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-2: Propagation model baseline | Tentative agreements: Refers to TR 38.863 Clause 6.2.5.1, 6.2.5.2 and 6.2.5.3 as baseline and further check TR38.821 and 38.811 in case of any difference between 2GHz and Ka-band. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-3: Additional losses between NTN SAN and UE | GTW Agreement (Oct.18): Option 1. |
| Issue 4-4: Propagation model between NTN UE and TN UE | Tentative agreements: N/A Candidate options: Existing options Recommendations for 2nd round: Further discuss Option 1. |
| Issue 4-5: TN UL TPC | Tentative agreements: Option 1 Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-7: NTN and TN DL TPC | Tentative agreements: Not applied Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-8: Received power model | Tentative agreements: Refers to TR38.863 section 6.2.7 to determine the reveived power model. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-9: Performance metric | Tentative agreements: For TN, the average throughput loss and 5%-ile throughput loss should be less than 5%. For NTN, the average throughput loss and 5%-ile throughput loss should be less than 5%. Candidate options: N/A Recommendations for 2nd round: N/A |
| Issue 4-10: Throughput ~ SNR mapping | Tentative agreements: Refer to TR38.863 section 6.2.9 to determine throughput to SNR mapping. Candidate options: N/A Recommendations for 2nd round: N/A |

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| N/A | N/A |

Discussion on 2nd round

Open issues and view collection

Issue 4-3:  Additional losses between NTN SAN and UE

- Proposals

  - Option 1(Ericsson): Following losses should be considered

| Atmospheric absorption | Figure 6 of ITU-R P.676 (XX) shows the corresponding zenith attenuation for frequencies between 1 and 350 GHz. For an elevation angle α, the corresponding attenuation is given by: |
| --- | --- |
| Rain and cloud attenuation | For system-level simulations, the baseline is to consider clear sky conditions only. No rain and cloud attenuation should then be considered. |
| Scintillation | The ionospheric scintillation corresponds to rapid fluctuations of the received signal amplitude and phase. Ionosphere propagation shall only be considered for frequencies below 6 GHz. This is not applicable then for coexistence system simulations for the NTN Ka-band. The tropospheric scintillation corresponds to rapid fluctuations of the received signal amplitude and phase. Tropospheric propagation shall only be considered for frequencies above 6 GHz. Table 6.6.6.2.1-1 from TR 38.821 could be used as starting point. |

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | Our proposal is based on TR 38.811, same approach as for FR1 then. The considered frequencies are different (FR1:2GHz vs FR2:20/30GHz) which also means some losses can’t be neglectable anymore (again, please TR 38.811…). We encourage companies to better study this. |
| Samsung | Option 1 |

Issue 4-4:  Propagation model between NTN UE and TN UE

- Proposals

  - Option 1(ZTE): Propagation model between NTN UE and TN UE should reference to the following propagation model,

- UE-to-UE: Umi (h_BS=1.5 m ~ 22.5 m)

+ penetration loss see TR 38.803

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | With the explanations given in the 1st round, ok with option 1. |
| Samsung | Option 1 |
| THALES | Depends if UMi or UMa for coexistence cases, not clear which one is more realistic. |

Summary for 2nd round

|  | Status summary |
| --- | --- |
| Issue 4-3: Additional losses between NTN SAN and UE | Agreements: Option 1 |
| Issue 4-4: Propagation model between NTN UE and TN UE | Agreements: N/A Candidate options: - Option 1: Propagation model between NTN UE and TN UE should reference to the following propagation model, - UE-to-UE: Umi (h_BS=1.5 m ~ 22.5 m) + penetration loss see TR 38.803 Recommendations for way forward: FFS |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-2217468 | WF on [313] NR_NTN_enh_Part2 | Samsung |  |
| R4-2217469 | Simulation assumptions for NTN co-existence in bands above 10GHz | Samsung |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| N/A |  |  |  |  |  |

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2217468 |  | Way forward on [313] NR_NTN_enh_Part2 | Samsung | Agreeable |  |
| R4-2217469 |  | Simulation assumptions for NTN co-existence study in bands above 10GHz | Samsung | Agreeable |  |

- Annex 1 TDOC list

A total of 5 TDOCs have been received for this agenda and listed as below.

| TDoc No. | Title | Source | Type | For | Agenda Item | Status |
| --- | --- | --- | --- | --- | --- | --- |
| R4-2215348 | VSAT UE Characteristics and Initial Simulation Parameters | THALES | discussion | Discussion | 6.22.2 | available |
| R4-2215352 | Discussion on Ka-band NTN-TN NR adjacent band coexistence scenarios | THALES, Lockheed Martin, Hispasat, Intelsat, Magister Solutions Ltd, Satellite Applications Catapult, ESA, Avanti, Hughes/EchoStar, Inmarsat, Eutelsat, Sateliot | discussion | Discussion | 6.22.2 | available |
| R4-2215777 | Simulation assumptions for above 10GHz NTN co-existence study | Samsung Electronics Nordic AB | discussion | Decision | 6.22.2 | available |
| R4-2216517 | NTN enhancement - coex simulations: scenarios and assumptions | Ericsson | other | Approval | 6.22.2 | available |
| R4-2216557 | Discussion on coexistence evaluation for NTN in Ka-band | ZTE Corporation | other | Approval | 6.22.2 | available |

- Annex 2 Issue 2-3 Option 2 of consideration of Network and UE deployment

Moderator’s Note: Case 1 to 6 are identical with the deployment for S-Band study, except the removal of the Note in Case 6. Case 7 & 8 are newly proposed.

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | All active TN clusters which has the NTN UE(s) at its edge. |

| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped at the edge of TN clusters | Consider an active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 3 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values) NTN UE: NTN UEs dropped outside or at the edge of TN clusters | TN clusters randomly placed in this NTN beam |  |

| 4 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters | TN randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |
| --- | --- | --- | --- | --- | --- | --- |

| 5 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 6 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |

| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which have the NTN UE(s) at its edge. |

| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs dropped outside or at the edge of TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| --- | --- | --- | --- | --- | --- | --- |