3GPP TSG-RAN WG4 Meeting # 104-bis-e 	R4-2217502

Electronic Meeting, October 10 – October 19, 2022

Source: 	Huawei, HiSilicon

Title: 	TP for deployment scenarios

Agenda Item:	6.4.1

Document for:	Approval

1 Introduction

New SID on NR BS RF requirement evolution was approved in RAN#95-e [1]. The TR skeleton for the SI dealing with FR2 multi-band BS deployments was approved in [2].

In this contribution, we provide a TP on deployment scenarios from benefit, limitation and example structure perspective.

2 TP for TR 38.877

4.2	Deployment scenarios

The scope of this study encompasses FR2 multi-band BS can support multiple FR2-1 mmWave bands transmission and/or reception through common active RF components. BS that supports multiple bands by means of separate antenna arrays and active components within the same enclosure are not in the scope of this study as they can already be supported by Rel-15 single band requirements and do not need further study.

FR2 BS capable of multi-band operation (whether those in the scope of this study, or BS supporting multiple bands by means of separate antenna arrays and active components within the same enclosure) should support evolution from single FR2-1 band to two FR2-1 bands application. Operators can configure for single band operation at one band initially and upgrade it to two bands sharing a common radio without further hardware investment.

Since a common active RF components are used for FR2 multi-band BS, the receiver and transmitter operate simultaneously would require very large isolation, which make it less possible for unsynchronized operation between bands. Hence inter-band synchronized operation is required for multi-band operation.

Possible FR2 multi-band BS structures and configurations based on combining different receiver/transmitter implementations (multi-band or single band), as well as mapping receive and transmit signals on a shared antenna or separate antennas should be considered in the study.

Scenarios 1), 4), 5) and 6) below should be considered as the target scenarios to be studied in this study while no study needed on the scenario 2) and 3) below:

1) Multi-band transmitter and/or receiver with common active RF components

2) Single-band transmitter and receiver

3) Configurable BS for different bands with the same hardware, i.e. only one band can be configured to operate at any time.

4) BS covers full-band or sub-band of band A and band B

5) BS covers consecutive spectrums with different band number, for example, n258+n261

6) BS covers two bands which have overlapping spectrums, for example, n258+n257, and BS support no overlapping frequency range in the two bands.

3 Conclusion

It is proposed to approve the TP for TR [2].

References

[1] RP-221001, “New SID: NR BS RF requirement evolution”, Huawei, HiSilicon

[2] R4-2214779, “TR skeleton for NR mmWave MB-BS”, Huawei, HiSilicon