3GPP TSG-RAN4 Meeting #104bis-e		R4-2217584

# Electronic Meeting, 10-19 October, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | DraftCR | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on band grouping for Cat-M1 for satellite access in 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2022-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The frequency bands for category M1 for satellite access for RRM requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following bands are being specified for the category M1 for satellite access for RRM requirements: - Bands 255 and 256 The above bands are defined and introduced in new band group in new section 3.5.1A in TS 36.133 for the applicability of RRM requirements. According to RF agreements in the WF in R4-2214467, band 255 for category M1 have the same REFSENS as for legacy cat-M1 band 24 i.e. -102.7 dBm for FDD and -102.5 dBm for HD-FDD. Therefore, bands 255 is added to the same group A in the new table. According to RF agreements: - For band 256, the REFSENS of NTN Cat-M1 is -102.2 dBm for FD-FDD and -103 dBm for HD-FDD per 1.4 MHz. REFSENS for band 256 is 0.5 dB higher than that of band 255. Therefore band 256 is assigned band group B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM requirements for category M1 with satellite access will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.5.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

### 3.5.1A	Groups of bands for satellite access

The grouping of bands for satellite access for category M1 is specified in Table 3.5.1A-2.

Table 3.5.1A-2: Band groups for category M1 for satellite access

| Group | E-UTRA FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | FDD-M1_SAB_A | 255 |
| B | FDD-M1_SAB_B | 256 |
| C | FDD-M1_SAB_C |  |
| D | FDD-M1_SAB_D |  |
| E | FDD-M1_SAB_E |  |
| F | FDD-M1_SAB_F |  |
| G | FDD-M1_SAB_G |  |
| H | FDD-M1_SAB_H |  |
| I | FDD-M1_SAB_I |  |
| J | FDD-M1_SAB_J |  |
| K | FDD-M1_SAB_K |  |
| L | FDD-M1_SAB_L |  |
| M | FDD-M1_SAB_M |  |
| N | FDD-M1_SAB_N |  |

----------------------END OF CHANGE----------------------------