3GPP TSG-RAN4 Meeting #104-bis-e	R4-2217593

# Electronic Meeting, 10 – 19 October, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2656 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for NR SDT performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Perf |  |  |  |  |  | Date: |  |  | 2022-10-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following draft CRs R4-2217244,R4-2217245 for NR SDT perf has been endorsed in RAN4#104bis-e meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add the test case for NR SDT in both FR1 and FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test requirement are defined for NR SDT in Rel-17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.2.1, A7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

### A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)

A.6.2.1.1 Test purpose and Environment

The purpose of this test is to verify that the UE properly perform TA validation for CG-SDT transmission in clause 5.5.3. In the test, UE is configured two CG-SDT configurations at different time. [The test consists of five successive time periods, with time duration of T1, T2, T3, T4, T5, and T6 repectively]. Figure A.6.2.1.1-1 shows the variation of the RSRP. Prior to the [time point A], the UE shall be fully synchronized to PCell. Before [Time point A] UE shall receive uplink data from TE.

Figure A.6.2.1.1-1: RSRP variation model for CG-SDT testing

A.6.2.1.2 Test Parameters

There is one cells in the test, the FR1 PCell. The test parameters for the PCell are given in Table A.6.2.1.2-1, Table A.6.2.1.2-2, and Table A.6.2.1.2-3.

Table A.6.2.1.2-1: NR configuration for FR1 SSB

| Config | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.2.1.2-2 : General test parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Duplex mode |  | FDD |
| TDD Configuration |  | N/A |
| BWchannel | MHz | 10: NRB,c = 52 |
| PDSCH Reference measurement channel |  | SR.1.1 FDD |
| RMSI CORESET Reference Channel |  | CR.1.1 FDD |
| Dedicated CORESET Reference Channel |  | CCR.1.1 TDD |
| SSB configuration |  | SSB.3 FR1 |
| OCNG Patterns |  | OP.1 |
| Initial BWP Configuration |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | SMTC.1 |
| DRX configuration |  | [DRX.3] |
| T1 | ms | [TBD] |
| T2 | ms | [TBD] |
| T3 | ms | [TBD] < 640 ms |
| T4 | ms | [TBD] |
| T5 | ms | [TBD] |
| T6 | ms | [640] ms |
| cg-SDT-RSRP-ChangeThreshold | dB | [TBD] |
| cg-SDT-TimeAlignmentTime |  | infinity |
| CG-SDT resource period | ms | [TBD] |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |
| Propagation condition |  | AWGN |

Table A.6.2.1.2-3: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T4 | T5 |
| Note1 | 1 | dBm/15kHz | -98 |  |  |  |
| Note1 | 1 | dBm/SSB SCS | -95 |  |  |  |
|  | 1 | dB | [14] | [3] | [3] | [-4] |
| SS RSRP Note2 | 1 | dBm/SSB SCS | [-81] | [-95] | [-95] | [-104] |
| Io Note3 | 1 | dBm/38.16 MHz | [-49.79] | [-62.2] | [-62.2] | [-64.49] |
|  | 1 | dB | [14] | [3] | [3] | [-4] |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.2.1.3 Test requirements

At time point A, UE shall receive RRC release message with CG-SDT configurations.

The detail description after [time point A] is FFS.

The rate of correct events observed during repeated tests shall be at least 90%.

<Next of Change 1>

A.7.2.1	Small Data Transmission

A.7.2.1.1	TA validation for CG-SDT in FR2

Editor’s Note: the contents in the clause will be updated based on further agreement on the test case design WF R4-2217588

A.7.2.1.1.1	Test Purpose and Environment

[The purpose of this test is to verify that the UE correctly performs TA validation for CG-SDT. This test will partly verify the TA validation requirements in clause 5.5.3.

Supported test configurations are shown in table A.7.2.X.1.1-1. The test parameters are given in Tables A.7.2.X.1.1-2 and A.7.2.X.1.1-3.

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of TBD successive time periods, with time duration of TBD respectively. Figure A.7.2.X.1.1-1 shows the variation of the RSRP level in the active cell during the test.

Table A.7.2.X.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table A.7.2.X.1.1-2: General test parameters for TA validation for CG-SDT in FR2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 DD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 kHz |
| PRACH Configuration | Config 1 |  | Table A.3.8.3.4 |
| OCNG parameters |  |  | OP.5 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| DRX |  | s | 1.28 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | [2] |
| cg-SDT-TimeAlignmentTime |  |  | infinity |
| CG-SDT resource period |  | ms | 40 |
| T1 |  | s | [0.8] |
| T2 |  | s | [1.2] |
| T3 |  | s | [2.5] |

Table A.7.2.X.1.1-3: Cell specific test parameters TA validation for CG-SDT in FR2

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Before X | X to Y | After Y |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |
| Assumption for UE beams Note 4 |  |  | Rough |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 1 | dB | 0 | 8.5 | 0 |
|  | Config 1 | dB | 0 | 8.5 | 0 |
| SS-RSRP | Config 1 | dBm/SCS | -98 | -89.5 | -98 |
| Io | Config 1 | dBm/95.04 MHz | -66.00 | -59.94 | -66.00 |
| Propagation condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |

Figure A.7.2.X.1.1-1: RSRP variation for TA validation for CG-SDT

]

A.7.2.1.1.2	Test Requirements

The UE behaviour in each test during time durations shall be as follows:

TBD

<End of Change 1>