3GPP TSG-RAN WG4 Meeting #104-bis-e                           R4-2217713

Electronic Meeting, 10 October– 19 October, 2022

Source: 	Huawei, HiSilicon

Title: 	TP and discussion on CA_n5-n28

Agenda Item:	6.2.3.2

Document for:	Approval

Introduction

In RAN#96e meeting, a WF [1] was approved with the following outcomes.

1) UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n28

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

2) CBW assumption

n5: 5, 10, 15, 20

n28: 5, 10, 15, 20, 25, 30

3) Feasibility issues need to be analysed

The following issues can be analyzed in the feasibility study.

o	The MSD due to cross band isolation

4) Other observations and proposals

The following proposal is agreed: No IMD for 2UL CA n5-n28

5) UL configuration

The CBW of UL configuration CA_n5-n28 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

We analyzed and summarized all the RF requirements for CA_n5-n28. In this paper, we provide the following text proposals to be captured into TR.

- References

[1] R4-2214445, WF on study on FS_NR_700800900, CATT

- Text proposal

- <<Start of Change for TR 38.872>>

## 5.2	CA_n5-n28

- 5.2.1	UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n28

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

- 5.2.2	Common for 1 band UL and 2 bands UL of CA_n5-n28

- 5.2.2.0	General

According to the current spec TS 38.101-1, DL CA_n5A-n28A_BCS0 has been specified. Thus, some of RF requirements can be reused as below.

- 5.2.2.1	Operating bands for CA

Table 5.2.2.1-1:  CA band combination of band n5+n28

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

- 5.2.2.2	Channel bandwidths per operating band for CA

Referring to table 5.5A.3.1-1e of TS 38.101-1, DL_n5A-n28A_BCS0 has been specified

- 5.2.2.3	UE co-existence studies

There is no need to specify harmonics and harmonics mixing exception for CA_n5-n28 as we didn’t specify them in current spec.

- 5.2.2.4	∆TIB,c and ∆RIB,c values

For CA_n5-n28, the TIB,c and RIB,c values which has been specified in TS 38.101-1

- 5.2.2.5	REFSENS requirements

FFS

- 5.2.3	Specific for 2 bands UL of fallback CA_n5-n28

- 5.2.3.1	Maximum output power for inter-band CA

- Power class 3 is assumed for UL CA_n5-n28.

- 5.2.3.2	UE co-existence studies

Table 5.2.3.2-1 lists Band n5 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.2.3.2-1: Band n5 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 869 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 166 | 76 | 1527 | 1617 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 900 | 1035 | 537 | 672 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2351 | 2486 | 2230 | 2365 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1724 | 1904 | 1240 | 1420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3175 | 3355 | 2933 | 3113 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 152 | 332 | 3054 | 3234 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2168 | 1943 | 2773 | 2548 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 596 | 371 | 1201 | 976 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3636 | 3861 | 3999 | 4224 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3757 | 3982 | 3878 | 4103 |

Based on Table 5.2.3.2-1, there is no IMD issue for CA_n5-n28.

Table 5.2.3.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.2.3.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n28 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 25, 26, 31, 34, 38, 40, 73 NR Band n79 | FDL_low | - | FDL_high | -50 |  |  |
|  | E-UTRA Band 4, 41, 42, 43, 50, 51, 52, 65, 66, 74 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

- 5.2.3.3	REFSENS requirements

FFS

- <<End of Change>>