3GPP TSG-RAN WG4 Meeting #104-bis-e                           	R4-2217720

Electronic Meeting, 10 October– 19 October, 2022

Source: 	Huawei, HiSilicon

Title: 	TP and Discussion on test burden reduction

Agenda Item:	6.3.3

Document for:	Approval

Introduction

In RAN4#104 meeting, we provided a discussion paper [1] to consider the potential directions about test burden reduction. The outcomes were captured into the Way forward [2] as below.

<Way forward>:

Proposal 1: Generally, if CA_nA-nB, DC_A_nB (DC_A_nD), DC_B_nA (DC_B_nC), DC_nB_A, DC_nA_B have same spurious emission requirements for UE to UE coexistence, there is no need to test the spurious emission requirements for UE to UE coexistence for each UL configuration again and again. Once one of these UL configurations is verified, the other UL configurations for different feature in same band combination can be considered as being capable of meeting these requirements.

NOTE: Band nC and nD are the corresponding SUL bands with same UL frequency range of band nA and nB, e.g. SUL band n80 has same UL frequency range of band n3.

FFS how to reduce the test burden for MOP requirements.

<Way forward>:

Proposal 2: reference sensitivity exception due to harmonic/harmonic mixing or cross band isolation specified for ENDC band combinations can be further simplified by using same approach of NR CA, as what we have done for R17 NR CA band combinations in BCS4 WI.

FFS how to address the different test configurations between band combinations.

<Way forward>:

Proposal 3: For band combinations DL_nA-nB_UL_nA-nB / DL_B_nA_UL_B_nA / DL_A_nB_UL_A_nB which have MSD due to IMD interference, FFS how to reduce the test burden.

FFS whether we only choose the worst MSD case.

<Way forward>:

Proposal 4: For band combinations DL_nA-nB_UL_nA-nB / DL_B_nA_UL_B_nA / DL_A_nB_UL_A_nB which doesn’t have any MSD requirements, it’s recommended to test one of them in order to reduce the test burden for REFSENS requirements.

FFS how to address the potential different delta Rib requirements.

In this meeting, we’d like to address the open issues and provide a text proposal.

- References

[1] R4-2213163, General discussion on Simplification of band combination specification, Huawei, HiSilicon

[2] R4-2214447, Way forward on test burden reduction for band combination, Huawei, HiSilicon

[3] R4-2210565, WF on criteria on Rel-17 enhanced MSD table format, Skyworks Solutions, Inc.

- Text proposal

- <<Start of TP for TR 38.846>>

- 7	Test burden reduction for band combinations

- 7.1	General

One of the objectives in this SI is to investigate the feasibility and optimize the specification structure and reduce the test burden. Currently, the main RF requirements related to specific band combinations include maximum output power (MOP), spurious emission for UE-to-UE coexistence, REFSENS and REFSENS exceptions due to harmonic/harmonic mixing/cross band isolation/IMD interference. Obviously, RF requirements for different features on the same band combination have some similarities and dependency. Especially, the RF implementations are similar and RF architectures can be reused for different features on the same band combination. It’s very meaningful to study the similarity and dependency of RF requirements for different features on the same band combination and find out the feasibility to further optimize the specification structure and reduce the test burden. For example, CA_nA-nB (NR CA), DC_nA-nB (NR-DC), DC_A_nB (EN-DC), DC_B_nA (EN-DC), DC_nB_A (NE-DC), DC_nA_B (NE-DC) (different features on same band combination) can use same RF implementation.

- 7.2	Similarity and Dependency of Tx RF requirements for different features on the same band combination

- 7.2.1	Maximum output power

As an example, PC3 MOP requirements for all the UL NR CA, UL NR DC, EN-DC and NE-DC band combinations in table 7.2.1-1 based on the TS 38.101-1-h60 and TS 38.101-3-h60. One band combination may be chosen to verify PC3 MOP testing for some band combinations in same row in table 7.2.1-1, as a result of reducing test burden. It’s suggested to randomly choose one band combination that UE support for MOP testing. The proposed test reduction could be considered for an informative annex in the TS since the final decision should be taken by RAN5 based on the industry certification testing needs.

Table 7.2.1-1 Band combination PC3 MOP requirements for the same frequency range with different features

| Uplink NR CA Configuration | Uplink NR DC Configuration | Uplink EN-DC Configuration | Uplink NE-DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | DC_n1A-n3A | DC_1A_n3A DC_3A_n1A DC_1A_n80A DC_3A_n84A | DC_n3A_1A | 23 | +2/-3 |
| CA_n1A-n5A |  | DC_1A_n5A |  | 23 | +2/-3 |
| CA_n1A-n7A | DC_n1A-n7A | DC_1A_n7A DC_7A_n1A |  | 23 | +2/-3 |
| CA_n1A-n8A |  | DC_1A_n8A DC_8A_n1A | DC_n8A_1A | 23 | +2/-3 |
| CA_n1A-n18A |  |  |  | 23 | +2/-3 |
| CA_n1A-n20A |  | DC_1A_n20A DC_20A_n1A |  | 23 | +2/-3 |
| CA_n1A-n28A | DC_n1A-n28A | DC_1A_n28A DC_28A_n1A | DC_n1A_28A | 23 | +2/-3 |
|  |  | DC_1A_n38A DC_38A_n1A |  | 23 | +2/-3 |
| CA_n1A-n40A |  | DC_1A_n40A DC_40A_n1A |  | 23 | +2/-3 |
| CA_n1A-n41A | DC_n1A-n41A | DC_1A_n41A DC_41A_n1A |  | 23 | +2/-3 |
|  |  | DC_1A_n50A |  | 23 | +2/-3 |
|  |  | DC_1A_n51A |  | 23 | +2/-3 |
|  |  | DC_1A_n71A |  | 23 | +2/-3 |
| CA_n1A-n74A |  |  |  | 23 | +2/-3 |
| CA_n1A-n77A | DC_n1A-n77A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A | DC_n77A_1A | 23 | +2/-3 |
| CA_n1A-n78A | DC_n1A-n78A | DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A | DC_n78A_1A | 23 | +2/-3 |
| CA_n1A-n79A | DC_n1A-n79A | DC_1A_n79A DC_1A_n84A_ULSUP-TDM_n79A |  | 23 | +2/-3 |
| CA_n2A-n5A | DC_n2A-n5A | DC_2A_n5A DC_5A_n2A |  | 23 | +2/-3 |
| CA_n2A-n7A |  | DC_2A_n7A DC_7A_n2A |  | 23 | +2/-3 |
| CA_n2A-n12A |  | DC_2A_n12A DC_12A_n2A |  | 23 | +2/-3 |
| CA_n2A-n14A |  | DC_14A_n2A |  | 23 | +2/-3 |
|  |  | DC_2A_n25A |  | 23 | +2/-3 |
|  |  | DC_2A_n28A DC_28A_n2A |  | 23 | +2/-3 |
| CA_n2A-n30A |  | DC_2A_n30A DC_30A_n2A |  | 23 | +2/-3 |
|  |  | DC_2A_n38A |  | 23 | +2/-3 |
|  |  | DC_2A_n41A |  | 23 | +2/-3 |
|  |  | DC_2A_n46A |  | 23 | +2/-3 |
| CA_n2A-n48A | DC_n2A-n48A | DC_2A_n48A DC_48A_n2A |  | 23 | +2/-3 |
| CA_n2A-n66A | DC_n2A-n66A | DC_2A_n66A DC_66A_n2A |  | 23 | +2/-3 |
|  |  | DC_2A_n71A DC_71A_n2A |  | 23 | +2/-3 |
| CA_n2A-n77A | DC_n2A-n77A | DC_2A_n77A |  | 23 | +2/-3 |
| CA_n2A-n78A |  | DC_2A_n78A |  | 23 | +2/-3 |
| CA_n3A-n5A |  | DC_3A_n5A |  | 23 | +2/-3 |
| CA_n3A-n7A |  | DC_3A_n7A DC_7A_n3A DC_7A_n80A |  | 23 | +2/-3 |
| CA_n3A-n8A |  | DC_3A_n8A DC_8A_n3A DC_8A_n80A | DC_n3A_8A DC_n8A_3A | 23 | +2/-3 |
| CA_n3A-n18A |  | DC_18A_n3A |  | 23 | +2/-3 |
| CA_n3A-n20A |  | DC_3A_n20A DC_3A_n82A DC_20A_n3A DC_20A_n80A |  | 23 | +2/-3 |
| CA_n3A-n28A | DC_n3A-n28A | DC_3A_n28A DC_28A_n3A | DC_n28A_3A | 23 | +2/-3 |
| CA_n3A-n34A |  |  |  | 23 | +2/-3 |
| CA_n3-n38A |  | DC_3A_n38A DC_38A_n3A |  | 23 | +2/-3 |
| CA_n3A-n40A |  | DC_3A_n40A |  | 23 | +2/-3 |
| CA_n3A-n41A | DC_n3A-n41A | DC_3A_n41A DC_3A_n80A_ULSUP-TDM_n41 DC_41A_n3A | DC_n41A_3A | 23 | +2/-3 |
|  |  | DC_3A_n50A |  | 23 | +2/-3 |
|  |  | DC_3A_n51A |  | 23 | +2/-3 |
|  |  | DC_3A_n71A |  | 23 | +2/-3 |
| CA_n3A-n74A |  |  |  | 23 | +2/-3 |
| CA_n3A-n77A | DC_n3A-n77A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A | DC_n77A_3A | 23 | +2/-3 |
| CA_n3A-n78A | DC_n3A-n78A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A | DC_n78A_3A | 23 | +2/-3 |
| CA_n3A-n79A | DC_n3A-n79A | DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A |  | 23 | +2/-3 |
|  |  | DC_4A_n2A |  | 23 | +2/-3 |
|  |  | DC_4A_n5A |  | 23 | +2/-3 |
|  |  | DC_4A_n7A |  | 23 | +2/-3 |
|  |  | DC_4A_n28A |  | 23 | +2/-3 |
|  |  | DC_4A_n38A |  | 23 | +2/-3 |
|  |  | DC_4A_n41A |  | 23 | +2/-3 |
|  |  | DC_4A_n78A |  | 23 | +2/-3 |
| CA_n5A-n7A |  | DC_5A_n7A DC_7A_n5A |  | 23 | +2/-3 |
| CA_n5A-n12A |  | DC_5A_n12A DC_12A_n5A |  | 23 | +2/-3 |
| CA_n5A-n14A |  | DC_14A_n5A |  | 23 | +2/-3 |
| CA_n5A-n25A |  |  |  | 23 | +2/-3 |
| CA_n5A-n30A |  | DC_5A_n30A DC_30A_n5A |  | 23 | +2/-3 |
|  |  | DC_5A_n38A |  | 23 | +2/-3 |
| CA_n5A-n40A |  | DC_5A_n40A |  | 23 | +2/-3 |
| CA_n5A-n48A | DC_n5A-n48A | DC_5A_n48A DC_48A_n5A |  | 23 | +2/-3 |
| CA_n5A-n66A | DC_n5A-n66A | DC_5A_n66A DC_66A_n5A |  | 23 | +2/-3 |
|  |  | DC_5A_n71A DC_71A_n5A |  | 23 | +2/-3 |
| CA_n5A-n77A | DC_n5A-n77A | DC_5A_n77A |  | 23 | +2/-3 |
| CA_n5A-n78A |  | DC_5A_n78A | DC_n78A_5A | 23 | +2/-3 |
| CA_n5A-n79A |  | DC_5A_n79A |  | 23 | +2/-3 |
|  |  | DC_7A_n8A DC_8A_n7A |  | 23 | +2/-3 |
|  |  | DC_7A_n20A DC_20A_n7A |  | 23 | +2/-3 |
| CA_n7A-n25A |  | DC_7A_n25A |  | 23 | +2/-3 |
| CA_n7A-n28A |  | DC_7A_n28A DC_28A_n7A |  | 23 | +2/-3 |
| CA_n7A-n40A |  | DC_7A_n40A |  | 23 | +2/-3 |
| CA_n7A-n46A | DC_n7A-n46A |  |  | 23 | +2/-3 |
|  |  | DC_7A_n51A |  | 23 | +2/-3 |
| CA_n7A-n66A |  | DC_7A_n66A DC_66A_n7A |  | 23 | +2/-3 |
|  |  | DC_7A_n71A |  | 23 | +2/-3 |
| CA_n7A-n77A |  | DC_7A_n77A |  | 23 | +2/-3 |
| CA_n7A-n78A | DC_n7A-n78A | DC_7A_n78A | DC_n78A_7A | 23 | +2/-3 |
|  |  | DC_7A_n79A |  | 23 | +2/-3 |
|  |  | DC_8A_n2A |  | 23 | +2/-3 |
|  |  | DC_8A_n20A DC_20A_n8A |  | 23 | +2/-3 |
|  |  | DC_8A_n28A DC_28A_n8A | DC_n28A_8A |  |  |
| CA_n8A-n34A |  | DC_8A_n34A |  | 23 | +2/-3 |
| CA_n8A-n39A |  | DC_8A_n39A |  | 23 | +2/-3 |
| CA_n8A-n40A |  | DC_8A_n40A |  | 23 | +2/-3 |
| CA_n8A-n41A |  | DC_8A_n41A DC_8A_n81A_ULSUP-TDM_n41 | DC_n41A_8A | 23 | +2/-3 |
| CA_n8A-n77A |  | DC_8A_n77A | DC_n77A_8A | 23 | +2/-3 |
| CA_n8A-n78A |  | DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A | DC_n78A_8A | 23 | +2/-3 |
| CA_n8A-n79A |  | DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A |  | 23 | +2/-3 |
|  |  | DC_11A_n1A |  | 23 | +2/-3 |
|  |  | DC_11A_n3A |  | 23 | +2/-3 |
|  |  | DC_11A_n28A |  | 23 | +2/-3 |
|  |  | DC_11A_n41A |  | 23 | +2/-3 |
|  |  | DC_11A_n77A |  | 23 | +2/-3 |
|  |  | DC_11A_n78A |  | 23 | +2/-3 |
|  |  | DC_11A_n79A |  | 23 | +2/-3 |
|  |  | DC_12A_n7A |  | 23 | +2/-3 |
|  |  | DC_12A_n25A |  | 23 | +2/-3 |
| CA_n12A-n30A |  | DC_12A_n30A |  | 23 | +2/-3 |
|  |  | DC_12A_n38A |  | 23 | +2/-3 |
|  |  | DC_12A_n41A |  | 23 | +2/-3 |
| CA_n12A-n66A |  | DC_12A_n66A DC_66A_n12A |  | 23 | +2/-3 |
|  |  | DC_12A_n71A |  | 23 | +2/-3 |
| CA_n12A-n77A |  | DC_12A_n77A |  | 23 | +2/-3 |
|  |  | DC_12A_n78A |  | 23 | +2/-3 |
|  |  | DC_13A_n2A |  | 23 | +2/-3 |
|  |  | DC_13A_n5A |  | 23 | +2/-3 |
|  |  | DC_13A_n7A |  | 23 | +2/-3 |
| CA_n13A-n25A |  | DC_13A_n25A |  | 23 | +2/-3 |
|  |  | DC_13A_n48A |  | 23 | +2/-3 |
| CA_n13A-n66A |  | DC_13A_n66A |  | 23 | +2/-3 |
|  |  | DC_13A_n71A |  | 23 | +2/-3 |
| CA_n13A-n77A |  | DC_13A_n77A |  | 23 | +2/-3 |
|  |  | DC_13A_n78A |  | 23 | +2/-3 |
| CA_n14A-n30A |  | DC_14A_n30A |  | 23 | +2/-3 |
| CA_n14A-n66A |  | DC_14A_n66A |  | 23 | +2/-3 |
| CA_n14A-n77A |  | DC_14A_n77A |  | 23 | +2/-3 |
| CA_n18A-n28A |  | DC_18A_n28A |  | 23 | +2/-3 |
| CA_n18A-n41A |  | DC_18A_n41A |  | 23 | +2/-3 |
| CA_n18A-n74A |  |  |  | 23 | +2/-3 |
| CA_n18A-n77A |  | DC_18A_n77A |  | 23 | +2/-3 |
| CA_n18A-n78A |  | DC_18A_n78A |  | 23 | +2/-3 |
|  |  | DC_18A_n79A |  | 23 | +2/-3 |
|  |  | DC_19A_n1A |  | 23 | +2/-3 |
|  |  | DC_19A_n77A |  | 23 | +2/-3 |
|  |  | DC_19A_n78A |  | 23 | +2/-3 |
|  |  | DC_19A_n79A |  | 23 | +2/-3 |
| CA_n20A-n28A |  | DC_20A_n28A DC_20A_n83A | DC_n28A_20A | 23 | +2/-3 |
|  |  | DC_20A_n38A |  | 23 | +2/-3 |
|  |  | DC_20A_n41A |  | 23 | +2/-3 |
|  |  | DC_20A_n50A |  | 23 | +2/-3 |
|  |  | DC_20A_n51A |  | 23 | +2/-3 |
|  |  | DC_20A_n77A |  | 23 | +2/-3 |
| CA_n20A-n78A |  | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A |  | 23 | +2/-3 |
|  |  | DC_21A_n1A |  | 23 | +2/-3 |
|  |  | DC_21A_n28A |  | 23 | +2/-3 |
|  |  | DC_21A_n77A |  | 23 | +2/-3 |
|  |  | DC_21A_n78A |  | 23 | +2/-3 |
|  |  | DC_21A_n79A |  | 23 | +2/-3 |
| CA_n24A-n41A |  |  |  | 23 | +2/-3 |
| CA_n24A-n48A |  |  |  | 23 | +2/-3 |
| CA_n24A-n77A |  |  |  | 23 | +2/-3 |
| CA_n25A-n38A |  | DC_25A_n41A |  | 23 | +2/-3 |
| CA_n25A-n41A |  |  |  | 23 | +2/-3 |
| CA_25A-n48A |  | DC_48A_n25A |  | 23 | +2/-3 |
| CA_n25A-n66A |  | DC_66A_n25A |  | 23 | +2/-3 |
| CA_n25A-n77A |  | DC_25A_n77A |  | 23 | +2/-3 |
| CA_n25A-n78A |  | DC_25A_n78A |  | 23 | +2/-3 |
|  |  | DC_26A_n25A |  | 23 | +2/-3 |
|  |  | DC_26A_n41A |  | 23 | +2/-3 |
| CA_n26A-n66A |  |  |  | 23 | +2/-3 |
| CA_n26A-n70A |  |  |  | 23 | +2/-3 |
|  |  | DC_26A_n77A |  | 23 | +2/-3 |
|  |  | DC_26A_n78A | DC_n78A_26A | 23 | +2/-3 |
|  |  | DC_26A_n79A |  | 23 | +2/-3 |
|  |  | DC_28A_n5A |  | 23 | +2/-3 |
| CA_n28A-n34A |  |  | DC_n28A_34A | 23 | +2/-3 |
| CA_n28A-n39A |  |  | DC_n28A_39A | 23 | +2/-3 |
| CA_n28A-n40A |  | DC_28A_n40A | DC_n28A_40A | 23 | +2/-3 |
| CA_n28A-n41A | DC_n28A-n41A | DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41A DC_41A_n28A |  | 23 | +2/-3 |
| CA_n28A-n46A | DC_n28A-n46A |  |  | 23 | +2/-3 |
| CA_n28A-n50A |  | DC_28A_n50A |  | 23 | +2/-3 |
|  |  | DC_28A_n51A |  | 23 | +2/-3 |
|  |  | DC_28A_n66A DC_66A_n28A |  | 23 | +2/-3 |
| CA_n28A-n74A |  |  |  | 23 | +2/-3 |
| CA_n28A-n77A | DC_n28A-n77A | DC_28A_n77A |  | 23 | +2/-3 |
| CA_n28A-n78A | DC_n28A-n78A | DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A |  | 23 | +2/-3 |
| CA_n28A-n79A | DC_n28A-n79A | DC_28A_n79A |  | 23 | +2/-3 |
| CA_n34A-n79A |  |  |  | 23 | +2/-3 |
| CA_n30A-n66A |  | DC_30A_n66A DC_66A_n30A |  | 23 | +2/-3 |
| CA_n30A-n77A |  | DC_30A_n77A |  | 23 | +2/-3 |
| CA_n34A-n40A |  |  |  | 23 | +2/-3 |
| CA_n34A-n41A |  |  | DC_n41A_34A | 23 | +2/-3 |
|  |  | DC_38A_n8A |  | 23 | +2/-3 |
|  |  | DC_38A_n28A |  | 23 | +2/-3 |
| CA_n38A-n66A |  | DC_66A_n38A |  | 23 | +2/-3 |
| CA_n38A-n78A |  | DC_38A_n78A |  | 23 | +2/-3 |
|  |  | DC_38A_n79A |  | 23 | +2/-3 |
| CA_n39A-n40A |  | DC_39A_n40A |  | 23 | +2/-3 |
| CA_n39A-n41A |  | DC_39A_n41A | DC_n41A_39A | 23 | +2/-3 |
|  |  | DC_39A_n78A |  | 23 | +2/-3 |
| CA_n39A-n79A |  | DC_39A_n79A |  | 23 | +2/-3 |
| CA_n40A-n41A |  | DC_40A_n41A | DC_n41A_40A | 23 | +2/-3 |
| CA_n40A-n77A |  | DC_40A_n77A |  | 23 | +2/-3 |
| CA_n40A-n78A |  | DC_40A_n78A |  | 23 | +2/-3 |
| CA_n40A-n79A |  | DC_40A_n79A |  | 23 | +2/-3 |
| CA_n41A-n48A |  |  |  | 23 | +2/-3 |
| CA_n41A-n50A |  |  |  | 23 | +2/-3 |
| CA_n41A-n66A |  | DC_66A_n41A |  | 23 | +2/-3 |
| CA_n41A-n70A |  |  |  | 23 | +2/-3 |
| CA_n41A-n71A |  | DC_71A_n41A |  | 23 | +2/-3 |
| CA_n41A-n74A |  |  |  | 23 | +2/-3 |
| CA_n41A-n77A | DC_n41A-n77A | DC_41A_n77A |  | 23 | +2/-3 |
| CA_n41A-n78A | DC_n41A-n78A | DC_41A_n78A |  | 23 | +2/-3 |
| CA_n41A-n79A |  | DC_41A_n79A |  | 23 | +2/-3 |
|  |  | DC_42A_n1A |  | 23 | +2/-3 |
|  |  | DC_42A_n3A |  | 23 | +2/-3 |
|  |  | DC_42A_n28A |  | 23 | +2/-3 |
|  |  | DC_42A_n51A |  | 23 | +2/-3 |
|  |  | DC_42A_n77A |  | 23 | +2/-3 |
|  |  | DC_42A_n78A |  | 23 | +2/-3 |
|  |  | DC_42A_n79A |  | 23 | +2/-3 |
| CA_n46A-n48A | DC_n46A-n48A |  |  | 23 | +2/-3 |
| CA_n46A-n48B | DC_n46A-n48B |  |  | 23 | +2/-3 |
| CA_n46A-n78A | DC_n46A-n78A |  |  | 23 | +2/-3 |
|  |  | DC_48A_n12A |  | 23 | +2/-3 |
| CA_n48A-n66A | DC_n48A-n66A | DC_48A_n66A DC_66A_n48A |  | 23 | +2/-3 |
| CA_n48A-n70A | DC_n48A-n70A |  |  | 23 | +2/-3 |
| CA_n48A-n71A | DC_n48A-n71A | DC_48A_n71A DC_71A_n48A |  | 23 | +2/-3 |
| CA_n48A-n96A | DC_n48A-n96A |  |  | 23 | +2/-3 |
| CA_n48B-n96A | DC_n48B-n96A |  |  | 23 | +2/-3 |
| CA_n48A-n96B |  |  |  | 23 | +2/-3 |
| CA_n50A-n78A |  |  |  | 23 | +2/-3 |
|  |  | DC_66A_n46A |  | 23 | +2/-3 |
| CA_n66A-n71A |  | DC_66A_n71A DC_71A_n66A |  | 23 | +2/-3 |
| CA_n66A-n77A | DC_n66A-n77A | DC_66A_n77A |  | 23 | +2/-3 |
| CA_n66A-n78A |  | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |  | 23 | +2/-3 |
| CA_n70A-n71A |  |  |  | 23 | +2/-3 |
| CA_n70A-n78A |  |  |  | 23 | +2/-3 |
|  |  | DC_71A_n38A |  | 23 | +2/-3 |
| CA_n71A-n77A |  |  |  | 23 | +2/-3 |
| CA_n71A-n78A |  | DC_71A_n78A |  | 23 | +2/-3 |
| CA_n74A-n77A |  |  |  | 23 | +2/-3 |
| CA_n74A-n78A |  |  |  | 23 | +2/-3 |
| CA_n77A-n79A | DC_n77A-n79A |  |  | 23 | +2/-3 |
| CA_n78A-n79A |  |  |  | 23 | +2/-3 |
| CA_n78A-n92A |  |  |  | 23 | +2/-3 |

- 7.2.2	Spurious emission for UE-to-UE coexistence

Generally, if CA_nA-nB, DC_A_nB (DC_A_nD), DC_B_nA (DC_B_nC), DC_nB_A, DC_nA_B have same spurious emission requirements for UE to UE coexistence, it may not be needed to test the spurious emission requirements for UE to UE coexistence for each UL configuration again and again. Once one of these UL configurations is verified, the other UL configurations for different feature in same band combination can be considered as being capable of meeting these requirements.

NOTE: Band nC and nD are the corresponding SUL bands with same UL frequency range of band nA and nB, e.g. SUL band n80 has same UL frequency range of band n3.

- 7.3	Similarity and Dependency of Rx RF requirements for different features on the same band combination

- 7.3.1	REFSENS exception due to harmonic/harmonic mixing interference for inter-band combinations (two bands)

For reference sensitivity exception due to harmonic/harmonic mixing specified for ENDC/NEDC band combinations, it’s suggested to follow the same principles as for NR CA BCS4 WI in WF R4-2210565 as a starting point.

- 7.3.2	REFSENS exception due to cross band isolation interference for inter-band combinations (two bands)

For reference sensitivity exception due to cross band isolation specified for ENDC band combinations, it’s suggested to follow the same principles as for NR CA BCS4 WI in WF R4-2210565 as a starting point.

- 7.3.3	REFSENS exception due to inter-modulation distortion for inter-band combinations (two bands)

- 7.3.4	REFSENS requirements without any degradation for inter-band combinations (two bands)

- <<End of TP>>