3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2217727

Online Meeting, 10-19 October 2022

Title: 	WF on BC in RRC_INACTIVE and initial access

Agenda Item:	6.7.3

Source: 	Nokia

Document for:	Approval

# <Topic Beam refinement assumption>

## <Sub-topic 1-1> Rough beam vs Fine beam

There have been different views on how the beam is refined in RRC_INACTIVE and initial access in RAN4#104-bis-e.

<Way forward/Agreement>: It is encouraged that companies contribute in RAN4#105 about their understanding of how the beam refinement is done in RRC_INACTIVE and initial access. Possible options (but not limited to) are listed in the following.

- Option 1: Beam refinement in RRC_INACTIVE and initial access is made in the same way as RRC_CONNECTED.

  - Option 1a: with the same SSB configuration as Rel-16 SSB BC (RRC_CONNECTED) case.

  - Option 2a: with some modification in SSB configuration.

- Option 2: It is allowed not to refine beams in RRC_INACTIVE and initial access.

  - Option 2a: It is allowed to use only one antenna element.

  - Option 2b: Beam gain is 7 dB lower than RRC_CONNECTED.

- Option 3: Somewhere in the middle.

  - Option 3a: Refinement is done but is not as good as RRC_CONNECTED.

  - Option 3b: Refinement is done in CG-SDT but is not in RA-SDT and initial access.

  - Option 3c: Refinement in DRX is not as good as continuous reception.

- Option 4: Rough beam or Fine beam used in IA is up to UE implementation and requirements should be implementation agnostic.

# <Topic 2> BC requirement

Possible options for each sub-topic are listed below but are not limited to them.

## <Sub-topic 2-1> min peak EIRP

<Way forward>: It is further discussed whether the minimum peak EIRP is used for the beam correspondence criteria and if the minimum peak EIRP value in RRC_INACTIVE and initial access is the same as RRC_CONNECTED.

- Option 1: min peak EIRP is included.

  - Option 1a: EIRP is the same as RRC_CONNECTED.

  - Option 2a: EIRP is lower in initial access and RRC_INACTIVE than RRC_CONNECTED.

- Option 2: min peak EIRP is not included.

## <Sub-topic 2-2> RAR

<Way forward>: It is further discussed whether RAR is included in BC requirement. Proponents of RAR test are encouraged to provide more analysis why spherical coverage used in RRC_CONNECTED is not sufficient and why RAR test complement the BC requirement in RRC_INACTIVE and initial access.

- Option 1: RAR is included.

- Option 2: RAR is not included.

## <Sub-topic 2-3> msg3

<Way forward>: It if further discussed if msg3 requirement is specified separately from msg1. Proponents of msg3 test are encouraged to provide more analysis why msg1 test is not enough to verify the beam correspondence in RRC_INACTIVE and initial access.

- Option 1: msg3 is included.

  - Option 1a: msg3 requirement is the same as msg1.

  - Option 1b: msg3 requirement is not the same as msg1.

- Option 2: msg3 is not included.

## <Sub-topic 2-4> msgA

<Way forward>: It is further discussed if msgA requirement specified separately from msg1. Proponents of msgA test are encouraged to provide more analysis why msg1 test is not enough to verify the beam correspondence in RRC_INACTIVE and initial access.

- Option 1: msgA is included.

  - Option 1a: msgA requirement is the same as msg1.

  - Option 2a: msgA requirement is the same as Rel-16.

- Option 2: msgA is not included

## <Sub-topic 2-5> applicability of Rel-16 SSB BC requirement

<Way forward>: It is further discussed the applicability of Rel-16 SSB BC requirement to initial access and RRC_INACTIVE.

- Option 1: Applicable to IA, RA-SDT, and CG-SDT.

- Option 2: Applicable to CG-SDT.

- Option 3: Not applicable.

## <Sub-topic 2-6> requirement scenario

<Way forward>: It is further discussed whether initial access, RA-SDT and CG-SDT are all specified. Proponents of multiple tests are encouraged to provide more analysis why the requirements are different among these cases.

- Option 1: Core requirement is introduced to all cases, i.e., IA, RA-SDT, and CG-SDT

  - Option 1a: Core requirement is the same for all cases and one set of requirements is appliable to all.

    - Requirement is verified only in one case.

  - Option 1b: Core requirement is specified for each case, IA, RA-SDT and CG-SDT.

    - Requirement is verified for each case.

- Option 2: Core requirement is only introduced to initial access.

## <Sub-topic 2-7> waveform

<Way forward>: It is for further discussion if DFT-s-QPSK is used as a baseline in the same way as the connected mode.

## <Sub-topic 2-8> BC tolerance

<Way forward>: It is further discussed whether the BC tolerance is applied or not in RRC_INACTIVE and initial access.

- Option 1: BC tolerance is applicable.

  - Option 1: The same as Rel-16.

  - Option 2: New tolerance is introduced.

    - Option 2a: New tolerance for long/short DRX scenarios needs to be clear.

- Option 2: BC tolerance is not applicable.

## <Sub-topic 2-9> UE capability

<Way forward>: It is further discussed whether a new UE capability in Rel-18 is introduced and whether BC in initial access and RRC_INACTIVE is mandated. It is also discussed how UE that does not support beamCorrespondenceSSB-based-r16 and beamCorrespondenceWithoutUL-BeamSweeping is covered.

## <Sub-topic 2-10> Side conditions

<Way forward>: As there was common understanding that determining the side condition of SSB for the EIRP spherical coverage test of Msg1 is required, it is encouraged that companies to contribute on the exact side conditions in initial access and RRC_INACTIVE.

# <Topic 3> Test Issues

Possible options for each sub-topic are listed below but are not limited to them.

## <Sub-topic 3-1> Feasibility to achieve maximum output power / New test functionality

<Way forward>: It is encouraged that companies to contribute on how to configure the well-defined parameters to make UE to transmit at the maximum output power. It is also encouraged if there is any test issue to verify beam correspondence in initial access and RRC_IDLE and if a new test functionality is required.

- Issue 3-1-1: BC can be verified with well-defined parameters already available from legacy releases.

  - Option 1a: Feasible by holding RAR.

  - Option 1b: Feasible already from the first preamble.

- Issue 3-1-2: whether new test functionality is needed?

  - Option 2a: Introduced a beam lock function to RRC_INACTIVE and initial access.

  - Option 2b: Introduced a new RA response timer.

## <Sub-topic 3-2> Test scenario

<Way forward>: It is further discussed if new test scenario specific to SDT is introduced.

- Option 1: A new test scenario with short/long DRX to trigger SDT mode in RRC_INACTIVE is introduced.

- Option 2: Existing test scenario from Rel-16 is reused.

## <Sub-topic 3-3> Polarization aspects

<Way forward>: It is further discussed whether and how testability issues should be addressed without beam lock function.

- Option 1: Testability limitation on polarization aspect shall be addressed.

  - Option 1a: EIRP compensation according to R4-2215702.

  - Option 1b: Separate communication and measurement antenna.

- Option 2: No need of them.