3GPP TSG-RAN WG4 Meeting #104-e-bis 								R4-2217740

Electronic Meeting, October 10 - October 19, 2022

Source:	China Telecom

Title:	WF on UL Tx switching across 3/4 bands with single TAG

Agenda Item:	6.19.2

Document for:	Approval

- Way Forward

- Sub-topic 1-1: Switching period for Tx switching across 3/4 bands

- Issue 1-1-1: Exact value of Tx switching period

Way forward:

For the exact value of Tx switching period for each band pair, further discuss the following options:

- Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18.

  - Note: with the understanding that the switching period in Rel-18 could be different for different band pairs, according to the granularity of per band pair per BC agreed in the last meeting.

- Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 band combination with 3/4 bands.

- Option 3: Option 1 for switchedUL, and option 2 for dualUL

- Sub-topic 1-2: Impact from switching of one Tx chain on the other Tx chain

- Issue 1-2: Impact on the band with the number of Tx chain unchanged due to switching (Case 2)

Way forward:

- For the impact on the band with the number of Tx chain unchanged due to switching, in addition to the baseline UE assumption agreed in RAN4 #104e, further discuss the two options in the next meeting:

  - Option 1: Introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching.

    - Potential proposals on the granularity of the optional UE capability can also be discussed.

  - Option 2: Do not define other optional features to allow the other Tx chain to be used for transmission on the band with the number of Tx chain unchanged during the switching period.

- Issue 1-2A: Issue of two Tx chains switched between two different band pairs

Way forward:

- When two Tx chains are switched between two different band pairs with different lengths of switching periods, as baseline UE assumption, neither of the two Tx chains is expected to be used for transmission during the larger one of the two switching periods.

- Whether advanced optional UE ability will be considered is related to the discussion in Issue 1-2.

- Sub-topic 1-3: RAN4 RF requirements

GTW Agreement

  - Following Rel-16/17 approach, common switching time mask requirements for “switched UL” case and “dual UL” case are to be defined.

  - Further discuss whether to define additional requirements for CA option 2, by considering the switching cases to be introduced in RAN1.

- Sub-topic 1-4: Location of switching period

GTW Agreement

  - For single-TAG case, RAN4 agrees to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1.

- Sub-topic 1-5: Applicability of DL interruption

Way forward:

- For combinations of SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required.

- For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

  - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

Further discuss in the next meeting:

- For SUL+TDD+FDD combinations, for Tx switching between SUL and TDD bands, whether or not DL interruption on the SUL and TDD bands is required.