3GPP TSG-RAN WG4 Meeting # 104bis-e 	R4-2217754

# Online, 10th – 19th October, 2022

Title:	TP to TS 36.102 clause 5 for IoT NTN

Agenda Item:	7.5.2

Source:	ZTE Corporation

Document for:		Approval

- Introduction

This document proposes a TP to TS36.102 to cover the agreements made in [1] at RAN4#104.

Based on an offline comment suggesting a reduction in the number of nested levels of suffix subclauses, we have proposed also to modify slightly the clause structure accordingly.

Proposal: It is proposed to agree to this TP.

3. Text proposal to TS36.102

<Start of text>

5	Operating bands and channel arrangement

5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

5.2	Operating bands

E-UTRA satellite access is designed to operate in the operating bands defined in Table 5.2-1.

Table 5.2-1 E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

5.2A	Operating bands for category M1

UE category M1 is designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1 in both half duplex FDD mode and full-duplex FDD mode.

5.2B	Operating bands for category NB1 and NB2

Category NB1 and NB2 UE are designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1.

Category NB1 and NB2 UE operate in HD-FDD duplex mode.

5.3	Channel bandwidth

5.3.1	General

The requirements in present document are specified for the channel bandwidth listed in Table 5.3A-1.

Table 5.3.1-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

Figure 5.3.1-1 shows the relation between the Channel bandwidth (BWChannel) and the Transmission bandwidth configuration (NRB). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Transmission

Center subcarrier (corresponds to DC in baseband) is not transmitted in downlink

Active Resource Blocks

Resource block

Transmission bandwidth configuration [NRB]

bandwidth [RB]

Channel bandwidth [MHz]

Figure 5.3.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one E-UTRA carrier

5.3B	Channel bandwidth for category NB1 and NB2

For category NB1 and NB2, requirements in present document are specified for the channel bandwidth listed in Table 5.3B-1.

Table 5.3B-1: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

Figure 5.3B-1 shows the relation between the Category NB1/NB2 channel bandwidth (BWChannel) and the Category NB1 /NB2 transmission bandwidth configuration (Ntone). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Figure 5.3B-1 Definition of Channel Bandwidth and Transmission Bandwidth configuration

5.4	Channel arrangement

5.4.1	Channel spacing

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent E-UTRA carriers is defined as following:

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective E-UTRA carriers. The channel spacing can be adjusted to optimize performance in a particular deployment scenario.

5.4.2	Channel raster, carrier frequency and EARFCN

The global frequency raster is defined for all frequencies. The granularity of the global frequency raster is 100 kHz, which means that the carrier centre frequency must be an integer multiple of 100 kHz. For each operating band, a subset of frequencies from the global frequency raster are applicable and forms a channel raster with a granularity ΔFRaster.

The carrier frequency in the uplink and downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143. The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in Table 5.4.2-1 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in Table 5.4.2-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

The applicable channel raster and EARFCNs for each operating band are specified in Table 5.4.2-1.

For operating bands with a channel raster of 100 kHz, every EARFCN within the operating band shall be applicable for the channel raster, and the step size for the channel raster in Table 5.4.2-1 is given as <1>. The broadcast parameter earfcn-LSB defined in [TS36.331] may be used to assist the UE in synchronizing to the cell.

Table 5.4.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 –<1>- 229375 | 1980 | 261844 | 261844 –<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 –<1>- 229075 | 1626.5 | 261504 | 261504 –<1>- 261843 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

5.4.3	TX–RX frequency separation

The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.4.3-1 for the TX and RX channel bandwidth defined in Table 5.3.1-1.

Table 5.4.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 256 | 190 MHz |
| 255 | -101.5 MHz |

5.4B	Channel arrangement for category NB1 and NB2

5.4B.1	Channel spacing

Nominal channel spacing for UE category NB1 and NB2 in stand-alone mode is 200 kHz.

5.4B.2	Channel raster, carrier frequency and EARFCN

The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4.2, and the channel raster per-frequency band shall be as defined in table 5.4.2-1.

The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 –262143. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

NOTE 1: Guard-band operation and in-band operation for NB-IoT are not supported in this version of the specification.

NOTE 2:	For the carrier including NPSS/NSSS for stand-alone operation, MDL = -0.5.

Editor’s note: FFS on MDL offsets values for non anchor carrier for DL

5.4B.3	TX–RX frequency separation

For UE category NB1/NB2 operation in stand-alone mode, the default TX-RX frequency separation shall be as specified in Table 5.4.3-1 for the NB-IoT TX and RX channel bandwidth defined in Table 5.3B-1.

<End of text>

References

[1] R4-2214380, WF on system parameter for IoT over NTN, Ericsson