3GPP TSG-RAN WG4 Meeting # 104-bis-e												R4-2217795

Electronic Meeting, 10– 19 October 2022

Agenda item:			7.5.7

Source:	Moderator (MediaTek Inc.)

Title:	Email discussion summary for [104-bis-e][143] LTE_NBeMTC_NTN_UERF

Document for:	Information

# Introduction

This email discussion covers the following sub-agenda items for WI LTE_NBIOT_eMTC_NTN_req (agenda item 7.5):

- 7.5.1: General and Work Plan

- 7.5.2: System parameters

- 7.5.5: UE RF requirements

The aim for the 1st round is to maximise agreement and common understanding on the above aspects.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Xiaomi | Shengxiang Guo | guoshengxiang@xiaomi.com |
| Qualcomm | Bin Han | binhan@qti.qualcomm.com |
| ZTE | Fei Xue | Xue.fei25@zte.com.cn |
| Sony | Kun Zhao | Kun.1.zhao@sony.com |
| Huawei | Jin Wang | jinwang@huawei.com |
| Ligado Networks | Ojas Choksi | ojas.choksi@ligado.com |
| Hughes/EchoStar | Munira Jaffar | munirajaffar@echostar.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: General issues (7.5.1)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216857 | Ericsson | <Moderator comment: These documents will be handled in the RRM email thread [226]> |
| R4-2216858 | Ericsson |  |
| R4-2216859 | Ericsson |  |

## Open issues summary

There are no issues to discuss in 1st Round for Topic 1. Therefore, no discussion is proposed.

## Discussion on 2nd round

### Preparation of remaining TPs for next meeting

This is to discuss the further work split to contribute remaining TPs for the next meeting. TS36.102 spec rapporteur invited to make a proposal here on how to organize further work among companies.

TS36.102 spec rapporteur proposal

There are bunch of TPs for next meeting. Here are the remaining sections that still need contents to be added and so we invite companies to volunteer. We will appreciate if companies could share the draft TP (even if some parts are still open) 2 weeks before next meeting to help consolidation. Clause/Subclause; Company. Definition/Abbreviation. MPR/A-MPR. SEM/ACLR; Huawei: we can prepare TP for this part.. Spurious emission band UE co-existence. Additional spurious emission. ACS; Sony (we are also fine to take other clause if anyone would like to take this part.). Annexes; Note: The Annex in TS 38.101-5 is not currently used. There was a proposal in this meeting..

### Company feedback on TS36.102 spec rapporteur proposal

| Company | Comments |
| --- | --- |

## Summary for 2nd round

|  | Status summary |
| --- | --- |
| Topic#1 | Some more volunteers still needed. Propose for Spec Rapporteur to follow up further offline with companies, and report the outcome prior to final deadline. |

# Topic #2: System Parameters (7.5.2)

This topic covers the proposals in agenda item 7.5.2 on System Parameters.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215487 | CMCC | Observation 1: for NB in-band deployment at 2G era, if same gNB transmit multiple carriers, 100kHz frequency gap is reserved. Besides, operators usually require all neighbor cells using different carriers. If so no frequency gap is reserved even when two carriers are adjacent to each other. Proposal 1: it’s suggested to study the possibility to reduce frequency gap for NB IoT NTN gNB. Observation 2: updated MDL+0.5 in R4-2213243 applies when we approve to shrink frequency gap and make it to be integral multiple of 15kHz tone. Proposal 2: the EARFCN value for band 255 and band 256 are suggested as in above table. |
| R4-2216546 | ZTE Corporation | Observation 1: for the legacy TN NB-IoT spec, 100kHz frequency gap between adjacent NB-IoT carrier within the RF bandwidth, it should be still applicable. Proposal 1: to postpone the discussion of guard band between IoT over NTN carriers until we get more concrete simulation results to validate its necessity. Proposal 2: to follow the same M_DL value of TN FDD band in Rel-18. Proposal 3: to agree the EARFCN value in the above table for Band 256 and B255. |
| R4-2216637 | Ericsson | Observation 1: In the legacy NB-IoT specification, MDL is selected from {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD, which results in unutilized spectrum resources for orthogonal arrangement between anchor and non-anchor carrier since between anchor and non-anchor carriers there are 8 unused subcarriers as the closest possible selection. Observation 2: In Rel-18 for NB-IoT over NTN, MDL is selected from -9.5, -8.5,-7.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, -0.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 6.5, 7.5, 8.5, 9.5} for FDD, which opens the possibility of having no unused subcarriers (i.e., no unutilized resources) between anchor and non-anchor carriers with orthogonality design. Proposal 1: Introducing a new set value of MDL -9.5, -8.5,-7.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, -0.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 6.5, 7.5, 8.5, 9.5}. Alternatively, re-using the existing set introducing a new note for deriving a new MDL value defining MDL = MDL + 0.5. |
| R4-2216778 | Huawei, HiSilicon | Proposal 1: Reuse the frequency offset Foffset of 200kHz as specified for NB-IoT TN at least for the current WI. Proposal 2: Follow the same M_DL values as TN FDD for NTN. Proposal 3: Define the ARFCN for IoT-NTN as shown in Table 1. |
| R4-2216799 | MediaTek (Chengdu) Inc. | Proposal 1: Existing principles for guardband on the UE side should be used at least as the baseline starting point. However, we welcome further feedback from stakeholders on the network deployment side regarding how to handle the Foffset. Proposal 2: Reuse the existing M-DL values defined for FDD (Option 1) as part of this work item also for non-anchor carriers. Proposal 3: Adopt the channel numbering approach starting from value 262143 and decrementing, starting from the upper edge for each band. Also adopt a similar framework to 38.101-1 in order to allow either a 100kHz or 200kHz EARFCN channel raster step size for future bands. |
| R4-2216682 | Ericsson | Table 5.4.3-1: E-UTRA channel numbers E-UTRA Operating Band; Downlink; Uplink. FDL_low [MHz]; NOffs-DL; Range of NDL; FUL_low [MHz]; NOffs-UL; Range of NUL. 255; 1525; 70656; 70656-70996; 1626.5; 134292; 134292-134632. 256; 2170; 70997; 70997-71297; 1980; 134633; 134633-134933. NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4 MHz.. |

## Open issues summary

### Sub-topic 2-1: F_offset for NB-IoT BS

The question of whether or not to apply Foffset from T36.104 also for 36.108 for the NB-IoT BS.

Open issues and candidate options before e-meeting are captured below.

Issue 2-1:

- Proposals

  - Option 1: Reuse Foffset also for NTN operation

  - Option 2: Continue to study/analyse further as to whether Foffset is needed or can be reduced in size (with conclusion at next RAN4 meeting)

- Recommended WF: TBD

### Sub-topic 2-2: MDL values for NB-IoT

The question as to the applicable MDL values for NB-IoT NTN.

Open issues and candidate options before e-meeting are captured below.

Issue 2-2:

- Proposals

  - Option 1: For NTN, follow the same MDL values as TN FDD

  - Option 2: For NTN, define an offset of +0.5 to the existing MDL values for FDD, applicable for non-anchor carrier operation

  - Option 3: For NTN, define new explicit values of of MDL {-9.5, -8.5,-7.5, -6.5, -5.5, -4.5, -3.5, -2.5 -1.5, -0.5, 0.5, 1.5, 2.5, 3.5, 4.5, 5.5, 6.5, 7.5, 8.5, 9.5}

- Recommended WF: TBD

### Sub-topic 2-3: EARFCN/channel numbering

The question as to the EARFCN/channel numbering format and table.

Open issues and candidate options before e-meeting:

- Channel numbering options:

  - Option 1 (CMCC): Channel numbering starting from b255.

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] | NOffs-UL | Range of NUL |
| 255 | 1525 | 130432 | 130432-130771 | 1626.5 | 261504 | 261504 -261843 |
| 256 | 2170 | 130772 | 130772-131071 | 1980 | 261844 | 261844-262143 |

  - Option 2 (Huawei):

| NTN Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 256 | 2170 | 229076 | 229076 – 229375 | 1980 | 261844 | 261844 – 262143 |
| 255 | 1525 | 228736 | 228736 – 229075 | 1626.5 | 261504 | 261504 – 261843 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |  |  |  |

  - Option 3 (ZTE): Different NOffs-DL compared to Huawei, and additional text added to note for cat-M1

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 256 | 2170 | 243544 | 243544 –<1>-– 243843 | 1980 | 261844 | 261844 –<1>- 262143 |
| 255 | 1525 | 243204 | 243204 –<1>-– 243543 | 1626.5 | 261504 | 261504 –<1>- 261843 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7, 15, 25, 50, 75 and 100 channel numbers at the lower operating band edge and the last 6, 14, 24, 49, 74 and 99 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4, 3, 5, 10, 15 and 20 MHz respectively. |  |  |  |  |  |  |

  - Option 4 (Ericsson): Continue the N_DL in LTE band list in TS 36.101

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] |  | NOffs-UL | Range of NUL |  |
| 255 | 1525 | 70656 | 70656-70996 | 1626.5 | 134292 |  |  | 134292-134632 |
| 256 | 2170 | 70997 | 70997-71297 | 1980 | 134633 |  |  | 134633-134933 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4 MHz. |  |  |  |  |  |  |  |  |

- Whether to add flexibility to framework (similar to 38.101-1) in order to allow either a 100kHz or 200kHz EARFCN channel raster step size to be selected for “future” bands.  (MediaTek) – also see related TP in RP-2216834

Issue 2-3-1: Channel numbering

- Proposals

  - Option 1: Channel numbering Option 1 above <Moderator, CMCC please can you confirm that you really intended to start from b255 as highest range?>  CMCC also start from b256 as highest range rather than b255.

  - Option 2: Channel numbering Option 2 above

  - Option 3: Channel numbering Option 3 above

  - Option 4: Channel numbering Option 4 above

- Recommended WF

  - Agree Option 3 version of NOTE 1 (applicable for cat-M1, not for NB1/2)

  - Further feedback invited on down-selection between Option 2, 3 and 4, noting that different NOffs-DL values are proposed for Options 2 and 3.

Issue 2-3-2: Flexibility in channel numbering framework for 100kHz or 200kHz raster later

- Proposals

  - Option 1: Adopt a similar framework to 38.101-1 in order to allow either a 100kHz or 200kHz EARFCN channel raster step size to be selected for “future” bands

  - Option 2: Other

- Recommended WF: TBD

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1: |
| Ericsson | We are fine to study future in next meeting. |
| Qualcomm | Issue 2-1: We understand the motivation of improving the spectrum efficiency. But we prefer Option 1 in this WI considering the target completion time is this Dec. Note that this is also connected to the MDL definition. We don’t see the issue to reuse the legacy Foffset. Hence we would like to agree with Option 1. |
| ZTE | Issue 2-1: Tend to agree withe Qualcomm, to reuse the option 1 should be no problem, whether further improvement needed which can be further discussed. |
| Huawei | We’re fine with option 1. But if companies are willing to run coexistence simulations for NTN vs NTN, we’re open to consider option 2. |

Sub topic 2-2

| Company | Comments |
| --- | --- |
| XXX | Issue 2-2: |
| Ericsson | Option 2 or option 3. Introducing the + 0.5 for M_DL can optimize the spectrum utilization at SAN when multiple NB carrier is configured. |
| Qualcomm | Issue 2-2: We prefer option 1. Option 2/3 are the enhancements for NB deployment, We don’t think it is necessary to introduce the new MDL for IoT NTN. It can be discussed in general way, i.e., for TN NB firstly. In addition, if the new MDL is introduced, it will also make the interference signal closer for ACS requirements. |
| MediaTek | Issue 2-2: We have a preference for Option 1. |
| Ericsson | To Qualcomm: we are not sure to understand the interference conderation for ACS, the different carrier is transmitting from the same SAN node and all the signal will experience the same doppler frequency when arrived at UE with the same level. In coexisting simulation, there is no guard band consideration so ACS is set without. |
| ZTE | Issue 2-2: We also support the option 1 at the current phase, any further improvement could be discussed in future release especially considering timeline for this WID. |
| Huawei | We can understand the motivation of option 2/3 proposed by Ericsson. Meanwhile, we have the following concerns: - The TN M_DL for FDD is targeted for inband/guard band deployment, and only 4 values (i.e., 3.5, -4.5, 7.5 and -8.5) are added for TDD to enable channel spacing of 180/240kHz between anchor and non-anchor carriers. - Given CMCC’s input, it seems that in practice at least one RB is reserved between the carriers from the same eNB. - So the question is that: do we need all the M_DL values in option 2/3 to enable 14 different channel spacings? We’re open to further discuss the issue. |
| Ligado Networks | Issue 2-2: As an operator, we prefer option 2 or 3 to maximize spectrum use as described in R4-2216637. |
| Hughes/EchoStar | 2-2: prefer option 2 but open for discussion |

Sub topic 2-3

| Company | Comments |
| --- | --- |
| XXX | Issue 2-3-1: Issue 2-3-2: |
| Ericsson | Issue 2-3-1: Option 4. Question on other option1 using the highest number of N_DL, is the NTN IoT band the last band which need counted from the highest number? We think continuing counting from existing legacy LTE Band is fine. Issue 2-3-2: Not sure if the format should consider 200kHz raster, maybe the format could be discussed in future when the 200kHz raster decide? |
| Qualcomm | Issue 2-3-1: We support option 2. It was agreed that Nul should start from 262143 with decreasing order. For Ndl, we suggest splitting the UL and DL in half. So option 2 is preferred. Issue 2-3-2: With option 1, it might lead to the confusion that all the bands have to support both 100khz and 200khz raster. Can proponent show the example for option 1? |
| MediaTek | Issue 2-3-1: Prefer either Option 2 or 3. Is the only difference between Option 2 and 3 how much gap we leave before starting the DL? The note in Option 3 would apply for cat-M1 in any case. Issue 2-3-2: @Qualcomm, please see the TP in R4-2216834, clause 5.4.2 for the framework. There is no risk that we end up with all bands supporting both (I copy the table from that TP below): Table 5.4.2-1: E-UTRA channel numbers E-UTRA Operating Band; ΔFRaster (kHz); Downlink; Uplink. FDL_low (MHz); NOffs-DL; Range of NDL; FUL_low (MHz); NOffs-UL; Range of NUL. 256; 100; 2170; TBD; TBD –<1>- TBD; 1980; TBD; TBD –<1>- TBD. 255; 100; 1525; TBD; TBD –<1>- TBD; 1626.5; TBD; TBD –<1>- TBD. NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz.. |
| CMCC | To moderator, thank moderator for your reminder. But I don’t understand your concern. option 1 also use band 256 as highest range not 255 and I have highlighted by yellow. The uplink range of EARFCN is the same for option 1, 2 and 3. I just list band 255 at first while option 2 and 3 list 256 at first. From my understanding, band 255 should be listed before 256, if my understanding is wrong, please correct me, thanks. Of cause we don’t have strong view on this order. list band 255 first or band 256 first are both OK for us. At least for the uplink, the EARFCN range in option 1, 2, 3 are the same and we support them. |
| ZTE | Issue 2-3-1: The only difference between the option 2 and option 3 is DL part. With further clarification from Qualcomm, we are fine with option 2 from Huawei to splitting the UL and DL in half. In addition, Note as mentioned by MTK should be added for Cat-M1; Issue 2-3-2: We are fine with option 1 from MTK. |
| Moderator | @CMCC: Yes, you are right. Apologies, I misread the numbers. |
| Huawei | Issue 2-3-1: prefer option 2. The rationale behind option 2 is as follows: - Start from the max channel number and continue in descending order as per the agreed WF; - Leave the space counting from B65 untouched (i.e. from 2^16 to 2^17+2^16-1); - Derive the remaining unused space and split it equally for both DL/UL (i.e. from 2^17+2^16 to 2^18-1). The figure below illustrates the design: Issue 2-3-2: Option 1 is acceptable if the group can agree that 200kHz raster size might be used in the future. |

### CRs/TPs comments collection

| CR/TP number | Comments collection <Moderator comment: Please do not repeat comments made for sub-topic 2-2 or 2-3 here. Those comments will be taken into account when deciding on way forward for these TPs> |
| --- | --- |
| R4-2216682 (Ericsson) | Huawei: The TP depends on the discussion of the open issues. |
|  | Company B |
| R4-2216834 (MediaTek) | AEricsson: Not sure we need mention 200kHz raster as there is no 200kHz raster band defined for now. |
|  | MediaTek: @Ericsson: The point was it was agreed in RAN1, so we feel it makes sense to include that and apply the RAN1 recommendation, for future consideration. |
|  | Huawei: This TP also depends on the discussion of the open issues. Additionally, we’d like to remind the group that E-UTRA stands for Evolved UMTS Terrestrial Radio Access, which is inherently for TN, not NTN. We suggest to avoid using “E-UTRA channel number” or “EARFCN” in the NTN spec (i.e. 36.102). Instead, we should follow NR-NTN (38.101-5), and use “NTN satellite operating band” and “NTN ARFCN”. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Tentative agreements: Agree on reuse of Foffset for NTN operation as baseline way forward. Further improvements can be discussed, but if not agreeable next meeting they would not be covered in this WI. Candidate options: Recommendations for 2nd round: - Capture the above into Clause 5/6/7 way forward (MediaTek). - Include Foffset in TP to TS36.108 in square brackets (in TP for next meeting). |
| Sub-topic#2-2 | No clear outcome between Option 1 and 2. Less support for Option 3. Tentative agreements: Further discuss between Option 1 and non-anchor carrier M-DL enhancement limited to Option 2 for this WI, as Option 3 would seem to require new signalling from eNB to UE. Recommendations for 2nd round: - Capture the above agreement into Clause 5/6/7 way forward (MediaTek) - Continue discussion on merits of Option 2 vs Option 1. No further Option 3 discussion. |
| Sub-topic#2-3 | Issue 2-3-1: Option 2 from Huawei received most support, with the ZTE note added for cat-M1. Tentative agreements: Proposed to agree Option 2 incorporating the ZTE note for cat-M1. Issue 2-3-2: Option 1 received some support and no objections. Tentative agreements: Proposed to agree Option 1. Moderator also comments to Huawei that the “EUTRA” terminology is all over the IoT NTN specifications already, including in the approved title of the RAN4 specifications. We would need to modify every 3GPP specification of every WG to remove the EUTRA term for IoT NTN or create completely new specs for each working group dedicated to IoT NTN. This is not something that RAN4 can unilaterally decide, and really such proposals should have been made during the Rel-17 study or WI phase. Also 38.101-5 does not use NTN-ARFCN as you commented? Recommendations for 2nd round: - Capture agreement into Clause 5 6, 7 way forward (MediaTek) - Incorporate the above agreements into the 36.102 baseline TP from R4-2216834 (ZTE). |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2216682 (Ericsson) | Not agreeable. |
| R4-2216834 (MediaTek) | Revised: To be updated with agreements from Topic #2, with TP provided by ZTE. |

## Discussion on 2nd round (if applicable)

### M-DL for non-anchor carrier

Any further comments welcome on Option 1 vs Option 2 from round 1 discussions:

| Company | Comments |
| --- | --- |
| Ligado Networks | If non-anchor carrier deployment is to be supported for IoT NTN in this release, then our preference is for Option 2 as it seems to be spectrally more efficient even from the perspective of a stand-alone deployment. |
| Huawei | It seems there’re misunderstanding/confusion on the issue. Suggest to allow more time to study and decide M_DL in the next meeting. Also, it’s premature to down select between option 1 and 2. Some compromised solution might be feasible and may not require new signaling, either. |

### Comments on Clause 5 TPs

Please make comments below.

| TP | Comments |
| --- | --- |
| TP on clause 5 of TS36.102 (ZTE) | Huawei: It seems strange to define E-UTRA requirements in the NTN spec. For example, E-UTRA operating band, E-UTRA channel bandwidth/channel number in 36.102? Maybe better to replace E-UTRA with NTN in the TP. |

## Summary for 2nd round

|  | Status summary |
| --- | --- |
| M-DL issue | Moderator proposes to follow up on this next meeting where we will make a final decision. It was requested after the 2nd round commenting phase to add an extended editor’s note to the TP below, which was agreeable to all. |
| Clause 5 TP | Moderator proposes to approve the latest version of the TP, with the extended editor’s note added on the M-DL issue. |

# Topic #3: UE RF requirements (7.5.5)

This covers the proposals in agenda item 7.5.5 on UE RF requirements.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215570 | Qualcomm Incorporated | Observation 1: UE is required to keep the same pre-compensation for the duration of the segment. Observation 2: Segment duration is configured by the network Proposal: NTN IoT UE frequency error requirement is defined such that UE is required to maintain estimated Doppler compensation constant for the duration of the segment. |
| R4-2215697 | Ligado Networks | Observation 1: Additional emissions requirements for NTN Band 255 are relaxed by 35 dB compared to that for TN Band 24. Observation 2: There are no additional emissions requirements for NTN Band 255 in the out of band emissions region of an eMTC channel. Observation 3: For frequency range of 1607 - 1626.5 MHz, NTN Band 255 requirement are much more relaxed than even the 3GPP general emissions requirements. Observation 4: Within the spurious emission region of 1541 – 1607 MHz, the NTN Band 255 requirements exceed those specified in 3GPP by up to 10 dB. Observation 5: 3GPP specifications within 1610 – 1626.5 MHz are adequate for CAT-M1 UEs without requiring A-MPR. Observation 6: IM products resulting from sub-carrier allocation are above the emissions requirements for TN band 24 but below that for NTN Band 255. Observation 7: PA emissions in the spurious region at maximum power are below the emissions requirements for both MBWs (700 Hz, 1 MHz). Proposal 1: It is proposed that A-MPR is not required for IoT NTN operation for NTN Band 255 to meet the additional spurious emissions requirements. |
| R4-2215779 | MediaTek India Technology Pvt. | Proposal 1: Reuse existing NB-IoT NTN requirements for MPR, SEM and ACLR. Proposal 2: (For spurious emission coexistence) use the NR bands as a starting point, and contribution invited on any additional E-UTRA bands to consider. Proposal 3: Agree the Frequency Error definition proposed in this document. Also agree to verify the Frequency Error with zero doppler. Proposal 4: Agree to reuse the NR NTN outcome for b256 OOB blocking. Agree the requirements defined in this document for NB-IoT NTN. Proposal 5: Reuse existing NB-IoT TN requirements for IoT NTN UE ACS and adapt the test parameters according to the -40dBm Maximum Input Level. Proposal 6: For Cat-M1 b256 REFSENS, reuse b255 value of –102.7 dBm (FDD) and -103.5 dBm (HD-FDD) Proposal 7: For 36.102 clause 4.1, follow the 38.101-5 approach. Proposal 8: For 36.102 clause 4.2: - Adapt bullets a) and b) to directly indicate the applicability of main clause and suffix for each UE category. - Apply bullets a), c) and g) from 36.101 - Remove bullet f), as well as bullet d) if not clarified. Proposal 9: Do not reserve a (clause 4 sub)-clause at this stage for “additional/suffix requirements”. Clause 4.2 can be split into different sub-clauses if necessary at a later stage. Proposal 10: Add a bullet to clause 4.2 to indicate to point to TS36.307 for applicability of requirements to earlier Releases. |
| R4-2216147 | Xiaomi | Proposal 1: For band 256, the REFSENS is -102.2 dBm for M1 FDD and -103 dBm for M1 HD-FDD. Proposal 2: the out of band requirement for for n256 could be reused for band 256 for cat M1 and NB1/2. |
| R4-2216254 | Sony | Observation 1: The REFSENS of NTN Cat-M1 can be scaled from NR NTN by considering the RB numbers, single Rx relaxation, and transmitter noise. Observation 2: Deriving the REFSENS of NTN Cat-M1 from similar TN bands would result in similar values to those obtained by scaling from NR NTN devices. Proposal 1: For band 256, the REFSENS of NTN Cat-M1 is -102.2 dBm for FD-FDD and -103 dBm for HD-FDD per 1.4 MHz. |
| R4-2216549 | ZTE Corporation | Proposal 1: to define NS_xxN for NTN NS naming for all the NTN bands, but only for new NSs specified in TS 36.102. Proposal 2: to define NS_01 with requirement defined in 6.6.2F.1 of TS 36.101 and NS_57N with requirement defined in 6.5.3.3.2 of TS 38.101-5 ( channel bandwidth to be updated as 200kHz for NB1 and NB2)for NB1/NB2 operating in Band n255. Proposal 3: to define NS_01 with requirement defined in 6.6.2F.1 of TS 36.101 and NS_24 and NS_100 for NB1/NB2 operating in Band n256. Proposal 4: define NS_01 with requirement defined in 6.6.2F.1 of TS 36.101 and NS_57N with requirement defined in 6.5.3.3.2 of TS 38.101-5 ( channel bandwidth to be updated as 1.4MHz for Cat M1)for Cat M1 operating in Band n255. Proposal 5: to define NS_01 with requirement defined in 6.6.2F.1 of TS 36.101 and NS_24 and NS_100 for Cat M1 operating in Band n256. Proposal 6: for Cat M1 REFSENS in band B255 and B256, to reuse the same requirement 1.4MHz of band 24 for B255 and B256. Proposal 7: to define -40dBm as maximum input power for IoT over NTN UE. Proposal 8: to follow the same requirement of OOBB requirement of band n256 for band B256. |
| R4-2216635 | Ericsson | Observation 1: The FCC 25.202(f) requirement is more stringent than the 3GPP SEM mask for Cat-M1 and NB-IoT. Proposal-2: The same NS value can be applied to the additional SEM and additional spurious requirement in TS 38.101-5 Proposal-3: Further investigate the A-MPR with the new additional SEM requirement for b255 and b256. Observation 3: It will be easier for TE to measure and compensate the pre-compensated doppler shift by UE relative to the nominal UL frequency Observation 4: Discuss further whether the current test methodology is good enough to measure the additional frequency error caused by frequency pre-compensation in UE. Proposal-4: Whether to have GNSS access at TE could leave to RAN5 to decide. Proposal-5: The doppler frequency should be specified in annex so that frequency error caused by the deviation from the UE estimated amount and TE pre-set would be minimized. Proposal-6: RAN4 discuss the above changes for the frequency error requirement considering the test discussion above. Proposal-7: Discuss the above annex for the doppler frequency measurement. Proposal-8: Legacy ACLR requirement should be reused if to reuse the legacy device directly to the NTN. Proposal-9: Reuse the NTN spurious requirement for coexisting with modification of operating band number. Proposal-10: Consider the NTN Cat-M1 REFSENS based on B65 REFSENS with half dB tightening in TS 36.101. Proposal-11: Considering the above OOB for NTN cat-M1 and NTN NB-IoT. |
| R4-2216884 | Qualcomm | Proposal: Use suffixes on 2nd level in the specification 36.102 and approve the TP in appendix. |

## Open issues summary

### Sub-topic 3-1: Frequency error

UE frequency error requirement conditions and definition are discussed here (note that at RAN4#104-e there was an agreement on the frequency error values already.

Open issues and candidate options before e-meeting are captured in the options below:

Issue 3-1:

- Proposals:

- Proposal 1: Agree on baseline text for frequency error requirement proposed in R4-2215779, and same approach for cat-M1.

- Proposal 2: Make the following modification to proposal 1 final sentence “…after compensation with ideally pre-compensated doppler frequency compared with nominal reference uplink carrier frequency”

- Proposal 3: Mandate the UE to maintain Doppler pre-compensation constant for the duration of an UL segment

- Proposal 4: Agree to verify the Frequency Error with zero Doppler.

- Proposal 5: The doppler frequency should be specified in annex so that frequency error caused by the deviation from the UE estimated amount and Test Equipment (TE) pre-set would be minimized.

- Recommended WF: TBD

NOTE: When replying, please provide views on Proposals 1 to 5, indicating for each:

  - Option 1: Agree

  - Option 2: Other

### Sub-topic 3-2: Emissions requirements

Different requirements are proposed in terms of SEM, Additional SEM/Additional Spurious emissions and associated A-MPR, and also ACLR. Proposals are captured below.

Issue 3-2:

- Proposals

- Proposal 1: Define NS_xxN for NTN NS naming for all the NTN bands, but only for new NSs specified in TS 36.102.

- Proposal 2: The same NS value can be applied to the additional SEM and additional spurious requirement as for 36.101 if the same requirement is applicable for the NTN band as for TN.

- Proposal 3: Reuse the NR NTN spurious requirement for coexisting with modification of operating band number

- Proposal 4: Define NS_01 with requirement defined in 6.6.2F.1 of TS 36.101 and NS_57N with requirement defined in 6.5.3.3.2 of TS 38.101-5 (channel bandwidth to be updated as 200kHz for NB1 and NB2) for NB1/NB2 operating in Band n255

- Proposal 5: Define NS_01 with requirement defined in 6.6.2F.1 of TS 36.101 and NS_24 and NS_100 for NB1/NB2 operating in Band n256

- Proposal 6: Define NS_01 with requirement defined in 6.6.2F.1 of TS 36.101 and NS_57N with requirement defined in 6.5.3.3.2 of TS 38.101-5 (channel bandwidth to be updated as 1.4MHz for Cat M1) for Cat M1 operating in Band n255

- Proposal 7: Define NS_01 with requirement defined in 6.6.2F.1 of TS 36.101 and NS_24 and NS_100 for Cat M1 operating in Band n256

- Proposal 8: A-MPR is not required for IoT NTN operation for NTN Band 255 to meet the additional spurious emissions requirements.

- Proposal 9: Further investigate the A-MPR with the new additional SEM requirement for b255 and b256.

- Proposal 10: Reuse TS36.101 NB-IoT UE requirements for MPR, SEM and ACLR for NTN operation.

- Recommended way forward: TBD

NOTE: When replying, please provide views on Proposals 1 to 10, indicating for each:

  - Option 1: Agree

  - Option 2: Other

### Sub-topic 3-3: Rx requirement proposals

Different proposals are made for:

- Rx sensitivity for cat-M1 in b256

- Out of band blocking for b256

- Maximum input level <Moderator: @ZTE: It seems we already agreed on -40dBm in the last meeting. I propose to consider this agreed with no further discussion>

- UE ACS

Issue 3-3-1: Rx sensitivity for cat-M1 b256

- Proposals

  - Option 1: For Cat-M1 b256 REFSENS, reuse b255 value of –102.7 dBm (FDD) and -103.5 dBm (HD-FDD)

  - Option 2: For band 256, the REFSENS of NTN Cat-M1 is -102.2 dBm for FD-FDD and -103 dBm for HD-FDD per 1.4 MHz.

NOTE: Different methodologies have been used to derive the result even when the same result is proposed, so please read the rationale in the corresponding input papers.

- Recommended WF: TBD

Issue 3-3-2: Maximum input level for all categories

- Recommended WF:

  - -40dBm was already agreed last meeting, so no need to discuss further this meeting.

Issue 3-3-3: Out of band blocking

- Proposals

  - Option 1: to follow the same requirement of OOBB requirement of band n256 for band B256 for category NB1/2 and category M1 (as in tables below)

  - Option 2: Other

- Recommended WF:

  - Agree Option 1, along with notes as in tables below.

NB-IoT:

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -152 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports supports in frequency range 1475.9 MHz < f < 2690 MHz.” |  |  |  |  |  |

Cat-M1:

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

Issue 3-3-4: UE ACS

- Proposals

  - Option 1: Reuse existing NB-IoT TN requirements for IoT NTN UE ACS and adapt the test parameters according to the -40dBm Maximum Input Level.

  - Option 2: Other

- Recommended WF: TBD

### Sub-topic 3-4: Clause 4 – Requirements applicability

A set of proposals are made for clause 4, as reflected below.

Issue 3-4-1:

- Proposals

  - Proposal 1: For 36.102 clause 4.1, follow the 38.101-5 approach (rather than the 36.101 approach)

  - Proposal 2: For 36.102 clause 4.2:

    - Adapt bullets a) and b) from 36.101 to directly indicate the applicability of main clause and suffix for each UE category.

    - Apply bullets a), c) and g) from 36.101

    - Apply neither bullet f) nor bullet d) from 36.101 if not clarified.

  - Proposal 3: Do not reserve a (clause 4 sub)-clause at this stage for “additional/suffix requirements”. Clause 4.2 can be split into different sub-clauses, if necessary, at a later stage.

  - Proposal 4: Add a bullet to clause 4.2 to indicate to point to TS36.307 for applicability of requirements to earlier Releases.

  - Proposal 5: Use suffixes on 2nd level in the specification 36.102.

- Recommended WF: TBD

NOTE: When replying, please provide views on Proposals 1 to 5, indicating for each:

  - Option 1: Agree

  - Option 2: Other

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1 |
| Ericsson | Proposal 1 may be modified with consideration of proposal 2. Proposal 3, our understanding is that UE allow to change between segmentation, but not sure RAN4 should mandate this if FE is met. Proposal 4: is only zero doppler or including the zero doppler frequency? Proposal 5: agree. |
| Qualcomm | Issue 3-1 P3 seems to be the assumed behavior from ran1/2. We would not be ok with proposal 4 since then the doppler pre-compensation is not tested. For P5, it would make sense to specify the Doppler as part used reference channel / RMC |
| MediaTek | Issue 3-1: Proposal 1: Support Proposal 2: Do not support. Not clear what is the added value here in fact it makes it less clear? And the associated TP does not include the part proposed to be changed here? Proposal 3: we are ok but for GSO for NB1/2 there is no concept of segmentation, so we would need to extend the text to add “…or for NB1/2 for the duration of the UL transmission”. Proposal 4: It seems from the proposals made for NR NTN and in proposal 5 below that we are not going to be testing pre-compensation algorithm in the UE anyway. Constant doppler seems not possible to predict in a real algorithm as gravity would need to be considered in predicting the trajectory. In the proposal 5 below other aspects of a realistic doppler channel such as “delay” have not been included (are we supposed to implement another algorithm just for testing?). Therefore, we propose to simply verify with zero doppler. Happy to consider that for GSO only (and yes pre-compensation would need to be used to predict a zero doppler, so it would be tested). Proposal 5: Seems this is related to the test configuration but does not consider aspects like delay properties. Maybe more meaningful to include in RAN5 spec once it is agreed how to test all of this. |
| Ericsson | To MTK: For P2, our understanding is that the FE requirement is compared with nomial UL frequency not the ideally pre-compensated frequency in 15779. The UE pre-compensation when verified, need TE to compensate this doppler frequency so the measured frequency error is still be within the 0.1ppm or 0.2 ppm. For P4, we agree with QC that we need test pre-compensation function, not testing eaqually means there is no FE requirement when UE enable the pre-compensation function. For P5, TE need to know how to compensate the doppler frequency, that is the reason for Annex. In our view, RAN4 can specify this as it is related to the FE calculation. |
| ZTE | Issue 3-1: We are fine with proposal 1 and proposal 2. For the proposal 3, we need more time to check it. For the proposal 4, we did have similar discussion for NTN UE before in Rel-17, it seems not conclusion made finally. For the propsoal 5, this cuold be further discussed for Annex. |
| Sony | Issue 3-1 We support proposal 3. The pre-compensation should be constant within each segment. We think this proposal aligns with RAN1 design on segmented uplink transmission. |
| Huawei | P1: Seems to be line with NR-NTN. Fine for us. P2: The wording seems confusing. The last “compensation” seems to mean to revert the pre-compensation operation. P1 seems better in this sense. Moreover, in NR-NTN, it also says “compared to ideally pre-compensated reference uplink carrier frequency”. P3 seems to be in line with RAN1 agreement indicated in LS R1-2205642. P4: If zero Doppler is used, how to test the pre-compensation function? P5: Based on Ericsson’s comments above, if the intention is to specify how to calculate the “ideally pre-compensated reference uplink carrier frequency”, we’re fine with P5. |
| Hughes/EchoStar | OK with proposal 1 and 5 |

Sub topic 3-2

| Company | Comments |
| --- | --- |
| XXX | Issue 3-2 |
| Ericsson | Proposal 2: agree. Proposal 3: agree. Proposal 4/5/6/7: NS_01 is reserved for all bands and should not be used for other purpose. Proposal 8: need to discuss the additional SEM for FCC 25.202(f) Proposal 9: agree. Proposal 10: Need additionally consider the FCC requirement. |
| Qualcomm | Issue 3-2 P1/P2: We should follow the same naming rule as NR NTN NS in TS38101-5. It says the same NS value can be applied to the additional SEM and additional spurious requirement as for 36.101 if the same requirement is applicable for the NTN band as for TN. For the requirements different from 36.101, define NS_xxN for IoT NTN requirements. P3: Agree P4: Need to clarify whether or not to capture the FCC requirement of 47 C.F.R. § 25.202 (f) in TS36102 pointed out in Ericsson’s paper R4-2216635. This requirement is not in the TS38101-5. P5: Agree. P6: Need to clarify whether or not to capture the FCC requirement of 47 C.F.R. § 25.202 (f) in TS36102 pointed out in Ericsson’s paper R4-2216635. This requirement is not in the TS38101-5. P7: Agree P8/9: Need to clarify whether or not to capture the FCC requirement of 47 C.F.R. § 25.202 (f) in TS36102 pointed out in Ericsson’s paper R4-2216635. This requirement is not in the TS38101-5. If yes, we need to check the A-MPR values. P10: Based on the co-ex simulation we submitted, we agree with P10. |
| ZTE | Issue 3-2 P1: we support the proposal 1 which is coming from TS 38.101-5. P2: we could follow the legacy approach used for TS 38.101-5 P3: Agree P4: similar comment as Qualcomm. P5: Agree. P6: similar understanding as Qualcomm.. P7: Agree P8/9: similar comment as Ericsson, for n57_N, it might be okay, however for new FCC requirement proposed by Ericsson, this need more analysis and confirmation from satellite vendors.. P10: this still depend on the conclusion of coexistence study and two thread should be aligned. |
| Huawei | P1/P2/P3: no strong views. P4/P5/P6/P7: NS_01 refers to requirements in 6.6.2F.1 of 36.101, which is equivalent to P10. Depending on the coexistence simulation results, we’re not ready to agree with them yet. P8/P9: P9 is preferred based on companies’ feedback. P10: Not ready to agree yet. |
| Ligado Networks | Issue 3-2: Proposal 1: Option 1 – Agree Proposal 2: Option 2 – NS value for NS_57N from TS 38.101-5 for n255 should be reused for Band 255. Proposal 3/4/6: Option 2 – This needs further discussion on whether MBW 4 kHz for 1610 – 1626.5 MHz and below need to be captured for regional specific requirements under NS_57N requirements, at least for Band 255 in compliance with 25.202(f). Proposal 8: Option 1 – Agree Proposal 9: Option 2 – this proposal seems to be based on improper interpretation of 25.202(f). The correct interpretation is included below; the 3 steps of 25 35, 43+10logP attenuations per 4 kHz are in relation to total power and not in relation to the PSD as interpreted to arrive at Observation 1. For MOP of 23 and 20 dBm, this translates to, -2 dBm per 4 kHz and -21 dBm per 4 kHz and – 13 dBm per 4 kHz. Once normalized to 30 kHz and 1 MHz, these are much more relaxed than the general 3GPP emission requirements. Therefore, no A-MPR should be required to meet 25.202(f) requirements close to the band edge. Proposal 10: Option 1Agree for MPR, ACLR; Option 2 for SEM: further discussion on how to capture 4 kHz MBW associated with 25.202(f) needed. |
| Hughes/EchoStar | Proposal 7: need further checking Proposal 9: need further checking |
| MediaTek | Proposal 1/2/3: agree Proposal 5/7: agree Proposal 10: Option 1 agree to reuse TS36.101 IoT MPR and ACLR requirements as baselines and we think modification of MPR and ACLR should be allowed if necessary. |

Sub topic 3-3

| Company | Comments |
| --- | --- |
| Xiaomi | Issue 3-3-1: Option 2. The value -102.2 dBm and -103 dBm has already considered the 0.5dB stringent requirement for band n256 compared to Band 65 based on our evaluation. Issue 3-3-2: already been agreed in the last meeting Issue 3-3-3:Option 1 Issue 3-3-4:Option 1 |
| Ericsson | Issue 3-3-1: Option 2. |
| MediaTek | Issue 3-3-1: We are okay with Option 2. Issue 3-3-2: We are okay with the recommended WF. Issue 3-3-3: Option 1 Issue 3-3-4: We are okay with Option 1 as baseline and think modification should be allowed if necessary. This approach addresses RAN4 colleagues’ opinions and at least some progress can be made considering the timeline for the WID. |
| Qualcomm | Issue 3-3-1: We are OK with Option 2. Issue 3-3-2: Agree with recommended WF from moderator Issue 3-3-3: Agree with recommended WF from moderator Issue 3-3-4: We support option 1 |
| ZTE | Issue 3-3-1: Our proposal is option 1, however we are also fine with option 2. Issue 3-3-2: Agree with recommended WF Issue 3-3-3: Agree with recommended WF Issue 3-3-4: similar comments as ACLR/SEM/MPR before. Two thread should be aligned, this could be confirmed in BS thread coexistence study. |
| Sony | Issue 3-3-1: we propose option 2 originally, which is mainly based on the scale from NR NTN. However, we are also fine to take option 1, to re-use b255 value. Issue 3-3-3: Support the recommended WF Issue 3-3-4: Option 1 |
| Huawei | Issue 3-3-4: The TN NB-IoT ACS requirements use GSM(GMSK) interferer. Not sure if this is still suitable for NTN. Some further discussion is needed. |
|  | Issue 3-3-1: Support Option 1. Issue 3-3-3: OK with Option 1 |

Sub topic 3-4

| Company | Comments |
| --- | --- |
| XXX | Issue 3-4: Regarding proposal 5, we support the idea to unify the style. However, it seems clause 6 and 7 are more suitable to use suffixes on 3rd level. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216636 (Ericsson) | Qualcomm: Needs to be aligned with discussion in this summary. There is an error in the level of the proposed FCC requirement for >2*BWchannel. It should be -36 for PC2 and -33 for PC5. MediaTek: The frequency error part is different from 6635, which version is proposed? |
|  | Company B |
|  | Ericsson: To Qaulcomm: Thanks for pointing this. Indeed, it is -36 for PC3 and -33 for PC5. Will be corrected in updated version. To MTK: thanks for pointing this. The Annex part is missed in 6636 and should follow the 6635, can correct this with updated version. |
| R4-2216680 (MediaTek) | Xiaomi: From the TP, it can been seen all the suffix B for NB-iot are included. could you provide a clarification why all the suffix A for eMTC requirements are missing in the TP? |
|  | AEricsson: Foob may need to mention somewhere and if the spurious emission could follow the NR NTN. |
|  | MediaTek: To Xiaomi : For clarification: In principle we did not add a suffix when there was no suffix used in 36.101, and this mainly applies to cat-M1. For further discussion: as we have no other UE types currently, we do wonder whether it is clearer to add a suffix A for all clauses, as it is maybe a bit unclear as to what the main clause is for currently To Ericsson: Thanks for comment. Content of Foob will be added in updated version. |
| R4-2216681 (MediaTek) | Company A |
|  | Company B |
| R4-2216884 (Qualcomm) | <Moderator comment: Only provide comments on the text in the Annex here please> |
|  | Company A |
| R4-2215779 (MediaTek) – not a formal TP, but text in Annex should be discussed | <Moderator comment: Only provide comments on the text in the Annex here please> |
|  | Ericsson: the highlighted text may not be needed. NTN IoT are FDD bands and narrow channel bandwidth. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Majority support for Proposals 1 and 3. Commented that for P3 for GSO “for the duration of the UL transmission” would need to apply. Proposals 2 and 5 would need further discussion. Proposal 4 received no support. Tentative agreements: Proposal 1 and 3 are agreeable, with the addition above for NB1/2 GSO. Recommendations for 2nd round: - Capture above agreements in clause 6/7 way forward (MediaTek) - Capture above agreements into TPs on Frequency Error for NB1/NB2 (Qualcomm) and Cat-M1 (Ericsson), using the Proposal 1 text as baseline. - Further discuss Proposals 2 and 5 as part of 2nd round. |
| Sub-topic#3-2 | Tentative agreements: - Proposal 1 – agree - Proposal 2 – agree to follow same approach as NR NTN - Proposal 3 – agree - Proposals 4/5/6/7 – agree on non-NS_01 parts as proposed. Further discussion needed on NS_01 at next meeting once coexistence aspects are clear. Also further discuss proposed mapping to FCC requirements (below). - Proposal 8/9 – more discussion needed. Also further discuss proposed mapping to FCC requirements (below). - Proposal 10 – come back next meeting on this. - FCC: In general, further discuss potential FCC requirement mapping of SEM and Spurious emissions requirements. Recommendations for 2nd round: - Capture status and try to make further progress via WF on A-MPR and Emissions requirements (ZTE) |
| Sub-topic#3-3 | Issue 3-3-1: Propose to agree option 2 due to the vast majority accepting that. Issue 3-3-2: Agree recommended WF Issue 3-3-3: Agree to Option 1 Issue 3-3-4: No agreement at this stage Recommendations for 2nd round: - Capture the agreements in WF on clause 5, 6 and 7 (MediaTek) - New TP for out of band blocking for b256 (Xiaomi) - Capture agreement in TP for Rx sensitivity for cat-M1 (Ericsson) |
| Sub-topic#3-4 | Tentative agreements: Agree to Proposals 1/2/3/4. FFS on proposal 5. Recommendations for 2nd round: - Evaluate further if clauses 6 and 7 should be 2nd level or 3rd level - Merge TP in R4-2216884 with R4-2215779 proposed text according to agreements (Qualcomm, MediaTek) |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2216636 (Ericsson) | To be revised to cover Cat-M1 Frequency Error and b256 Rx sensitivity only. |
| R4-2216680 (MediaTek) | To be revised to add suffix A clauses where previously was main clause. |
| R4-2216681 (MediaTek) | To be revised to add suffix A clauses where previously was main clause. |
| R4-2216884 (Qualcomm) | To be revised and merged with text from R4-2215779 without the yellow highlighted text, with suffix levels for clause 6 and 7 left as [TBD - 2nd or 3rd]. |

## Discussion on 2nd round (if applicable)

### Comments on Clause 6 and 7 TPs

Please make comments below.

| TP | Comments |
| --- | --- |
| TP for NB-IoT UE Frequency Error (Qualcomm) | Huawei: Should “shorter” in the statement below be “longer”? Since the proceeding clauses require no changes of pre-compensation during ongoing repetition period or during a segment, it seems logical to use “whichever is the longer”. When repetitions and segmented transmissions are overlapping, UE shall update the Doppler pre-compensation between segments or repetitions periods, whichever is the shorter. |
|  | MediaTek: Regarding “When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period”, there is a maximum transmission length of 256ms for the UE in uplink, after which there is a gap and later on it may resume repetitions. While we acknowledge that our text regarding “UL transmission” was maybe unclear and this was maybe a way of clarifying it, we think that we should not prevent the UE from updating during the gap. We also would not like to mandate the UE to update after every repetition as this may not be needed, as it depends on UE implementation. So we would propose to remove “When repetitions and segmented transmissions are overlapping, UE shall update the Doppler pre-compensation between segments or repetitions periods, whichever is the shorter.” In general, it might be best not to be so prescriptive about when things are done- at least in the GSO case. |
|  | Qualcomm: UE should update the pre-compensation in the gaps either between segments or repetitions, when ever it can. If segments are shorter, then between segments during the repetions and if there are no segments configured, then between repetition periods. So shorter, maybe we can use “can” instead of “shall” |
|  | Huawei: “can” is a better word, however, it seems that different pre-compensations can be applied for one segment if the repetition is shorter as per Qualcomm’s suggestion. This seems to contradict to the LS from RAN1, which says: - Different values (e.g., TA) for pre-compensation may be used per segment, where UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment |
| TP for Out-of-band blocking for b256 (Xiaomi) | MediaTek: Editorial - it says “support support” in Note 3 for NB1/2. |
|  | Huawei: Since NOTE2 from the TN spec (36.101) is kept, it seems that NOTE 4 should also be copied over. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |
| TP on Cat-M1 UE NTN Frequency Error and b256 UE Rx sensitivity (Ericsson) | MediaTek: Frequency Error: - In order to align with TN cat-M1, the observation period should be 0.5ms not 1ms. - You have included category M2 UEs, but this is out of scope of the current WI. - Proposed to clarify 2nd paragraph text: “For category M1 FD-FDD UEs, and for category M1 HD-FDD UEs with continuous uplink transmissions of duration ….” - Proposed to clarify 3rd paragraph: For category M1 HD-FDD UEs with continuous uplink transmissions of duration ….” - Comments on proposal 2 and 5 included in section 3.5.2 below, as they were not agreed yet. Rx sensitivity: - Why did you not add Note 1 to the HD-FDD table, like in 36.101? - Suggest to remove TDD until we have some TDD bands. |
|  | Huawei: Regarding the wording for frequency error, why not use the same one as NR-NTN? The new wording sounds very confusing. The pre-compensation needs to be done twice? [after compensation with ideally pre-compensated doppler frequency compared with reference uplink carrier frequency]. In the Annex Z, it says the doppler frequency f(T) could be linear extrapolated with f(T1) and f(T2). Does it mean that UE need to adapt the doppler pre-compensation during the transmission duration? It also says that the time between T1 and T2 could be set to be short. It’s not clear how to achieve this. According to the formula in this section, the UE needs to acquire its position, the satellite position and velocity at both T1 and T2 in order to estimate the doppler frequency f(T1) and f(T2). Not sure if it’s feasible to make it shorter since complex operations are involved. And it seems not necessarily that shorter interval would result in better estimation of doppler. In summary, our preference is to reuse the NR-NTN wording for frequency error requirements, and postpone the decision on Annex Z to the next meeting. |
| TP for clause 6 of TS36.102 (MediaTek) | Company A |
| TP for clause 7 of TS36.102 (MediaTek) | Company A |
| TP for clause 4 of 36.102 (Qualcomm) | Company A |

### Further comments on Frequency Error ”Proposals 2 & 5”

Please make comments below.

| Company | Comments |
| --- | --- |
| MediaTek | PROPOSAL 2: “after compensation” proposal. This proposal seems to be putting a requirement on the TE or eNB rather than the UE. I believe that you mean “after the doppler channel has been applied”. But this is very much a test configuration question, which companies told us was a RAN5 question. However, I think it makes sense to clarify that the ideal pre-compensation is applied at the start of the segment, and as such that any doppler channel variations that may impact frequency error after the UL transmission/segment starts have not been included in the frequency error tolerance by RAN4. PROPOSAL 5: For the Annex, this seems to be defining test equipment behaviour. I would suggest not putting this into the specification until there has been some liaison with RAN5 about how they plan to test this requirement. Also, an ideal channel would also have variable delay as the satellite is moving. Doppler frequency variation is approx. 0.5Hz/ms at 2GHz for LEO. Only changing the frequency in the channel would mean that a UE that (correctly) pre-compensates delay and doppler together would likely fail the test. |
| Ericsson | To MTK: We are open to discuss how to capture this in wording. The point is that the 0.1ppm is after TE introduce the compensation on the UE pre-compensated doppler shift. Such compensation could by test configuration which could be stated as “TE should compensate the ideally doppler frequency with Annex Z” There is another point in P2, which we would also to clarify, the compared frequency should be the UL configured frequency and not to be a frequency with ideally pre-compensated reference frequency. The reasons are below: - In FCC frequency tolerance clause, there is no doppler frequency pre-compensation but the reference frequency. Modification of the reference frequency meaning on regulatory part need further discussed. 47 C.F.R. § 25.202: - (d) Frequency tolerance, Earth stations. The carrier frequency of each earth station transmitter authorized in these services shall be maintained within 0.001 percent of the reference frequency. - When received at network (reference at the SAN antenna), the doppler should be corrected by satellite movement and with UE pre-compensation, the frequency received at network should be 0.1ppm compared with is the configured UL frequency. - In frequency error or frequency tolerance performance, the frequency error always compared to a reference frequency, it is not clear to us in current wording, that the reference frequency contains an error component which neither algin with the regulatory nor expectation from 0.1 ppm meaning explained above. P5: there is “ideal doppler frequency” wording in current NTN FE requirement, as such, the TE needs to know what is the meaning and how to derive it, in our understanding, it should be RAN4 task to explain it as RAN4 define it in requirement. For the Time advance verification, we donot have the requirement discussion in RF requirement, so clarification of the how to handle this needs some guidance. |
| Huawei | Same comments as for the TP on Cat-M1 UE NTN Frequency Error. [UPDATE after GTW]: We see some valid points from Ericsson and ZTE’s comments. The root cause of the confusion seems to be that the wording of the NR-NTN spec mixes the UE behavior with that of the TE. There’re two pre-compensation operations in the spec, of which the first one is performed by the UE and the 2nd one is executed by the TE. The hidden assumption is that the Doppler shift of the propagation channel is not emulated by the TE. Hence the TE needs to manually apply the Doppler shift to the configured uplink carrier frequency, resulted in the so-called “ideally pre-compensated reference uplink carrier frequency”. If the propagation channel is properly emulated, the 2nd compensation wouldn’t be needed. So, an improvement to the NR-NTN wording may be: in the formal UE requirement, the frequency error measurement results are “compared to reference uplink carrier frequency”. Then add a note to say that if the Doppler shift of the propagation channel is not emulated, the TE needs to pre-compensate the uplink carrier frequency before computing the frequency error. In this way, the UE requirements would be separated from the TE behavior. |

### Further comments on 2nd level vs 3rd level clause 6 and 7 split

Please make comments below.

| Company | Comments |
| --- | --- |

## Summary for 2nd round

|  | Status summary |
| --- | --- |
| TP for NB-IoT UE Frequency Error (Qualcomm) | Still more discussion needed. Moderator proposes that as part of final checking we identify which parts are agreeable and leave an editor’s note for the remaining aspects to finalise next meeting if no agreement in extended round. |
| TP for Out-of-band blocking for b256 | Moderator proposes to approve the latest version of the TP. |
| TP on Cat-M1 UE NTN Frequency Error and b256 UE Rx sensitivity | Still more discussion needed on frequency error part. Moderator proposes that as part of final checking we identify which parts are agreeable and leave an editor’s note for the remaining aspects to finalise next meeting if no agreement in extended round. Rx sensitivity part is agreeable. |
| TP for clause 6 of TS36.102 | The updated version was provided late, however the changes are quite small (purely structural). Moderator proposes to approve this after final checking. Further discussion anyway needed on 2nd or 3rd level suffix rule. |
| TP for clause 7 of TS36.102 | The updated version was provided late, however the changes are quite small (purely structural). Moderator proposes to approve this after final checking. Further discussion anyway needed on 2nd or 3rd level suffix rule. |
| TP for clause 4 of 36.102 | Moderator proposes to agree the latest version with changes provided by MediaTek |
| Frequency Error proposal 2 and 5 | No agreement. Moderator proposes that Proposal 5 would need further discussion (see WF) Moderator proposes that Proposal 2 is handled directly as part of TP drafting for frequency error. |
| 2nd and 3rd level clause split for 36.102 | No discussion on this in 2nd round. Moderator proposes to further finalise on this next meeting. |
| A-MPR and Spurious Emission Way forward | WF not agreed. But confirm the principles of proposal 1, 2, 3 from sub-topic 3-2 in 1st round as a basis for further discussion. |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on 36.102 Clause 5, 6 and 7 general | MediaTek | Capture agreements made on topic 2 and sub-topics 3-1, 3-3. |
|  | WF on UE A-MPR and Emissions requirements | ZTE | Capture more concretely the status and outcome of sub-topic 3-2. |
|  | TP for NB-IoT UE Frequency Error | Qualcomm | Capture outcome of 3-1 |
|  | TP for Out-of-band blocking for b256 | Xiaomi | Capture outcome from 3-3 |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2215487 |  | discussion on IoT NTN system parameter | CMCC | Noted |  |
| R4-2215570 |  | Freq error with segmentation | Qualcomm Incorporated | Noted |  |
| R4-2215697 |  | Evaluating emission requirements and whether AMPR needs to be specified for IoT NTN operation in Band 255 or not | Ligado Networks | Noted |  |
| R4-2215779 |  | Discussion on UE RF requirements for IoT NTN | Mediatek India Technology Pvt. | Noted |  |
| R4-2216147 |  | Discussion on UE RF requirement for NB-iot/eMTC NTN | Xiaomi | Noted |  |
| R4-2216254 |  | REFSENS for NTN eMTC | Sony | Noted |  |
| R4-2216546 |  | Further discussion on system parameters | ZTE Corporation | Noted |  |
| R4-2216549 |  | Further discussion on UE RF requirements for IoT over NTN | ZTE Corporation | Noted |  |
| R4-2216635 |  | IoT UE RF remaining issues | Ericsson | Noted |  |
| R4-2216636 |  | TP for UE RF requirement (modified to “TP on Cat-M1 UE NTN Frequency Error and b256 UE Rx sensitivity”) | Ericsson | Revised |  |
| R4-2216637 |  | On System parameter for IoT NTN | Ericsson | Noted |  |
| R4-2216680 |  | TP for IoT NTN UE clause 6 | Mediatek India Technology Pvt. | Revised |  |
| R4-2216681 |  | TP for IoT NTN UE clause 7 | Mediatek India Technology Pvt. | Revised |  |
| R4-2216682 |  | TP for system parameter | Ericsson | Not pursued |  |
| R4-2216778 |  | Discussions on IoT NTN system parameters | Huawei, HiSilicon | Noted |  |
| R4-2216799 |  | Remaining issues for system parameters for IoT NTN | MediaTek (Chengdu) Inc. | Noted |  |
| R4-2216834 |  | TP to TS 36.102 clause 5 for IoT NTN | MediaTek (Chengdu) Inc. (changed to “ZTE”) | Revised |  |
| R4-2216884 |  | Discussion and TP on Suffix information for 36.102 (modified to “TP for clause 4 of TS36.102”) | Qualcomm Incorporated (changed to “Qualcomm Incorporated, MediaTek”) | Revised, merging with text in R4-2215779 |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2217747 |  | WF on UE RF and System Parameters for IoT NTN | MediaTek | Agreeable |  |
| R4-2217748 |  | WF on UE A-MPR and Emissions requirements | ZTE | Agreeable |  |
| R4-2217749 |  | TP for NB-IoT UE Frequency Error | Qualcomm | TBD | Pending extended round |
| R4-2217750 |  | TP for Out-of-band blocking for b256 | Xiaomi | Agreeable |  |
| R4-2217751 |  | TP on Cat-M1 UE NTN Frequency Error and b256 UE Rx sensitivity | Ericsson | TBD – Rx sensitivity part is agreeable | Pending extended round |
| R4-2217752 |  | TP for IoT NTN UE clause 6 | Mediatek India Technology Pvt. | Agreeable |  |
| R4-2217753 |  | TP for IoT NTN UE clause 7 | Mediatek India Technology Pvt. | Agreeable |  |
| R4-2217754 | R4-2217807 | TP to TS 36.102 clause 5 for IoT NTN | ZTE | Agreeable | After extended round |
| R4-2217755 |  | TP for clause 4 of TS36.102 | Qualcomm Incorporated, MediaTek | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents