3GPP TSG-RAN WG4 Meeting #104-Bis-e												R4-2217796

Electronic Meeting, Oct. 10 – 19, 2022

Agenda item:			8.4

Source:	Moderator (Apple)

Title:	Email discussion summary for [104-bis-e][144] NR_reply_LS_UE_RF

Document for:	Information

# Introduction

This email thread treats the following topics:

- Lower humidity limit in normal temperature test environment (R5-221604)

- Modified MPR-Behaviour clarification for different power classes (R5-223635)

- UE power limitation for STxMP in FR2 (R1-2205639)

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Sumant Iyer | sumanti@qti.qualcomm.com |
| OPPO | Jinqiang | xingjinqiang@oppo.com |
| Xiaomi | Juan Zhang | zhangjuan8@xiaomi.com |
| Huawei | Ye Liu | leo.liuye@huawei.com |
| Nokia (PV) | Petri Vasenkari | petri.j.vasenkari@nokia.com |
| InterDigital (IDC) | Virgil Comsa | virgil.comsa@interdigital.com |
| Ericsson | Christian Bergljung | Christian.Bergljung@ericsson.com |
| vivo | Ruixin Wang | Ruixin.wang@vivo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: Lower humidity limit in normal temperature test environment (R5-221604)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215706 | Samsung | Observation 1: there is not only humidity inconsistency among specifications, but also inconsistency between normal condition and extreme condition inside TS 38.101 itself. It is not reasonable that the humidity range is even more restrict for normal condition than extreme condition in terms of upper limit (75% vs 50%). Proposal 1: it is agreed in RAN4 that the humidity inconsistency issue among specifications should be resolved. Proposal 2: It is proposed to remove the explicit humidity range and normal temperature test is required to be performed under room humidity condition unless otherwise stated. +15C to +35C; for normal conditions (under room humidity conditions unless otherwise stated). -10C to +55C; for extreme conditions (see IEC publications 68-2-1 and 68-2-2). A draft reply LS is provided in Annex. |
| R4-2216618 | ZTE | Observation 1: The relative humidity for normal test conditions should be uniformly specified for GERAN/UTRA/E-UTRA/NR systems. Observation 2: It is observed that to keep the core specification stable is preferable to the industry. Observation 3: The high or low humidity limit for the normal conditions may have potential impacts on some test items and cannot be ignored. Proposal 1: It is suggested to select Option 3, i.e. to keep the current description in RAN4 spec with the relative humidity range of “25% ~ 75%” as the solution to resolve the inconsistencies among the specs. A draft reply LS to RAN5 is attached |

## Open issues summary

### Sub-topic 1-1: On possible ways to resolve the inconsistency

- Proposals

  - Option 1: It is proposed to remove the explicit humidity range and normal temperature test is required to be performed under room humidity condition unless otherwise stated..

  - Option 2: keep the current description in RAN4 spec with the relative humidity range of “25% ~ 75%” as the solution to resolve the inconsistencies among the specs.

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Samsung | Support option 1. In RAN4 specification TS 38.101-1: +15C to +35C; For normal conditions (with relative humidity of 25 % to 75 %). -10C to +55C; For extreme conditions (see IEC publications 68-2-1 and 68-2-2). In RAN4 specification TS 38.101-2: + 25 ⁰C ± 10 ⁰C; For normal (room temperature) conditions with relative humidity of 25 % to 75 %. -10C to +55C; For extreme conditions. There is no humidity requirement for extreme condition or refer to IEC publications in which humidity is up to around 50% without humidity lower limit. It is not reasonable that the humidity range is even more restrict for normal condition than extreme condition in terms of upper limit (75% vs 50%). We think the humidity inconsistency even within RAN4 specification should be resolved first of all. |
| OPPO | Option 2 probably. This has been discussed for serval meetings, we see no big impact from RAN4 perspective to keep it as it is, and leave it to RAN5 on the testing decide. |
| ZTE | Option 2. We agree with OPPO that keep RAN4 spec as it is now and leave it to RAN5 to decide since it is testing related issues. For option 2 the impact will be only within RAN5. Considering that we are now in the stage of Rel-18, we believe a stable core specification is more preferable to the industry. Furthermore, as to Option 1, it seems not clear what exactly the room humidity condition refers to. There is no explicit definition for the room humidity condition in the specifications which may lead to confusion in understanding. |
| Huawei | Option 2 is preferred. As ZTE pointed out, there might be no common understanding of room humidity. The humidity at different locations in the world and in different seasons could vary greatly. It would be clearer to have a fixed range in the technical specification. |
| Ericsson | Option 2. |
| Nokia (PV) | Option 2. |
| Apple | The inconsistency in the specifications is certainly evident and should be resolved. Could we agree that normal conditions and extreme conditions should match between 38.101-1 and 38.101-2? We are fine with Option 1. As a possible compromise, we could consider an alternative Option 3. Since the temperature ranges already match, we then decide how to set the humidity conditions. If we consider room humidity, then at least some common definitions of the ideal indoor humidity range exist (e.g. the US Environmental Protection Agency defines the ideal indoor humidity range to be between 30% and 50%). Since this upper limit matches the IEC publications (which are already applicable to the extreme conditions in 38.101), then it seems that humidity conditions can be simply set according to IEC publications in both normal and extreme conditions, and we can remove the 25%~75% range. This is one way to resolve the ambiguity. |

## Companies views’ collection for 1st round

### Open issues

Comments are collected in section “Open issues summary” above.

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Fiven companies supported Option 2 and two companies supported Option 1. Meanwhile, Option 3 is proposed (humidity conditions can be simply set according to IEC publications in both normal and extreme conditions, and we can remove the 25%~75% range). Recommendations for 2nd round: Continue the discussion in the WF in the second round. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

### Necessity to resolve the inconsistency.

Regarding to the necessity to resolve the inconsistency, the main issue is that if the humidity do impacts the test stability and reliability of devices. Furthermore, from Table 1, we see that the inconsistencies appear not only in the specs of different RATs but also in different RAN groups. However, during the discussion, it seems companies still have different views on whether the test environment should be the same in different RATs or in different RAN groups. Companies also have different views if it is reasonable to have different humidity ranges in normal condition and extreme condition.

- Question 3.1-1  Does the humidity environment have potential impacts on the test stability and reliability of devices?

  - Option 1:  Yes

  - Option 2:  No

  - Option 3:  Others

Discussion:

Agreement:

- Yes, the humidity environment has potential impacts on the test stability and reliability of devices although the degree of impact is FFS.

- Question 3.1-2  Do we need to unify the test environment for humidity in different RATs / different RAN groups?

  - Option 1:  Yes

  - Option 2:  No

  - Option 3:  Others

Discussion:

Agreement:

- Yes, there is a need or at least it is better to unify the test environment for humidity in different RATs / different RAN groups.

- Question 3.1-3  Is it reasonable to have different humidity ranges in normal condition and extreme condition?

  - Option 1:  Yes

  - Option 2:  No

  - Option 3:  Others

Discussion:

- No consensus on if it is reasonable to have different humidity ranges in normal condition and extreme condition.

- Option 1:  ZTE, HW

- Option 2:  OPPO, Apple

- Option 3:  Samsung

Recommendations for 2nd round (Please provide your comments below)

| Company | Comments on Question 3.1-1 & Question 3.1-2 & Question 3.1-3 |
| --- | --- |
| Samsung | - Question 3.1-1 Does the humidity environment have potential impacts on the test stability and reliability of devices? - There is low dependency except when humidity is too high. As we can see humidity is just a side condition for temperature. - Question 3.1-2 Do we need to unify the test environment for humidity in different RATs / different RAN groups? - Option 1: Yes. The humidity inconsistency does exist now even for specifications within the same work group. - Question 3.1-3 Is it reasonable to have different humidity ranges in normal condition and extreme condition? - Option 3: Others. It depends on RAN4 further discussion on how to solve the specification inconsistent issue. |
| OPPO | Question 3.1-1 Does the humidity environment have potential impacts on the test stability and reliability of devices? Option 1: Yes, in principle, but FFS on the impacts. Question 3.1-2 Do we need to unify the test environment for humidity in different RATs / different RAN groups? Align is good if in the beginning of Rel-15 when define requirements, now see no much benefits. Test environment can be handled in RAN5. Question 3.1-3 Is it reasonable to have different humidity ranges in normal condition and extreme condition? Align is good if in the beginning of Rel-15 when define requirements, now see no much benefits. |
| ZTE | - Question 3.1-1 Does the humidity environment have potential impacts on the test stability and reliability of devices? Option 1, Yes. From the perspective of the propagation characteristics of wireless signals, temperature and humidity may have potential impacts on the radio test results. - Question 3.1-2 Do we need to unify the test environment for humidity in different RATs / different RAN groups? Option 1,Yes. The relative humidity for normal test conditions should be uniformly specified for GERAN/UTRA/E-UTRA/NR systems. Also it’s better to align among different RAN groups. - Question 3.1-3 Is it reasonable to have different humidity ranges in normal condition and extreme condition? Option 1, Yes. There is no need to have the same humidity ranges for normal condition and extreme condition. |
| Apple | - Question 3.1-1 Does the humidity environment have potential impacts on the test stability and reliability of devices? - In principle, yes, although the degree of the impact is not currently quantified. - Question 3.1-2 Do we need to unify the test environment for humidity in different RATs / different RAN groups? - Support Option 1. - Question 3.1-3 Is it reasonable to have different humidity ranges in normal condition and extreme condition? - Support Option 2. As we commented in the first round, the best candidate approaches might be as Samsung had proposed or to simply use the IEC recommendation for both normal and extreme conditions. |
| Huawei | - Question 3.1-1 Does the humidity environment have potential impacts on the test stability and reliability of devices? Option 1, Yes. The extremely low or high humidity needs to be avoided. - Question 3.1-2 Do we need to unify the test environment for humidity in different RATs / different RAN groups? No strong opinion. It’s preferred to not change E-UTRA and earlier specifications as those have been commonly used for quite long time. - Question 3.1-3 Is it reasonable to have different humidity ranges in normal condition and extreme condition? Option 1, Yes. The humidity condition requirement could be different for normal and extreme condition. |

### If resolution is necessary, in which RAN group to resolve the inconsistency?

In the previous discussion, some solutions have been proposed to resolve the inconsistency. The main disagreement lies in the aspects in which RAN group the inconsistencies will be solved.

- Question 3.2-1  In which RAN group we intend to resolve the inconsistency?

  - Option 1:  RAN4

  - Option 2:  RAN5

  - Option 3:  Both RAN4 and RAN5

  - Option 4:  Others

Discussion:

- No consensus on the RAN group to resolve the inconsistency.

- Option 1:  Samsung, Apple

- Option 2:  OPPO, ZTE

Recommendations for 2nd round (Please provide your comments below)

| Company | Comments on Question 3.2-1 |
| --- | --- |
| Samsung | Finally the inconsistency should be resolved in all 3GPP specifications. Currently we should focus on solving the inconsistency in RAN4 specifications firstly. |
| OPPO | Option 2 as the triggering and impact group. |
| ZTE | Option 2. Considering that the test environment has been specified in RAN4 for many years, to keep the core spec stable is preferable to the industry. |
| Apple | RAN4 should resolve the inconsistency in the RAN4 specifications first, and we can communicate with RAN5 as the next step. Whether RAN5 adopts the solution is up to RAN5 WG discussion, however. |

### If resolution is necessary, which option is preferred?

To resolve the inconsistency on humidity range, companies are welcome to provide their preference among the following options, any other solutions are not precluded.

- Question 3.3-1  Which option is the preference to resolve inconsistency?

  - Option 1:  It is proposed to remove the explicit humidity range and normal temperature test is required to be performed under room humidity condition unless otherwise stated.

  - Option 2:  Keep the current description in RAN4 spec with the relative humidity range of “25% ~ 75%” as the solution to resolve the inconsistencies among the specs.

  - Option 3:  Humidity conditions can be simply set according to IEC publications in both normal and extreme conditions, and we can remove the 25%~75% range.

Discussion:

- No consensus on which option to resolve inconsistency.

- Option 1:  Samsung

- Option 2:  OPPO, ZTE, HW

- Option 3:  Apple

Recommendations for 2nd round (Please provide your comments below)

| Company | Comments on Question 3.3-1 |
| --- | --- |
| Samsung | Prefer option 1. Option 3 can also be further discussed, but the point is RAN4 need to achieve the consensus that the humidity inconsistency need to be solved, it is suggested that the WF can capture such agreement: “Humidity inconsistency does exist in RAN4 specifications and RAN4 agreed to solve the humidity inconsistency issue”. Options could be captured in WF for further discussion if down-selection could not be achieve this meeting. |
| OPPO | Option 2. |
| ZTE | Option 2. To keep the core specification stable, it is preferable to keep the current RAN4 spec with the relative humidity range of “25% ~ 75%” as it is now. |
| Apple | Prefer Option 3 and can agree to Option 1 if that is the group’s preference. Option 2 seems to be a step backward, since it would retain all of the inconsistencies we have already identified. |
| HW | Option 2. The humidity range of “25%~75%” has been used and referred by the industry for many years. It’s preferred to keep it stable. |

# Topic #2: Modified MPR-Behaviour clarification for different power classes (R5-223635)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215317 | Nokia, Qualcomm Inc, Skyworks Inc, Ericsson | - For Rel-15 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE supports modifiedMPR-Behaviour bit 0 UE capability? Proposed answer: RAN4 has agreed that if UE supports modifiedMPR-Behaviour bit 0 UE capability it shall support latest requirements over all releases and specification versions bit relates to unless new modified MPR-Behaviour bit is defined for same requirement the bit relates to. - For Rel-15 PC2 and 4 UEs, is modifiedMPR-Behaviour bit 0 capability applicable? Proposed answer: Yes it is. - For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? Also, is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true? Proposed answer: RAN4 has agreed [2] that in REL16 this feature is mandatory and support shall be signalled. RAN4 is aware that there are currently UEs in the market that do not support this bit functionality. As an exceptional case due to late change to specification Rel16 are allowed to support MPR defined in REL15. RAN4 leaves it to RAN5 judgement how to do this. - For Rel-16 PC2, 4 and 5 UEs, is the PC3 MPR as defined in 38.101-2 v16.2.0 applicable? Also, is modifiedMPR-Behaviour bit 0 capability applicable? Proposed answer: Same as answer in c. - Is any kind of Rel-16 UE supposed to support MPR as defined in 38.101-2 version v16.11.0? - No, see [2] - For Rel-17 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE signals modifiedMPR-Behaviour bit 0=true? Proposed answer: Rel17 38.101 Annex-H is not correct. It should state This bit SHALL be set to 1 instead of MAY. There is a RAN4 CR on this [1] - For Rel-17 PC3 UE, what is the MPR requirement if the UE signals modifiedMPR-Behaviour bit 0=false? Proposed answer: See answer in c |
| R4-2216355 | Xiaomi | Proposal 1: Answer c): The MPR as defined in 38.101-2 v16.2.0 is optional for Rel-16 PC3 UE, the Rel-16 UE is optional to signal modifiedMPR-Behaviour bit 0=true. Answer d): For Rel-16 PC3 UE, 38.101-2 v16.1.0 and earlier versions are taken as default MPR requirement. The PC3 UE need meet the MPR as defined in 38.101-2 v16.2.0 if the PC3 UE signals modifiedMPR-Behaviour bit 0=ture, if not, the PC3 UE just need meet the default MPR requirement. |
| R4-2216442 | OPPO | Observation 1: Improved MPR was introduced in Rel-16 38.101-2 v16.2.0 which makes two different MPRs exist in Rel-16 specs, and modifiedMPR-Behaviour IE was used to indicate which MPR UE supports. Observation 2: Rel-16 UE can comply with either the MPR defined before v16.2.0 or after that. Observation 3: It is unclear the meaning of “open release” and “freeze release” in RAN4, usually only ASN.1 matter and RAN4 WI can close before or after the ASN.1 frozen as long as no signaling impact. Proposal 1: For Rel-16 PC3 UE, the MPR defined in 38.101-2 v16.2.0 is optional. Proposal 2: Encourage proponents to clarify the meaning of “open release” and “freeze release” in RAN4, and align the understanding of requirement applications for different spec versions. Observation 4: If the modifiedMPR-Behaviour refer to the fixed version v16.2.0, then the changes after that will no longer be used, and there is no way for RAN4 to update the MPR section after v16.2.0. This is not reasonable. Observation 5: It was agreed in FR1 that latest spec version will be referred in the modifiedMPR-Behavior definition, and remove the fixed version from the table. Proposal 3: Align the spec version handling between FR1 and FR2, and in FR1 the specific version has been removed, similar can be done in FR2. Proposal 4: For Rel-16 PC3 UE, the latest version of specification is used when modifiedMPR-Behaviour bit 0=true. And the 38.101-2 v16.1.0 is used when UE signals modifiedMPR-Behaviour bit 0=false. Observation 6: In Rel-17, only one MPR defined, there is no point for Rel-17 UE to comply with Rel-16 MPR specification. Even remove the Rel-16 version information from Rel-17 spec, still there is no more information this modifiedMPR-Behavior can provide since it is basic principle that Rel-17 UE will comply with Rel-17 specification. Proposal 5: For Rel-15 and Rel-16, the modifiedMPR-Behaviour pointing to the latest version of specification by using “from v16.2.0”. For Rel-17, remove the modifiedMPR-Behaviour table which pointing to the improved MPR requirements defined since v16.2.0. |
| R4-2216677 | Huawei, HiSilicon | Observation 1: From the current specification, it is observed that modified MPR is not mandatory for the release when it is introduced. |
| R4-2216678 | Huawei, HiSilicon | Apart from the previous reply in R4-2215091, RAN4 made further conclusion for the remaining questions as follows: Question c) For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? In case it is mandatory then is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true? Answer: For Rel-16 PC3 UE, the MPR as defined in 38.101-2 v16.2.0 is optional according to the current specification. Question d) For Rel-16 PC3 UE, which version of specification is taken as default MPR requirement, 38.101-2 v16.2.0 or latest version (v16.11.0 released in Apr 2022)? What are the Rel-16 MPR requirements if the UE signals respectively modifiedMPR-Behaviour bit 0=false and modifiedMPR-Behaviour bit 0=true? Answer: For Rel-16 PC3 UE, 38.101-2 v16.1.0 and earlier versions are taken as default MPR requirement. The PC3 UE needs to meet the MPR as defined in 38.101-2 v16.2.0 if the UE signals modifiedMPR-Behaviour bit 0=ture; if the bit is set to false, the PC3 UE just needs to meet the default MPR requirement, i.e. MPR specified in 38.101-2 v16.1.0. |

## Open issues summary

### Sub-topic 2-1: Question c) For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? In case it is mandatory then is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true?

- Proposed answers

  - Option 1: RAN4 has agreed [2] that in REL16 this feature is mandatory and support shall be signalled. RAN4 is aware that there are currently UEs in the market that do not support this bit functionality. As an exceptional case due to late change to specification Rel16 are allowed to support MPR defined in REL15. RAN4 leaves it to RAN5 judgement how to do this.

  - Option 2: For Rel-16 PC3 UE, the MPR as defined in 38.101-2 v16.2.0 is optional according to the current specification.

  - Option 3: Others

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| OPPO | Option 2. The current spec is clear on this. Improved MPR was introduced in Rel-16 38.101-2 v16.2.0 which makes two different MPRs exist in Rel-16 specs, and modifiedMPR-Behaviour IE was used to indicate which MPR UE supports. Rel-16 UE can comply with either the MPR defined before v16.2.0 or after that. |
| Xiaomi | Option 2, Rel-16 UE can indicate which MPR UE support by IE modifiedMPR-Behaviour |
| Huawei | Option 2. For the bands supporting modified MPR, the usual way adopted in the spec is to mandate the capability from the next release when the modified MPR is specified according to the current specification. It’s not possible to make changes for a long existing optional capability to be mandatory. We agree with OPPO, the current spec is clear that for Rel-16, the modified MPR is optional. |
| Nokia (PV) | Whether Rel-16 MPR is mandatory or not seems to require more discussion, note that the proposal is to allow for this case UE pass the tests with Rel-15 MPR |
| Ericsson | Option 1: a Rel-16 UE shall meet requirements specified in the Rel-16 specification unless optional according to the main body of the specification, no matter if introduced when this release was open or as part of Rel-16 maintenance. MPR requirements are mandatory. The intention of the modifiedMPRbehavior bitmap is to make possible for an early UE (e.g. Rel-15) to indicate to the network that a specific MPR behavior specified in a later version (e.g. v16.2.0) is supported, it is not for indicating applicability of core requirements in conformance tests. RAN5 can apply a grace period for Rel-16 IEs only compliant with the Rel-15 behavior, if needed. RAN5 has only recently started working on the new MPR, it will take a while before this test is validated by GCF |
| Apple | Option 2 |

### Sub-topic 2-2: Question d) For Rel-16 PC3 UE, which version of specification is taken as default MPR requirement, 38.101-2 v16.2.0 or latest version (v16.11.0 released in Apr 2022)? What are the Rel-16 MPR requirements if the UE signals respectively modifiedMPR-Behaviour bit 0=false and modifiedMPR-Behaviour bit 0=true?

- Proposed answers

  - Option 1: For Rel-16 PC3 UE, 38.101-2 v16.1.0 and earlier versions are taken as default MPR requirement. The PC3 UE needs to meet the MPR as defined in 38.101-2 v16.2.0 if the UE signals modifiedMPR-Behaviour bit 0=ture; if the bit is set to false, the PC3 UE just needs to meet the default MPR requirement, i.e. MPR specified in 38.101-2 v16.1.0.

  - Option 2: Others

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| OPPO | Option 1 is aligned with our understanding. The improved MPR is an optional feature comparing to the previous MPR. |
| Xiaomi | Option 1 |
| Huawei | Option 1 |
| Ericsson | Option 2: a Rel-16 UE shall meet the latest version of the Rel-16 specification. Only a Rel-15 UE can optionally indicate to the network that it supports the v16.2.0, a Rel-16 UE shall indicate bit 0 = ‘true’. |
| Apple | Option 1 |

### Sub-topic 2-3: Comments on the following proposal:

Proposal 1: ”For Rel-15 and Rel-16, the modifiedMPR-Behaviour pointing to the latest version of specification by using “from v16.2.0”.

Proposal 2: For Rel-17, remove the modifiedMPR-Behaviour table which pointing to the improved MPR requirements defined since v16.2.0.”

| Company | Comments |
| --- | --- |
| OPPO | We support the two proposals. It was agreed in FR1 that latest spec version will be referred in the modifiedMPR-Behavior definition, and removed the fixed version from the table. In FR2, we can apply similar approach by using “from v16.2.0” which means the latest version is applied. In Rel-17, only one MPR exist in the spec, there is no point for Rel-17 UE to refer back to Rel-16 MPR specification. What Rel-17 UE should comply is the Rel-17 spec. |
| Xiaomi | Support these two proposal |
| Huawei | Sometimes the clause of MPR requirement could be adjusted later, thus it would be better to use clear version information for the modified MPR when it is specified, thus “from” may cause some ambiguity for later release. We prefer not to make the change as in proposal 1. The modifiedMPR-Behaviour table provides the bit mapping information, which is important for a UE to indicate the applicable requirements. We think the table should not be removed even the bit shall be set to 1. |
| Nokia (PV) | from v16.2.0 is a good proposal, if this is already agreed for FR1 sorry for missing this. To Huawei MPR in n16.2.0 is not anymore valid as there has been 2 CRs after that, one editorial and one correction. How Huawei would solve this? Do we have common understanding that for Rel-17 MPR is mandatory and a CR is definitely needed? |
| Ericsson | None of the two proposals. For a Rel-15 UE (or any early implementation) the requirement in the later release cannot be floating, the bit must point at a requirement in a specific version, 16.2.0 in this case. A Rel-17 UE shall meet the latest version of the Rel-17 specification, a Rel-17 UE shall also indicate bit 0 = ‘true’ (the definition of bit 0 should remain unchanged like all other capability bits in the RRC specification unless extended). Given all these issues, the modifiedMPR-Behavior should not be used for minor modifications but rather for changes that result in a different UE behavior in the field. |
| Apple | Agree with Ericsson’s comment. The behavior of the bit should not change from release to release: just its optionality. Thus, the Rel-17 specification should mandate the bit to be set to “true” and also specify the corresponding requirements if it is set to “true” (latest Rel-17 spec) and if it is set to “false” (v16.1.0 of the spec) |

## Companies views’ collection for 1st round

### Open issues

Comments are collected in section “Open issues summary” above.

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2215316 | Company A |
|  | Company B |
| R4-2215970 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1, 2-2, 2-3 | 2-1: Four companies supported Option 2 and two companies supported Option 1. 2-2: Four companies supported Option 1 and one company supported Option 2. 2-3: There were no agreement. Recommendations for 2nd round: Continue the discussion in the WF in the second round. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### modifiedMPR-Behaviour for Rel-16 UE (remaining issues for the reply LS to RAN5)

### Question c) For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? In case it is mandatory then is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true?

The following options have been discussed in 1st round.

Option 1: RAN4 has agreed that in REL16 this feature is mandatory and support shall be signalled. RAN4 is aware that there are currently UEs in the market that do not support this bit functionality. As an exceptional case due to late change to specification Rel16 are allowed to support MPR defined in REL15. RAN4 leaves it to RAN5 judgement how to do this. (E//, Nokia)

Option 2: For Rel-16 PC3 UE, the MPR as defined in 38.101-2 v16.2.0 is optional according to the current specification (OPPO, Xiaomi, HW, Apple)

Summary of 2nd round discussion

Views are still diversified. Given the current situation and limited time in this meeting, it is suggested to further discuss this issue with the listed options in next meeting.

- Recommended WF

FFS in next meeting.

| Company | Comments |
| --- | --- |
| Qualcomm | (Related to next discussion) |
| Huawei | Option 2. In Rel-16 specification, not only FR2 we discussed in the reply LS but also many other FR1 bands support modified MPR. It is clear in the spec that the bit 0 may be set to 1 when the modified MPR specified and in the next release for most cases the bit shall be set to 1. In addition, it’s too late to mandate a Rel-16 UE to comply with a different requirement. The UE design is not from scratch, usually band specific design are often inherited from release to release unless there is critical requirement change. Some transition time is needed for UE to reconsider the design to comply with the changes. |
| Nokia | Option 1 |
| Xiaomi | Option 2. In Rel-16, when the modifiedMPR-Behaviour was discussed to introduce for FR2, some company had tried to use ‘must’ instead of ‘may’, but it wasn’t agreed. So the MPR for Rel-16 PC3 UE as defined in 38.101-2 v16.2.0 should still keep optional as current spec defined. |
| OPPO | Option 2. |
| Ericsson | Option 1. A grace period can be allowed for Rel-16 UEs implemented against the existing Rel-16 version with the incorrect modified-MPR table should RAN5 consider this necessary. |
| Apple | Option 2 |

### Question d) For Rel-16 PC3 UE, which version of specification is taken as default MPR requirement, 38.101-2 v16.2.0 or latest version (v16.11.0 released in Apr 2022)? What are the Rel-16 MPR requirements if the UE signals respectively modifiedMPR-Behaviour bit 0=false and modifiedMPR-Behaviour bit 0=true?

The following options have been discussed in 1st round.

Option 1: For Rel-16 PC3 UE, 38.101-2 v16.1.0 is taken as default MPR requirement. The PC3 UE needs to meet the MPR as defined in 38.101-2 v16.2.0 if the UE signals modifiedMPR-Behaviour bit 0=ture; if the bit is set to false, the PC3 UE just needs to meet the default MPR requirement, i.e. MPR specified in 38.101-2 v16.1.0. (OPPO, Xiaomi, HW, Apple)

Option 2: A Rel-16 UE shall meet the latest version of the Rel-16 specification. Only a Rel-15 UE can optionally indicate to the network that it supports the v16.2.0, a Rel-16 UE shall indicate bit 0 = ‘true’. (E//)

Summary of 2nd round discussion

Views are still diversified. Given the current situation and limited time in this meeting, it is suggested to further discuss this issue with the listed options as well as the proposed RAN5 test solution in next meeting.

- Recommended WF

FFS in next meeting.

| Company | Comments |
| --- | --- |
| Qualcomm | We appreciate both sides of the problem. We think option 2 packaged with option 1 from the previous topic (advising RAN5 to accommodate Rel-16 UEs with Rel-15 MPR) may be a compromise to resolve the disconnect between RAN4 intent and signaling concerns from some companies. |
| Huawei | Option 1. The modified MPR for a Rel-16 UE shall be optional. If the bit is absent or bit 0 is false, the original MPR should be applied. |
| Nokia | Option 2 but with exception that Rel16 can be tested with REL15 MPR. Option 1 has a problem which is that it states UE shall meet MPR 16.2.0 if bit is set true, however 16.2.0 is not valid as there has been correction/editorial CRs later for MPR section. |
| Xiaomi | Option 1, the MPR as defined in 38.101-2 v16.2.0 should still keep optional as current spec defined. |
| OPPO | Option 1. |
| Ericsson | Option 2, otherwise the release handling would be broken. A modification of a mandatory feature (like MPR) in the open release is mandatory (Rel-16 was open when the CR against 16.2.0 was agreed). |
| Apple | Option 1 |

### modifiedMPR-Behaviour for Rel-15 and Rel17 UE (MPR changed in Rel-16 v16.2.0)

#### Rel-15 UE, spec version for modifiedMPR-Behaviour

Option 1: For a Rel-15 UE, the modifiedMPR-Behaviour pointing to the latest version of specification by using “from v16.2.0”.

Option 2: For a Rel-15 UE (or any early implementation) the requirement in the later release cannot be floating, the bit must point at a requirement in a specific version, 16.2.0 in this case.

Summary of 2nd round discussion

Views are still diversified. With the clarifications during discussion, companies can further think about the issue in next meeting.

- Recommended WF

FFS in next meeting.

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1. In our view, option 2 has no mechanism to accommodate running changes to the requirement. |
| Huawei | Considering the possible running changes after the MPR is modified, once the bit 0 is set to 1, we are open to use the latest version by using “from v16.2.0”. But the running change should not be significant, otherwise the Rel-15 UE may not support such changes. In that sense, option 2 is safer for a Rel-15 UE. |
| Nokia | Option 1, option 2 has the issue Qualcomm is explaining |
| OPPO | Option 1. Agree with QC that when set to 1 the latest version should be used. |
| Ericsson | Option 2, otherwise the network would have no idea about the version of the Rel-16 specification that an early UE (Rel-15) is supporting. The purpose of the bit is not for applicability of conformance requirements, it to indicate to the network that an early UE supports a change – supposedly an improvement – specified in a later release. |
| Apple | Both conditions of the modifiedMPR bit (true and false) need to identify which requirements the UE shall fulfill without ambiguity. For the case it is set to false, then we can refer to items 1 and 2 in this WF for the clarification. For the case it is set to true, then we can either rely on the specification not to ever change the requirements after 16.2.0, in which case we can simply refer to the latest version of the specification, or we can link the true condition to a specific version of the Rel-16 specification. From the perspective of reducing the probability of incompatibility between NW and UE behavior, Option 2 seems to be the most reasonable. Furthermore, in our understanding, the mechanism to accommodate running changes of the requirement is the modifiedMPR framework. If the requirements need to be modified in the future, then a new modifiedMPR bit would have to be defined, since legacy UEs would not be aware of any future changes. |

#### Rel-17 UE, whether modifiedMPR-Behaviour table could be removed

Option 1: For Rel-17, remove the modifiedMPR-Behaviour table which pointing to the improved MPR requirements defined since v16.2.0.”

Option 2: The modifiedMPR-Behaviour table shall be kept. A Rel-17 UE shall meet the latest version of the Rel-17 specification, a Rel-17 UE shall also indicate bit 0 = ‘true’ (the definition of bit 0 should remain unchanged like all other capability bits in the RRC specification unless extended).

Summary of 2nd round discussion

Views are still diversified. At least all companies agree the modifiedMPR-Behaviour table shall be kept. Whether the entry in the table could be revised can be further discussed.

- Recommended WF

FFS in next meeting.

| Company | Comments |
| --- | --- |
| Qualcomm | We presume the discussion is about the entry in the table rather than the table itself. (We would like to retain the table). Prefer Option 1: remove bit and use ‘reserved, shall be set to 1’ or similar Problem with Option 2: Consider the eventual Rel-18 requirement: it is ambiguous if the Rel-16 MPR should apply to a Rel-18 UE or the Rel-18 MPR, if wording referencing Rel-16 is retained. |
| Huawei | Option 2. If we understand correctly, the proposal of option 1 is to remove the table completely, which is not favored by us. For Rel-17, we agree that the UE shall set bit 0 = true and the corresponding Rel-17 category F CR can be agreed as well. |
| Nokia | Table should be kept and QC proposal of remove bit and use ‘reserved, shall be set to 1’ or similar could be considered. Then it is understood in this specific case that - REL15 may support latest spec of REL16 or should this be latest over all releases?’ - REL16 may or shall support modified MPR is under discussion but testing exception has been discussed as an compromise - REL17 UE shall support REL17 MPR hence referencing to v16.2.0 MPR is not good, stating “from v.16.2.0” is ambiguous what is the last version then is it 16.11.0 or latest running spec - REL18 same discussion as REL17 |
| OPPO | Option 1 with clarification that this is not to remove the table, but the corresponding bit (QC alternative is ok with us). As explained in our paper, in Rel-17 there is only one set MPR (the improved one), no other alternatives, therefore, Rel-17 UE shall comply with Rel-17 MPR (the improved one). There is no point to comply with Rel-16 MPR. |
| Ericsson | Option 2. |
| Apple | Option 2. In the Rel-18 specification it is straightforward to accommodate the modifiedMPR table and to indicate which parameters shall be set to “true” as part of compliance with the Rel-18 spec. |

# Topic #3: UE power limitation for STxMP in FR2 (R1-2205639)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215652 | Apple | Response to Q1: Power limitation from regulatory perspective is always per UE; power control per panel (e.g. Pmax configuration from the network or TPC command) should be per TCI state, and based on this observation Assumption 1 seems feasible. If the UE maps different TCI states to different panels, then it is up to UE’s implementation how to implement this mapping. Response to Q2: Assumption 2 is already implemented in 38.101-2 (i.e. power limitation per UE); for a UE capable of simultaneous Tx on multiple TCI states, this power limitation should be met over the sum of all simultaneously transmitted TCI states Response to Q3: Assumption 2 total must equal to the existing power limitation for a given power class (this has regulatory implications); Assumption 1 (i.e. Tx power associated with a particular TCI state) should be limited by the existing power limitation for a given power class and any applicable network configuration; the sum of Tx powers associated with all TCI states which are transmitted simultaneously shall be limited by the existing power class limitation (this has regulatory implications) Response to Q4: Both Assumption 1 and Assumption 2 are feasible and can be applied to the same UE, and the sum of per-panel power limitations shall not exceed the existing power limitations for the UE’s supported power class. Proposal 1: It is proposed to take the suggested responses to the RAN1 questions provided in this contribution for consideration in the RAN4 response. |
| R4-2216122 | vivo | Observation 1: RAN1 is doing research of per-panel power control scheme, and RAN4 is asked to take part in the feasibility study and requirements definition. Observation 2: Current RAN4 discussion is already touch the core port of the WI but no specific TU and agenda except for this LS, thus making difficult to converge. Observation 3: RAN1 is still making progress even without RAN4’s feedback. Proposal 1: A preliminary LS which answers some of the questions and provide some background information for alignment may be enough for current stage, and further details can be left to the WI starts in RAN4. Proposal 2: The concept of “panel” may be referenced from RAN1 or defined RAN4 specific more implementation orientated concept. The details can also be postponed to later stage. Proposal 3: Detailed power limitation may not be that necessary to be sent to RAN1 in this stage. Proposal 4: For the sum up the multiple “EIRP” in different directions, the following concept of “per-UE” EIRP can be considered: To sum up of all respective beams in a certain direction as a function of a certain angle as “per-UE” EIRP "per UE" EIRP(θ,φ)=EIRP(beam1,θ,φ)+EIRP(beam2,θ,φ)+…+EIRP(beamn,θ,φ) Note: Those EIRP items for different beams are only for concept illustration and cannot be differentiated if configured in one CC. Proposal 5: A simplified version of draft LS was attached aiming at easier agreement. If new agreements such as the definition of “panel” or the summing up method of EIRP in different direction can be reached, they can also be incorporated in the LS. Question 1: From RAN4 perspective, is Assumption 1 is feasible? Answer: Yes. Question 2: From RAN4 perspective, is Assumption 2 is feasible? Answer: Yes. Question 3: In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? Answer: RAN4 confirm that existing UE RF requirements are framed so standards compliance implies regulation compliance (clause 6.5x in TS38.101-2). For any additional limitation like the sum over all panels of the per-panel power limitation for STxMP, would be defined in RAN4 if necessary after the WI started in RAN4. Question 4: If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can/shall be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? Answer: It is believed that both assumptions are feasible, and both assumptions shall be applied to a same UE. The per-panel power limitation would be defined if necessary in latter stage, and the per-UE power limitation should be applicable at all the time. [Editor’s note: Clarifications on how to calculate sum up two different beams, e.g. “per UE” EIRP, can be added if agreed] |
| R4-2216354 | Xiaomi | Proposal 1: RAN4 need further check with RAN1 whether one panel means one beam. Proposal 2: max EIRP, min peak EIRP and EIRP spherical coverage should be limited per-panel, max TRP can be limited either per-panel or per UE over all UE panels, it depends on UE types. Proposal 3: The output power for the UE types which don’t need consider the total power consumption, can be limited per-panel. And the per-panel power limitation for STxMP including max TRP, max EIRP, min peak EIRP can be the same with the existing power limitation for a given power class. Proposal 4: The output power for the UE types which need consider the total power consumption, the total power limitation should be considered per UE over all UE panels. How to consider the total power limitation based on per UE over all UE panels need further study. |
| R4-2216411 | InterDigital | Observation 1: The current base station testing methodology (38.141-2) allow for EIRP and TRPmax testing for multiple beams simultaneous transmissions. Observation 2: Current power class definitions from 38.10-2 are clear and are applicable. Observation 3: Two beams may have an insufficient angular difference, so the panels may share the same EIRPmax limit. Observation 4: The panel-beam (or TCI state) and antenna ports relation may not be mutual exclusive within STxMP RF requirements context. Observation 5: In RAN4 understanding, for Assumptions 1 and 2, EIRP limits versus total power radiation limit (TRPmax) are part of the Pcmax equations. However, this it is a power limit or range targeting a single UL transmission and the only relation with UL MIMO are the specific side conditions mentioned in 6.2.4D subclause of 38.101-2 specification. Observation 6: RAN4 current Pcmax definition and applicability: - Pcmax definition for FR2 has a directional (EIRP) component and a per UE (TRP) component. - For UL MIMO the measured radiated power is related to nrofSRS-ports which is set to 2 and different TPMIs that are related to ULFPTx modes and 2 MIMO UL layers. - RAN4 didn’t studied two simultaneous transmissions requirements for FR2. Proposal 1: From RAN4 perspective both Assumption 1 and 2 are feasible. Proposal 2: RAN4 discusses question 3 by considering regulatory perspective. Proposal 3: For Question 4, RAN4 discusses how power class parameters EIRPmin, EIRPmax and TRPmax can be captured in Pcmax parameter for Assumptions 1 & 2. |
| R4-2216586 | Huawei, HiSilicon | RAN4 would like to thank RAN1 for LS on UE power limitation for STxMP in FR2. RAN4 responses to RAN1 questions are below. We continue to use the following definitions for Assumptions 1 and 2: - Assumption 1: Power limitation per panel for STxMP - Assumption 2: A total power limitation per UE over all UE panels used for STxMP Before RAN4 can analyze the questions in the LS, RAN4 kindly requests clarifications for terms used: - Panel: RAN4 currently does not have a definition for this entity. - Power limitation: From RAN4 perspective, there are several concepts related to the power radiated by UE: - Power class: The definition of power class (e.g. TS 38.101-2 clause 6.2.1.x), which is a package composed of below requirements - Min peak EIRP (The lower limit of EIRP at Tx beam peak direction); - Max EIRP (This is derived from regulatory requirements) and Max TRP; - Spherical coverage (The minimum EIRP at the Nth percentile of the distribution of power measured over the full sphere around the UE). - Configured transmitted power: PCMAX, f, c, which is used in RAN1 spec TS 38.213 power control part, and also applied in TS 38.101-2 clause 6.2.4: “The configured UE maximum output power PCMAX, f, c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215”. It is noted that PCMAX used by RAN1 power control mechanism for FR2 is considered at the virtual antenna connector which is not testable from RAN4 perspective. - Total power concept: In Rel-17, RAN4 had a discussion about “total power concept”, which is a maximum output power limitation for a FR2 UE from implementation perspective, but there is no conclusion and the related discussion is still ongoing in Rel-18. In general, it means the actual radiation power of a UE considering multiple implementation aspects e.g. MPE limitation and heat dissipation. In this sense, the actual TRP limitation of the UE is not identical to the max limitation, i.e. Max TRP as defined for the power class of the UE. - P-max: The parameter p-Max (i.e. p-UE-FR2) similar to FR1 p-UE-FR1 was introduced by RAN2 spec, which is the maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2) across all cell groups. However, the feasibility of P-max for FR2 is questionable in RAN4, which is not reflected in RAN4 requirements yet. In addition, whether such parameter to limit the max output power of UE should be EIRP based or TRP based is not clear during previous discussion in RAN4. With the RAN1 definition of panel, RAN4 would like to ask which above option (or multiple options) would match the concept of power limitation in the LS. Moreover, are the “per panel power limitation”, “per UE power limitation”, “total power limitation” and “existing power limitation” identical to this concept of power limitation? If not, please further explain those definitions from RAN1 perspective. |
| R4-2216783 | Qualcomm | Proposal 1: RAN4 to adopt the definition of panel as used in RAN1 discussions: ‘Panel’ is defined as one or multiple as combination of below depending on different UE implementation: - Unit of antenna group to control beam independently - Within a panel, one beam can be selected and used for UL transmission. - Across different panels, multiple beams (each selected per panel) may be used for UL transmission - ‘Beam’ is assumed to mean spatial filter associated with transmission or reception - Unit of antenna group to control its transmission power - Unit of antenna group to have a common UL timing. Proposal 2: RAN4 to focus on the configured Tx power requirement while addressing ‘power limitation’ for STxMP in FR2. Proposal 3: RAN4 to consider the following formulation for the configured power requirement as it would apply to STxMP. ### 6.2x.4 Configured transmitted power for [STxMP] The UE can configure its maximum output power. The configured UE maximum output power PCMAX,f,c,k for TCI state k of carrier f and serving cell c defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k for TCI state k is within the following bounds PPowerclass + PIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k) + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k)), T(P-MPRf,c,k)} -[∆TSTxMP] ≤ PUMAX,f,c,k ≤ EIRPmax And where the corresponding measured peak EIRP for carrier f of a serving cell c, over all active TCI states configured for [STxMP], PUMAX,f,c satisfies PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax [where ∆TSTxMP is a relaxation specific to STxMP operation]. Observation: RAN1’s classification as ‘per-UE’ or ‘per-panel’ may be difficult to ascertain before RAN4 have a common understanding on how to construct the requirement(s) that may interest RAN1. |

## Open issues summary

### Sub-topic 3-1: On panel definition

- Proposals

  - Option 1: RAN4 to adopt the definition of panel as used in RAN1 discussions:

‘Panel’ is defined as one or multiple as combination of below depending on different UE implementation: - Unit of antenna group to control beam independently - Within a panel, one beam can be selected and used for UL transmission. - Across different panels, multiple beams (each selected per panel) may be used for UL transmission - ‘Beam’ is assumed to mean spatial filter associated with transmission or reception - Unit of antenna group to control its transmission power - Unit of antenna group to have a common UL timing

  - Option 2: Seek RAN1 clarification as proposed by Huawei

  - Option 3: Others

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| XXX |  |
| Qualcomm | Option 1 will allow us to move forward. If it is not agreed, we can go with option 2. |
| Xiaomi | Prefer to reply RAN1’s LS based on Option 1, if not agreed, option 2 is OK. |
| Huawei | Option 2. |
| InterDigital | Option 1 seems to define well RAN4 understanding while not limiting the implementation options. Also, if there is a desire to bring panel definition to RAN1 attention, then Option 1 is a very good start. |
| Ericsson | For the Reply LS and a first liaison, the characteristics in the list under Option 1 can be used |
| Apple | Option 3: with Option 1 as baseline, add the following line and also ask RAN1 to confirm the RAN4 understanding (and to provide clarifications where necessary): ‘Panel’ shall not be explicitly used in a requirement to ensure maximum flexibility for different UE implementations. |
| vivo | Option 3: The definition from RAN1 is helpful, and may serve as baseline. However, if we try to have a baseline understanding, this understanding is better to consider Rx at the same time. A unified understanding, if possible, would be quite helpful not just here for this WI. E.g. The following text based on RAN1 assumption is provided for consideration, with the high lightened part added for unified concept between UL and DL: ‘Panel’ is defined as one or multiple as combination of below depending on different UE implementation: - Unit of antenna group to control beam independently - Within a panel, one beam can be selected and used for UL transmission or DL Reception. - Across different panels, multiple beams (each selected per panel) may be used for UL transmission or DL Reception - ‘Beam’ is assumed to mean spatial filter associated with transmission or reception - Unit of antenna group to control its transmission power for UL transmission Unit of antenna group to have a common UL timing |

### Sub-topic 3-2: Is it agreeable that RAN4 to focus on the configured Tx power requirement while addressing ‘power limitation’ for STxMP in FR2? If so, what is to be included in the reply LS?

- Proposals

  - Option 1: Yes

  - Option 2: No

  - Option 3: Others

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| XXX |  |
| Qualcomm | Option 1, as proponents. On what is to be included in an LS: Add to reply to question 4: ‘RAN4 can confirm that a configured power requirement can be established per-TCI state’ |
| Xiaomi | Option 1, and agree Qualcomm: the configured power requirement can be established per TCI-state |
| Huawei | Option 2. Any discussion regarding RAN4 requirement shall be started strictly according to the TU arrangement. We should focus on the LS reply only. |
| InterDigital | Option 1. Agree with Qualcomm. The anchor shall be the TCI state. Maybe the panel definition from sub-topic 3.2.1 will fit well here. |
| Ericsson | Option 1: the reply should focus on power control as RAN1 is asking RAN4 about power limitations for a ‘panel’ and for the UE. For this purpose, the list of characteristics for the ‘panel’ used for RAN1 discussion can be used. However, RAN4 can of course ask questions for clarification of the ‘panel’ concept. |
| Apple | We are fine to concentrate on power control per TCI state. However, we should be careful about UE implementation of TCI states, since some pairs of TCI states can map to the same panel (and in which case power control per TCI state is not possible), while other pairs can map to different panels (and in which case power control per TCI state is possible). We suggest to refine the wording as follows: “power control can be established per TCI state, provided the UE is capable of simultaneous transmission based on these TCI states” |
| vivo | We may consider adding some tentative information about possible configured power requirement per-TCI state. However, it might be premature to determine this key new RAN4 requirements in the WI in current stage, anyway it is only a discussion for LS and the official TU is not started yet. If we can have more clear understanding on their power control scheme, RAN4 can confirm defining related requirements, e.g. per TCI-state. This might be a more usual process. |

### Sub-topic 3-3: Which draft LS is used as baseline?

- Proposals

  - Option 1: R4-2215652 (Apple)

  - Option 2: R4-2216122 (vivo)

  - Option 3: R4-2216586 (Huawei)

  - Option 4: R4-2216411 (InterDigital, proposals 1,2,3)

  - Option 5: Others

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| XXX |  |
| Qualcomm | Option 2, if we can add detail on feasibility of a per-TCI state configured power requirement. Option 3 is the fallback option if we cannot agree on how to answer the LS – it needs to be trimmed however, there is a lot of information in the draft that warrants further discussion on whether to include. |
| Xiaomi | Option 2 with some modifications |
| Huawei | Option 3 as the proponent. |
| InterDigital | Option 2 is a good start and we agree with Qualcomm that we have to bring in the TCI linkage and probably the panel definition. |
| Ericsson | The scope proposed in Option 4 appears a good start, but no strong view. RAN4 should emphasize the regulatory limits and constraints that might not be widely known in RAN1: there can be a power limitation per beam/panel/TCI-state (perhaps similar to the BS declared beam directions), but the EIRP from the equipment/device has to meet regulatory requirements no matter the number of simultaneous antenna groups used. In practice, the exposure requirements limit the achievable power (in this case CPE/FWA/industrial), the requirement a power-flux density measured at a certain distance and averaged across the device for any transmitter configuration. The power limitation for the UE is presumably for specification of a priority mechanism between the beams/panel power similar to that in 7.5 of 38.213 for multiple serving cells. |
| Apple | Considering the responses so far, we are fine to use Option 2 as the baseline. The answer to Question 3 should include the following aspect related to per-TCI power control: “power control can be established per TCI state, provided the UE is capable of simultaneous transmission based on these TCI states” |
| vivo | Option 2, as the proponent. Option 3 is somewhat too conservative. It is believed that the option 2 is already not that controversial. We may also add something more if can be agreed. E.g. tentative understanding of panel for RAN1 to confirm, or some tentative requirements under discussion. |

## Companies views’ collection for 1st round

### Open issues

Comments are collected in section “Open issues summary” above.

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1, 3-2, 3-3 | There was no agreement. More companies agreed to use R4-2216122 as the baseline for the reply LS. Recommendations for 2nd round:continue the discussion on the reply LS using R4-2216122 as a baseline in the second round |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### The following comments are made on the draft LS based on R4-2216122

| Company | Comments |
| --- | --- |
| Qualcomm | We would like to point out that in our development of the per-TCI state configured power framework we showed that it was indeed feasible, and further, that a per-UE configured power framing does not work (due to need for independent MPRs, etc). In other words, when the UE is configured ‘for StxMP’, a per-UE configured power is not feasible. (R4-2216783). We think this message is beneficial to send. We appreciate that work has not begun in RAN4 on this topic, and we are ok to soften the language as suggested by vivo. Finally, to accommodate Apple’s view, we propose to preface with ‘when UE is configured for STxMP’, which is standard wording for features, and it is clear that any new requirement would not replace legacy requirements. We would like to clarify reply 4 as: It is believed that both assumptions are feasible, and both assumptions shall be applied to a same UE. The per-panel power limitation would be defined if necessary in latter stage, and the per-UE power limitation should be applicable at all the time. ‘Limitation’ here applies to regulatory compliance rather than a configured power requirement. RAN4 is discussing ‘per-TCI state’ as the applicable framework for the configured power requirement for a UE configured for STxMP operation. |
| vivo | A typo has been corrected and highlighted in the LS text, in that it is “RAN4” that had a discussion in this meeting for the definition. The intention to add this assumption explicitly is try to have some alignment between RAN1/4 and adding confidence for RAN1 to accept RAN4’s answers. To Qualcomm: Currently it seem ok to add some clarification on the “Limitation” if companies are fine with it. However, it may not that essential, since we already had some relating explanation for Question 3. For the newly proposed wording related to “per-TCI state”, it is also suggested to use a more soft wording, such as “…as an tentative framework …”, since we do not really have too much discussion yet, and the framework is an important decision to make. |
| Samsung | We have a similar view with vivo. Basically, we are fine with the current answers. However, as we have discussed, it would be better if we can add a sentence to check whether our guesses on “panel” or “power limitation” is correct or not. For the last sentence about our future work on STxMP, we don’t think it is the answer they are asking, nor the plan RAN4 agrees to. Detailed method to support STxMP can be discussed after the WI gets started in RAN4. |
| Huawei | We are NOT OK with involving the RAN1 assumption of ‘panel’. One straight forward reason is that this the WID for UL Tx, but that assumption is also mentioned DL reception… Like also mentioned by Vivo and Samsung, RAN4 discussion related to requirement shall be started when there is allocated TU for RAN4, right now we think the discussion shall focus on the LS reply itself only based on the existing RAN4 requirement. In our contribution, we listed four concepts related to power from RAN4 perspective. We understand that none of them can be a solid proof that per panel power limitation is feasible from RAN4 perspective. (needless to say there is no RAN4 agreement on panel yet) |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on resolving humidity inconsistency | ZTE |  |
| WF on Modified MPR-Behaviour clarification for different power classes | Huawei |  |
| Reply LS on UE power limitation for STxMP in FR2 | vivo | To: RAN1 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2215706 |  | Samsung | Noted |  |
| R4-2216618 |  | ZTE | Noted |  |
| R4-2215317 |  | Nokia, Qualcomm Inc, Skyworks Inc, Ericsson | Noted |  |
| R4-2216355 |  | Xiaomi | Noted |  |
| R4-2216442 |  | OPPO | Noted |  |
| R4-2216677 |  | Huawei, HiSilicon | Noted |  |
| R4-2216678 |  | Huawei, HiSilicon | Noted |  |
| R4-2215316 | CR to 38.101-2: Correction to modified MPR information | Nokia, Qualcomm Inc, Skyworks Inc, Ericsson | Return to |  |
| R4-2215970 | CR to 38.101-2: Correction to modified MPR information R16 | Nokia, Qualcomm Inc, Skyworks Inc, Ericsson | Return to |  |
| R4-2215652 |  | Apple | Noted |  |
| R4-2216122 |  | vivo | Noted |  |
| R4-2216354 |  | Xiaomi | Noted |  |
| R4-2216411 |  | InterDigital | Noted |  |
| R4-2216586 |  | Huawei, HiSilicon | Noted |  |
| R4-2216783 |  | Qualcomm | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2217757 | WF on Modified MPR-Behaviour clarification for different power classes | Huawei | Noted |  |
| R4-2217756 | WF on resolving humidity inconsistency | ZTE | Agreeable |  |
| R4-2217758 | Reply LS on UE power limitation for STxMP in FR2 | vivo | TBD | Still under discussion |
| R4-2215316 | CR to 38.101-2: Correction to modified MPR information | Nokia, Qualcomm Inc, Skyworks Inc, Ericsson | Not purpued |  |
| R4-2215970 | CR to 38.101-2: Correction to modified MPR information R16 | Nokia, Qualcomm Inc, Skyworks Inc, Ericsson | Not purpued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents