3GPP TSG-RAN WG4 Meeting #104bis-e											R4-2217798

Electronic Meeting, 10–19 October 2022

Agenda item:			9.3, 9.4

Source:	Moderator (OPPO)

Title:	Email discussion summary for [104-bis-e][146] RAN_task_UERF_part2

Document for:	Information

Introduction

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Peng (Henry), Zhang | zhangpeng169@huawei.com |
| Ericsson | Christian Bergljung | Christian.Bergljung@ericsson.com |
| Qualcomm Incorporated | Gene Fong | gfong@qti.qualcomm.com |
| Nokia(HO) | Hisashi Onozawa | hisashi.onozawa@nokia.com |
| Apple | James Wang | fucheng_wang@apple.com |
| Meta | Suhwan Lim | suhlim@meta.com |
| Samsung | Yuanyuan Zhang | Tina55.zhang@samsung.com |
| Huawei | Hu Dan (intra-band EN-DC) | hudan11@huawei.com |
| Google | Clement Huang | clementhuang@google.com |
| OPPO | Jinqiang | xingjinqiang@oppo.com |
| MediaTek | Daniel Hsieh | daniel.hsieh@mediatek.com |
| Nokia(HU) | Hiromasa Umeda | hiromasa.umeda@nokia.com |
| Intel | Richard Burbidge | richard.c.burbidge@intel.com |
| ZTE | Zhifeng Ma | ma.zhifeng@zte.com.cn |
| AT&T | Ron Borsato | ronald.borsato@att.com |
| T-Mobile USA | Bill Shvodian | bill.shvodian@t-mobile.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: Inconsistency issue for intra-band EN-DC band combinations

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215668 | Apple | Observation 1: The validity of Case 3 and Case 4 configurations can be interpreted differently, depending on which aspect is being used for judgement. Observation 2: In a band combination, the UL configuration is either the same as DL configuration or belongs to the DL fallback configurations. Observation 3: 3GPP has never allowed a contiguous configuration to fall back to a non-contiguous configuration. Observation 4: Despite the existing signaling design for CA would allow UE to signal DL configuration and UL configuration separately, there has never been any intra-band CA combination specified with contiguous DL configuration paired with a non-contiguous UL configuration. Observation 5: Since the validity of the concerned configurations can be interpreted differently, it may not be so meaningful to continue the long debate in RAN4 which may just render the issue unresolved. Observation 6: Non-contiguous UL configuration in general would be much less efficient than contiguous UL configuration due to the potential higher MPR or A-MPR. Observation 7: If reconfiguration by the network is always needed when certain part of the spectrum is occupied by the other network, it seems to make more sense to always configure the 2 UL carriers as contiguous when the 3 DL carriers are contiguous. Observation 8: CA_n48A(A-B) has been specified only with CA_n48B as UL CA, but not CA_n48A-n48A. It is unclear why DC_48A_n48A is needed for DC_48A_(n)48AA. Observation 9: For B41/n41 EN-DC combination without the concern of spectrum sharing, it is unclear what motivates the network to configure non-contiguous UL configuration when all the DL carriers are contiguous. Observation 10: The Case 3 and Case 4 configurations cannot be supported on the UE side based on the existing signaling design. The scope to enable these configurations is much wider than a TEI can handle as it may involve the whole concept change in intra-band configuration fallback. Proposal 1: The Case 3 and Case 4 proponent companies to reconsider whether there would be any merit to operate those configurations if reconfiguration by the network is always needed when certain part of the spectrum is occupied by the other network. Proposal 2: Remove Case 3 and Case 4 configurations from current RAN4 specifications to close the issue. Proposal 3: If the proponent companies still see the demand for Case 3 and Case 4 configurations, it is proposed to start a new study item or work item to seek for the solution. |
| R4-2215933 | Nokia, Nokia Shanghai Bell | Observation 1: Case 3 and Case 4 are needed for the flexibility of CBRS GAA operation. Proposal 1: Consider removing Case 3 for band 41 and n41 if no use case is identified. Observation 2: Fallback is defined to support reduced UE capability signalling for lower order CA/DC and is not restricting Case 3 or Case 4 configuration if there is a demand to support them. Observation 3: The existing signalling can be reused if EN-DC config including a (n) sub-block in both DL and UL is categorized to “contiguous”, EN-DC config without any (n) sub-block is categorized to “non-contiguous”, “EN-DC config including a (n) sub-block in DL and without it in UL is categorized to “both” Proposal 2: It is further discussed if we wait for RAN2 to provide a flexible signalling or indicate RAN2 to implement a CR according to Observation 3. |
| R4-2215956 | Ericsson | Observation 1: existing intra-band EN-DC configurations specified in Rel-17 can be indicated by existing signaling (with suitable restrictions for Case 4). and propose Proposal 1: for case 3, remove non-contigous UL configurations that are paired with contigousn DL configurations - The UE must support non-contiguous EN-DC also in the DL, the combinations discussed already exist except DC_41A-n41B - Case 3 ‘amended’ can then be indicated by a single BC entry e.g. {41C, n41A} (DL) and {41A, n41A} (UL) and intraBandENDC-Support = ‘both’ Proposal 2: for Case 4, Rel-17 combinations of contiguous and non-contigous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous CA configurations in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contigous and non-contigous EN-DC in the UL. Proposal 3: changes of signaling can be made for combinations specified in Rel-18, if needed, but configurations should still comply with the existing fallback rules. |
| R4-2216356 | Xiaomi | Proposal 1: According to previous common understanding, intra-band contiguous ENDC should have contiguous ENDC configurations both for UL and DL, case3 is invalid. Proposal 2: If RAN4 tries to make case 3 is valid, RAN4 need first discuss whether the rule in section 4.2 of Spec 38.101-3 applies to case3, or whether case3 means a new UE capability. Proposal 3: According to previous common understanding, case 4 is valid. |
| R4-2216412 | Google Inc., Comcast, CableLabs | Proposal 1: The following intra-band EN-DC DL/UL configurations are valid configurations. - DL DC_(n)48CA with UL DC_48A_n48A - DL DC_(n)48DA with UL DC_48A_n48A - DL DC_48A_(n)48AA with UL DC_48A_n48A - DL DC_48A_(n)48AA with UL DC_(n)48AA Observation 1: The intra-band EN-DC combination to support contiguous DL EN-DC configuration with non-contiguous UL EN-DC configurations by reporting an additional band combination does not violate the fallback band combination rule. Observation 2: An intra-band contiguous band combination is considered to be a fallback band combination of an intra-band non-contiguous band combination. Hence, there is no issue for the band combination DC_48A_(n)48AA to have the configuration DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A. Proposal 2: For Case 3, we propose the following compromise solutions. - In Rel-16 and Rel-17, report an additional band combination to support the Case 3 configurations. - In Rel-18, introduce a UE capability signaling to support the Case 3 configurations. |
| R4-2216421 | Google Inc., Comcast, CableLabs | Draft CR for 38.101-3 Rel-16 intra-band contiguous EN-DC band combination |
| R4-2216427 | Google Inc., Comcast, CableLabs | Draft CR for 38.101-3 Rel-17 intra-band contiguous EN-DC band combination |
| R4-2216443 | OPPO | Observation 1: Case 4 has been solved in RAN4 by moving this kind of band combination to a separate table, and if UE support this band combination should indicate “both” for IntrabandENDC-Support capability. Observation 2: The fallback band combination restriction in RAN2 is not between UL and DL, but for either UL fallbacks or DL fallbacks. This should not be used to preclude UE having contiguous DL but non-contiguous UL. Observation 3: UE with separate PAs can support UL non-contiguous EN-DC with each PA per CC. And usually DL is not the bottle neck and can be supported easily by Rx paths. Observation 4: UL and DL actually have different restrictions and can be considered independently on the supporting of contiguous and non-contiguous. Proposal 1: UL and DL considered independently on the supporting of contiguous and non-contiguous capability, due to different restrictions on UL and DL. Proposal 2: Check with RAN4 group whether there is clear demand from operators on the non-contiguous UL but contiguous DL configurations, and then consider the following options: - Option 1: if no clear demands, then RAN4 consider to remove these configurations. - Option 2: if there is clear demand, then RAN4 inform RAN2 to consider support these different UL and DL configurations from signaling and detailed signaling design is up to RAN2. |
| R4-2216617 | ZTE | Observation 1: Whether the E-UTRA band or NR band with the same band number are regarded as the same band, it has different understanding for EN-DC intra-band configuration and EN-DC inter-band configuration in current RAN4 spec. Proposal 1: A note to inform that “intra-band” EN-DC configurations are considered as the same frequency spectrum in E-UTRA and NR band which should be added to TS 38.101-3. Observation 2: The aspects of contiguous or non-contiguous EN-DC configurations are currently only categorized by DL in RAN4. Observation 3: The main confusion of intra-band contiguous or non-contiguous EN-DC configurations comes from the scenarios (b) and (c) which need to be further confirmed from the operators. Proposal 2: Further confirmation of the requirements from the operator with the scenarios of DL contiguous and UL Non-contiguous (or UL both) is recommended. If no such requirements, then no ambiguity any more, otherwise, further distinguish the continuousness from UL and DL may be required. |
| R4-2216656 | Huawei, HiSilicon | Observation 1: Case 3 and Case 4 are valid band combinations from the perspective of operators. Observation 2: Case 3 and Case 4 are valid band combinations from the perspective of specifications. Proposal 1: The configurations in Case 3 and Case 4 are valid from RAN4 and RAN2 point of view. Proposal 2: A solution is necessary in RAN2 to address the ambiguity issue for configurations on some intra-band EN-DC band combinations with more than 2 carriers from Rel-15 |
| R4-2216657 | Huawei, HiSilicon | [DRAFT] LS on intra-band EN-DC combination |

Open issues comment collection

Sub-topic 1-1: Case 3 and case 4 demands

- Issue 1-1-1: Whether Operator has demands for Case 3 in the fields

Moderator note: There are proposals to clarify whether case 3 and 4 are real demands in the NW, and if no then remove it, and if yes then consider how to accommodate it in the spec.

- Option 1: Yes, scenario and benefits are…

  - Option 1a: Case 3 is valid scenario (HW)

  - Option 1b: Case 3 for band 48 and n48 are valid scenario (Google Inc., Comcast, CableLabs)

  - Option 1c: Case 3 for band 41 and n41 are invalid scenario according to R4-2114890 (WF on intraBandENDC-support) (Nokia)

- Option 2: No

| Company | Comments |
| --- | --- |
| Ericsson | Option 1c. The combination is not valid if case 3 is indicated as a BC without support if non-contiguous in the DL as shown in the figure below. The UE must also indicate support for non-contiguous in the DL; a restriction to non-contiguous in the UL for any carrier separation in the band is not valid, there must be a corresponding DL for the carrier unless a SUL. If there are demands for Case 3, the most straightforward would be a configuration with a BCS that works for all configurations, or indicate two separate BCs, one contiguous and one non-contiguous, if DC_41C-n41A is not supported. |
| Xiaomi | Option 1c |
| Nokia(HO) | Option 1b and 1c. We think supporting Case 3 would benefit GAA scenarios for band n48 due to dynamic channel allocation. We’d like to hear from band n41 operators if Case 3 is needed. |
| Apple | The validity of Case 3 configurations can be interpreted differently, depending on which aspect is being used for judgement. We would like to know how dynamic the spectrum availability may vary in Band n48 and how much efficiency is gained with only DL reconfiguration as compared to both DL and UL reconfiguration and whether such efficiency gain is sufficient to justify the UL performance loss due to higher MPR/A-MPR for non-contiguous UL configuration as compared to contiguous UL configuration. |
| Meta | It is dependent on how to define the MPR/A-MPR requirements for UL configuration in DC_41A_n41A. In TS38.101-3, RAN4 already define MPR/A-MPR to support the non-contiguous DC band combinations. So we think option 1a is feasible when UE indicate to support non-contiguous UL/DL DC combos. But we also think that the smallest MPR/A-MPR is more beneficial to operate these specific operations. |
| Samsung | Made aware by RAN2 colleague that below fallback rule captured in 38.306 exists date back to LTE and it appears imposed by RAN4(due to different architectures and requirements between contiguous and non-contiguous), Ericsson also mentioned it in their paper, hence essentially it is still RAN4 fallback rule(Name it RAN4 fallback rule#1) “An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination” We think below inter-band EN-DC RAN4 fallback rule is also applicable to intra-band EN-DC (Name it fallback rule#2) “A terminal which supports an inter-band EN-DC or NE-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC or NE-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.” If we deem Case 3 as contiguous band combination and we say they are valid. Situation becomes RAN4 fallback rule#1 is contradict with RAN4 fallback rule#2. However as clarified by operators the 48 related combos in case 3 indeed has special deployment need, to satisfy their request we think a joint solution between RAN2 and RAN4 is needed to address the RAN4 fallback rule confliction, along with the new signalling. For 41 related combos in Case 3, if no operators clarify the motivation, we suggest to remove them. For 48 related combos in Case 3, we propose to add note into the configuration table, as below, further modification is welcome. In addition, the applicability of the note should be determined, and clearly recorded into FS_BC_simlication TR(ZTE) to forbid other combos flexibility adding this note. With this note, we agree Case 3 for 48 is valid. DC_(n)48CAx; DC_48A_n48A. DC_(n)48DAx; DC_48A_n48A. Note X: The fallback rule in 38.306 “ An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination” is not applicable.. |
| Huawei | Based on the input from RAN#97e, the scenario of band 48 and n48 is valid, which means Case3 is valid. Whether band 41 and n41 is supported depends on the proponent’s confirmation. |
| Google | Option 1b and 1a. Agree with Case 3 would benefit GAA scenarios for band n48 due to dynamic channel allocation. The dynamic channel allocation may be frequently happened around US Naval Base such as San Diego and Norfolk. |
| OPPO | Option 1c was already confirmed for Rel-15, i.e. Case 3 for b41+n41 is no demands. If no clear demands now, they can be removed also from Rel-16 onwards. Option 1b no strong view, but seems is needed from operator inputs. |
| ZTE | Whether case 3 is valid or not should be decided by operators’ requirements. We would like to hear the feedback from operators who are deploying or ready to deploy the network on band 41/n41 or 48/n48. |
| Moderator summary: No operator demand is shown in the 1st round for b41+n41, thus Case 3 for b41+n41 can be removed. For b48+n48, several companies (no operator?) expressed the view that this is desired for US dynamic channel allocation scenario. And one company questioned the efficiency gain with only DL reconfiguration as compared to both DL and UL reconfiguration. One company would like to add one note to this band combination to clarify that RAN2 fallback rule is not applied to this band combination. Therefore, moderator suggest to make following conclusion: 1. Case 3 for b41+n41 to be removed 2. Case 3 for b48+n48 is kept and considered a valid band combination, and further discuss whether the note below can be added for Case 3 b48+n48 in the solution discussion Note X: The fallback rule in 38.306 “An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination” is not applicable. |  |

- Issue 1-1-2: Whether Operator has demands for Case 4 in the fields

- Option 1: Yes, scenario and benefits are…

  - Option 1a: Case 4 for band 48 and n48 are valid scenario (Google Inc., Comcast, CableLabs, Xiaomi)

  - Option 1b: Case 4 is valid scenario (HW)

  - Option 1c: Case 4 has been solved in RAN4 by moving this kind of band combination to a separate table, and if UE support this band combination should indicate “both” for IntrabandENDC-Support capability. (OPPO)

- Option 2: No

| Company | Comments |
| --- | --- |
| Ericsson | Option 1c (not answering if there are further operator requests). The case 4 (as specified) can be supported using existing signaling, this with the understanding that new band combinations of this type are specified in a similar way: support of both contiguous and non-contiguous and up to two sub-blocks if one consists of both LTE and NR carriers. |
| Xiaomi | Option 1c |
| Nokia(HO) | We think supporting Case 4 would benefit GAA scenarios for band n48 due to dynamic channel allocation. |
| Apple | For Case 4, our understanding is that only the configuration DL DC_48A_(n)48AA with UL DC_(n)48AA can be supported by the existing signaling. For DL DC_48A_(n)48AA with UL DC_48A_n48A, if intraBandENDC-Support = ‘both’, that means UE should also be capable of supporting DL DC_48A-48A_n48A. However, this may not be true as UE’s implementation may only be able to support one sub-block gap. Therefore, we do not agree that the Case 4 with non-contiguous UL configuration can be supported using existing signaling. |
| Meta | Agree with option 1c. |
| Huawei | The contiguity of intra-band combination is up to UE implementation. It is possible that the UE supports ‘both’ in DL, but only supports ‘contiguous’ in UL. Currently, RAN2 signaling cannot report such scenario. |
| Google | Support Option 1a, 1b and 1c. For Option 1c, there could be a compromised solution to specify “both” for DC_48A_(n)48AA with UL DC_48A_n48A and UL DC_(n)48AA and to specify “non-contiguous” for DC_48A-48A_n48A with UL DC_48A_n48A. |
| OPPO | Option 1c. But the demands from operators may still be valuable here. The meaning of intrabandENDC-support IE in RAN2 pointing to RAN4 spec where the band combinations are defined in table with contiguous/non-congituous/both definitions. This means the interpretation of this IE is depends on how RAN4 spec classify these band combinations. Putting Case 4 band combinations to the table with “both” capability can solve the problem. “Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3” |
| ZTE | Same comments as Case 3. We would like to hear the feedback from operators. To our understanding there is no inconsistent issue in Case 4, our preference is 1c. |
| Huawei | Option 1a and Option 1b are same since Case4 only includes band 48 and n48 for the moment. Case 4 is a valid scenario and the problem is unsolved by Option 1c UE reporting ‘both’ with intrabandENDC-Support. The solution is not only related to signaling, but also the UE implementation architecture. For example, to support DL non-contiguous CA is to implement separate chains. But the requirements of dual PA in UL to support NC was just specified in Rel-17. Most UEs may not support NC CA in UL considering the cost and implementation complexity. According to the endorsed CR R4-2214967, the intra-band EN-DC configuration of DC_48A-(n)48AA is moved to the table for mixed intra-band contiguous and non-contiguous EN-DC. For a UE only support ‘contiguous’ in UL, when gNB configures UL DC_48A_n48A after receiving the UE capability of ‘both’, UE is not able to perform the configuration. It would be a typical UE implementation for Case 4 with the ambiguity issue. |
| Moderator summary: For Case 4 where only b48+n48 specified, no company deny this scenario though no operator express demands. And question is whether current signaling can support such reporting and if not how to make the signaling possible. The solutions will be discussed in another issue. Therefore, moderator recommend to confirm Case 4 is a valid scenario. |  |

Sub-topic 1-2: Case 3 and case 4 fallback rules

- Issue 1-2-1: Whether contiguous or non-contiguous EN-DC configurations are only categorized by DL in RAN4?

- Option 1: Yes (ZTE)

- Option 2: No

| Company | Comments |
| --- | --- |
| Ericsson | Option 2. An intra-band EN-DC configuration part of an inter-band BC can be non-contiguous in the DL within the band but not in the UL (when there is only one UL in this band). |
| Xiaomi | Option2, pure intra-band contiguous EN-DC configuration for aggregated CCs larger than 2CCs should be categorized by both of DL and UL. |
| Nokia(HO) | We need an overall consensus/agreement how to sort out this issue. At least, when non-contiguous UL for DL contiguous EN-DC was first introduced into the spec, Option 1 was the intention of the proponent of such EN-DC. That is why non-contiguous UL was included in the table of the contiguous EN-DC. However, the issue was overlooked at that time and thus we have had a long discussion on this issue. |
| Apple | Option 2 Clarification is needed for this question. Is the question based on RAN4’s agreement or based on what is being defined in the current RAN4 specifications? If it is based on RAN4’s agreement, our answer is Option 2. |
| Meta | Option 2 |
| Samsung | Option 2, share similar view with Apple, based on current RAN’4 agreement, it is No. In addition, maybe we do not need to argue on this. It is also ok to classify case 3 as contiguous ENDC, judge contiguous/non-contiguous is mainly for the purpose to judge whether case 3 violates the fallback rule, by adding the note suggested in Issue 1-1-1, there would be no problem from fallback rule perspective. |
| Huawei | In the current spec 38.101-3, the contiguity of EN-DC configurations are only categorized by DL in RAN4. When it comes to the inconsistency of DL and UL configurations in contiguity, the UE capability follows that of DL configurations. We share the similar view with Nokia for the background of introduction on the band combination configuration in Case 3 and Case 4. |
| OPPO | Option 2. |
| ZTE | Option 1. Currently in RAN4 spec it is categorized by DL, if not, further clarification may be needed. |
| Moderator summary: Majority company think the contiguous or non-contiguous EN-DC configuration is NOT only categorized by DL in RAN4. This understanding can be confirmed. |  |

- Issue 1-2-2: How to interpretate the fallback rule in RAN2 38.306

- Option 1: In a band combination, the UL configuration is either the same as DL configuration or belongs to the DL fallback configurations. (Apple)

- Option 2: Fallback is defined to support reduced UE capability signalling for lower order CA/DC and is not restricting Case 3 or Case 4 configuration if there is a demand to support them. (Nokia)

- Option 3: UL and DL considered independently on the supporting of contiguous and non-contiguous capability, due to different restrictions on UL and DL. (OPPO)

| Company | Comments |
| --- | --- |
| Ericsson | Option 1, an Scell must at least have a DL part, and Option 3, band combination fallback applies to release of Scells or an UL part of an Scell. |
| Xiaomi | Option 1, the UL configuration should be either the same as DL configuration or belongs to the DL fallback configurations, even through UL and DL considered independently on the supporting of contiguous and non-contiguous capability. |
| Nokia(HO) | We are open to support Case 3 and 4, rather than forbidding them due to the rules applied to the legacy combinations. |
| Apple | Option 1 The fallback rule was defined for the purpose to minimize the signaling to indicate UE’s capability to support various CA/DC configurations. For UE with the capability to support one highest order band combination, all the lower order configurations by releasing one or more carriers from the highest order configuration which UE is also capable of supporting without additional signaling are viewed as the fallback configurations for this highest order configuration. Based on this rule, non-contiguous configuration would not be allowed as a fallback configuration from a higher order contiguous configuration as they represent two different UE capability where additional signaling is needed. Without considering the existing fallback rule, a mixture of contiguous and non-contiguous CA configurations in both UL and DL may already be supported by the signaling as long as UE is capable of supporting it. However, to enable the support of Case 3 configurations and Case 4 with non-contiguous UL configuration, in our view the scope is much wider than a TEI can handle as it may involve in the whole concept change in intra-band configuration fallback. Therefore, if there is still demand for Case 3 and Case 4 configurations, it is proposed to start a new study item or work item to seek for the solution. |
| Meta | Option 1 |
| Samsung | Option 1 Note that it appears this rule is imposed by RAN4 (due to architecture and RF requirement are different between contiguous and non-contiguous) and exists since LTE, essentially it is still RAN4 fallback rule. “An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination” From my observation, RAN2 does not care about it since it is RF architecture related. |
| Huawei | We prefer option 3 The fallback rule in RAN2 does not aligned with RAN4 spec. The definition of fallback rule is not for the restriction of scenarios. The fallback rule defined in RAN2 spec should be based on the scenarios in RAN4. If there are un-alignments, the rule needs modification. And since the configurations of DL and UL are configured separately, the contiguity of DL and UL should also be independent. |
| Google | Same view as Nokia. We are also open to support Case 3 and 4 instead of forbidding them due to the rules applied to the legacy combinations. |
| OPPO | Option 1 is ok Option 2 is also ok since this means if UE with further indication it can also support Case 3/4 Option 3 is the key in our view, whether we consider the UL and DL separately in the fallback handling. And this is not contradicting with Option1 since if UE support both contiguous and non-contiguous in DL then this UE can fallback to non-contiguous in DL CA which can be same as non-contiguous UL. Meanwhile, if this UE only support contiguous fallback in DL CA then Case 3 probably is invalid. From this point of view, this Case 3 UE should support both contiguous and non-contiguous capability in DL. |
| ZTE | Option 1. |
| Moderator summary: Majority company agree with Option 1, and two companies support Option 3. Moderator recommends to go with majority view, i.e. In a band combination, the UL configuration is either the same as DL configuration or belongs to the DL fallback configurations. Moderator also notice that one potential issue with this agreed UL and DL configuration relation is that it may lead to Case 3 as invalid scenario, since the DL contiguous cannot fallback to non-contiguous configuration. How to solve this issue is up to Issue 1-3-1 solution discussion. |  |

- Issue 1-2-3: Views on the fallback rule in RAN4 38.101-3 section 4.2 below:

“A terminal which supports an inter-band EN-DC or NE-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC or NE-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.”

- Proposal 1:

  - This rule leads to: UE must support both of DL contiguous configuration and DL non-contiguous configuration with a certain UL non-contiguous configuration. (Xiaomi)

- Proposal 2:

  - If Case 3 is made valid scenario, whether the rule in section 4.2 of Spec 38.101-3 applies to case3, or whether case3 means a new UE capability needs to be discussed. (Xiaomi)

| Company | Comments |
| --- | --- |
| Ericsson | Proposal 2: a similar rule should apply to intra-band EN-DC configurations. |
| Xiaomi | If the fallback rule in RAN4 38.101-3 applies to intra-band EN-DC configuration, it will leads proposal 1, but it will be against RAN2’s fallback rule. So we prefer to discuss the request of CBRS GAA operation separately with general intra-band EN-DC configuration, i.e., band 41 and n41. |
| Nokia(HO) | If Case 3 and 4 are forbidden because of this rule, amendment can be considered. |
| Apple | Our understanding is that this rule for inter-band EN-DC currently also applies for intra-band EN-DC. If we would intend to enable Case 3 configurations, then we may have to revisit the fallback rule as we commented above. |
| Samsung | Agree with Proposal 2, should also applicable to intra-band EN-DC. |
| Huawei | The rule is irrelevant to the topic. The referred rule is for inter-band EN-DC configurations. If the rule is applied to the case of intra-band configuration and not able to solve the problems of Case3 and Case 4, it should be revised. |
| Google | We share the same view as Nokia. |
| OPPO | We are not sure whether this fallback rule defined in 38.101-3 can leads to preclude or include the Case 3/4 discussed in this topic since at least this is for inter-band EN-DC. |
| ZTE | We agree with Proposal 2, a similar rule should be applied to intra-band EN-DC, but detail should be further discussed. |
| Moderator summary: Different views on whether this rule defined in 38.101-3 are connected with the cases discussed, but one common view is that current rule is only apply to inter-band EN-DC or NE-DC. Thus moderator recommend no more discussion in 2nd round. |  |

Sub-topic 1-3: Case 3 and case 4 Solutions

- Issue 1-3-1: Solutions for Case 3

- Option 1: Remove Case 3 configurations

  - Option 1a: Remove Case 3 configurations from current RAN4 specifications. (Apple, Ericsson, Xiaomi)

  - Option 1c: Remove Case 3 if no use case is identified. (OPPO)

  - Option 1b: Remove Case 3 for band 41 and n41 if no use case is identified. (Nokia)

- Option 2: Reuse existing signalling to indicate Case 3

  - Option 2a: EN-DC config including a (n) sub-block in both DL and UL is categorized to “contiguous”, EN-DC config without any (n) sub-block is categorized to “non-contiguous”, “EN-DC config including a (n) sub-block in DL and without it in UL is categorized to “both” (Nokia)

  - Option 2b: In Rel-16 and Rel-17, report an additional band combination DC_48A_n48A to support the Case 3 configurations DL DC_(n)48CA with UL DC_48A_n48A and DL DC_(n)48DA with UL DC_48A_n48A (Google Inc., Comcast, CableLabs)

- Option 3: New signalling

  - Option 3a: Be considered in Rel-18 to support Case 3 (Ericsson, Google Inc., Comcast, CableLabs)

  - Option 3b: A solution is necessary in RAN2 to address the ambiguity issue for configurations on some intra-band EN-DC band combinations with more than 2 carriers from Rel-15. (HW)

  - Option 3c: If there is clear demand for Case 3, then RAN4 inform RAN2 to consider support these different UL and DL configurations from signalling and detailed signalling design is up to RAN2. (OPPO)

- Option 4: Wait for RAN2 to provide a flexible signalling (Nokia)

- Option 5: If the proponent companies still see the demand, start a new study item or work item to seek for the solution. (Apple)\

- Option 6: A joint solution between RAN2 and RAN4: Adding note to the configuration table for Case 3, along with the new signalling defined in RAN2 (Samsung)

| DC_(n)48CAx | DC_48A_n48A |
| --- | --- |
| DC_(n)48DAx | DC_48A_n48A |
| Note X: The fallback rule in 38.306 “ An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination” is not applicable. |  |

| Company | Comments |
| --- | --- |
| Ericsson | Option 1a: remove the non-contiguous UL for the configurations with only contiguous in the DL. Option 2b: then also remove the non-contiguous UL for the configurations with only contiguous in the DL hoping that the BCS would consistent between the two BC entries indicated The most straightforward would be a configuration that can be indicated as a single BC (with intraBandENDC-Support = ‘both’) with a BCS that works for all configurations. Option 3a: no new signaling earlier than Rel-18. Option 4: anything expected from RAN2? |
| Xiaomi | Support Option 1a and Option 1c, if most companies want to keep the flexibility of CBRS GAA operation, we prefer to first agree Option 1c. |
| Nokia(HO) | Support Option 1b, 2a, 3a, 3c 4, and/or 5. Not support Option 1a, and 1c. 3b would be NBC so maybe it is not possible. 2b is not clear to us. Does it mean if both DL DC_(n)48CA and DL DC_48A_n48A are supported by UE through UE capability signalling, UL DC_48A_n48A is supported for DL DC_(n)48CA? Why is (Rel-18 signalling) simultaneously proposed, if the issue can be solved in Rel-16/17 by reporting an additional band combination? Or is it ok as compromise only to support from Rel-18? |
| Apple | Option 1a (including removing DL DC_48A_(n)48AA with UL DC_40A_n48A in Case 4) Option 2b is possible using the implied capability signaling which is without the consideration of the fallback rule. Option 5: A designated SI or WI may be better to revisit the fallback rule and seek for a more well-thought signaling solution. |
| Meta | Option 1b, 1c, 2a, 3c and option 5 are acceptable. |
| Samsung | At least Option 1b is acceptable. Regarding Option 3: Support Option 3c and 3a. About release independence, there might be some informal signaling already exists to implement these configuration, as operator request to be Release independent from 18. We support Option 6 as proponent. (Sorry, Jinqiang, I add it without your permission) |
| Huawei | We prefer Option 3b and we are open to Option 3a. |
| Google | Not Support Option 1a and 1c. Support Option 2b, 3a and 3c. To Nokia, According to previous meeting controversial discussion, Option 2b and Option 3a are our compromised proposal. |
| OPPO | For remove Case 3 option, we would like to respect operator inputs, if no inputs then we can go with Option 1c (remove whole Case 3). If there is demands, then we can go with Option 1b (remove b41+n41) For reuse existing signalling to support Case 3, we are ok with Option 2b (report an additional band combination DC_48A_n48A). If new signalling is needed, then Option 3a (define in Rel-18), and 3c (different UL/DL configuration and up to RAN2 design signalling) is ok. And from RAN2 conclusion in this meeting seems new signalling design is possible and even for the frozen release. Then among the above different approaches, the most efficient and future proof approach is introducing new signalling to support different UL/DL continuity capability as long as there is Operator demands on this. RAN2 concludes that the discussed cases are not currently supported by signalling and new signalling is needed. Case validity is up to RAN4, and if RAN4 concludes they are valid, RAN2 can then attempt to find a signalling solution. RAN4 can also develop a preference as to what release should be applicable. |
| ZTE | We support Option 1c: Remove Case 3 if no use case is identified. Option 3c: If there is clear demand for Case 3, then RAN4 inform RAN2 to consider support these different UL and DL configurations from signalling and detailed signalling design is up to RAN2. |
| Moderator summary: With the confirmation of demands for Case 3 b48+n48 and also no demands for b41+n41, the Option 1b can be confirmed. For design new signalling: Option 3a (Ericsson, Nokia, Samsung, [HW], Google, OPPO) Option 3b (HW) Option 3c (Meta, Samsung, Google, OPPO, ZTE) To combine Option 3a and 3c, moderator recommends to consider design new signalling in Rel-18 for Case3 if necessary and detailed signalling design is up to RAN2. And rely existing signalling to indicate Case 3 before Rel-18. For reusing current signalling: Option 2a (Nokia, Meta) Option 2b (Ericsson, Apple, Google, OPPO) - Clarification question from Nokia: Does it mean if both DL DC_(n)48CA and DL DC_48A_n48A are supported by UE through UE capability signalling, UL DC_48A_n48A is supported for DL DC_(n)48CA? Moderator recommends to go with Option 2b with high priority and further consider Option 2a if feasibility issue found for Option 2b. Option 4 and 5 didn’t got many supports, can be droped for the time being. Option 6 was proposed in the 1st round comment. It can be further considered in 2nd round. |  |

- Issue 1-3-2: Solutions for Case 4

- Option 1: Remove Case 4 configurations (Apple)

- Option 2: Reuse existing signalling to indicate Case 4

  - Option 2a: EN-DC config including a (n) sub-block in both DL and UL is categorized to “contiguous”, EN-DC config without any (n) sub-block is categorized to “non-contiguous”, “EN-DC config including a (n) sub-block in DL and without it in UL is categorized to “both” (Nokia)

  - Option 2b: Rel-17 combinations of contiguous and non-contiguous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous CA configuration in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contiguous and non-contiguous EN-DC in the UL. (Ericsson)

- Option 3: New signalling

  - A solution is necessary in RAN2 to address the ambiguity issue for configurations on some intra-band EN-DC band combinations with more than 2 carriers from Rel-15. (HW)

- Option 4: Wait for RAN2 to provide a flexible signalling (Nokia)

- Option 5: If the proponent companies still see the demand, start a new study item or work item to seek for the solution. (Apple)

- Option 6: Case 4 has been solved in RAN4 by moving this kind of band combination to a separate table, and if UE support this band combination should indicate “both” for IntrabandENDC-Support capability. (OPPO)

| Company | Comments |
| --- | --- |
| Ericsson | Option 2b: support as proponent, Case 4 can be kept as specified if similar (new) combinations are restricted as proposed. Option 3: not agreeable. Option 4: anything expected from RAN2? Option 6: we propose to combine this with Option 2b (add to the new table) |
| Xiaomi | Support Option 2b and Option 6, at least for current existing band combinations the new signaling is not needed. |
| Nokia(HO) | Support 2a, 2b, 3, 4, or 5. Not support 1 and 6, because Case 4 would be useful for GAA use cases. Option 6 cannot solve Case 3 (unless Case 3 are deleted). |
| Apple | Our proposal in Option 1 is to only remove UL DC_48A-n48A, but still keeping DL DC_48A_(n)48AA with UL DC_(n)48AA as this configuration can be supported by the existing signaling. It is also consistent with DL CA_n48(A-B) where we only have UL CA_48B specified, but not CA_48A-48A. We do not think the existing signaling can be used to indicate to only support DC_48A_(n)48AA with UL DC_48A_n48A. As we commented above, if intraBandENDC-Support = ‘both’, that means UE should also be capable of supporting DL DC_48A-48A_n48A. However, this may not be true as UE’s implementation may only be able to support one sub-block gap. |
| Meta | Support Option 2a, 2b and Option 5 |
| Samsung | Support Option 2b and Option 6. |
| Huawei | Option 1: disagree Option 2: existing signalling cannot fully solve the problem. Option 3: support Option 4: RAN2 has drawn a conclusion to wait for RAN4’s input. Option 5: RAN has already tasked RAN4 and RAN2 for possible solution. Option 6: It is clearer to move this kind of band combinations to a separate table. However, the problem still unsolved. |
| Google | Option 6 is our preference as a compromised solution to specify “both” for DC_48A_(n)48AA with UL DC_48A_n48A and UL DC_(n)48AA and to specify “non-contiguous” for DC_48A-48A_n48A with UL DC_48A_n48A. We are also fine with Option 2a and 2b. |
| OPPO | Option 2b and Option 3. Based on RAN2 conclusion that new signaling can be designed in RAN2 even for the frozen release, then the most simple approach is to design new signaling for UL and DL separately, and allow UE to indicate its capability. |
| ZTE | Support Option 1 and Option 6. |
| Moderator summary: With the confirmation of valid for Case 4 b48+n48, the Option 1 can be omitted. The new signalling approach got support also object, Option 3 can be considered not agreeable as a solution to solve the exiting release problem. Option 4 is not agreeable to some company especially considering RAN2 is waiting for RAN4 conclusion. Option 5 and 6 don’t get majority support. For the Options reuse existing signalling to indicate Case 4: Option 2a (Nokia, Meta, Google) Option 2b (Ericsson, Xiaomi, Nokia, Meta, Samsung, Google, OPPO) Moderator recommend to go with Option 2b in 2nd round considering all the companies support Option 2a also support Option 2b. Meanwhile, there are also some comments on whether current signalling can be used/fully solve the problem. Further check in 2nd round. |  |

Sub-topic 1-4: ”intra-band” EN-DC Notes

- Issue 1-4-1: Note for “intra-band” EN-DC configurations

- Proposal: A note to inform that “intra-band” EN-DC configurations are considered as the same frequency spectrum in E-UTRA and NR band which should be added to TS 38.101-3. (ZTE)

| Company | Comments |
| --- | --- |
| Ericsson | The proposal is somewhat unclear, does it refer to a range within a given operating band (the NR band a migrated LTE band with the same range)? |
| Nokia(HO) | We do not understand where this note is proposed to be added. As we have tables for intra-band EN-DC, it is clear which is supposed to be intra-band EN-DC in our view. The combinations in Table 5.5B.4.2-1 which ZTE pointed out are the inter-band EN-DC; however, precisely speaking it is mixed intra- and inter-band EN-DC. We understand that IntrabandENDC-Support is applicable to the intra-band component of the mixed intra- and inter-band EN-DC. As only one IntrabandENDC-Support can be signaled per BC, you cannot have multiple intra-band components. We can request RAN2 to clarify this more because this is not clear enough in RAN2 spec today. |
| Apple | We can more or less understand why this note is proposed as currently DC_1A-3A_n3A and other similar combinations are defined as 3-band combinations. But spectrum wise, there are only two bands. This may imply that DC_3A_n3A is equivalent to inter-band EN-DC combination where the fallback rule defined for inter-band EN-DC/NE-DC would also apply for intra-band EN-DC. In our view, DC_1A-3A_n3A should be defined as 2-band combination. The fallback rule currently should be applicable to both inter-band and intra-band configurations. With that said, we do not think this note is necessary as the definition of intra-band is already very clear. |
| ZTE | The question we raised in our proposal is what Apple explained above. The concept of “intra-band” for E-UTRA band and NR band with the same frequency range is inconsistent in TS 38.101-3. In the inter-band EN-DC configuration table having the same frequency range E-UTRA/NR bands, we consider E-UTRA band and NR band are different bands, for example in Table 5.5B.4.2-1, band 3 and band n3 are different bands since they are in the configuration table of three bands. However, in the section of “intra-band contiguous EN-DC” or “intra-band non-contiguous EN-DC”, the E-UTRA band and NR band are considered as the same band, otherwise why we call them “intra-band”? Here, we think the “intra-band” actually means the same frequency range, not the same band number. We did not find a formal definition for “intra-band EN-DC” in TS 38.101-3 in clause 3.1 of Definitions. We only find a definition for “intra-band CA” in TS 38.101-1. In this proposal, we suggest to add a definition of what is intra-band contiguous/non-contiguous EN-DC in section 3.1 of Definitions in TS 38.101-3 to mention that the intra-band actually refers to E-UTRA band and NR band having the same frequency range. By this, we can understand correctly why inter-band EN-DC configuration DC_1A-3A_n3A is categorized as “three bands”. |
| Moderator summary: No consensus on introducing this Note, and majority think it is not necessary. Moderator recommend to not pursue this note in this meeting. |  |

CRs/TPs/ comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216421 | Draft CR for 38.101-3 Rel-16 intra-band contiguous EN-DC band combination Moderator Note: depends on outcome of Option 2b in Issue 1-3-1. |
|  | Xiaomi: at least, for band 41 and band n41, disagree the note. |
| R4-2216427 | Draft CR for 38.101-3 Rel-17 intra-band contiguous EN-DC band combination Moderator Note: depends on outcome of Option 2b in Issue 1-3-1. |
|  | Xiaomi: at least, for band 41 and band n41, disagree the note. |

Summary for 1st round

Open issues

|  | Status summary Moderator notes: Detailed summary can be found in each issue in section 1.2 |
| --- | --- |
| - Issue 1-1-1: Whether Operator has demands for Case 3 in the fields | Tentative agreements: 1. Case 3 for b41+n41 to be removed 2. Case 3 for b48+n48 is kept and considered a valid band combination Candidate options: Recommendations for 2nd round: Further discuss whether the note below can be added for Case 3 b48+n48 in the solution discussion: - Note X: The fallback rule in 38.306 “An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination” is not applicable. |
| - Issue 1-1-2: Whether Operator has demands for Case 4 in the fields | Tentative agreements: Case 4 is a valid scenario Candidate options: Recommendations for 2nd round: |
| - Issue 1-2-1: Whether contiguous or non-contiguous EN-DC configurations are only categorized by DL in RAN4? | Tentative agreements: Contiguous or non-contiguous EN-DC configuration is NOT only categorized by DL in RAN4 Candidate options: Recommendations for 2nd round: |
| - Issue 1-2-2: How to interpretate the fallback rule in RAN2 38.306 | Tentative agreements: In a band combination, the UL configuration is either the same as DL configuration or belongs to the DL fallback configurations. Candidate options: Recommendations for 2nd round: Potential issue with this agreed UL and DL configuration relation is that it may lead to Case 3 as invalid scenario, since the DL contiguous cannot fallback to non-contiguous configuration. How to solve this issue is up to Issue 1-3-1 solution discussion. |
| - Issue 1-2-3: Views on the fallback rule in RAN4 38.101-3 section 4.2 below | Tentative agreements: None, views are divergent. Candidate options: Recommendations for 2nd round: No more discussion in 2nd round. |
| - Issue 1-3-1: Solutions for Case 3 | Tentative agreements: - Consider design new signalling in Rel-18 for Case3 if necessary and detailed signalling design is up to RAN2. And rely existing signalling to indicate Case 3 before Rel-18. - Go with Option 2b with high priority and further consider Option 2a if feasibility issue found for Option 2b. Candidate options: Recommendations for 2nd round: - Further discuss clarification question: Does it mean if both DL DC_(n)48CA and DL DC_48A_n48A are supported by UE through UE capability signalling, UL DC_48A_n48A is supported for DL DC_(n)48CA? - Further check Companies view on Option 6 proposed in 1st round. |
| - Issue 1-3-2: Solutions for Case 4 | Tentative agreements: Go with Option 2b in 2nd round Candidate options: Recommendations for 2nd round: Further clarify whether current signalling can be used/fully solve the problem. |
| - Issue 1-4-1: Note for “intra-band” EN-DC configurations | Tentative agreements: No consensus on introducing this Note, not pursue this note in this meeting. Candidate options: Recommendations for 2nd round: |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2216421 | Draft CR for 38.101-3 Rel-16 intra-band contiguous EN-DC band combination Moderator summary: It depends on outcome of Option 2b in Issue 1-3-1, recommend to focus on Issue 1-3-1 in this meeting and postpone this draft CR to next meeting. |
| R4-2216427 | Draft CR for 38.101-3 Rel-17 intra-band contiguous EN-DC band combination |
|  | Moderator summary: It depends on outcome of Option 2b in Issue 1-3-1, recommend to focus on Issue 1-3-1 in this meeting and postpone this draft CR to next meeting. |

Discussion on 2nd round

Open issues

|  | Comment collection |
| --- | --- |
| - WF on IntrabandENDC-Support | Meta: we add our comments in WF directly. |

Summary for 2nd round

|  | Status summary |
| --- | --- |
| - WF on IntrabandENDC-Support | Comments are received on the WF in 2nd round, suggest to further discuss in GTW 18 and approve it. |

Topic #2: CRs for Canada and US band n77

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215334 | Nokia, Nokia Shanghai Bell | Observation 1: In LTE RRC, the NS-value for PCell which is obtained in single carrier or after HO is not used when UL CA is configured and the NS-value is configured only for Scell value and the NS-value for Scell is applicable to Pcell instead. In other words, an NS-value for Pcell follows an NS-value configured for Scells when UL CA is configured. Observation 2: NS value handling in NR specifications should follow the same way as LTE spec And proposed the following: Proposal 1: Adopt option 2 and send an LS to RAN2 to share RAN4’s preference. Proposal 2: Clarify that RAN4 has no intention to turn over the agreement on n77 US and Canada issues made in RAN2 in RAN2#119-e in the LS. Proposal 3: No RAN4 spec changes are needed due to RAN2 spec changes to fix NS mapping issue. |
| R4-2215527 | Nokia, Nokia Shanghai Bell | CR: Introduction of intra band NC UL CA in the n77 frequency range in Canada [n77 Canada] |
| R4-2215794 | Mediatek | Observation 1: Regarding intra-band UL CA for band n77 in US or Canada, the operation of NS_55, NS_57, CA_NC_NS_01, and CA_NS_01 with additionalSpectrumEmission and signalling is clearly indicated in RP-222682, RP-222683, R2-2209137 and R2-2209139. Additionally, the RP-222682 and RP-222683 CRs’ new added texts also follow the agreed RAN2 CRs R2-2209137 and R2-2209139, respectively. Observation 2: Regarding NS mapping from RAN2 and RAN4, the RAN2 agreements (i.e., CRs of R2-2209137 and R2-2209139) can be accommodated by either option 1 or option 2 in [3] although the implementation may slightly differ. Option 1: The additionalSpectrumEmission associated with PCC applies for NS, CA_NS, and CA_NC_NS. RAN4 specification update needed. Option 2: The additionalSpectrumEmission associated with configuration/activation of the SCC applies (analogous to LTE with a separate parameter) for CA_NS and CA_NC_NS. RAN2 specification update needed. Proposal 1: Based on the agreed intra-band UL CA CRs (i.e., RP-222682 and RP-222683) associated with Canada and US band n77, the RAN4 specification update and NS mapping in TS 38.101-1 is already applicable. As for NS (i.e., NS, CA_NS, and CA_NC_NS) mapping issue associated with additionalSpectrumEmission, it can be solved by either option 1 or option 2 from R4-2214409. As for additionalSpectrumEmission NS mapping on n77 or other bands, option according to RAN4 solution below is recommended as it is straightforward and updated directly in RAN4 specification if needed. Option: For intra-band UL CA on band n77 or other bands, the additionalSpectrumEmission associated with PCC applies for NS, CA_NS, and CA_NC_NS. RAN4 specification update if needed. |
| R4-2216063 | Huawei, HiSilicon | Observation 1: since “NS_XX, CA_NS_XX, CA_NC_NS_XX” represent the RF requirements, RAN4 still need to associate the specific RF requirements “NS_XX, CA_NS_XX, CA_NC_NS_XX” to the network signalling label. Otherwise, UE may not know what the accurate RF requirements are for a specific network work signalling lable. Observation 2: As Rel-17 has been finalized, it’s too late to introduce a new IE which is analogous to LTE with a separate parameter due to NBC issue. Option 2 in WF R4-2214409 is not preferred. Observation 3: The reason why RAN2 stress “Network configures the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured” is that specification or network can’t require or configure two different sets of RF requirements to UE for intra-band UL CA. Otherwise, UE may be confused and doesn’t know which set of RF requirements need to be met. Observation 4: the demands for NS_47 are only applicable to single carrier instead of intra-band UL CA. That’s why CA_NS_47 wasn’t specified. If we remove CA_NS and CA_NC_NS as suggested in WF R4-2214409, it will cause some ambiguity. Proposal 1: introducing a new IE which is analogous to LTE with a separate parameter will cause NBC issue, so option 2 in WF R4-2214409 is not preferred. Proposal 2: “CA_NS_XX and CA_NC_NS_XX” should be kept to indicate the specific RF requirements, which is used in RAN4 specification. In addition, “CA_NS_XX and CA_NC_NS_XX” need to be associated to network signalling label additionalSpectrumEmission. Proposal 3: In order to indicate the same RF requirements for all uplink carrier(s) of the same band, Network is required to configure the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured. However, some exceptions are allowed as below. With these exceptions, the same RF requirements for all uplink carrier(s) of the same band are also applicable. Proposal 4: in order to reflect the proposals above, the specification can be further improved as Annex. |
| R4-2216649 R4-2216650 | Qualcomm | CR: Remove network signalling labels for CA_NS and CA_NC_NS |

Open issues comment collection

Sub-topic 2-1:

Moderator Note: In RAN4#104e two options are given for the mapping of RAN2 signaling to RAN4 NSs [R4-2214409]. Option 1: The additionalSpectrumEmission associated with PCC applies for NS, CA_NS, and CA_NC_NS. RAN4 specification update needed. Option 2: The additionalSpectrumEmission associated with configuration/activation of the SCC applies (analogous to LTE with a separate parameter) for CA_NS and CA_NC_NS. RAN2 specification update needed.

- Issue 2-1-1: Whether Option 2 in R4-2214409 (as below) means new signaling is designed?

Option 2: The additionalSpectrumEmission associated with configuration/activation of the SCC applies (analogous to LTE with a separate parameter) for CA_NS and CA_NC_NS. RAN2 specification update needed.

- Yes

- No

| Company | Comments |
| --- | --- |
| Huawei | Since this option 2 is analogous to LTE with a separate parameter, in my understanding, a new signaling (e.g. additionalSpectrumEmissionScell) is needed as what we did in LTE phase. However, it’s too late to introduce a new signaling which will cause NBC issue. But proponent can further clarify this option and its intention. |
| Ericsson | Yes. For LTE a separate indication additionalSpectrumEmissionScell was needed since the MPR specification is different for LTE, the A-MPR is added to the MPR for the non-CA case while for LTE CA the total back-off is set by A-MPR (and MPR = 0 dB). This is not a problem for NR, the same value of additionalSpuriousEmission can be indicated on all uplink serving cells (with exceptions). |
| Qualcomm | New signaling would be needed if separate indexing is required for CA compared to single carrier, as we had it for LTE. However, aside from the consistency with LTE, no such need has been identified for NR as far as we understand it. But if there is a need identified, we would be willing to consider it. |
| Apple | Yes |
| Meta | Yes, need to update RAN2 specification with new signaling |
| OPPO | Yes |
| MediaTek | Yes |
| Nokia(HU) | No, why do we need new signalling? Only what we need to do is additionalSpuriousEmission follows SCell value. Even now, the same name of additionalSpuriousEmission is indicated in different containers, and additionalSpuriousEmission in FrequencyInfoUL corresponds to that in additionalSpectrumEmissionSCell in LTE. In LTE, the spec just split additionalSpectrumEmission into additionalSpectrumEmissionSCell and additionalSpectrumEmissionPCell. But the latter was not actuall used at all. Would companies that said “Yes” explain why new signalling is needed? Only what we need to do is just clarify that additionalSpuriousEmission in FrequencyInfoUL in SCell determines the value that UE shall follow. |
| Intel | Yes, our understanding is that new signaling would need to be added to RRC in order to configure a CA_NS or CA_NC_NS value for intra-band uplink CA operation, where the values are able to be configured independently from the cell specific NS values that are used for the signal UL carrier (per band) case. It is not yet clear to us whether there is a need for independent configuration of the CA_NS/CA_NC_NS for intra-band UL CA and the NS value for single UL carrier case. So far no company has identified a strong need for this. Whether this needs to be supported by the specs is probably the first question that needs to be answered before debating details of the CRs. To respond to the Nokia question, the additionalSpectrumEmission for both PCell and SCells are provided in the ServingCellConfigCommon/ServingCellConfigCommonSIB. These contain common configuration for the cell and are expected be configured in the same way whether provided via SIB or dedicated signaling. These are not UE specific configurations. Consequently if additionalSpectrumEmission=n is set in the SIB of a cell, then the same value n should be provided for that cell when UL CA is configured. If I remember correctly this was pointed out by an earlier Nokia RAN plenary paper on the n77 specific issue. To provide a different additionalSpectrumEmission value to be applied when UL CA is configured then additional signalling would be required. |
| AT&T | We would not want to see any NBC change introduced at this time. |
| T-Mobile USA | No. We believe that in the past RAN4 told RAN2 that the same NS value and additional emissions requirments for all parts of a band and RAN2 based the signalling on that. We are not aware of any situation where additional spectrum emissions requirments only applies to part of a band or only apply for CA, although there may be different MPR. Also, we now use NS for barring for part of the band for n77, but there are no associated emissions requirments. RAN23 has decided that either NS_01 or NS_55/57 can be signalled for CA, so there is no need to change the RAN2 specs. |
| Moderator summary: Different views on whether the Option 2 in R4-2214409 means new signaling or not. However, this is for alignment understanding purpose which can be discussed further in other issues. |  |

- Issue 2-1-2: Whether CA_NS_XX and CA_NC_NS_XX should be kept in RAN4 spec to indicate the specific RF requirements?

- Option 1: Yes (HW)

- Option 2: No

| Company | Comments |
| --- | --- |
| Huawei | Option 1. As we explained in our contribution R4-2216063, CA_NS_XX and CA_NC_NS_XX represent RF requirements including not only general SE/SEM or ASE/ASEM requirements, but also MPR or AMPR requirements. If we remove them as QC proposed in R4-2216649, we will have three different RF requirements sets for one label NS_04, i.e. UL single carrier, intra-band UL contiguous and intra-band UL non-contiguous. It will cause some confusion and ambiguities for readers. |
| Ericsson | Option 1: CA_NS_xy (NS_xy indicated/configured for both the Pcell and Scell) does not indicate the same additional emission requirement as NS_xy indicated on the Pcell only. |
| Qualcomm | Option 2. If it is agreed that separate indexing and separate signaling is not needed for CA, then it would be better to remove the CA_NS and CA_NC_NS indexing since a common indexing with single carrier is used. Leaving the indexing separate implies that they can be independently indexed. Of course, the requirements and A-MPR if needed for CA_NS and CA_NC_NS could differ, but these requirements would be indexed by the common additionalSpectrumEmissions IE. |
| Apple | Option 1 |
| Meta | Option 1 |
| OPPO | If different additional requirements are indicated by CA_NS comparing to single band NS, then probably they should be kept. But the problem might be how to signal it. |
| Nokia(HU) | As commented in the previous enquiry, we are ok with Option 2b and SCell/PCell discussion should leave to RAN2. |
| Intel | If independent configuration of the CA_NS/CA_NC_NS for intra-band UL CA and the NS value for single UL carrier case is needed then CA_NS_XX and CA_NC_NS_XX must remain in the specs. If such independent configuration flexibility is not needed then it would be possible to remove CA_NS_XX and CA_NC_NS_XX as suggested by the Qualcomm CRs. However, this will likely cause a larger specification change then necessary with an approach that keeps CA_NS_XX and CA_NC_NS_XX. Our (mild) preference is to minimise specification changes and keep CA_NS_XX and CA_NC_NS_XX. |
| AT&T | We do not have a strong view as long as there is a way for RAN4 to clearly define the RF requirements for the n77 case when different NS values are signalled but the alternate NS value(s) is(are) used for barring purposes only. |
| T-Mobile USA | Option 2. While clearly there are different MPR and A-MPR requirements for CA, the UE knows that the CA requirments apply for CA, and the non-CA requirements apply for non-CA. The network doesn’t need to tell the UE that since CA is enabled it needs to apply CA_NS_04, vs. when CA is not applied it needs to apply NS_04. The UE knows when CA is applied and when it is not so it knows what MPR and A-MPR to apply. We don’t want the network to have to tell the UE to use different MRP and A-MPR every time CA is enabled or disabled. If we had different emissions requirments for CA that would be different. But since we don’t, there is no need to have separate NS signalling for CA. |
| Moderator summary: There are still different preferences in whether to keep CA_NS/CA_NC_NS, but it is clear that in current spec the MPR/AMPR defined for intra-band CA is different from single band and is associated with CA_NS/CA_NC_NS. Meanwhile, it is also commented that specific MPR/AMPR for intra-band CA can also be indicated via single CC NS which makes the CA_NS/CA_NC_NS is not necessary. Option 1, keep (HW, Ericsson, Apple, Meta, OPPO, Intel) Option 2, remove (Qualcomm, [Nokia?], T-Mobile USA) Moderator recommend that it is confirmed CA_NS_XX and CA_NC_NS_XX in RAN4 spec are associated with specific RF requirements. Whether to keep or remove the CA_NS/CA_NC_NS is pending further discussion. |  |

- Issue 2-1-3: Which solution is preferred

- Option 1: Solve the NS mapping issue in RAN2, and change 38.331 as below.

  - The additionalSpectrumEmission associated with configuration/activation of the SCC applies (analogous to LTE with a separate parameter) for CA_NS and CA_NC_NS. (Nokia)

    - Note: Proposed RAN2 changes in R4-2215334 Annex.

- Option 2: Solve the NS mapping issue in RAN4 (MTK, HW, QC)

  - Option 2a: In order to indicate the same RF requirements for all uplink carrier(s) of the same band, Network is required to configure the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured. However, some exceptions are allowed as below. With these exceptions, the same RF requirements for all uplink carrier(s) of the same band are also applicable. (HW)

    - Note: Proposed RAN4 changes in R4-2216063 Annex.

  - Option 2b: Instances of CA_NS and CA_NC_NS are removed but pointed to the NS signaled in the band. (QC)

    - Note: Proposed RAN4 changes in CR R4-2216649

| Company | Comments |
| --- | --- |
| Huawei | We prefer option 2a. Comments on option 1. Why are “uplink carrier(s)” replaced by “SCell”? For RAN4 spec and table’s reference, generally I’m fine with them. But editorial errors can be corrected, e.g. 6.2A.3.1.1-2. However, it’s better to discuss this RAN2 CR in RAN2’s maintenance agenda which is covered by RAN2 experts. Comments on option 2b. Firstly, the demands for NS_47 are only applicable to single carrier (30MHz) instead of intra-band UL CA. That’s why CA_NS_47 wasn’t specified. If we remove CA_NS and CA_NC_NS as suggested in WF R4-2214409 and add them into CA clauses proposed in R4-2216649, it will cause some ambiguities and UE vendors have to take the additional costs/test/requirements, which are not necessary. Secondly, as we commented above, CA_NS_XX and CA_NC_NS_XX represent RF requirements including not only general SE/SEM or ASE/ASEM requirements, but also MPR or AMPR requirements. If we remove them as QC proposed in R4-2216649, we will have three different RF requirements sets for one label NS_04, i.e. UL single carrier, intra-band UL contiguous and intra-band UL non-contiguous. It will cause some confusion and ambiguities for readers. |
| Ericsson | Option 2a: the CA_NS could be clarified but the RAN4 specifications should not set any requirements on the configuration by the network (RAN2). The case with more than one NS value indicated in the SI (and obtained by the Pcell) and one of these configured for the Scell could also be covered (not an issue for the current n77 cases). |
| Qualcomm | Option 2b. The CA_NS and CA_NC_NS are referred to as “network signaling label”. Since a common signaling label or indexing is used between single carrier and CA, then there is no separate network signaling label for CA. Keeping it in the RAN4 spec is at best redundant and possibly confusing. In fact, having it separate is what caused the confusion in the first place from last meeting. We don’t understand the point from Huawei, but the requirements are listed as “N/A” so there shouldn’t be any ambiguity or burden on the UE. In fact, not having the “N/A” suggests that there might be a requirement specified or that it was simply overlooked. |
| Apple | Option 2a RAN2 signaling is used to indicate the emission requirement for the band. Therefore, there is no need to introduce another emission requirement index for SCell. RAN4 NS index mapping includes the information of emission requirement and A-MPR requirements where A-MPR can be different among single carrier, contiguous UL, and non-contiguous UL. Therefore, we prefer to keep the existing NS index mapping. For n77, we may need to include the following mappings in the table: NR CA band; Value of additionalSpectrumEmission. 0; 1; 2; 3; 4; 5; 6; 7. CA_n41; CA_NS_01; CA_NS_04. CA_n48; CA_NS_01; CA_NS_27. CA_n7; CA_NS_01; CA_NS_46. CA_n77; CA_NS_01; CA_NS_01; CA_NS_01. NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7].. NR CA band; Value of additionalSpectrumEmission. 0; 1; 2; 3; 4; 5; 6; 7. CA_n41; CA_NC_NS_01; CA_NC_NS_04. CA_n77; CA_NC_NS_01; CA_NC_NS_01; CA_NC_NS_01. NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7].. |
| Meta | Firstly, we prefer to solve this issues in RAN4. And the option 2a is our preference. |
| MediaTek | We can understand the intention of Option 2a and 2b. Both options have the point. Currently, we slightly prefer Option 2a. |
| Nokia | Option 1 and Option 2b. What is the point to compare Option 1 and Option 2b? Option 2b and Option 1 are not exclusive. |
| Intel | We think the issue can be addressed mainly in RAN4 specs and the CR associated with option 2a provides the best starting point. We are not certain at this stage that no change in the RAN2 specs will be needed but we can progress the RAN4 CRs and later consider if any RAN2 alignment is needed. Furthermore, we think that both options 2a and 2b have a deficiency, which is best explained by an example. Selecting a random example of band n5 for which 2 NS values are defined, NS_01 and NS_100, and consider the case that all cells in the band set additionalSpectrumEmission to 1 (which is interpreted as NS_100). Now if uplink intra-band contiguous CA is configured, with the additionalSpectrumEmission value for both the PCell and SCell set to 1, neither the 2a or 2b CRs explain how to map additionalSpectrumEmission=1 to an CA_NS value (CA_NS_0 corresponding to additionalSpectrumEmission=0 is the only value defined for band n5). This can easily be addressed by stating that the UE applies CA_NS_0 for all bands not listed in the table. Finally band n41 has 3 possible NS values so the additionalSpectrumEmission value can be 0,1,2. the current 2a CRs do not explain how to map additionalSpectrumEmission=2 to an CA_NS value. This should be added to the table (not sure if CA_NS_01 or CA_NS_04 is the correct requirement to apply for this case). The 2b CRs do address this case. |
| AT&T | We do not have a strong view as long as we do not introduce NBC changes at this stage and there is a way for RAN4 to clearly define the RF requirements for the n77 case when different NS values are signalled but the alternate NS value(s) is(are) used for barring purposes only. |
| T-Mobile USA | We would be fine with Option 2a or option 2b. We think that Option 1 could lead to NBC issues/ |
| Moderator summary: No consensus can be reached, but seems majority companies prefer Option 2a though revision might be needed. In the 2nd round companies can further discuss whether there is technical issue if use Option 2a as baseline and discuss the detailed changes. Option 2a (HW, Ericsson, Apple, Meta, MTK, Intel, TMO) Option 2b (Qualcomm, Nokia, TMO) |  |

CRs/TPs/ comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2215527 | Introduction of intra band NC UL CA in the n77 frequency range in Canada [n77 Canada] Moderator Note: CR is created based on the approved Rel-17 CR of RP-222686 for US. |
|  | Huawei: see comments above |
|  | Meta: These contents will be updated based on issue 2-1-3 |
|  | Intel: Please see comments above |
|  | AT&T: We are OK with the content of the CR since it follows the agreed text utilized for the NS55 case. It should be revised as noted on the coversheet to take the official 38.101-1 v17.7.0 content as the baseline. We would prefer to provisionally agree to the revision and address any modifications necessary once RAN4 concludes on the generic mapping issue. |
|  | T-Mobile USA: We are OK with the CR. We agree with the comments from AT&T. |
| R4-2216649 R4-2216650 (CAT-A) | Remove network signalling labels for CA_NS and CA_NC_NS Moderator Note: depends on outcome of Option 2b in issue 2-1-3. |
|  | Huawei: see comments above |
|  | Meta: These contents will be updated based on issue 2-1-3 |
|  | Intel: Please see comments above |
|  | T-Mobile USA: We think it would be simpler to just modify the table as follows: |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| - Issue 2-1-1: Whether Option 2 in R4-2214409 (as below) means new signaling is designed? | Tentative agreements: None Candidate options: Recommendations for 2nd round: |
| - Issue 2-1-2: Whether CA_NS_XX and CA_NC_NS_XX should be kept in RAN4 spec to indicate the specific RF requirements? | Tentative agreements: It is confirmed CA_NS_XX and CA_NC_NS_XX in RAN4 spec are associated with specific RF requirements Candidate options: Recommendations for 2nd round: Whether to keep or remove the CA_NS/CA_NC_NS |
| - Issue 2-1-3: Which solution is preferred | Tentative agreements: No consensus can be reached, but majority companies prefer Option 2a. Candidate options: Recommendations for 2nd round: Further discuss whether there is technical issue if use Option 2a as baseline and discuss the detailed changes with WF. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2215527 | Introduction of intra band NC UL CA in the n77 frequency range in Canada [n77 Canada] |
|  | Agreed. Nokia: we don’t see the necessity of revision. The change just follows existing spec. The CR should not be taken as a hostage. If QC’s CR was agreed, we could revise this accordingly, though the agreement of QC CR was postponed. |
| R4-2216649 R4-2216650 (CAT-A) | Remove network signalling labels for CA_NS and CA_NC_NS |
|  | Postponed |

Discussion on 2nd round

Open issues

|  | Comment collection |
| --- | --- |
| - WF on NS mapping for intra-band CA | Qualcomm: We do not agree with the WF. In our understanding, there is a common signaling of additionalSpectrumEmissions for all carriers in a band. Therefore, this common signaling NS_XX is used to map to appropriate emission and A-MPR specifications. It is superfluous to create redundant labels CA_NS_XX and CA_NC_NS_XX. For example, for Band n48 and all intra-band CA configurations within Band n48, there is a single signaling value according to NS_27 for example. There is no need to create a new label called CA_NS_27 and CA_NC_NS_27 since they are all the same thing already indicated by NS_27. Creating separate labels implies that they could be different things distinctly signaled which would be incorrect. Hence, we propose to remove CA_NS and CA_NC_NS and just use the one NS_XX for indexing in single carrier and intra-band CA. |
|  | T-Mobile USA: We agree with Qualcomm. We also don't support the WF. As we proposed in round 1, we think it would be simpler to just modify the tables as follows: Huawei: As we explained in our contribution R4-2216063, CA_NS_XX and CA_NC_NS_XX represent RF requirements including not only general SE/SEM or ASE/ASEM requirements, but also MPR or AMPR requirements. From RAN4 perspective, CA_NS_XX and CA_NC_NS_XX are different from NS_XX. If we follow Qualcomm’s method to remove them, it will have an big impact on RAN4’s spec and will cause some confusion on RF requirements’ indication. For example, NS_47 is only used for single carrier, but not for intra-band. However, following QC’s method will change current requirements and mix the requirements between single carrier and UL intra-band requirements. |
|  | Nokia: We cannot agree with the WF due to following reasons. For the observation, we need to reiterate our comments in the 1st round. We don’t observe NBC issues and we don’t expect we need a new IE. The existing RAN2 spec has just said that all the NS values across serving cells the same. We have used that. Even if we clarified that UE should follow a value in SCell, there must not be NBC issue since thus far, the same value has been used across carries for dedicated signaling. How can we see NBC issue? For the WF1, we don’t see specific reasons to keep the CA_NS_xxx and CA_NC_NS_xx so that we don’t agree with the WF1. For the WF2, we don’t understand what the last text intends for. We need to be careful about it. The proposal says if he intra band UL CA RF requirements are the same across uplink carriers, different NS values are allowed. This broaden the RAN2 agreement significantly. The exceptions should not be allowed due to such reasons. NS_55 and NS_57 are used also for cell camping restriction on the legacy UEs. The existing exceptions are highly band specific. However, exceptions are also allowed if the same intra-band UL CA RF requirements for all uplink carrier(s) of the same band are indicated by network. For the WF3, the original mapping discussion is how CA_NS_xx/CA_NC_NS_xx and NS_xx are mapped since RAN2 spec has referred to only NS_xx for the NS to be used for UL CA. Hence which table RAN2 spec should refer to and/or RAN4 spec is changed to make RAN2 spec stay or make RAN2 spec change minimum must have been the discussion point. Now suddenly companies try to include SCell/PCell discussion in RAN4. Note that since companies brought this in the last meeting, we included this aspect into our contribution. |
|  | Meta: we prefer to keep the CA_NS_XX/CA_NC_NS_XX for intra-band CA UE to comply the regional specific emission requirements. We also have similar view with Huawei how to apply the NS mapping for intra-band CA UE and shall update RAN2 specification based on the revised RAN4 agreements. |
|  | Ericsson: WF1: agreed. WF2: if a clarification is made in the 38.101-1, there should be no requirement on what the network shall configured or is expected to be configured; this belongs to 38.331. However, it could be clarified that the CA_NS is a mapping from a band number and a single additionalSpectrumEmission value configured for more than one uplink serving cell. There are exceptional cases with different values, the applicable requirements for these cases are clear following the changes at RAN. The NS value is also a RAN4-only notion, a mapping from a band number and a an additionalSpectrumEmission value for a single uplink serving cell. The requirement indicated by the NS implied is different from that indicated by the CA_NS. Removing the latter would create ambiguity as to the applicable emission requirement. WF3: not agreed, no further RAN2 changes are needed at this point. |
|  | Intel: WF1: Agree - As we commented in the 1st round, we think it would be possible to remove the separate labels for CA_NS_XX and CA_NC_NS_XX, and merge the requirements for the UL CA cases to the NS_XX table (because all 3 tables are pointed to by the same signaled value). However, although it would be possible to do this we don't see any strong technical need for doing so. We prefer to minimize spec impact and hence prefer to keep CA_NS_XX and CA_NC_NS_XX. WF2: Not really sure what WF2 is saying. 38.331 already says " Network configures the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured, except for additionalSpectrumEmission value corresponding to NS_55/NS_57. If NS_55/NS_57 ". Is the way forward proposal simply suggesting to duplicate this in RAN4 specs? WF3: Agree the NS mapping issue needs to be solved. I would suggest that we develop the RAN4 spec changes that would be required and then inform RAN2 to let them conclude on the RAN2 spec changes. |
|  | Apple: WF1: Support WF2: Not sure about the purpose of this WF. WF3: RAN4 spec update can be further discussed if found necessary. Not sure about what needs to be updated in TS 38.306. MediaTek: WF1: We see points from Qualcomm and T-Mobile. The decision does not need to be binary. We think more discussion is still needed. WF2: We are not sure about the purpose currently. WF3: Given that no further RAN2 changes are needed at this point, we are not okay with WF3 at this stage. |

CRs/TPs

| CR/TP number | Comment collection |
| --- | --- |
| Rev R4-2215527 | Introduction of intra band NC UL CA in the n77 frequency range in Canada [n77 Canada] |
|  | Intel: Our first round comments relating to this CR were not correct - they were referring to the mapping issue, not this specific CR. This CR looks OK. Although obviously needs to be recreated based on the September spec. |

Summary for 2nd round

|  | Status summary |
| --- | --- |
| - WF on NS mapping for intra-band CA | Comments are received on the WF in 2nd round, suggest to further discuss in GTW 18 and try to approve it. |

| CR/TP number | Status summary |
| --- | --- |
| Rev R4-2215527 | No revision is needed, thus withdrawn. |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-2217759 | WF on IntrabandENDC-Support | OPPO |  |
| R4-2217760 | WF on NS mapping for intra-band CA | Huawei |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2215668 |  | Issue for intra-band EN-DC combinations | Apple | Noted |  |
| R4-2215933 |  | Issues on intra-bandENDC-Support | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2215956 |  | The inconsistency issue for intra-band EN-DC band combinations | Ericsson | Noted |  |
| R4-2216356 |  | Discussion on intrabandENDC-Support | Xiaomi | Noted |  |
| R4-2216412 |  | Discussion on Intra-Band EN-DC band combinations | Google Inc., Comcast, CableLabs | Noted |  |
| R4-2216421 |  | Draft CR for 38.101-3 Rel-16 intra-band contiguous EN-DC band combination | Google Inc., Comcast, CableLabs | postpone |  |
| R4-2216427 |  | Draft CR for 38.101-3 Rel-17 intra-band contiguous EN-DC band combination | Google Inc., Comcast, CableLabs | postpone |  |
| R4-2216443 |  | R16 Discussion on IntrabandENDC-Support capability handling | OPPO | Noted |  |
| R4-2216617 |  | Discussion on intra band ENDC combination inconsistency issues | ZTE | Noted |  |
| R4-2216656 |  | Discussion on intra-band EN-DC combination | Huawei, HiSilicon | Noted |  |
| R4-2216657 |  | [DRAFT] LS on intra-band EN-DC combination | Huawei, HiSilicon | Noted |  |
| R4-2215334 |  | Impact of NS mapping between RAN2 and RAN4 specs on n77 issue | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2215527 |  | Introduction of intra band NC UL CA in the n77 frequency range in Canada [n77 Canada] | Nokia, Nokia Shanghai Bell | Agreed |  |
| R4-2215794 |  | Discussion on additionalSpectrumEmission for NS mapping and CRs associated with Canada and US band n77 | Mediatek | Noted |  |
| R4-2216063 |  | Discussion on NS mapping for intra-band UL CA | Huawei, HiSilicon | Noted |  |
| R4-2216649 R4-2216650 (CAT-A) |  | Remove network signalling labels for CA_NS and CA_NC_NS | Qualcomm | postpone |  |

2nd round

|  | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2217759 | WF on IntrabandENDC-Support | OPPO | [Approved], Pending GTW discussion |
| R4-2217760 | WF on NS mapping for intra-band CA | Huawei | [Approved], Pending GTW discussion |

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2217761 | Introduction of intra band NC UL CA in the n77 frequency range in Canada [n77 Canada] | Nokia, Nokia Shanghai Bell | Withdrawn |  |