| 3GPP TR 38.872 V0.2.0 (2022-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on enhancement for 700/800/900MHz band combinations (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	7

4.1	TR Maintenance	8

5	Specific Band combinations	8

5.1	CA_n5-n8	8

5.1.1	UE RF architecture assumption	8

5.1.2	Common for 1 band UL and 2 bands UL of CA_n5-n8	8

5.1.2.1	Operating bands for CA	8

5.1.2.2	Channel bandwidths per operating band for CA	9

5.1.2.3	UE co-existence studies	9

5.1.2.4	∆TIB,c and ∆RIB,c values	9

5.1.2.5	REFSENS requirements	10

5.1.3	Specific for 2 bands UL of CA_n5-n8	10

5.1.3.1	Maximum output power for inter-band CA	10

5.1.3.2	UE co-existence studies	10

5.1.3.3	REFSENS requirements	11

5.2	CA_n5-n28	12

5.2.1	UE RF architecture assumption	12

5.2.2	Common for 1 band UL and 2 bands UL of CA_n5-n28	12

5.2.2.0	General	12

5.2.2.1	Operating bands for CA	12

5.2.2.2	Channel bandwidths per operating band for CA	12

5.2.2.3	UE co-existence studies	12

5.2.2.4	∆TIB,c and ∆RIB,c values	12

5.2.2.5	REFSENS requirements	12

5.2.3	Specific for 2 bands UL of fallback CA_n5-n28	12

5.2.3.1	Maximum output power for inter-band CA	12

5.2.3.2	UE co-existence studies	12

5.2.3.3	REFSENS requirements	14

5.3	CA_n8-n20-n28	14

5.3.1	UE RF architecture assumption	14

5.3.2	CA_n8-n20	15

5.3.2.0	General	15

5.3.2.1	Operating bands for CA	15

5.3.2.2	Channel bandwidths per operating band for CA	15

5.3.2.3	UE co-existence studies	15

5.3.2.4	∆TIB,c and ∆RIB,c values	15

5.3.2.5	REFSENS requirements	15

5.3.2.6	Maximum output power for UL inter-band CA	15

5.3.2.7	UE co-existence studies for UL inter-band CA	15

5.3.2.8	REFSENS requirements for UL inter-band CA	17

5.3.3	CA_n8-n28	17

5.3.3.0	General	17

5.3.3.1	Operating bands for CA	17

5.3.3.2	Channel bandwidths per operating band for CA	17

5.3.3.3	UE co-existence studies	18

5.3.3.4	∆TIB,c and ∆RIB,c values	18

5.3.3.5	REFSENS requirements	18

5.3.3.6	Maximum output power for UL inter-band CA	18

5.3.3.7	UE co-existence studies for UL inter-band CA	18

5.3.3.8	REFSENS requirements for UL inter-band CA	20

5.3.4	CA_n20-n28	20

5.3.4.1	General	20

5.3.5	CA_n8-n20-n28	21

5.3.5.1	Operating bands for CA	21

5.3.5.2	Channel bandwidths per operating band for CA	21

5.3.5.3	∆TIB,c and ∆RIB,c values	21

5.3.5.4	UE co-existence studies for UL inter-band CA	21

5.3.5.5	REFSENS requirements	25

Annex A: Change history	26

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document …

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

<defined term>: <definition>.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background

In RAN#95e, SI “enhancement for 700800900MHz band combinations” was approved. The objectives of SI are as follows:

1) Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands and simultaneous reception on two or three bands for the band combination of 700, 800 and 900MHz spectrum for smart phone form factor

2) The following band combinations will be considered. And the feasibility study on three band combination will start after the completion of feasibility study of all the fallback band combinations.

-  CA_n8-n20-n28 with uplink configurations of CA_n8-n20, CA_n8-n28, CA_n20-n28, and the fallback modes

-  CA_n5-n8 with uplink configuration of CA_n5-n8, and the fallback modes

Note1: Spectrum restrictions should be studied to solve overlap of band n5 downlink and band n8 uplink

Note2: The current filter is used as the baseline. Further study whether or not to have new solutions.

- CA_n5-n28 (full range) with uplink configuration of CA_n5-n28.

Summary of band combinations considered in the SI

| Configuration | Uplink configuration | Supported operators |
| --- | --- | --- |
| CA_n8-n20-n28 | CA_n8-n20, CA_n8-n28, CA_n20-n28 | Vodafone, Telecom Italia, Orange, Deutsche Telekom |
| CA_n5-n8 | CA_n5-n8 | China Telecom, Spark NZ, China Unicom |
| CA_n5-n28 | CA_n5-n28 | Spark NZ |

The following aspects need be studied

-  UE architecture including n-plexing, PA

-  Study feasibility of low band wideband antenna

-  Performance due to impacts including inter-modulation products

-  Method to manage the inter-modulation product impacts

Note: Revisit in RAN#98 whether additional aspects need to be added.

Power class 3 (PC3) is considered in this study

3) Identify potential impacts to relevant RAN4 requirements.

The present document is the technical report for this Study Item.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor.

5	Specific Band combinations

5.1	CA_n5-n8

5.1.1	UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n8

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

5.1.2	Common for 1 band UL and 2 bands UL of CA_n5-n8

5.1.2.1	Operating bands for CA

UE should support the full range of spectrum for single band operations on both n5 and n8

Use the following frequency ranges for further discussion for spectrum restriction to support uplink CA_n5-n8.

Table 5.1.2.1-1: CA band combination of band n5+n8

|  | UL | DL |
| --- | --- | --- |
| Frequency 1 (800MHz) | 824MHz - 835MHz | 869MHz - 880MHz |
| Frequency 2 (900MHz) | 904MHz - 915MHz | 949MHz - 960MHz |

The frequency relation between band n8 and n5 is shown below as UL band n8 is overlapping with DL band n5

Figure 5.1.2.1-1: The frequency relation between band n8 and n5

5.1.2.2	Channel bandwidths per operating band for CA

Based on WF R4-2214445, BCS0 for CA_n5A-n8A can be specified as below.

Table 5.1.2.2-1: Supported bandwidths per CA band combination of band n5+n6

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n8A | CA_n5A-n8A | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |

5.1.2.3	UE co-existence studies

Table 5.1.2.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n8.

Table 5.1.2.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 | 4400 | 4575 |

Table 5.1.2.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 | 4625 | 4800 |

Based on the analysis above, there is no need to specify harmonics and harmonics mixing exception for CA_n5-n8 as we didn’t specify them in current spec.

5.1.2.4	∆TIB,c and ∆RIB,c values

For CA_n5-n8, the TIB,c and RIB,c are given in the tables below.

Table 5.1.2.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n8 | n5 | FFS |
|  | n8 | FFS |

Table 5.1.2.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n8 | n5 | FFS |
|  | n8 | FFS |

5.1.2.5	REFSENS requirements

FFS

5.1.3	Specific for 2 bands UL of CA_n5-n8

5.1.3.1	Maximum output power for inter-band CA

Power class 3 is assumed for UL CA_n5-n8.

5.1.3.2	UE co-existence studies

Table 5.1.3.2-1 lists Band n5 + Band n8 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.1.3.2-1: Band n5 and Band n8 UL IMD products with frequency range restriction

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 835 | 904 | 915 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 69 | 91 | 1728 | 1750 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 733 | 766 | 973 | 1006 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 2585 | 2632 | 2665 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1557 | 1601 | 1877 | 1921 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3420 | 3536 | 3580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 182 | 138 | 3456 | 3500 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2836 | 2781 | 2436 | 2381 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1097 | 1042 | 697 | 642 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4440 | 4495 | 4200 | 4255 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4360 | 4415 | 4280 | 4335 |

Based on Table 5.1.3.2-1, there is no IMD issue for CA_n5-n8 with the following frequency range restriction.

n5	UL: 824 MHz – 835 MHz	DL: 869 MHz – 880 MHz;

n8	UL: 904 MHz – 915 MHz	DL: 949 MHz – 960 MHz.

Table 5.1.3.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.1.3.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n8 | E-UTRA Band 1, 28, 31, 34, 38, 40, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.1.3.3	REFSENS requirements

FFS

5.2	CA_n5-n28

.

5.2.1	UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n28

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

5.2.2	Common for 1 band UL and 2 bands UL of CA_n5-n28

5.2.2.0	General

According to the current spec TS 38.101-1, DL CA_n5A-n28A_BCS0 has been specified. Thus, some of RF requirements can be reused as below.

5.2.2.1	Operating bands for CA

Table 5.2.2.1-1:  CA band combination of band n5+n28

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

5.2.2.2	Channel bandwidths per operating band for CA

Referring to table 5.5A.3.1-1e of TS 38.101-1, DL_n5A-n28A_BCS0 has been specified

5.2.2.3	UE co-existence studies

There is no need to specify harmonics and harmonics mixing exception for CA_n5-n28 as we didn’t specify them in current spec.

5.2.2.4	∆TIB,c and ∆RIB,c values

For CA_n5-n28, the TIB,c and RIB,c values which has been specified in TS 38.101-1.

5.2.2.5	REFSENS requirements

FFS

5.2.3	Specific for 2 bands UL of fallback CA_n5-n28

5.2.3.1	Maximum output power for inter-band CA

Power class 3 is assumed for UL CA_n5-n28.

5.2.3.2	UE co-existence studies

Table 5.2.3.2-1 lists Band n5 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.2.3.2-1: Band n5 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 869 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 166 | 76 | 1527 | 1617 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 900 | 1035 | 537 | 672 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2351 | 2486 | 2230 | 2365 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1724 | 1904 | 1240 | 1420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3175 | 3355 | 2933 | 3113 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 152 | 332 | 3054 | 3234 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2168 | 1943 | 2773 | 2548 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 596 | 371 | 1201 | 976 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3636 | 3861 | 3999 | 4224 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3757 | 3982 | 3878 | 4103 |

Based on Table 5.2.3.2-1, there is no IMD issue for CA_n5-n28.

Table 5.2.3.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.2.3.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n28 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 25, 26, 31, 34, 38, 40, 73 NR Band n79 | FDL_low | - | FDL_high | -50 |  |  |
|  | E-UTRA Band 4, 41, 42, 43, 50, 51, 52, 65, 66, 74 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

5.2.3.3	REFSENS requirements

FFS

5.3	CA_n8-n20-n28

5.3.1	UE RF architecture assumption

The following UE RF architectures can be assumed to analyze and study for CA_n8-n20-n28.

2 antenna, 3 antenna, 4 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

5.3.2	CA_n8-n20

5.3.2.0	General

According to the current spec TS 38.101-1, DL CA_n8A-n20A_BCS0 has been specified. Thus, some of RF requirements can be reused as below. Both DL_n8A-n20A_BCS0 and DL_n8A-n20A_UL_n8A-n20A are fallback CA band combinations of DL_n8A-n20A-n28A_UL_n8A-n20A.

5.3.2.1	Operating bands for CA

Table 5.3.2.1-1:  CA band combination of band n8+n20

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |

5.3.2.2	Channel bandwidths per operating band for CA

Referring to table 5.5A.3.1-1e of TS 38.101-1, DL_n8A-n20A_BCS0 has been specified.

5.3.2.3	UE co-existence studies

There is no need to specify harmonics and harmonics mixing exception for CA_n8-n20 as we didn’t specify them in current spec.

5.3.2.4	∆TIB,c and ∆RIB,c values

For CA_n8-n20, the TIB,c and RIB,c values which has been specified in TS 38.101-1.

5.3.2.5	REFSENS requirements

As CA_n8-n20 has been specified into current spec TS 38.101-1, there is no harmonics, cross band isolation and harmonics mixing exception.

5.3.2.6	Maximum output power for UL inter-band CA

Power class 3 is assumed for UL CA_n8-n20.

5.3.2.7	UE co-existence studies for UL inter-band CA

Table 5.3.2.7-1 lists Band n8 + Band n20 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.3.2.7-1: Band n8 and Band n20 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 832 | 862 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 83 | 18 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898 | 998 | 749 | 844 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2592 | 2692 | 2544 | 2639 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1778 | 1913 | 1581 | 1706 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3472 | 3607 | 3376 | 3501 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 36 | 166 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2568 | 2413 | 2828 | 2658 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 826 | 666 | 1081 | 916 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4208 | 4363 | 4352 | 4522 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4256 | 4416 | 4304 | 4469 |

Based on Table 5.3.2.6-1, 3rd and 5th order IMD may also fall into Rx frequencies of bands n8 or band n20.

Table 5.3.2.7-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.3.2.7-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n8-n20 | E-UTRA Band 1, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 NR Band n100, n104 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 38, 42, 43, 52, 69 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8, 20 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.3.2.8	REFSENS requirements for UL inter-band CA

FFS

5.3.3	CA_n8-n28

5.3.3.0	General

According to the current spec TS 38.101-1, DL CA_n8A-n28A_BCS0 has been specified. Thus, some of RF requirements can be reused as below. Both DL_n8A-n28A_BCS0 and DL_n8A-n28A_UL_n8A-n28A are fallback CA band combinations of DL_n8A-n20A-n28A_UL_n8A-n28A.

5.3.3.1	Operating bands for CA

Table 5.3.3.1-1:  CA band combination of band n8+n28

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

5.3.3.2	Channel bandwidths per operating band for CA

Referring to table 5.5A.3.1-1e of TS 38.101-1, DL_n8A-n28A_BCS0 has been specified as below.

Table 5.3.3.2-1: Supported bandwidths per CA band combination of band n8+n28

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n28A | CA_n8A-n28A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |

5.3.3.3	UE co-existence studies

There is no need to specify harmonics and harmonics mixing exception for CA_n8-n28 as we didn’t specify them in current spec.

5.3.3.4	∆TIB,c and ∆RIB,c values

For CA_n8-n28, the TIB,c and RIB,c values which has been specified in TS 38.101-1.

5.3.3.5	REFSENS requirements

As CA_n8-n28 has been specified into current spec TS 38.101-1, there is no harmonics, cross band isolation and harmonics mixing exception.

5.3.3.6	Maximum output power for UL inter-band CA

Power class 3 is assumed for UL CA_n8-n28.

5.3.3.7	UE co-existence studies for UL inter-band CA

Table 5.3.3.7-1 lists Band n8 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.3.3.7-1: Band n8 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 212 | 132 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 1127 | 491 | 616 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463 | 2578 | 2286 | 2411 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892 | 2042 | 1194 | 1364 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343 | 3493 | 2989 | 3159 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 424 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2112 | 1897 | 2957 | 2772 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 484 | 279 | 1339 | 1144 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 | 3907 | 4223 | 4408 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869 | 4074 | 4046 | 4241 |

Based on Table 5.3.3.7-1, there is no IMD issue for CA_n8-n28.

Table 5.3.3.7-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.3.3.7-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n8-n28 | E-UTRA Band 20, 31, 34, 38, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 65, 73, 74, 75, 76 NR Band n77, n78, n79, n100 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 15 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

5.3.3.8	REFSENS requirements for UL inter-band CA

There is no REFSENS exception due to IMD interference for CA_n8A-n28A.

5.3.4	CA_n20-n28

5.3.4.1	General

According to the current spec TS 38.101-1, DL_n20A-n28A_UL_n20A-n28A_BCS0 and DL_n20A-n28A_UL_n20A-n28A_BCS1 have already been specified. Thus, all of the related RF requirements have been analysed in clause 6.31 from TR 38.716-02-00 and can be reused as below. Both DL_n20A-n28A_BCS0 and DL_n20A-n28A_UL_n20A-n28A_BCS0 are fallback CA band combinations of DL_n8A-n20A-n28A_UL_n20A-n28A.

1) The operating bands for CA_n20-n28 were specified in table 5.2A.2.1-1 from TS 38.101-1 with note 2 that The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.

2) The UL/DL configurations and bandwidth combination sets for DL_n20A-n28A_UL_n20A-n28A_BCS0 and DL_n20A-n28A_UL_n20A-n28A_BCS1 were specified in table 5.5A.3.1-1g from TS 38.101-1.

3) 23dBm Power class 3 for UL CA_n20A-n28A was specified in Table 6.2A.1.3-1 from TS 38.101-1.

4) 0.5dB TIB,c for CA_n20-n28 was specified in Table 6.2A.4.2.3-1 from TS 38.101-1.

5) Spurious emissions for UE co-existence for Inter-band CA_n20-n28 were specified in Table 6.5A.3.2.3-1 from TS 38.101-1.

6) Referring to UE co-existence studies in clause 6.31.2.1 from TR 38.716-02-00, there are no harmonic/harmonic mixing or IMD issues affecting own Rx frequencies of either band n20 or n28. Therefore, there is no need to specify the additional REFSENS exceptions for CA_n20-n28.

7) 0dB RIB,c for CA_n20-n28 was specified in Table 6.31.1.4-2 from TR 38.716-02-00.

5.3.5	CA_n8-n20-n28

5.3.5.1	Operating bands for CA

Table 5.3.5.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

It’s noted that the following sentence was clarified in clause 5.5A.0 from TS 38.101-1.

“For a higher order band combination of which CA_n20-n28 is a subset, the frequency range in band n28 is restricted for the higher order band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.”

Thus, this frequency range restriction is also applicable to CA_n8-n20-n28.

5.3.5.2	Channel bandwidths per operating band for CA

Table 5.3.5.2-1: Supported bandwidths per CA band combination of band n8+n20+n28

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n20A-n28A | CA_n8A-n20A CA_n8A-n28A CA_n20A-n28A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20 |  |

5.3.5.3	∆TIB,c and ∆RIB,c values

For CA_n8-n20-n28, the TIB,c and RIB,c values are FFS

Table 5.3.5.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | FFS |
|  | n20 | FFS |
|  | n28 | FFS |

Table 5.3.5.3-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | FFS |
|  | n20 | FFS |
|  | n28 | FFS |

5.3.5.4	UE co-existence studies for UL inter-band CA

Table 5.3.5.4-1 lists Band n8 + Band n20 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n28.

Table 5.3.5.4-1: Band n8 and Band n20 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 832 | 862 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 83 | 18 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898 | 998 | 749 | 844 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2592 | 2692 | 2544 | 2639 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1778 | 1913 | 1581 | 1706 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3472 | 3607 | 3376 | 3501 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 36 | 166 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2568 | 2413 | 2828 | 2658 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 826 | 666 | 1081 | 916 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4208 | 4363 | 4352 | 4522 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4256 | 4416 | 4304 | 4469 |

Table 5.3.5.4-2 lists Band n8 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n20.

Table 5.3.5.4-2: Band n8 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 212 | 132 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 1127 | 491 | 616 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463 | 2578 | 2286 | 2411 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892 | 2042 | 1194 | 1364 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343 | 3493 | 2989 | 3159 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 424 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2112 | 1897 | 2957 | 2772 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 484 | 279 | 1339 | 1144 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 | 3907 | 4223 | 4408 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869 | 4074 | 4046 | 4241 |

Table 5.3.5.4-3 lists Band n20 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n8.

Table 5.3.5.4-3: Band n20 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 703 | 733 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 159 | 99 | 1535 | 1595 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 931 | 1021 | 544 | 634 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2367 | 2457 | 2238 | 2328 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1763 | 1883 | 1247 | 1367 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3199 | 3319 | 2941 | 3061 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 198 | 318 | 3070 | 3190 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2100 | 1950 | 2745 | 2595 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 535 | 385 | 1180 | 1030 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3644 | 3794 | 4031 | 4181 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3773 | 3923 | 3902 | 4052 |

Based on Table 5.3.5.4-1, 3rd and 5th order IMD may also fall into Rx frequencies of third DL band n28 when CA_n8-n20 is the UL configuration.

Based on Table 5.3.5.4-2, there is no IMD interference which may also fall into Rx frequencies of third DL band n20 when CA_n8-n28 is the UL configuration.

Based on Table 5.3.5.4-3, 3rd order IMD may also fall into Rx frequencies of third DL band n8 when CA_n20-n28 is the UL configuration.

5.3.5.5	REFSENS requirements

TBD.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-10 | RAN4#104be | R4-2217712 |  |  |  | TR skeleton | 0.1.0 |
| 2022-10 | RAN4#104be | R4-2217713 |  |  |  | TP and discussion on CA_n5-n28 | 0.2.0 |
| 2022-10 | RAN4#104be | R4-2217714 |  |  |  | TP and discussion on CA_n8-n20-n28 | 0.2.0 |
| 2022-10 | RAN4#104be | R4-2217715 |  |  |  | TP and discussion on CA_n5-n8 | 0.2.0 |