3GPP TSG-RAN WG4 #105	R4-2218047

November 14th ‒ 18th, 2022

Face to face and electronic Meeting

Agenda item:	7.27.2

Source: 	Spark NZ

Title: 	UE RF parameters for n 105

Document for:	Approval

# Introduction

A WF on UE RF parameters for n 105 was discussed during RAN 104ebis meeting [1].   It was proposed that :

- For bandwidths 20 MHz and greater, two options of RFSENS are being considered:

  - One option is to keep them same as n 71

  - A second option is to improve them relative to n 71.

RFSENS values for 20 MHz and wider bandwidths

This contribution provides comments and suggestions on 1(a) and 1(b) above.

For channel bandwidths 20 MHz or more, it is proposed in the WF to keep the same RFSENS values as n 71 or improve them.

Table 7.3.2-1a in [2] gives the  two antenna port reference sensitivity QPSK PRFSENS for FDD bands for n 71 as below:

| Band | SCS KHz | 20 MHz bandwidth | 25 MHz bandwidth | 30 MHz bandwidth | 35 MHz bandwidth |
| --- | --- | --- | --- | --- | --- |
| n71 | 15 | -86.0 dBm | -84.1 dBm | -82.5 dBm | -80.7 dBm |
|  | 20 | -87.4 dBm | -84.2 dBm | -82.6 dBm | -80.8 dBm |

It is observed that the above numbers do not bear any resemblance with the corresponding numbers in options 2-1 and 2-2 in [1]. It is not clear how the values in the tables for options 2-1 and 2-2  are derived?

It is proposed that RFSENS for all channel bandwidths for n 105 should be the same as that for n 71 as shown in Table 7.3.2-1a [2] (now reproduced below for all operating bandwidths).

| Band | SCS KHz | 5 MHz bandwidth | 10 MHz bandwidth | 15 MHz bandwidth | 20 MHz bandwidth | 25 MHz bandwidth | 30 MHz bandwidth | 35 MHz bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n 105 | 15 | -97.2 dBm | -94.0 dBm | -91.6 dBm | -86.0 dBm | -84.1 dBm | -82.5 dBm | -80.7 dBm |
|  | 20 |  | -94.3 dBm | -91.9 dBm | -87.4 dBm | -84.2 dBm | -82.6 dBm | -80.8 dBm |

Table 1: Proposed RFSENS values for n 105

# Proposals

- The RFSENS values shown in Table 1 above should be adopted for n 105.

# References

[1] R4- 2217806 WF on UE  RF requirements

[2] 3GPP TS 38.101-1 v 17.5.0