3GPP TSG-RAN WG4 Meeting #105	R4-2218048

Electronic Meeting, 14 Nov. – 18 Nov. 2022

Agenda Item:	7.28.3

Source:	Charter Communications, Inc.

Title:	Simulation results on UE RF MPR and A-MPR for PC3

Document for:	Approval

# Introduction

In RAN4#104-e meeting, a WF for introducing Power class 3 (PC3) in NR-unlicensed spectrum was approved [1]. In RAN4#104-bis-e meeting, a WF with a few agreements and some way forward from initial results by companies [2]. In this paper we will provide simulation results for MPR and A-MPR for NS_30, NS_53, and NS_54. The paper will start off recapping the requirements for all three NS values.

Tentative agreement tables for NS_53 and for NS_54 respectively. Both are with simulations of 1Tx by most companies [2].

The way forward for NS_54 following points:

- Evaluate A-MPR for 2Tx to understand if one or two A-MPR tables are needed to be specified.

  - Merging two non-equal tables are not precluded.

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | QPSK | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 16 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 64 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 256 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
| CP-OFDM | QPSK | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 16 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 64 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [5.5] | ≤ [8.5] | ≤ [4.5] | ≤ [7.5] |
|  | 256 QAM | ≤ [12.0] | ≤ [14.5] | ≤ [9.0] | ≤ [11.5] | ≤ [7.0] | ≤ [10.0] | ≤ [6.5] | ≤ [8.5] | ≤ [6.5] | ≤ [7.5] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

The way forward for NS_54 contained following discussion points:

- Discuss what values to add for PI/2 BPSK.

- Discuss if value for CP-OFDM-64QAM should be 5.0, as the average from the two papers.

- Discuss why (any) backoff is needed for allocations away from the edges.

- Evaluate and discuss if a dedicated 2Tx A-MPR tables is needed.

  - Merging two non-equal tables are not precluded.

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | QPSK |  | ≤ [3.0] | ≤ [5.0] |
|  | 16 QAM |  | ≤ [3.5] | ≤ [5.0] |
|  | 64 QAM |  | ≤ [3.5] | ≤ [5.0] |
|  | 256 QAM |  | ≤ [5.0] | ≤ [6.0] |
| CP-OFDM | QPSK |  | ≤ [4.5] | ≤ [6.0] |
|  | 16 QAM |  | ≤ [5.0] | ≤ [6.0] |
|  | 64 QAM |  | ≤ [5.5] | ≤ [6.0] |
|  | 256 QAM |  | ≤ [6.5] | ≤ [7.0] |
| NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz], and 100 MHz channels centered at the nearest NR-ARFCN corresponding to [5995 MHz]. |  |  |  |  |

# Discussion

## PC3 MPR

Simulations were done for unlicensed spectrum with PC3. The PA was calibrated with the agreed QPSK DFT-s-OFDM waveform. The detailed list of the setup is provided in the following:

- Calibration: QPSK DFT-s-OFDM 100RB with 1dB backoff

- ACLR: 27dB

- SEM: NR-U

- Image rejection: -28dB

- Carrier leakage rejection: -28dBc

Table 1: Setup for MPR

| ID | Modulation | Waveform | Allocation |
| --- | --- | --- | --- |
| 1 | Pi/2 BPSK | DFT-s-OFDM | Full |
| 2 | Pi/2 BPSK | DFT-s-OFDM | Interlaced |
| 3 | QPSK | DFT-s-OFDM | Full |
| 4 | QPSK | CP-OFDM | Full |
| 5 | QPSK | DFT-s-OFDM | Interlaced |
| 6 | QPSK | CP-OFDM | Interlaced |
| 7 | 16QAM | DFT-s-OFDM | Full |
| 8 | 16QAM | CP-OFDM | Full |
| 9 | 16QAM | DFT-s-OFDM | Interlaced |
| 10 | 16QAM | CP-OFDM | Interlaced |
| 11 | 64QAM | DFT-s-OFDM | Full |
| 12 | 64QAM | CP-OFDM | Full |
| 13 | 64QAM | DFT-s-OFDM | Interlaced |
| 14 | 64QAM | CP-OFDM | Interlaced |
| 15 | 256QAM | DFT-s-OFDM | Full |
| 16 | 256QAM | CP-OFDM | Full |
| 17 | 256QAM | DFT-s-OFDM | Interlaced |
| 18 | 256QAM | CP-OFDM | Interlaced |

## MPR Simulation Results

Figure 1: Simulation results for MPR PC3 with 27dB ACLR

Table 2: MPR for PC3 with 27dB ACLR

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 0.5 | ≤ 1.0 |
|  | QPSK | ≤ 1.0 | ≤ 1.5 |
|  | 16 QAM | ≤ 1.5 | ≤ 2.0 |
|  | 64 QAM | ≤ 2.5 | ≤ 3.0 |
|  | 256 QAM | ≤ 4.5 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 2.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 2.5 | ≤ 3.0 |
|  | 64 QAM | ≤ 4.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Proposal 1: Proposed MPR PC3 simulation results presented in Table 2 for shared spectrum.

## PC3 A-MPR

The networking signaling value NS_30 is used when operating in band n46, while the networking signaling values NS_53 and NS_54 is used when operating in band n96. Agreement that Power class 3 will be introduced for both of these unlicensed bands, as well as band n102.

## In-Band emissions

There are additional requirements for the transmit power density as seen in Table 6.2F.1-2 in TS 38.101-1 [3] for all unlicensed bands. A part of Table 6.2F.1-2 is shared below.

Approximately, for a full allocation in a 20MHz channel bandwidth, with maximum Power for PC3 (23dBm), the maximum mean power density (dBm/MHz) would be around 10, this means the in-band emission requirements for NS_53, will be very much leading the simulation results, while for NS_30 and NS_54 will probably not be restrictive/stringent requirement. The wider bandwidth is simulated, the more the power density is spread out in the channel bandwidth.

Table 6.2F.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | NS_30 | 20, 40, 60, 80 | 5150 – 5350 | 11 |
|  |  |  | 5470 – 5725 |  |
| n96 | NS_53 | 20, 40, 60, 80 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |

## Out-of-Band emissions

When "NS_30" is indicated in the cell, the power of any UE emission for channels assigned within 5150-5350 MHz, 5470-5725 MHz and 5725-5850 MHz shall not exceed the levels specified in Table 6.5F.3.3.3-1-1, Table 6.5F.3.3.3-1-2 and Table 6.5F.3.3.3-1-3, respectively.

Table 6.5F.3.3.3-1: Additional requirements for shared access channels assigned within 5150-5350 MHz

| Protected range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| 4500 ≤ f ≤ 5150 | -41 | 1 MHz |
| 5350 ≤ f ≤ 5460 | -41 |  |

Table 6.5F.3.3.3-2: Additional requirements for shared access channels assigned within 5470-5725 MHz

| Protected range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| 4500 ≤ f ≤ 5150 | -41 | 1 MHz |
| 5350 ≤ f ≤ 5460 | -41 |  |
| 5460 < f ≤ 5470 | -27 |  |
| 5725 ≤ f | -27 |  |

Table 6.5F.3.3.3-3: Additional requirements for shared access channels assigned within 5725-5850 MHz

| Protected range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80, [100] MHz |  |
| f < 5650 | -27 | 1 MHz |
| 5650 ≤ f < 5700 | -27 to 10 |  |
| 5700 ≤ f < 5720 | 10 to 15.6 |  |
| 5720 < f ≤ 5725 | 15.6 to 27 |  |
| 5850 ≤ f ≤ 5855 | 27 to 15.6 |  |
| 5855 < f ≤ 5875 | 15.6 to 10 |  |
| 5875 < f ≤ 5925 | 10 to -27 |  |
| 5925 < f | -27 |  |
| NOTE: The minimum requirement when specified as a range denotes the emission requirement at the end points of the protected range. The requirement within the protected range is obtained by linear interpolation between the requirements at the end points. |  |  |

Additional requirements for the spurious emission limit for operations using the NS_53 and NS_54 the TS 38.101-1 [3] says; “When "NS_53" or "NS_54" or "NS_60" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5F.3.3.5-1.”

Table 6.5F.3.3.5-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

Based on the defined NR-ARFCN number for band n96 (Table 5.4.2.3-3 in TS 38.101-1 [3]) , the lowest number for each bandwidth using the formula

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

This gives center frequencies for each edge channel depending of bandwidth of the lower edge of band n96:

- 20MHz Bandwidth: 5955MHz

- 40MHz Bandwidth: 5965.02MHz

- 60MHz Bandwidth: 5974.98MHz

- 80MHz Bandwidth: 5985MHz

- 100MHz Bandwidth: 5995.02MHz

For band n96, the additional requirements are offset from the channel bandwidth edge by 20MHz. Also, there is only the first and perhaps also the second edge channels of each bandwidth that will be affected by the additional band specific requirements. All other channels will follow the MPR backoff limits.

For band n46, simulations are done with channels centered based on the two Notes in the A-MPR table for power class 5 for NS_30, Table 6.2F.3.4-1 in TS 38.101-1 [3]. The notes are:

NOTE 2:	Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz.

NOTE 3:	Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5230 and 5270 MHz.

This will be the affected channels by the additional requirements. All other channels will use the MPR table for maximum backoff.

## Simulation agreements

From the Way Forward [1], simulation agreements were

### Way Forward on PC3 A-MPR for 1Tx:

- PC3 calibration point: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB0 waveform at 30dB ACLR

- Post PA losses 4dB

Impairment assumptions

- CIM3: 60dBc

- CIM5: 70dBc

- IQ image: -28 dB

- Carrier leakage: -28dBc

## A-MPR Simulation Results

Charter’s simulation results for NS_54 with Power Class 3, see Table 3. This simulation is with 1Tx antenna.

Table 3: A-MPR for NS_54 power class 3

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK | See Table PC3 MPR | ≤ 2.0 | ≤ 4.5 |
|  | QPSK |  | ≤ 3.0 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 5.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 6.5 | ≤ 7.0 |
| NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 and 5995 MHz, 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz, [and 100 MHz channels centered at the nearest NR-ARFCN corresponding to 5995 MHz.] |  |  |  |  |

For NOTE 2, all the other valid channels and bandwidths other than those enumerated in NOTE 3 will follow the MPR table for Power class 3 for shared spectrum.

From the WF [2], there are some open issues for NS_54. In the Table 3, we have added our simulation results for PI/2 BPSK. We are fine to define CP-OFDM 64QAM full to 5.0dB, which is the average value between our results and Apple’s results from the last meeting.

According to the TR 37.890 [4], the Standard Power (SP) the Client devices is allowed the maximum EIRP of 30dBm and maximum EIRP PSD of 17dBm/MHz. This means SP may handle higher Power classes. However, the WID has only stated to introduce PC3. Hence, we would like to FFS higher Power classes (e.g., PC2 and PC1.5) for SP.

Observation 1: SP may hold Power Classes PC2 (26dBm) and PC1.5 (29dBm), but the agreement in the WID is to only introduce PC3.

Proposal 2: FFS higher Power classes e.g., PC2 and PC1.5 for SP.

Proposal 3: Proposed A-MPR PC3 simulation results presented in Table 3 for NS_54.

Charter’s simulation results for NS_53 with Power Class 3, see Table 4. This simulation is with 1Tx antenna.

Table 4: A-MPR for NS_53 power class 3

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 9.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | QPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 9.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 9.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 9.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 9.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 9.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 9.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 9.5 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 9.5 | ≤ 6.5 | ≤ 8.5 | ≤ 6.5 | ≤ 7.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

The driving requirement for NS_53 is the maximum mean power density (dBm/MHz) at -1dBm/MHz to set the A-MPR.  Our simulation results are very much aligning with Apple simulation results from the last meeting [5]. Only difference is our results for 60MHz bandwidth with partial allocation.

Proposal 4: Proposed A-MPR PC3 simulation results presented in Table 4 for NS_53.

Charter’s simulation results for NS_30 with Power Class 3, see Table 5. This simulation is with 1Tx antenna.

Table 5: A-MPR for NS_30 power class 3

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-OFDM | PI/2 BPSK | ≤ 12.0 | ≤ 15.5 | ≤ 3.5 | ≤ 5.5 | See Table MPR table for PC3 |
|  | QPSK | ≤ 12.5 | ≤ 16.0 | ≤ 4.0 | ≤ 6.5 |  |
|  | 16 QAM | ≤ 12.5 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 13.0 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 13.0 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
| CP-OFDM | QPSK | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 16 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 256 QAMr | ≤ 14.0 | ≤ 16.5 | ≤ 6.5 | ≤ 7.5 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5230 and 5270 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 2 and NOTE 3. |  |  |  |  |  |  |

Proposal 5: Proposed A-MPR PC3 simulation results presented in Table 5 for NS_30.

# Conclusion

In this paper, Charter has the following observations and proposals:

Observation 1: SP may hold Power Classes PC2 (26dBm) and PC1.5 (29dBm), but the agreement in the WID is to only introduce PC3.

Proposal 1: Proposed MPR PC3 simulation results presented in Table 2 for PC3 MPR for shared spectrum.

Proposal 2: FFS higher Power classes e.g., PC2 and PC1.5 for SP.

Proposal 3: Proposed A-MPR PC3 simulation results presented in Table 3 for NS_54.

Proposal 4: Proposed A-MPR PC3 simulation results presented in Table 4 for NS_53.

Proposal 5: Proposed A-MPR PC3 simulation results presented in Table 5 for NS_30.

# Reference

[1] R4-2214437, WF on NR-U PC3 requirements, Charter Communications Inc.

[2] R4-2217131, WF on NR-U PC3 requirements and MPR/A-MPR, Charter Communications Inc.

[3] TS 38.101-1: User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V17.6.0 (2022-06)

[4] TR 37.890, Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations V0.16.0 (2022-06)

[5] R4-2215651, NR-U MPR and A-MPR for PC3, Apple Inc.