3GPP TSG-RAN WG4 Meeting #105											         R4-2218159

Toulouse, France, November 14 – 18, 2022

Agenda item:	8.22.2.4

Source:	Apple

Title:	On R18 MUSIM - network B requirements

Document for:	Discussion

# Introduction

NW B requirements were initially discussed in RAN4#104e-bis. The latest agreements can be found in [1]. In this contribution, we will further discuss the open issues.

# Discussion

The summary of NW B requirements related agreement are duplicated here:

Issue 1-6-1: Whether to define network B requirements - Proposals: - Option 1: Deprioritize NW B requirement in R18 - Option 2: No measurement requirements in network B will be defined by RAN4 - Option 3: RAN4 does not define new UE idle/inactive measurement requirements for measurements on Network B for a UE configured with MUSIM gaps (exiting UE measurement requirement for Idle/Inactive mode applies) - Option 4: RAN4 to define measurement requirement for NW-B Idle mode which is helpful for both NW-A and NW-B Agreements: No Issue 1-6-2: Network B requirements if it will be defined - Proposals: - P1: RAN4 has to define some requirement for NW B in R18, RAN4 shall focus on cell reselection - P2: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP) - P3: UE measurement requirements for idle/inactive mode in network B shall be the same as existing Idle/Inactive measurement requirements. - UE measurement requirement for Idle/Inactive mode on network B, when assigned with requested MUSIM gaps, are the same as for network A. Hence, same as legacy requirements Agreements: Postpone discussion until there is outcome of issue 1-6-1

MUSIM gaps can be used for RRM measurement, paging reception, system information reception and so on. In general, we are fine with deprioritizing network B requirements. if necessary, RAN4 shall focus on idle/inactive mode cell reselection/measurement requirements.

Some companies proposed to reuse network A idle/inactive mode requirements. however, we believe existing requirements cannot be directly reused. Some additional delay can be expected due to gap collision and cancellation. MUSIM gap for NW B RRM measurement sometimes will be dropped due to collision with NW A gap or other DL/UL signal. This is similar to LBT failure in NR-U design.

Here is an example of serving cell measurement requirements:

Proposal 1: RAN4 shall deprioritize NW B requirements. If necessary, only idle/inactive mode measurement/cell reselection requirements need to be considered.

Proposal 2: framework of idle/inactive mode RRM requirements for NR-U can be used as starting point to accommodate MUSIM gap cancellation. Take serving cell measurement as an example:

# Conclusion

In this contribution, we provide further discussion on NW B requirements. after discussion the following conclusions are provided:

Proposal 1: RAN4 shall deprioritize NW B requirements. If necessary, only idle/inactive mode measurement/cell reselection requirements need to be considered.

Proposal 2: framework of idle/inactive mode RRM requirements for NR-U can be used as starting point to accommodate MUSIM gap cancellation. Take serving cell measurement as an example:

# References

- R4-2217261 WF on RRM requirements for Rel-17 MUSIM gaps, vivo