3GPP TSG-RAN WG4 Meeting # 105		R4-2218178

Toulouse, France, November 14 – November 18, 2022

Agenda item:	6.2.6.3.1

Source:	Apple

Title:	Discussion on PDSCH demod requirements for NTN

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-bis-e UE demodulation requirements for NTN were discussed and way forward [1] was agreed. In this contribution we present our views on the open issues and simulation results for requirements.

# 2. Discussion

In RAN4#104-e many open issues were resolved, and agreements reached as captured in [1]:

General Aspects Channel model for NTN-TDLC (LOS) - DS=5ns, Doppler=200Hz. K-factor=8.05dB. Delay resolution - Adopt 5 ns as the delay resolution Delay profile for NTN-TDLA Tap #; Delay [ns]; Power [dB]; Fading distribution. 1; 0; 0; Rayleigh. 2; 110; -4.7; Rayleigh. 3; 285; -6.5; Rayleigh. Delay profile for NTN-TDLC Tap #; Delay; Power in [dB]; Fading distribution. 1; 0; -0.6; LOS path. 0; -8.9; Rayleigh. 2; 60; -21.5; Rayleigh. PDSCH Demod K_offset value - Confirm K_offset equal to 8 slots for 15kHz SCS for all the HARQ configurations Doppler shift for LOS Path in the NTN TDL-C Channel - The same scaling factor as the TN channel models, i.e., (0.7*fD) with AoA of 45 degree is assumed for UE side in the simulation. No need to explicitly reflect this agreement in the TS 38101-5. Modulation order - Do not define PDSCH performance requirements for 64QAM SCS/CBW set - De-prioritize requirements with 30KHz /20MHz SCS/CBW set in Rel-17 and propose to revisit in later release based on demand and deployment plans.

In addition, we also agreed on the HARQ configuration for different MCS, propagation channel as below:

| Prop. Channel | MCS | HARQ Config |
| --- | --- | --- |
| NTN-TDLA100-200 | MCS4 | Disabled HARQ |
|  | MCS4 | 16 HARQ Proc |
| NTN-TDLC5-200 | MCS4 | 32 HARQ Proc |
|  | MCS13 | 16 HARQ Proc |

The open issues related to PDSCH demod requirements are:

Issue 2-6: Testability of the disabled HARQ scenario

- Proposals

  - Option 1(Ericsson, Qualcomm, Apple, Nokia): 4 disabled, 12 enabled in 16 HARQ processes. The peak throughput with scaling 3/4 can be used as the metric for this test.

  - Option 2 (Huawei): Keep previous agreements that all HARQ feedback are disabled with 16 HARQ processes. How to perform test will up to RAN5 discussion.

The motivation for introducing requirements with disabled HARQ feature is to test UE handling or receiver processing with disabled HARQ for some of the HARQ processes. Based on the design of this feature HARQ is disabled for some of the features not all. Even in actual deployment HARQ will not be disabled for all processes.

Observation #1: With disabled HARQ feature, HARQ is disabled for some of the HARQ processes, not all.

With HARQ re-transmission disabled on the HARQ processes with HARQ enabled, the performance/ throughput measured on HARQ processes with enabled HARQ approximates to the performance of disabled HARQ.  With some of the HARQ processes disabled, the test case will verify functionality and evaluate performance with disabled HARQ feature.

Observation #2: With some processes with disabled HARQ and some with HAR-ACK enabled with no re-transmissions, the functionality and performance will be verified with disabled HARQ feature.

Hence, we propose to introduce requirements with 4 disabled and 12 enabled HARQ processes. In the test throughput shall only be measured on processes with HARQ-ACK feedback.

Proposal #1: Define requirements with 4 disabled, 12 enabled in 16 HARQ processes for disabled HARQ test cases. Throughput shall be measured on processes with HARQ enabled.

Based on the agreed simulation parameters in [1] we provide simulation results for PDSCH demod in the table below.

Table 1: Simulation results for PDSCH demod for NTN

| Prop. Channel | MCS | HARQ Config | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- |
|  |  |  | Alignment Results | Impairment Results |
| NTN-TDLA100-200 | MCS4 | Disabled HARQ | -0.7 | 1.3 |
|  | MCS4 | 16 HARQ Proc | -1.4 | 0.6 |
| NTN-TDLC5-200 | MCS4 | 32 HARQ Proc | -3.0 | -1.0 |
|  | MCS13 | 16 HARQ Proc | 5.4 | 7.4 |

# 3. Conclusion

In this paper, we provide our views NTN UE demod requirements and simulation results for requirements definiton. Our observations and proposals are captured below:

Observation #1: With disabled HARQ feature, HARQ is disabled for some of the HARQ processes, not all.

Observation #2: With some processes with disabled HARQ and some with HAR-ACK enabled with no re-transmissions, the functionality and performance will be verified with disabled HARQ feature.

Proposal #1: Define requirements with 4 disabled, 12 enabled in 16 HARQ processes for disabled HARQ test cases. Throughput shall be measured on processes with HARQ enabled.

# Reference

- R4-2217344, “WF for NTN demodulation requirements - general and PDSCH”, Qualcomm Incorporated.