3GPP TSG-RAN WG4 Meeting # 105		R4-2218181

Toulouse, France, November 14 – November 18, 2022

Agenda item:	6.3.6.2.2

Source:	Apple

Title:	On PDCCH and PBCH demod requirements for 52.6 - 71 GHz

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-bis-e UE demodulation requirements for 52.6 - 71 GHz was discussed and way forward [1] was agreed. In this contribution we present simulation results for alignment for PDCCH and PBCH demodulation requirements.

# 2. PDCCH Demod

In [1] we made several agreements on PDCCH demodulation requirements. The simulation assumptions for PDCCH demod requirements were agreed. Based on the agreed simulation parameters, we present results for PDCCH performance in FR2-2.

Table 1: Simulation assumptions for PDCCH demod in FR2-2

| Test number | CBW/SCS | CORESET duration | CORESET RB | Intl. size | REG bundle size | Agg level | Prop. Condition | DCI format | Payload size | Ant. Config and corr Mtx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA30-200 | 1_0 | 40 | 1x2 Low |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA10-650 | 1_1 | 56 | 1x2 Low |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA10-200 | 1_1 | 56 | 2x2 Low |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | TDLA30-650 | 1_0 | 40 | 2x2 Low |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 7 | 400/480 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low |

Table 2: Simulation results for PDCCH demod in FR2-2

| Test number | CBW/SCS | Prop. Condition | Ant. Config and corr Mtx | 1% Pm-dsg |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Alignment | Impairment |
| 1 | 100/120 | TDLA30-200 | 1x2 Low | 5.5 | 7.6 |
| 2 | 100/120 | TDLA10-650 | 1x2 Low | 1.1 | 3.2 |
| 3 | 100/120 | TDLA10-200 | 2x2 Low | -1.1 | 0.9 |
| 4 | 100/120 | TDLA30-650 | 2x2 Low | -4.4 | -2.4 |
| 5 | 400/480 | TDLA10-200 | 1x2 Low | 1.1 | 3.1 |
| 6 | 400/480 | TDLA10-200 | 1x2 Low | -1.8 | 0.2 |
| 7 | 400/480 | TDLA10-200 | 2x2 Low | -4.3 | -2.3 |

# 2. PBCH Demod

For PBCH demod requirements, the channel model for requirements with 120Khz SCS is FFS:

Channel Model for FR2-2 PBCH Requirements Test 1-1 (120kHz SCS);

- Proposals

  - Option 1: TDLA30-200;

  - Option 3: TDLA30-650;

Based on the agreed simulation parameters, we provide simulation results for PBCH for FR2-2.

Table 3: Simulation results for PBCH demod in FR2-2

| Test number | SCS/CBW | Antenna configuration | Propagation condition | SS/PBCH block index | PBCH SNR (dB) @ 1% Pm-bch |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | TDLA30-200 | unknown | -7.5 |
|  |  |  | TDLA30-650 | unknown | -7.5 |
| 1-2 | 480kHz/400MHz |  | TDLA10-200 | unknown | -7.5 |

Based on our results, we find that propagation channel doesn’t impact the performance significantly for PBCH.  We propose to define requirements with TDLA30-200 for PBCH demod.

Proposal #1 Introduce PBCH demod requirements with 120KHz SCS with TDLA30-200 propagation channel.

# Conclusion

In this paper we present simulation results for PDCCH and PBCH demodulation requirements for 52.6-71GHz. Our observations and proposals are captured below:

Proposal #1 Introduce PBCH demod requirements with 120KHz SCS with TDLA30-200 propagation channel.

# Reference

- R4-2217391, “WF on FR2-2 UE demodulation requirements”, Qualcomm Incorporated.