3GPP TSG-RAN WG4 Meeting # 105		R4-2218189

Toulouse, France, November 14 – November 18, 2022

Agenda item:	6.5.3.1.1

Source:	Apple

Title:	Simulation results for demod requirements with HST-SFN Scheme-A

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-bis-e demodulation and CSI requirements FeMIMO was discussed and way forward [1] was agreed. In this contribution we present simulation results for requirements with HST Scheme-A.

# 2. Discussion

Based on the agreed simulation assumptions captured in Table 1, we present simulation results for PDSCH demod for HST Scheme-A.

Table 1: Simulation Assumptions for HST Scheme-A

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | FDD | TDD |  |
| SCS/CBW | 15 kHz /10 MHz | 30 kHz /40 MHz |  |
| Antenna configuration | 2x2; 2x4 |  |  |
| Max Doppler Shift | 870 Hz |  | 1667 Hz |
| Rank | 2 |  |  |
| MCS | 13 |  |  |
| DMRS type | Type 1 |  |  |
| Number of DMRS symbols | 1+1+1 |  |  |
| TDD pattern | N/A | 7D1S2U, S: 6D 4G 4U |  |
| TRS configuration | 10ms, 2 slot pattern |  |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |  |

Table 2: Simulation results for HST Scheme-A

|  | Ant Config | SNR @ 70% Max TP |  |
| --- | --- | --- | --- |
|  |  | Alignment Result | Impairment Result |
| FDD | 2x2 | 9.8 | 11.8 |
|  | 2x4 | 7.0 | 9.0 |
| TDD | 2x2 | 10.5 | 12.5 |
|  | 2x4 | 7.8 | 9.8 |

# 3. Conclusion

In this paper, we provide simulation results for requirements for PDSCH with HST Scheme-A.

# Reference

- R4-2217440, “WF on UE demodulation and CSI requirements for FeMIMO”, Samsung.