3GPP TSG-RAN WG4 Meeting #105	R4-2218284

Toulouse, France, November 14 – November 18, 2022

Agenda item:	11.3

Source:            	Google Inc., CableLabs, Federated Wireless

Title:                	Discussion on Intra-Band EN-DC band combinations

Document for:	Approval

# Introduction

In RAN4#104bis-e meeting, due to RAN tasking to RAN4, two configurations Case 3 and Case 4 for intra-band EN-DC band combinations have been discussed. Case 3 is the intra-band contiguous DL EN-DC configuration with intra-band non-contiguous UL EN-DC configuration, and Case 4 is the intra-band non-contiguous DL EN-DC configuration which LTE and NR adjacent carriers are contiguous with intra-band contiguous UL EN-DC configuration and intra-band non-contiguous UL EN-DC configuration.

- Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)41AB DC_(n)41CA DC_(n)41DA | DC_41A_n41A |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |

- Case 4: LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, it will has two kinds of UL ENDC configurations:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

    - DL DC_48A_(n)48AA with UL DC_(n)48AA

    - DL DC_48A_(n)48AA with UL DC_48A_n48A

The WF has been approved by RAN4 to keep b48+n48 for case 3 and case 4 as valid configurations and to remove case 3 for b41+n41. Besides, RAN4 also agree to seek the solution for case 3 and case 4.

Agreement:

- Keep b48+n48 in the RAN4 specification as it is

  - RAN4 seeks the solution to make such band combination workable. Depending on the solution, RAN4 decide how to handle this band combination in the specification.

- It is agreed that Case 3 for b41+n41 to be removed

For Case 3, it is agreed in the WF to indicate Case 3 configuration by existing signaling before Rel-18 and considering introduce a new signaling in Rel-18. Two options and interpretations are captured in the WF but some companies still have different view for Case 3 solution.

### Issue 1-3-1: Solutions for Case 3

WF are as below:

- Consider design new signalling in Rel-18 for Case3 if necessary and detailed signalling design is up to RAN2. And rely existing signalling to indicate Case 3 before Rel-18.

- FFS following solutions in next meeting

  - Option 2b: In Rel-16 and Rel-17, report an additional band combination DC_48A_n48A in both UL and DL to support the Case 3 configurations DL DC_(n)48CA with UL DC_48A_n48A and DL DC_(n)48DA with UL DC_48A_n48A, i.e.

UE indicate “contiguous” capability for DL DC_(n)48CA with UL DC_(n)48AA

UE additionally indicate “non-contiguous” capability for DC_48A_n48A in both UL and DL

    - Interpretation A:

Case 3 is considered as exceptional configuration with DC_48A_n48A in UL and DC_(n)48CA in DL, and a NOTE can be added in RAN4 spec to clarify this.

    - Interpretation B:

DL DC_48A_n48A is not considered as a fallback combination of DC_(n)48CA, but only a lower order combination of DC_(n)48CA.

  - Option 2c: In Rel-16 and Rel-17, UE shall also support non-contiguous operation in the DL (DC_48C_n48A), then the network can configure DL_(n)48CA with the middle LTE cell DL-only and the UL with a gap (non-contiguous)

For Case4, three options which include using existing signaling in Rel-16 and Re-17 and introducing a new signaling in Rel-18 are captured in the WF. However, for the solutions in Rel-16 and Rel-17, there are still different views on how to interpret the mapping between intraBandENDC-Support={contiguous, non-contiguous, both} and case 4 configuration, and different views on whether to introduce a new signaling in Rel-18.

### Issue 1-3-2: Solutions for Case 4

WF are as below:

- FFS following solutions in next meeting

- Option 2b: Rel-16 and 17 combinations of contiguous and non-contiguous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous EN-DC configuration in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contiguous and non-contiguous EN-DC in the UL, i.e.

  - UE indicate “both” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A

- Option 2c: In Rel-16 and Rel-17, report an additional band combination DC_48A_n48A in both UL and DL to support the Case 4 configuration DL DC_48A_(n)48AA with UL DC_48A_n48A and DC_(n)48AA with UL DC_48A_n48A, i.e.

  - UE indicate “contiguous” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA

  - UE additionally indicate “non-contiguous” capability for DC_48A_n48A in both UL and DL

- Option 3: New signalling

  - A solution is necessary in RAN2 to address the ambiguity issue for configurations on some intra-band EN-DC band combinations with more than 2 carriers from Rel-15.

In this paper, we would like to share our view in the following.

# Discussion

Solution for Case 3

In Table 1, we summarize all low order configurations for Case 3 configurations DL DC_(n)48CA/DC_(n)48DA with UL DC_(n)48AA and UL DC_48A_n48A. According to the fallback band combination definition in TS38.306, it is defined that “An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination”. Hence, all low order configurations for Case 3 can be divided into two categories which are fallback low order configurations and non-fallback low order configurations. The fallback low order configurations are captured as Note-1 in Table 1 and the non-fallback low order configurations are captured as Note-2, Note3 and Note-4.

Fallback band combination: A Uu band combination that would result from another Uu band combination (parent band combination) by releasing at least one SCell or uplink configuration of SCell, or SCG, or SUL. A PC5 band combination that would result from another PC5 band combination (parent band combination) by releasing at least one sidelink carrier. An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination. A fallback band combination supports the same channel bandwidth(s) for each carrier as its parent band combination(s).

For fallback low order chain with Note-1, the UE which indicates [LTE:CA_48D, NR:n48A, intraBandENDC-Support=not reported(contiguous)] to the network can support all the configurations including DL DC_(n)48DA with UL DC_(n)48AA, the first low order configuration DL DC_(n)48CA with UL DC_(n)48AA and the second low order configuration DL DC_(n)48AA with UL DC_(n)48AA. Similarly, the UE which indicates [LTE:CA_48C, NR:n48A, intraBandENDC-Support=not reported(contiguous)] to the network can support all the configurations including DL DC_(n)48CA with UL DC_(n)48AA and the first low order configuration DL DC_(n)48AA with UL DC_(n)48AA.

For non-fallback low order chain with Note-2, the UE which indicates [LTE CA_48D, NR n48A, intraBandENDC-Support=not reported(contiguous)] to the network does not imply that the UE supports the configurations DL DC_(n)48DA with UL DC_48A_n48A, the first low order configuration DL DC_(n)48CA with UL DC_48A_n48A and the second low order DL DC_48A_n48A with UL DC_48A_n48A. The UE should additionally indicate “DL DC_48A_n48A with UL DC_48A_n48A” such as [LTE:48A, NR:n48A, intraBandENDC-Support=non-contiguous] to enable these non-fallback configurations. Similarly, the UE which indicates [LTE:CA_48C, NR:n48A, intraBandENDC-Support=not reported(contiguous)] to the network does not imply that the UE supports the configurations DL DC_(n)48CA with UL DC_48A_n48A and the first low order configuration DL DC_48A_n48A with UL DC_48A_n48. The UE should additionally indicate “DL DC_48A_n48A with UL DC_48A_n48A” such as [LTE:48A, NR n48A, intraBandENDC-Support=non-contiguous] to enable these non-fallback configurations.

For non-fallback low order chain with Note-3, the UE which indicates [LTE:CA_48D, NR:n48A, intraBandENDC-Support=not reported(contiguous)] to the network does not imply that the UE supports the configurations DL DC_(n)48DA with UL DC_48A_n48A, the first low order configuration DL DC_48C_n48A with UL DC_48A_n48A and the second low order DL DC_48A_n48A with UL DC_48A_n48. The UE should additionally indicate both “DL DC_48C_n48A with UL DC_48A_n48A” such as [LTE:CA_48C, NR:n48A, intraBandENDC-Support=non-contiguous] and “DL DC_48A_n48A with UL DC_48A_n48A” such as [LTE:CA_48A, NR:n48A, intraBandENDC-Support=non-contiguous] to enable these non-fallback configurations.

For non-fallback low order chain with Note-4, the UE which indicates [LTE:CA_48D, NR:n48A, intraBandENDC-Support=not reported(contiguous)] to the network does not imply that the configurations DL DC_(n)48DA with UL DC_48A_n48A, the first low order configuration DL DC_48A_(n)48AA with UL DC_48A_n48A and the second low order DL DC_48A_n48A with UL DC_48A_n48. The UE should additionally indicate both “DL DC_48A_(n)48AA with UL DC_48A_n48A” such as [LTE:CA_48A-48A, NR:n48A, intraBandENDC-Support=non-contiguous or both (depending on Case 4 solution)] and “DL DC_48A_n48A with UL DC_48A_n48A” such as [LTE:48A, NR:n48A, intraBandENDC-Support=non-contiguous] to enable these non-fallback configurations.

In summary, to support Case 3 configuration, if the UE indicates [LTE:CA_48D, NR:n48A, intraBandENDC-Support=not reported(contiguous)] and additionally indicates “DL DC_48C_n48A with UL DC_48A_n48A” such as [LTE:CA_48C, NR:n48A, intraBandENDC-Support=non-contiguous], the configuration DL DC_(n)48DA with UL DC_48A_n48A, the first low order configuration DL DC_(n)48CA with UL DC_48A_n48A and the second low order DL DC_48A_n48A with UL DC_48A_n48A are all supported by the UE. As for other first lower configurations such as DL DC_48C_n48A with UL DC_48A_n48A and DC_48A_(n)48AA with UL DC_48A_n48A, it is not mandatory for the UE to support these two configurations without any corresponding additional signaling reporting.

Proposal 1: For Case 3, we propose the following solution

- In Rel-16 and Rel-17

  - Report an additional band combination “DL DC_48A_n48A with UL DC_48A_n48A” to support Case 3 configuration

  - For the configuration DL DC_(n)48DA with UL DC_48A_n48A, whether to support the first low order configurations DL DC_48C_n48A with UL DC_48A_n48A and/or DC_48A_(n)48AA with UL DC_48A_n48A should depend on the corresponding UE capability signaling reporting.

- In Rel-18

  - Whether introduce a new UE capability signaling to support the Case 3 configuration should follow the majority view in RAN4.

| Case 3 Configuration for b48+n48 (LTE:CA_48D/CA_48C, NR:n48A, intraBandENDC-Support=not reported(contiguous)) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DL EN-DC Configuration | UL EN-DC Configuration | First Low Order DL Configuration | First Low Order UL Configuration | Second Low Order DL Configuration | Second Low Order UL Configuration | Note |
| DC_(n)48DA | DC_(n)48AA | DC_(n)48CA | DC_(n)48AA | DC_(n)48AA | DC_(n)48AA | Note-1 |
| DC_(n)48DA | DC_48A_n48A | DC_(n)48CA | DC_48A_n48A | DC_48A_n48A | DC_48A_n48A | Note-2 |
|  |  | DC_48C_n48A | DC_48A_n48A | DC_48A_n48A | DC_48A_n48A | Note-3 |
|  |  | DC_48A_(n)48AA | DC_48A_n48A | DC_48A_n48A | DC_48A_n48A | Note-4 |
| DC_(n)48CA | DC_(n)48AA | DC_(n)48AA | DC_(n)48AA |  |  | Note-1 |
| DC_(n)48CA | DC_48A_n48A | DC_48A_n48A | DC_48A_n48A |  |  | Note-2 |
| Note-1: The low order configurations follow fallback band combination rule Note-2: The low order configurations does not follow fallback band combination rule and additional signaling reporting for DL DC_48A_n48A with UL DC_48A_n48A can be used to support these low order configurations Note-3: The low order configurations does not follow fallback band combination rule and additional signaling reporting for DL DC_48A_n48A with UL DC_48A_n48A and DL DC_48C_n48A with UL DC_48A_n48A can be used to support these low order configurations Note-4: The low order configurations does not follow fallback band combination rule and additional signaling reporting for DL DC_48A_n48A with UL DC_48A_n48A and DL DC_48A_(n)48AA with UL DC_48A_n48A can be used to support these low order configurations |  |  |  |  |  |  |

Table 1. Low order configurations for b48+n48 Case 3 configuration

Solutions for Case 4

In Table 2, we summarize how to interpret intraBandENDC-Support for Option 2b and Option 2c in Case 4 solutions when the UE indicates [LTE:CA_48A-48A, NR:n48A] to the network.

For Option 2b, the interpretation of intraBandENDC-Support is to follow 38.101-3 Table 5.3B.1.2-1, Table 5.3B.1.3-1 and Table 5.3B.1.3-2. There is no intra-band DL/UL EN-DC configuration mapping to intraBandENDC-Support=not reported(contiguous), so it is not allowed to indicate intraBandENDC-Support=not reported(contiguous). The configuration DL DC_48A-48A_n48A with UL DC_48A_n48A can be indicated by intraBandENDC-Support=non-contiguous, and the Case 4 configuration can be indicated by intraBandENDC-Support=both.

For Option 2c, the basic idea for the interpretation of intraBandENDC-Support is to use UL configuration to differentiate contiguous or non-contiguous for intra-band EN-DC band combination. Different from Option 2b, the configuration DL DC_48A-(n)48AA with UL DC_(n)48AA can be indicated by intraBandENDC-Support=not reported(contiguous), and  the configuration DL DC_48A-(n)48AA with UL DC_48A_n48A can be indicated by intraBandENDC-Support=non-contiguous. Both the configuration DL DC_48A-48A_n48A with UL DC_48A_n48A and Case 4 configuration can be indicated by intraBandENDC-Support=both. In addition, the exact Case 4 configuration can be indicated by intraBandENDC-Support=not reported(contiguous) and the additional signaling reporting “DL DC_48A_n48A with UL DC_48A_n48A” such as [LTE:48A, NR:n48A, intraBandENDC-Support=non-contiguous].

| Case 4 Configuration for b48+n48 (LTE:CA_48A-48A, NR:n48A) |  |  |  |
| --- | --- | --- | --- |
| Solution | intraBandENDC-Support | DL EN-DC Configuration | UL EN-DC Configuration |
| Option 2b | contiguous | Not allowed. | Not allowed. |
|  | non-contiguous | DC_48A-48A_n48A | DC_48A_n48A |
|  | both (Case 4) | DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |
| Option 2c | contiguous | DC_48A-(n)48AA | DC_(n)48AA |
|  | non-contiguous | DC_48A-(n)48AA | DC_48A_n48A |
|  | both | DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |
|  |  | DC_48A-48A_n48A | DC_48A_n48A |
|  | contiguous+additional signaling DL DC_48A_n48A with UL DC_48A_n48A (Case 4) | DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |
| Proposed Solution | contiguous | DC_48A-(n)48AA | DC_(n)48AA |
|  | non-contiguous | DC_48A-48A_n48A | DC_48A_n48A |
|  | both (Case 4) | DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

Table 2. intraBandENDC-Support for b48+n48 Case 4 configuration

In our view point, both Option 2b and Option 2c are workable solutions. However, in order to reduce the coupled relation  between intraBandENDC-Support and intra-band DL/UL EN-DC configuration, we propose the compromised solution that the intra-band DL/UL EN-DC configuration for intraBandENDC-Support=not reported(contiguous) in Option 2b can be replaced with Option 2c. Which means that the configuration DL DC_48A-(n)48AA with UL DC_(n)48AA can be indicated by intraBandENDC-Support=not reported(contiguous), and  the configuration DL DC_48A-48A_n48A with UL DC_48A_n48A can be indicated by intraBandENDC-Support=non-contiguous. The Case 4 configuration can be indicated by intraBandENDC-Support=both.

Proposal 2: For Case 4, we propose the following solution

- In Rel-16 and Rel-17

  - The configuration DL DC_48A-(n)48AA with UL DC_(n)48AA can be indicated by intraBandENDC-Support=not reported(contiguous)

  - The configuration DL DC_48A-48A_n48A with UL DC_48A_n48A can be indicated by intraBandENDC-Support=non-contiguous

  - The Case 4 configuration DL DC_48A-(n)48AA with UL DC_(n)48AA and DL DC_48A-(n)48AA with UL DC_48A_n48A can be indicated by intraBandENDC-Support=both

- In Rel-18

  - Whether introduce a new UE capability signaling to support the Case 4 configurations depends on Case 3 solution in Rel-18.

# Conclusion

The observations and proposals in this contribution are summarized in the following.

Proposal 1: For Case 3, we propose the following solution

- In Rel-16 and Rel-17

  - Report an additional band combination “DL DC_48A_n48A with UL DC_48A_n48A” to support Case 3 configuration

  - For the configuration DL DC_(n)48DA with UL DC_48A_n48A, whether to support the first low order configurations DL DC_48C_n48A with UL DC_48A_n48A and/or DC_48A_(n)48AA with UL DC_48A_n48A should depend on the corresponding UE capability signaling reporting.

- In Rel-18

  - Whether introduce a new UE capability signaling to support the Case 3 configuration should follow the majority view in RAN4.

Proposal 2: For Case 4, we propose the following solution

- In Rel-16 and Rel-17

  - The configuration DL DC_48A-(n)48AA with UL DC_(n)48AA can be indicated by intraBandENDC-Support=not reported(contiguous)

  - The configuration DL DC_48A-48A_n48A with UL DC_48A_n48A can be indicated by intraBandENDC-Support=non-contiguous

  - The Case 4 configuration DL DC_48A-(n)48AA with UL DC_(n)48AA and DL DC_48A-(n)48AA with UL DC_48A_n48A can be indicated by intraBandENDC-Support=both

- In Rel-18

  - Whether introduce a new UE capability signaling to support the Case 4 configuration depends on the Case 3 solution in Rel-18.

# Reference

- R4-2217798, “Email discussion summary for [104-bis-e][146] RAN_task_UERF_part2”, Moderator (OPPO)

- R4-2217759, “WF on IntrabandENDC-Support”, OPPO

- R4-2216412, “Discussion on Intra-Band EN-DC band combinations”, Google Inc., Comcast, CableLabs