3GPP TSG-RAN WG4 Meeting #105	R4-2218307

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	6.3.6.2.1

Source:	Ericsson

Title:	The remaining issues of the PDSCH requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the remaining issues of the PDSCH demodulation requirements for FR2-2. Furthermore, in RAN4 meeting #104-bis-e, WF [1] underlined the progress on PDSCH as well as the pending agreeable requirements and test cases.

Agreement on PDSCH Requirement Table SCS (KHz)/ CBW (MHz); MCS; Propagation Channel; Antenna Conf.; Throughput; Num PRB. 120kHz/100MHz; MCS4; TDLA30-650; 2x2 Low; 70%; 66. MCS13; TDLA30-200; 2x2 Low; 70%; 66. MCS13; TDLA30-650; 2x2 Low; 30%; 66. MCS17; TDLD30-200; 2x2 Low; 70%; [66, 32]. 480kHz/400MHz; MCS4; TDLA10-200; 2x2 Low; 70%; 66. MCS13; TDLD10-200; 2x2 Low; 70%; [20,32]. PDSCH Requirements for CA Bandwidth choice and test setup for FR1+FR2-2 CA requirements - Option 1: - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration. - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). - CQI requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is 66RBs - Option 2: - 100MHz/120KHz - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration. - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). - CQI requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). The RB allocation for FR2-2 CC is 66RBs - 400MHz/480KHz - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration. - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). - CQI requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is 66RBs Applicability rule for FR1+FR2-2 CA requirements - Option 1: - Copy all cases of single CC to CA and define applicability rules as follows: - If UE support standalone operation, UE can only be tested cases with single CC. - If UE don’t support standalone operation, UE can only be tested cases with CA Configurations for FR1 CC for FR1+FR2-2 CA requirements - Companies can provide configurations for FR1 CC for FR1+FR2-2 CA requirements next meeting. Test cases for FR1+FR2-2 CA requirements - Option 1: Reuse all test cases of single CC to CA requirements How to define FR1+FR2-2 CA requirements - Only define the requirements for FR2-2 CC - Option 1: Apply all the requirements of single CC to CA requirements - Option 2: Define separate requirements for CA Companies can provide simulation results or analysis next meeting to support your views.

# 2	Remaining issues for PDSCH requirements

Some pending issues still need to be discussed in this meeting.

## 2.1	General issues

An updated version of Table 7.2.3-2 in [2] which contains the Maximum DL testable SNR.

Table 7.2.3-2: Maximum DL testable SNR preliminary extension for band n263 CBW (MHz); Test method. IFF. Single band UE; 100; [9.8]. 400; [2.6]. 800; [-2.3]. 1600; < -20 (NOTE 1). 2000; < -20 (NOTE 1). Multi band UE; 100; TBD. 400; TBD. 800; TBD. 1600; TBD. 2000; TBD. NOTE 1: Result does not converge.

Accordingly, and based on the agreed test cases in [1] using full and partial RB allocation, we propose the following.

Proposal 1: RAN4 defines the PDSCH demodulation requirements using the extended maximum testable SNR:

|  | CBW (MHz) | Num RBs | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100 | 66 | [9.8] |
|  |  | [32] | [13.2] |
|  | 400 | 66 | [2.6] |
|  |  | [32] | [6.6] |
|  |  | [20] | [8.9] |

## 2.2	Remaining issues on PDSCH requirements

NR operations in 52.5 GHz – 71 GHz band are susceptible to phase noise (PN) effects that should be assessed and addressed. However, RAN4 was not able to agree on a single representative PN model during the study item and WF [1]. In TR 38.808, two PN models have been defined and referred to as Set 1 and Set 2.

In the previous meetings, it has been agreed to set the maximum achievable MCS following these two conditions

- Use CPE compensation only

- Consider performance degradation due to PN less than 1dB

That should be satisfied. Consequently, and based on our simulation results [3], we provide our view in the following proposal.

Proposal 2: Define max MCS for PDSCH demodulation requirements for UE at 70% of the peak throughput as

- 120 kHz/100 MHz: MCS 17 with full RB allocation (66 RBs)

- 480 kHz/400 MHz: MCS 13 with partial RB allocation (32RBs)

Proposal 3: Consider a margin of 3 dB for RF impairments when defining the PDSCH demodulation requirements for UE.

Observation 1: PDSCH demodulation parameters at 70% of the peak throughput has been agreed as

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 30 ns delay spread) TDLD (10 ns and 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). |
| NOH | 6 |
| Number of HARQ Processes | 8 for 120kHz and 16 for 480kHz SCS |

Observation 2: Based on our simulation results in [3], the max MCS that satisfy the following criteria: Performance degradation due to PN is less than 1 dB with CPE compensation and considering an additional margin of 3 dB for RF impairments are

- 120 KHz/100 MHz: MCS 17 with full RB allocation under TDLD30-200

- 480 KHz/400 MHz: MCS 13 with partial RB allocation of 32 under TDLD10-200

Proposal 4: Referring to Observation 2 and the Table in Observation 1, consider PDSCH test cases at 70% of the peak throughput for 120 kHz and 480 kHz SCS as follow

FR2-2 TDD, SCS 120 KHz/100 MHz, Metric: 70% Peak Throughput

| Test case | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compensation | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100MHz / 120kHz (66) | QPSK, 0.30 Rank 1 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | CPE | [-2.9 + 3.0] | R.PDSCH.5-1.1 TDD |
| 1-2 | 100MHz / 120kHz (66) | 16QAM, 0.48 Rank 1 | FR2.120-1 | TDLA30-200 | 2x2 ULA Low | CPE | [4.8 + 3.0] | R.PDSCH.5-2.1 TDD |
| 1-3 | 100MHz / 120kHz (66) | 64QAM, 0.43 Rank 1 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | CPE | [4.7 + 3.0] | R.PDSCH.5-3.2 TDD |

FR2-2 TDD, SCS 480 KHz/400 MHz, Metric: 70% Peak Throughput

| Test case | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- sation | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | 400MHz / 480kHz (66) | QPSK, 0.30 Rank 1 | FR2.480-1 | TDLA10-200 | 2x2 ULA Low | CPE | [-2.9 + 3.0] | R.PDSCH.7-1.1 TDD |
| 2-2 | 400MHz / 480kHz (32) | 16QAM 0.48 Rank 1 | FR2.480-1 | TDLD10-200 | 2x2 ULA Low | CPE | [1.0 + 3.0] | R.PDSCH.7-2.1 TDD |

In the sequel, our proposals for the test cases’ parameters to be used when considering PDSCH at 30% of the peak throughput.

Observation 3: Define PDSCH demodulation requirements for UE at 30% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): MCS 13 |
| NOH | 6 |
| Number of HARQ Processes | 16 |

Proposal 5: Referring to the Table in Observation 3, consider PDSCH test cases at 30% of the peak throughput using the following test setup.

FR2-2 TDD, SCS 120 KHz/100 MHz, Metric: 30% Peak Throughput

| Test number | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- sation | SNR (dB) @ 30% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | 100MHz / 120kHz (66) | 16QAM, 0.48 Rank 1 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | CPE | [-1.3 + 3.0] | R.PDSCH.5-2.1 TDD |

## 2.3	PDSCH requirements for CA

For CA, we should make sure to reconsider the channel bandwidths that have been considered in defining PDSCH requirements for single CC.

Observation 4: Consider CBW1 = 100 MHz and nx100 MHz for 120 kHz SCS, and CBW2 = 400 MHz and nx400 MHz for 480 kHz SCS.

According to Observation 4, we propose the following proposals.

Proposal 6: Define PDSCH requirements for FR1+FR2-2 CA, with FR1 TDD 30 kHz SCS, for the following cases

- 100MHz/120KHz

- PDSCH requirements: FR1(40MHz, single CC) + FR2-2(100MHz, single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration.

- PDCCH requirements: FR1(40MHz, single CC) + FR2-2(100MHz, single CC).

- CQI requirements: FR1(40MHz, single CC) + FR2-2(100MHz, single CC). The RB allocation for FR2-2 CC is 66RBs

- 400MHz/480KHz

- PDSCH requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration.

- PDCCH requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC).

- CQI requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is 66RBs

Proposal 7: Use same PDSCH requirements of single CC for CA, where

- In case of SA, only single CC test cases will be considered.

- In case of NSA, UE will be tested considering CA configurations.

# 3	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: RAN4 defines the PDSCH demodulation requirements using the extended maximum testable SNR:

|  | CBW (MHz) | Num RBs | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100 | 66 | [9.8] |
|  |  | [32] | [13.2] |
|  | 400 | 66 | [2.6] |
|  |  | [32] | [6.6] |
|  |  | [20] | [8.9] |

Proposal 2: Define max MCS for PDSCH demodulation requirements for UE at 70% of the peak throughput as

- 120 kHz/100 MHz: MCS 17 with full RB allocation (66 RBs)

- 480 kHz/400 MHz: MCS 13 with partial RB allocation (32RBs)

Proposal 3: Consider a margin of 3 dB for RF impairments when defining the PDSCH demodulation requirements for UE.

Observation 1: PDSCH demodulation parameters at 70% of the peak throughput has been agreed as

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 30 ns delay spread) TDLD (10 ns and 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). |
| NOH | 6 |
| Number of HARQ Processes | 8 for 120kHz and 16 for 480kHz SCS |

Observation 2: Based on our simulation results in [3], the max MCS that satisfy the following criteria: Performance degradation due to PN is less than 1 dB with CPE compensation and considering an additional margin of 3 dB for RF impairments are

- 120 KHz/100 MHz: MCS 17 with full RB allocation under TDLD30-200

- 480 KHz/400 MHz: MCS 13 with partial RB allocation of 32 under TDLD10-200

Proposal 4: Referring to Observation 2 and the Table in Observation 1, consider PDSCH test cases at 70% of the peak throughput for 120 kHz and 480 kHz SCS as follow

FR2-2 TDD, SCS 120 KHz/100 MHz, Metric: 70% Peak Throughput

| Test case | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compensation | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100MHz / 120kHz (66) | QPSK, 0.30 Rank 1 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | CPE | [-2.9 + 3.0] | R.PDSCH.5-1.1 TDD |
| 1-2 | 100MHz / 120kHz (66) | 16QAM, 0.48 Rank 1 | FR2.120-1 | TDLA30-200 | 2x2 ULA Low | CPE | [4.8 + 3.0] | R.PDSCH.5-2.1 TDD |
| 1-3 | 100MHz / 120kHz (66) | 64QAM, 0.43 Rank 1 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | CPE | [4.7 + 3.0] | R.PDSCH.5-3.2 TDD |

FR2-2 TDD, SCS 480 KHz/400 MHz, Metric: 70% Peak Throughput

| Test case | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- sation | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | 400MHz / 480kHz (66) | QPSK, 0.30 Rank 1 | FR2.480-1 | TDLA10-200 | 2x2 ULA Low | CPE | [-2.9 + 3.0] | R.PDSCH.7-1.1 TDD |
| 2-2 | 400MHz / 480kHz (32) | 16QAM 0.48 Rank 1 | FR2.480-1 | TDLD10-200 | 2x2 ULA Low | CPE | [1.0 + 3.0] | R.PDSCH.7-2.1 TDD |

Observation 3: Define PDSCH demodulation requirements for UE at 30% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): MCS 13 |
| NOH | 6 |
| Number of HARQ Processes | 16 |

Proposal 5: Referring to the Table in Observation 3, consider PDSCH test cases at 30% of the peak throughput using the following test setup.

FR2-2 TDD, SCS 120 KHz/100 MHz, Metric: 30% Peak Throughput

| Test number | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- sation | SNR (dB) @ 30% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | 100MHz / 120kHz (66) | 16QAM, 0.48 Rank 1 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | CPE | [-1.3 + 3.0] | R.PDSCH.5-2.1 TDD |

Observation 4: Consider CBW1 = 100 MHz and nx100 MHz for 120 kHz SCS, and CBW2 = 400 MHz and nx400 MHz for 480 kHz SCS.

Proposal 6: Define PDSCH requirements for FR1+FR2-2 CA, with FR1 TDD 30 kHz SCS, for the following cases

- 100MHz/120KHz

- PDSCH requirements: FR1(40MHz, single CC) + FR2-2(100MHz, single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration.

- PDCCH requirements: FR1(40MHz, single CC) + FR2-2(100MHz, single CC).

- CQI requirements: FR1(40MHz, single CC) + FR2-2(100MHz, single CC). The RB allocation for FR2-2 CC is 66RBs

- 400MHz/480KHz

- PDSCH requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration.

- PDCCH requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC).

- CQI requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is 66RBs

Proposal 7: Use same PDSCH requirements of single CC for CA, where

- In case of SA, only single CC test cases will be considered.

- In case of NSA, UE will be tested considering CA configurations.

# References

- R4-2217391, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- 3GPP TR 38.884, “Study on enhanced test methods for FR2 NR UEs”, v18.2.0.

- R4-2218308, “Simulation results for PDSCH demodulation in 52.6 – 71 GHz band”, Ericsson.

# Appendix	Simulation assumption

## A.1	PDSCH Mapping Type A

### FR2-1

Please refer to 38.101-4 Tables A.3.2.2.5

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| SCS |  | kHz | 120 |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 160 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 for DL slots 9 for special slots |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

## A.2	Propagation environments

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger  than 200MHz;

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | [Maximum Number of Taps] |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Companies can bring simulation results based on following simplified TDL model:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |