3GPP TSG-RAN WG4 Meeting #105	R4-2218311

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	6.3.6.2.3

Source:	Ericsson

Title:	CSI reporting requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the CSI reporting requirements in 52.5 GHz – 71 GHz band, where the related aspects have been gathered in the WF [1]. It is worth reminding that only CSI reporting under static conditions will be considered, where some remaining issues should be discussed

Rank to use for the definition of CQI Requirements - Option 1: Rank 1; - Option 2: Rank 2; Whether to define CQI requirements including 64QAM - Option 1: Yes; - Option 2: No SNR Test points for 64QAM - Option 1: 8/9 dB with Rank 1; - Option 2: Use Rank 2 as in FR2-1 and do not consider 64QAM; SNR Test points for 16QAM - Option 1: 8/9 dB; - Option 2: 2/3 dB; SNR Test points for QPSK - Option 1: -2/-1 dB; - Option 2: 0/1 dB;

# 2	CSI Reporting Requirements

RAN4 has defined CQI, PMI and RI reporting requirements in Rel-15. For Rel-17 and FR2-2 band, we will follow RAN1 where PDSCH MIMO rank has been agreed to be equal to 1. Consequently, we propose to define channel quality indicator (CQI) reporting requirements only, since we do not need to verify the rank indicator (RI). Furthermore, we need to define the CQI reporting requirements using 64QAM for 2Rx UE. We have measured the median CQI and PDSCH BLER at the fixed median CQI to find the SNR test points corresponding to CQI index for both rank 1 and rank 2. Based on our results in [2], we propose the following.

Observation 1: Following RAN1 and RAN4 PDSCH agreements where only Rank 1 has been considered, our view is to define CQI reporting with Rank 1 where 64QAM can be considered.

Proposal 1: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 kHz/100 MHz with Rank 1.

- Consider PN with CPE compensation only.

- Tx EVM = 6%.

- Test points:

- SNR = 8/9 dB for 64QAM

- SNR = 2/3 dB for 16QAM.

- SNR = -2/-1 dB for QPSK.

Observation 2: Some companies raised the interest to consider CQI reporting with Rank 2 following FR2-1. In the following Proposal we show that only 16QAM can be considered.

Proposal 2: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 kHz/100 MHz with Rank 2.

- Consider PN with CPE compensation only.

- Tx EVM = 6%.

- Test points:

- SNR = 5/6 dB for 16QAM

- SNR = -2/-1 dB for QPSK.

Proposal 3: We prefer considering CQI reporting under static conditions and considering Rank 1 only for the following reason

- Align with RAN1 conclusions

- Consider 64QAM.

# 4	Summary

In this paper, we tried to contribute with the following proposal for further discussions:

Observation 1: Following RAN1 and RAN4 PDSCH agreements where only Rank 1 has been considered, our view is to define CQI reporting with Rank 1 where 64QAM can be considered.

Proposal 1: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 KHz/100 MHz with Rank 1.

- Consider PN with CPE compensation only.

- Tx EVM = 6%

- Test points:

- SNR = 8/9 dB for 64 QAM

- SNR = 2/3 dB for 16QAM.

- SNR = -2/-1 dB for QPSK.

Observation 2: Some companies raised the interest to consider CQI reporting with Rank 2 following FR2-1. In the following Proposal we show that only 16QAM can be considered.

Proposal 2: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 kHz/100 MHz with Rank 2.

- Consider PN with CPE compensation only.

- Tx EVM = 6%.

- Test points:

- SNR = 5/6 dB for 16QAM

- SNR = -2/-1 dB for QPSK.

Proposal 3: We prefer considering CQI reporting under static conditions and considering Rank 1 only for the following reason

- Align with RAN1 conclusions

- Consider 64QAM.

# References

- R4-2217391, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- R4-2218312, “Simulation results for CSI reporting requirements in FR2-2”, Ericsson.

# Appendix

## A.1	CQI Tables

We consider Table A.4-1 (TS 38.101-4) mapping CQI index to the information bit payload, given by

Table A.4-1: Mapping of CQI Index to Information Bit payload (CQI table 1)

| TBS Scheme |  |  |  | TBS.1-1 | TBS.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 66 | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |  |  |  |  |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 |  |  |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |  |  |  |  |
| Overhead for TBS determination |  |  |  | 6 | 6 |  |  |  |  |
| Available RE-s |  |  |  | 7590 | 7590 |  |  |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A |  |  |  |  |
| 1 | 0.2344 | 0 | QPSK | 1800 | 3624 |  |  |  |  |
| 2 | 0.2344 | 0 |  | 1800 | 3624 |  |  |  |  |
| 3 | 0.3770 | 2 |  | 2856 | 5640 |  |  |  |  |
| 4 | 0.6016 | 4 |  | 4480 | 8968 |  |  |  |  |
| 5 | 0.8770 | 6 |  | 6528 | 13064 |  |  |  |  |
| 6 | 1.1758 | 8 |  | 8712 | 17928 |  |  |  |  |
| 7 | 1.4766 | 11 | 16QAM | 11016 | 22032 |  |  |  |  |
| 8 | 1.9141 | 13 |  | 14343 | 28680 |  |  |  |  |
| 9 | 2.4063 | 15 |  | 17928 | 35856 |  |  |  |  |
| 10 | 2.7305 | 18 | 64QAM | 20496 | 40976 |  |  |  |  |
| 11 | 3.3223 | 20 |  | 25104 | 50184 |  |  |  |  |
| 12 | 3.9023 | 22 |  | 29192 | 58384 |  |  |  |  |
| 13 | 4.5234 | 24 |  | 33816 | 67584 |  |  |  |  |
| 14 | 5.1152 | 26 |  | 38936 | 77896 |  |  |  |  |
| 15 | 5.5547 | 28 |  | 42016 | 83976 |  |  |  |  |

where the CQI indexes can be mapped to the corresponding MCS as given by Table 5.1.3.1-1 (TS 38.214)

| CQI Index | MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- | --- |
| 1 - 2 | 0 | 2 | 120 | 0.2344 |
| 3 | 2 | 2 | 193 | 0.3770 |
| 4 | 4 | 2 | 308 | 0.6016 |
| 5 | 6 | 2 | 449 | 0.8770 |
| 6 | 8 | 2 | 602 | 1.1758 |
| 7 | 11 | 4 | 378 | 1.4766 |
| 8 | 13 | 4 | 490 | 1.9141 |
| 9 | 15 | 4 | 616 | 2.4063 |
| 10 | 18 | 6 | 466 | 2.7305 |
| 11 | 20 | 6 | 567 | 3.3223 |
| 12 | 22 | 6 | 666 | 3.9023 |
| 13 | 24 | 6 | 772 | 4.5234 |
| 14 | 26 | 6 | 873 | 5.1152 |
| 15 | 28 | 6 | 948 | 5.5547 |

## A.2	CQI reporting in static condition

### A.2.1	Test metric

- The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

| For FR2-2 Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |
| Subcarrier spacing |  |  | kHz | 120 |
| Duplex Mode |  |  |  | TDD |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |
| SNRBB |  |  | dB | TBD |
| Propagation channel |  |  |  | AWGN |
| Antenna configuration |  |  |  | 2x2 with static channel specified in Annex B.1 |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | CSI-RS periodicity and offset |  | Slot | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | fd-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |
| ReportConfigType |  |  |  | Periodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 111111111 |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |
| aperiodicTriggeringOffset |  |  |  | Not configured |
| Codebook configuration |  | Codebook Type | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |
|  |  | CodebookSubsetRestriction |  | 010000 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8.375 |
| Maximum number of HARQ transmission |  |  |  | 1 |
| Measurement channel |  |  |  | As specified in Table A.4-1 |