3GPP TSG-RAN WG4 Meeting # 105	R4-2218326

Toulouse, France, 14 – 18 November 2022

Agenda Item:	8.7.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Link level simulation results for FR2-1 UL 256QAM

Document for:	Information

# 1.	Introduction

The revised work item on NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved at TSG RAN#96 [1]. One of the objectives of this work item is to investigate and enable UL 256QAM for FR2-1.

One of the main tasks to investigate and enable UL 256QAM for FR2-1 is to study the gain, operating SNR, phase noise model and implementation aspects. This topic was discussed at TSG RAN4#104-e and the link level simulation assumptions were included in the agreed WF [2]. This contribution provides link level simulation results for FR2-1 UL 256QAM according to the agreed simulation assumptions.

# 2.	Simulator assumptions

The simulation results at 29 GHz (n257) and 39 GHz (n260) were presented in [3], and the simulation results at 48 GHz (n262) are presented in this contribution. The simulation results are obtained with a subset of the agreed simulation cases in [2], because of the time constraint to obtain the simulation results with the whole set of cases. The selected subset of cases is yellow-highlighted in table 1, which is a copy of the simulation assumptions agreed in [2].

Table 1. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257), 39 GHz (n260) and 48GHz (n262) |
| CBW | 50 MHz, 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: CP-OFDM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. DFT-s-OFDM: MCS 22, 23 in TS 38.214 Table 6.1.4.1-1, other MCSs are not precluded. 256QAM: CP-OFDM/DFT-s-OFDM: MCS 21, 23 in TS 38.214 Table 5.1.3.1-2, other MCSs are not precluded. Baseline: fixed MCSs |
| Symbol type | CP-OFDM; DFT-s-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM: KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) DFT-s-OFDM: (,)=(4, 4), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (UE) + example1(BS) Option b): example2 (UE) + example2(BS) Option d): example1 (UE) + example2(BS) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 3%, 3.5%, 4%, rxEVM: 3%, 3.5%, 4% Option 1: txEVM >= rxEVM; |
| Other parameters | follow assumptions in TS38.104 Section 11.2.2 . |

# 3.	Simulation results and observations

The link level simulation results at 48 GHz (n262) using the highlighted parameters in table 1 with phase noise model option a and option d are shown in figures 1 and 2, respectively.

Figure 1. Simulation results with phase noise model option a

Figure 2. Simulation results with phase noise model option d

It can be observed from figures 1 and 2 that at 48 GHz (n262):

-	256QAM does not provide any throughput gain over 64QAM below 40 dB SNR.

# 4.	Conclusion

This contribution has provided link level simulation results for FR2-1 UL 256QAM according to the agreed simulation assumptions, with the observations summarized as follows:

Observation 1: For the simulation results at 48 GHz (n262):

-	256QAM does not provide any throughput gain over 64QAM below 40 dB SNR.

# References

[1]	RP-221862, “Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi.

[2]	R4-2214453, “WF on UL 256QAM”, Xiaomi.

[3]	R4-2215577, “Link level simulation results for FR2-1 UL 256QAM”, Nokia, Nokia Shanghai Bell.