3GPP TSG-RAN WG4 Meeting # 105	R4-2218329

Toulouse, France, 14 – 18 November 2022

Agenda Item:	8.8.2.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception

Document for:	Approval

# 1.	Introduction

The revised work item on Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception was approved at TSG RAN#96 [1]. One of the objectives of this work item is to:

•	Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs

Moreover, it is stated in the WID [1] that:

•	The legacy spherical coverage requirement for reception from a single direction will be kept

And that:

•	PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized.

The subject was discussed at TSG RAN4#104-bis-e and the WF was agreed [2]. This contribution provides a proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception comparing to the legacy spherical coverage requirement for reception from a single direction for PC3 UE.

# 2.	Discussion

A number of proposals for joint 2AoA sensitivity spherical coverage requirement was discussed at TSG RAN4#104-bis-e and the WF [2] stated:

FFS what is the key performance gain need to be verified for multi-Rx DL reception and how to construct the requirement:

Option 1: Coverage performance, e.g., proposal 1,2,3,4,7 in Annex2

Option 2: Simultaneous reception, e.g., proposal 5,6 in Annex2

Option 3: Others

The WID [1] clearly stated spherical coverage requirements as the main RF requirements to be specified. Moreover, proposal 5 in Annex 2 in [2] cannot provide assurance that the signal received from at least one of the two directions for FR2-1 multi-Rx chain DL reception will maintain or closely maintain the legacy spherical coverage requirement for reception from a single direction, i.e., the spherical coverage for both AoAs may sink to as much as the EIS tolerance compared to reception from a single direction. On the other hand, proposal 6 in Annex 2 in [2] cannot provide assurance on spherical coverage with simultaneous reception, as throughput/demod requirements will only be fulfilled when the side condition like received SNR of both AoAs is met, and thus high wanted signal power level (i.e., high EIS value) will be needed when noise/interference power level is high to meet the received SNR for the requirements to be met. Therefore, it is proposed to focus on option 1 to specify the spherical coverage requirements for FR2-1 multi-Rx chain DL reception.

The legacy spherical coverage requirement for reception from a single direction for PC3 UE is specified in clause 7.3.4.3 of TS 38.101-2 [3]:

<Start of extraction>

#### 7.3.4.3	EIS spherical coverage for power class 3

The reference measurement channels and throughput criterion shall be as specified in clause 7.3.2.3

The maximum EIS at the 50th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-1 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

For the UEs that support multiple FR2 bands, the minimum requirement for EIS spherical coverage in Table 7.3.4.3-1 shall be increased per band, respectively, by the EIS spherical coverage relaxation parameter ∆MBS,n as specified in clause 6.2.1.3.  The requirement for the UE which supports a single FR2 band is specified in Table 7.3.4.3-1. The requirement for the UE which supports multiple FR2 bands is specified in both Table 7.3.4.3-1 and Table 6.2.1.3-4.

Table 7.3.4.3-1: EIS spherical coverage for power class 3

| Operating band | EIS at 50th %-tile CCDF (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257 | -77.4 | -74.4 | -71.4 | -68.4 | N.A | N.A | N.A |
| n258 | -77.4 | -74.4 | -71.4 | -68.4 | N.A | N.A | N.A |
| n259 | -71.9 | -68.9 | -65.9 | -62.9 | N.A | N.A | N.A |
| n260 | -73.1 | -70.1 | -67.1 | -64.1 | N.A | N.A | N.A |
| n261 | -77.4 | -74.4 | -71.4 | -68.4 | N.A | N.A | N.A |
| n262 | -69.7 | -66.7 | -63.7 | -60.7 | N.A | N.A | N.A |
| n263 | N.A | -66.2 | N.A | -60.2 | -57.2 | -54.2 | -53.2 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |  |  |  |  |  |

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

<End of extraction>

It can be seen from the extraction above that the legacy spherical coverage requirement includes two target values with which the 95% throughput of the reference measurement channel shall be met: the maximum EIS and the 50th percentile of the CCDF. The requirement is specified for reception from a single direction, where it is assumed that a single receive beam is formed using one or more panels depending on UE implementation. For devices with simultaneous multi-Rx chain DL reception from two different directions, using the ‘panel’ understanding in assumptions for deriving UE RF requirements in the WF [2], it can be assumed that two receive beams are formed (each using one or more panels depending on UE implementation) to receive the signals from two different directions, where signal from one direction is received by the main lobe of one beam pattern while the signal from the other direction is received by the main lobe of the other beam pattern.

There would be inter-beam interference between the two receive beam patterns depending on UE implementation and AoA separation (e.g., inter-beam interference should be neglectable if they are pointing toward directions with large AoA separation). Hence spherical coverage relaxation may need to be allowed for both signals from the two different directions. However, any spherical coverage relaxation will mean coverage sinkage for devices, and thus degrade user experience which is supposed to be improved by multi-Rx chain DL reception. Therefore, it is proposed that the signal received from at least one of the two directions should maintain or closely maintain the legacy spherical coverage requirement for reception from a single direction to maintain the coverage of the device, while the signal received by the other direction may be allowed some spherical coverage relaxation due to the inter-beam interference with small AoA separation.

# 3.	Conclusion

This contribution has a proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception comparing to the legacy spherical coverage requirement for reception from a single direction for PC3 UE, with the following proposals.

Proposal 1: To focus on option 1 to specify the spherical coverage requirements for FR2-1 multi-Rx chain DL reception.

Proposal 2: The signal received from at least one of the two directions should maintain or closely maintain the legacy spherical coverage requirement for reception from a single direction to maintain the coverage of the device.

Proposal 3: The signal received by the other direction may be allowed some spherical coverage relaxation due to the inter-beam interference with small AoA separation.

# References

[1]	RP-221753, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated.

[2]	R4-2217731, “WF on FR2 UE RF requirements for 2AoA DL Rx”, vivo.

[3]	3GPP TS 38.101-2 v17.7.0: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.