3GPP TSG-RAN WG4 Meeting # 105	R4-2218463

Toulouse, FR, November 14 – November 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | - | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.41-2, On FR2-2 BS RF conformance testing in clauses 7.1-7.5 and respective MU and TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce BS RF RX conformance testing for FR2-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Add MU for FR2-2 for OTA reference sensitivity level, OTA ACS, and OTA in-band blocking (General) in Table 4.1.2.3-2. 2) Add OTA reference sensitivity requirement for FR2-2 in Table 7.3.5.3-3. 3) Add OTA ACS requirement for FR2-2 in Table 7.5.1.5.3-1. 4) Aad OTA ACS interferer frequency offset for FR2-2 in Table 7.5.1.5.3-2. 5) Add ΔfOOB offset for FR2-2 in Table 7.5.2.5.3-0. 6) Add general OTA blocking requirement in Table 7.5.2.5.3-1. 7) Add TTOTA for OTA reference sensitivity level for FR2-2 in Table C.2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BS RF Rx conformance testings for FR2-2 in TS 38.141-1 would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.3, 7.3.5.3, 7.5.1.5.3, 7.5.2.5.3, C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.1.2.3	Measurement of receiver

The maximum OTA Test System uncertainty for OTA receiver tests minimum requirements are given in tables 4.1.2.3-1 and 4.1.2.3-2. Details for derivation of OTA Test System uncertainty are given in corresponding clauses in TR 37.941 [29].

Table 4.1.2.3-1: Maximum OTA Test System uncertainty for FR1 OTA receiver tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 7.2 OTA sensitivity | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz ±1.6 dB, 4.2 GHz < f ≤ 6.0 GHz ±1.9 dB, 6.0 GHz < f ≤ 7.125 GHz ±1.9 dB for band n96 |
| 7.3 OTA reference sensitivity level | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz ±1.6 dB, 4.2 GHz < f ≤ 6.0 GHz ±1.9 dB, 6.0 GHz < f ≤ 7.125 GHz |
| 7.4 OTA dynamic range | ±0.3 dB |
| 7.5.1 OTA adjacent channel selectivity | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.4 dB, 4.2 GHz < f ≤ 6.0 GHz ±2.8 dB, 6.0 GHz < f ≤ 7.125 GHz |
| 7.5.2 In-band blocking (General) | ±1.9 dB, f ≤ 3.0 GHz ±2.2 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.5 dB, 4.2 GHz < f ≤ 6.0 GHz ±2.9 dB, 6.0 GHz < f ≤ 7.125 GHz |
| 7.5.2 In-band blocking (Narrowband) | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.4 dB, 4.2 GHz < f ≤ 6.0 GHz |
| 7.6 OTA out-of-band blocking (General) | fwanted ≤ 3.0 GHz: ±2.0 dB, finterferer ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < finterferer ≤ 6.0 GHz ±3.5 dB, 6.0 GHz < finterferer ≤ 12.75 GHz 3 GHz < fwanted ≤ 4.2 GHz: ±2.0 dB, finterferer ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < finterferer ≤ 6.0 GHz ±3.6 dB, 6.0 GHz < finterferer ≤ 12.75 GHz 4.2 GHz < fwanted ≤ 7.125 GHz: ±2.2 dB, finterferer ≤ 3.0 GHz ±2.3 dB, 3.0 GHz < finterferer ≤ 6.0 GHz ±3.6 dB, 6.0 GHz < finterferer ≤ 12.75 GHz |
| 7.6 OTA out-of-band blocking (Co-location) (NOTE 1) | fwanted ≤ 3.0 GHz: ±3.4 dB, finterferer ≤ 3.0 GHz ±3.5 dB, 3.0 GHz < finterferer ≤ 4.2 GHz ±3.7 dB, 4.2 GHz < finterferer ≤ 6.0 GHz 3 GHz < fwanted ≤ 4.2 GHz: ±3.5 dB, finterferer ≤ 3.0 GHz ±3.6 dB, 3.0 GHz < finterferer ≤ 4.2 GHz ±3.7 dB, 4.2 GHz < finterferer ≤ 6.0 GHz 4.2 GHz < fwanted ≤ 6.0 GHz: ±3.6 dB, finterferer ≤ 3.0 GHz ±3.7 dB, 3.0 GHz < finterferer ≤ 4.2 GHz ±3.8 dB, 4.2 GHz < finterferer ≤ 6.0 GHz 6.0 GHz < f ≤ 7.125 GHz: ±3.6 dB, finterferer ≤ 3.0 GHz ±3.8 dB, 3.0 GHz < finterferer ≤ 4.2 GHz ±3.9 dB, 4.2 GHz < finterferer ≤ 6.0 GHz |
| 7.7 OTA receiver spurious emissions | ±2.5 dB, 30 MHz ≤ f ≤ 6.0 GHz ±4.2 dB, 6.0 GHz < f ≤ 26 GHz |
| 7.8 OTA receiver intermodulation | ±2.0 dB, f ≤ 3.0 GHz ±2.6 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.2 dB, 4.2 GHz < f ≤ 6.0 GHz ±3.5 dB, 6.0 GHz < f ≤ 7.125 GHz |
| 7.9 OTA in-channel selectivity | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.4 dB, 4.2 GHz < f ≤ 6.0 GHz ±2.8 dB, 6.0 GHz < f ≤ 7.125 GHz |
| NOTE 1: Fulfilling the criteria for CLTA selection and placement in clause 4.12 is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as specified in the appropriate measurement uncertainty budget in TR 37.941 [29], shall be used for evaluating the test system uncertainty. NOTE 2: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

Table 4.1.2.3-2: Maximum OTA Test System uncertainty for FR2 OTA receiver tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 7.3 OTA reference sensitivity level | ±2.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±2.4 dB, 37 GHz < f ≤ 43.5 GHz ±[3.5] dB, 43.5 GHz < f ≤ 48.2 GHz ±[TBD] dB, 52.6 GHz < f ≤ 71 GHz |
| 7.5.1 OTA adjacent channel selectivity | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 43.5 GHz ±[5.1] dB, 43.5 GHz < f ≤ 48.2 GHz ±[TBD] dB, 52.6 GHz < f ≤ 71 GHz |
| 7.5.2 In-band blocking (General) | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 43.5 GHz ±[5.1] dB, 43.5 GHz < f ≤ 48.2 GHz ±[TBD] dB, 52.6 GHz < f ≤ 71 GHz |
| 7.6 OTA out-of-band blocking | ±3.6 dB, 24.25 GHz < f ≤ 43.5 GHz ±[4.5] dB, 43.5 GHz < f ≤ 48.2 GHz |
| 7.7 OTA receiver spurious emissions | ±2.5 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |
| 7.8 OTA receiver intermodulation | ±3.9 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.9 dB, 37 GHz < f ≤ 43.5 GHz ±[5.4] dB, 43.5 GHz < f ≤ 48.2 GHz |
| 7.9 OTA in-channel selectivity | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 43.5 GHz ±[5.1] dB, 43.5 GHz < f ≤ 48.2 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

<End of Change 1>

<Start of Change 2>

7.3.5.3	Test requirements for BS type 2-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz BS channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50 MHz BS channel bandwidth it does not imply that BS has to support 50 MHz BS channel bandwidth.

For Wide Area BS, EISREFSENS_50M is an integer value in the range -96 to -119 dBm. The specific value is declared by the vendor.

For Medium Range BS, EISREFSENS_50M is an integer value in the range -91 to -114 dBm. The specific value is declared by the vendor.

For Local Area BS, EISREFSENS_50M is an integer value in the range -86 to -109 dBm. The specific value is declared by the vendor.

Table 7.3.5.3-1 FR2-1 OTA reference sensitivity requirement applicable in the frequency range 24.25 – 43.5 GHz

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (annex A.1) | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 2.4 + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 2.4 + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + 2.4 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified in table 10.3.3-2. |  |  |  |

Table 7.3.5.3-2 FR2-1 OTA reference sensitivity requirement applicable in the frequency range 43.5 – 48.2 GHz

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (annex A.1) | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + [3.5] + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + [3.5] + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + [3.5] + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified in table 10.3.3-2. |  |  |  |

Table 7.3.5.3-3 FR2-2 OTA reference sensitivity requirement applicable in the frequency range 52.6 – 71 GHz

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (annex A.1) | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 100,400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + [TBD] + ΔFR2_REFSENS |
| 400, 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + 9 + [TBD] + ΔFR2_REFSENS |
| 400, 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + 9 + [TBD] + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified in table 10.3.3-2. |  |  |  |

<End of Change 2>

<Start of Change 3>

7.5.1.5.3	Test requirements for BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2, the OTA wanted and the interfering signal are specified in table 7.5.1.5.3-1 and table 7.5.1.5.3-2 for ACS. The reference measurement channel for the OTA wanted signal is identified in clause 7.3.5.3 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.5.1.5.3-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 7.5.1.5.3-1: OTA ACS requirement for BS type 2-O

| Frequency Range |  |  | BS channel bandwidth of the |  | Wanted signal mean power (dBm) |  | Interfering signal mean |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | lowest/highest carrier received (MHz) |  |  |  | power (dBm) |
| FR2-1 |  |  | 50, 100, 200, 400 |  | EISREFSENS + 6 dB (Note 3) |  | EISREFSENS_50M + 27.7 + ΔFR2_REFSENS (Note 1) EISREFSENS_50M + 26.7 + ΔFR2_REFSENS (Note 2) |
| FR2-2 | 100, 400, 800, 1600, 2000 | EISREFSENS + 6 dB (Note 3) | EISREFSENS_50M + 28.7 + ΔFR2_REFSENS (Note 4) |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 3: EISREFSENS is specified in TS 38.104 [2], clause 10.3.3. NOTE 4: Applicable to bands defined within the frequency spectrum range of 57 – 71 GHz |  |  |  |

Table 7.5.1.5.3-2: OTA ACS interferer frequency offset for BS type 2-O

| Frequency Range |  | BS channel bandwidth of the lowest/highest carrier received (MHz) |  | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) |  | Type of interfering signal |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 50 | ±24.29 | 50 MHz DFT-s-OFDM NR |
|  | 100 | ±24.31 | signal, 60 kHz SCS, 64 RBs |
|  | 200 | ±24.29 |  |
|  | 400 | ±24.31 |  |
| FR2-2 | 100 | ±48.58 | 100 MHz DFT-s-OFDM NR signal,120 kHz SCS, 64 RBs |
|  | 400 | ±48.58 |  |
|  | 800 | ±48.62 |  |
|  | 1600 | ±48.58 |  |
|  | 2000 | ±48.62 |  |

<End of Change 3>

<Start of Change 4>

7.5.2.5.3	Test requirements for BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For BS type 2-O, the OTA wanted and OTA interfering signals are provided at RIB using the parameters in table 7.5.2.5.3-1 for general OTA blocking requirements. The reference measurement channel for the OTA wanted signal is identified in clause 7.3.5.3 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA blocking requirements are applicable outside the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

For BS type 2-O the OTA blocking requirement shall apply in the in-band blocking frequency range, which is defined within frequency range from FUL_low - ΔfOOB to FUL_high + ΔfOOB, where the ΔfOOB for BS type 2-O is defined in table 7.5.2.5.3-0.

Table 7.5.2.5.3-0: ΔfOOB offset for NR operating bands in FR2

| BS type | Frequency Range | Operating band characteristics (MHz) | ΔfOOB (MHz) |
| --- | --- | --- | --- |
| BS type 2-O | FR2-1 | FUL_high – FUL_low ≤ 4000 | 1500 |
|  | FR2-2 | 4000 < FUL_high – FUL_low ≤14000 | 3500 |

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in table 7.5.2.5.3-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 7.5.2.5.3-1: General OTA blocking requirement for BS type 2-O

| Frequency Range |  | BS channel bandwidth of the |  | OTA wanted signal mean power (dBm) |  |  | OTA interfering signal mean | OTA interfering signal centre |  |  | Type of OTA interfering |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | lowest/highest carrier received (MHz) |  |  |  |  | power (dBm) | frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) |  |  | signal |
| FR2-1 |  | 50, 100, 200, 400 |  | EISREFSENS + 6 dB |  |  | EISREFSENS_50M + 33 + ΔFR2_REFSENS dB | ±75 |  |  | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| FR2-2 | 100, 400, 800, 1600, 2000 | EISREFSENS + 6 dB | EISREFSENS_50M + 36 + ΔFR2_REFSENS | ±150 | 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs |
| NOTE: EISREFSENS and EISREFSENS_50M are given in TS 38.104 [2], clause 10.3.3. |  |  |  |  |  |

<End of Change 4>

<Start of Change 5>

C.2	Measurement of receiver

Table C.2-1: Derivation of test requirements (FR1 OTA receiver tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.2 OTA sensitivity | See TS 38.104 [2], clause 10.2 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz 1.6 dB, 4.2 GHz < f ≤ 6.0 GHz 1.9 dB, 6.0 GHz < f ≤ 7.125 GHz | Formula: Declared Minimum EIS + TT |
| 7.3 OTA reference sensitivity level | See TS 38.104 [2], clause 10.3 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz 1.6 dB, 4.2 GHz < f ≤ 6.0 GHz 1.9 dB, 6.0 GHz < f ≤ 7.125 GHz | Formula: EISREFSENS + TT |
| 7.4 OTA dynamic range | See TS 38.104 [2], clause 10.4 | 0.3 dB, f ≤ 7.125 GHz | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.1 OTA adjacent channel selectivity | See TS 38.104 [2], clause 10.5.1 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.2 In-band blocking (General) | See TS 38.104 [2], clause 10.5.2 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.2 In-band blocking (Narrowband) | See TS 38.104 [2], clause 10.5.2 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.6 OTA out-of-band blocking (General) | See TS 38.104 [2], clause 10.6 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.6 OTA out-of-band blocking (Co-location) | See TS 38.104 [2], clause 10.6 | 0 dB | Formula: Wanted signal power unchanged Interferer signal power - TT. |
| 7.7 OTA receiver spurious emissions | See TS 38.104 [2], clause 10.7 | 0 dB | Formula: Minimum Requirement + TT |
| 7.8 OTA receiver intermodulation | See TS 38.104 [2], clause 10.8 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged |
| 7.9 OTA in-channel selectivity | See TS 38.104 [2], clause 10.9 | 1.7 dB, f ≤ 3.0 GHz 2.1 dB, 3.0 GHz < f ≤ 4.2 GHz 2.4 dB, 4.2 GHz < f ≤ 6.0 GHz 2.8 dB, 6.0 GHz < f ≤ 7.125 GHz | Formula: Wanted signal power + TT Interferer signal power unchanged |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table C.2-2: Derivation of test requirements (FR2 OTA receiver tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.3 OTA reference sensitivity level | See TS 38.104 [2], clause 10.3 | 2.4 dB, 24.25 GHz < f ≦ 33.4 GHz 2.4 dB, 37 GHz < f ≦ 43.5 GHz [3.5] dB, 43.5 GHz < f ≦ 48.2 GHz [TBD] dB, 52.6 GHz < f ≤ 71 GHz | Formula: EISREFSENS+ TT |
| 7.5.1 OTA adjacent channel selectivity | See TS 38.104 [2], clause 10.5.1 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.2 In-band blocking | See TS 38.104 [2], clause 10.5.2 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.6 OTA out-of-band blocking | See TS 38.104 [2], clause 10.6 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged |
| 7.7 OTA receiver spurious emissions | See TS 38.104 [2], clause 10.7 | 0 dB | Formula: Minimum Requirement + TT |
| 7.8 OTA receiver intermodulation | See TS 38.104 [2], clause 10.8 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.9 OTA in-channel selectivity | See TS 38.104 [2], clause 10.9 | 3.4 dB, 24.25 GHz < f ≦ 33.4 GHz 3.4 dB, 37 GHz < f ≦ 43.5 GHz [5.1] dB, 43.5 GHz < f ≦ 48.2 GHz | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

<End of Change 5>