3GPP TSG-RAN WG4 Meeting # 105                                                     R4-2218478			R4-22xxxxx

# Electronic Meeting, November 14 –November 18, 2022

Title: 	Further discussion on BS feasibility study for duplex evolution

Source: 	CATT

Agenda item:	8.18.2.2.1

Document for:	Discussion

- Background

This contribution provides our analysis for the self-interference analysis methodology, modeling and the RF requirement impact. The analysis is conducted using FR1 as the example; FR2 can be analyzed using the similar methodology when the assumptions are aligned.

- Discussion

  - gNB Self-interference

2.1.1 The whole SI contribution

For the gNB self-interference, we analysed the 1dB degradation requirement for FR1. The followings are the assumptions for the analysis.

- BWchannel:100MHz

- Subband configuration：｛DUD｝， 40MHz DL+20MHz UL+40MHz DL

- Tx power over whole channel  PTx :46dBm

- Subband ACLR：45dB

- Noise Figure: 5dB

- UL REFSENS degradation due to SI: 1dB

- Rx in-band blocking interfering signal power: -43dBm

Figure1: subband assignment

Figure 2: gNB Tx antenna output and Rx antenna input of self-interference

Interfering signal power can be calculated as in Table 1 according to the concept shown in Figure 2.

Table 1: Interfering signal mean powers of receiver

| Parameters | Value |
| --- | --- |
| Rx subband BWUL (MHZ) | 20 |
| Noise figure (dB) | 5 |
| Noise power (dBm/ BWUL) | -96 |
| Receiver sensitivity degradate (dB) | 1 |
| Receiver total noise power (dBm/ BWUL) | -95 |
| Rx subband interfering signal power (dBm/ BWUL) | -102 |

From the above calculation, it can be seen that the if 1 dB sensitivity degradation is the target, the total of Tx leakage and Rx path contribution should be 1 dB, i.e. -102 dBm. Then each contribution is assumed to be -105 dBm in the following separate analysis.

Observation 1: The Tx leakage and Rx contribution requirements for 1 dB SI target can be divided to -105 dBm each if each of them contributes to half of the noise.

2.1.2 RSIC capability for Tx leakage

Table 2: RSIC requirement for Tx leakage of SI

| Parameters | Value |
| --- | --- |
| Tx subband BWDL (MHZ) | 40 |
| Rx subband BWUL (MHZ) | 20 |
| Tx power over whole channel PTx（dBm) | 46 |
| Rx subband interfering signal mean power (dBm/ BWUL) | -105 |
| Overall RSIC requirement for Tx leakage (dB) | 148 |

The overall RSIC requirement 148 dB can be estimated to be achieved by the following performance:

| Overall RSIC requirement for Tx leakage (dB) | 148 |
| --- | --- |
| Subband ACLR (dB) | 45 |
| Spatial isolation (dB) | 83 |
| Beam nulling /isolation (dB) | 10 |
| Digital IC for Tx leakage(dB) | 10 |

Observation 2: The overall RSIC requirement for Tx leakage should be at least 148 dB for WA SBFD BS

2.1.3 Rx path noise contribution

2.1.3.1 Rx blocking issue

Assume Rx in-band blocking interfering signal power is -43 dBm for Wide Area BS, Table 3 shows the RSIC requirement for Rx blocking issue.

Table 3: RSIC requirement for Rx in-band blocking of SI

| Parameters | Value |
| --- | --- |
| Tx subband BWDL (MHZ) | 40 |
| Rx subband BWUL (MHZ) | 20 |
| Tx power over whole channel PTx（dBm) | 46 |
| BS in-band blocking requirement (dBm) | -43 |
| RSIC requirement for Rx blocking issue (dB) | 89 |

Observation 3: The RSIC requirement for Rx blocking issue should be 89 dB for WA SBFD BS.

2.1.3.2 Rx intermodulation

Assuming IM3 contributes half of the Rx noise contribution, then IM3 should be below -108 dBm, in-band blocking signal is -43dBm, the required IIP3 is -10.5 dBm.

IIP3=Pinterf+(Pinterf-PIM3)2

If the IBB signal power  increases 1dB，IIP3 requirement increases 1.5dB.

Observation 4: If IM3 contribution is -108 dBm for WA BS SBFD Rx noise, IIP3 should be at least-10.5 dBm in whole operation temperature range, which is very challenging.

2.1.3.3 AGC and ADC dynamic range

ADC dynamic range needs to be analysed for Rx path to see if the large interference and small wanted signal can be covered without any AGC operation. Assuming the largest interference signal is the IBB signal, i.e. -43 dBm, the minimum wanted signal in 20MHz Rx SB is -95.6 dBm, then the dynamic range of the Rx input signal is 52.6 dB. 52.6 plus 10 dB PAPR budget, 2dB gain discordance and 2dB gain change over the operation temperature range，it is 66.6dB dynamic range requirement for ADC. 12 bit ADC has 74 dB dynamic range, so 14 bit ADC has sufficient margin for the current assumption.

Observation 5: If Rx blocking level is assumed to be the current WA BS requirement, ADC dynamic range can cover the whole signal level including IBB blocking and the Rx wanted signal.

Then when ADC dynamic range is sufficient, there’s no motivation to have any AGC operation considering the NF and the implementation complexity.

Observation 6: AGC is not need to be analysed for BS Rx path because ADC dynamic is not a problem.

2.1.3.4 Adjacent Channel Selectivity (ACS)

Similar with IM3 contribution, assuming ACS contribution is half of the Rx noise contribution, i.e. -108 dBm.Then ACS requirement should be -43 – (-108) = 65 dB. It may be a little challenging for Rx digital filter. ~ 200 taps digital filter is needed for 20 MHz Rx filter if SU is the same with current specification.

Observation 7: 65 dB ACS capability is needed for WA BS. It is challenging if SU is the same with current specification.

2.1.4 SI analysis parameters table

From the above analysis, the SI analysis should be separated as two parts, i.e. Tx leakage and Rx path noise. For Rx path noise, Rx blocking, Rx IM3 (IIP3) and ACS should be analysed. So the following table can be used as the SI analysis parameters table to do the alignment between companies.

Table 4 : SI analysis parameters

| Item |  | Parameters | Value |
| --- | --- | --- | --- |
| General assumption |  | BS total Tx power @ DL SB (dBm) |  |
|  |  | SBFD configuration |  |
|  |  | Guard band assumption |  |
|  |  | Rx in-band blocking capability (dBm) |  |
|  |  | Rx ADC dynamic range (dB) |  |
|  |  | BS NF (dB) |  |
| Tx leakage contribution |  | Tx SB ACLR (dB) |  |
|  |  | Antenna isolation (dB) |  |
|  |  | Beam nulling/isolation (dB) |  |
|  |  | Digital IC for Tx leakage (dB) |  |
|  |  | Overall RSIC for Tx leakage (dB) |  |
|  |  | Tx leakage contribution to SI (dBm) |  |
| Rx path noise contribution | Blocking analysis | Overall RSIC for blocking capability (dB) |  |
|  |  | Tx SB interference signal level at Rx path (dBm) |  |
|  | Rx IMD | Rx IIP3 capability (dBm) |  |
|  |  | Rx IM3 contribution (dBm) |  |
|  | Rx ACS | ACS capability (dB) |  |
|  |  | Rx ACS contribution (dBm) |  |
|  | Rx path overall noise contribution (dBm) |  |  |
| Total SI contribution (dBm) |  |  |  |
| Overall REFSENS degradation (dB) |  |  |  |

Proposal 1: Table 4 can be used for SI analysis parameters alignment.

  - Co-channel inter-cell co-site inter-sector gNB-to-gNB inter-subband CLI

2.2.1 The whole CLI contribution

For co-site inter-sector co-channel inter-subband CLI, the followings are the assumptions for the analysis.

- Aggressor gNB and victim gNB configurations are same as SI gNB

- Assume UL receiver sensitivity degradation due to CLI is 1dB.

The analysis is similar with SI but antenna isolation is replaced by spatial isolation of different sectors.

If 1 dB sensitivity degradation is the target, the total of Tx leakage and Rx path contribution should be 1 dB, i.e. -102 dBm. Then each contribution is assumed to be -105 dBm, because it is the sum contributions from all co-site sectors, so -108dBm  is used in the following separate analysis.

Figure 3:  Co-channel co-site inter-sector gNB-to-gNB inter-subband CLI

Figure 4: gNB2 Tx antenna output and gNB1 Rx antenna input of inter-subband CLI

Observation 8: The Tx leakage and Rx contribution requirements to the CLI can be divided to -108 dBm each if each of them contributes to half of noise.

    - Tx leakage contribution

Table 5: Interference cancellation requirement for Tx leakage of CLI

| Parameters | value |
| --- | --- |
| Tx subband BWDL-gNB2 (MHZ) | 40 |
| Rx subband BWUL-gNB1 (MHZ) | 20 |
| Tx power over whole channel PTx-gNB2(dBm) | 46 |
| Rx subband interfering signal mean power (dBm/ BWUL-gNB1) | -108 |
| Isolation or receiver performance improved requirement (dB) | 151 |

The overall interference cancellation requirement 151 dB can be estimated to be achieved by the following performance:

| Overall RSIC requirement for Tx leakage (dB) | 151 |
| --- | --- |
| Subband ACLR (dB) | 45 |
| Spatial isolation (dB) | 106 |

Observation 9: The interference cancellation requirement for Tx leakage issue should be 151 dB for co-channel co-site inter-sector CLI, which is very challenging.

2.2.3 Rx path noise contribution

2.2.3.1 Rx blocking issue

Assume Rx in-band blocking interfering signal power is -43 dBm for Wide Area BS.

Table 6: RSIC requirement for Rx in-band blocking of SI

| Parameters | Value |
| --- | --- |
| Tx subband BWDL (MHZ) | 40 |
| Rx subband BWUL (MHZ) | 20 |
| Tx power over whole channel PTx（dBm) | 46 |
| BS in-band blocking requirement (dBm) | -43 |
| Interference cancellation (dB) | 92 |

Observation 10: The interference cancellation requirement for Rx blocking issue should be 92 dB for co-channel co-site inter-sector CLI.

2.2.3.2 Rx intermodulation

Assuming IM3 contributes half of the Rx noise contribution, then IM3 should be below -111 dBm, in-band blocking signal is -43dBm, the required IIP3 is -9 dBm.

Observation 11: If IM3 contribution is -108 dBm for co-channel co-site inter-sector CLI Rx noise, IIP3 should be at least -9 dBm in whole operation temperature range, which is very challenging.

2.1.3.3 Adjacent Channel Selectivity (ACS)

Similar with IM3 contribution, assuming ACS contribution is half of the Rx noise contribution, i.e. -111 dBm.Then ACS requirement should be -43 – (-111) = 68 dB. It may be challenging for Rx digital filter. ~ 200 taps digital filter is needed for 20 MHz Rx filter if SU is the same with current specification.

Observation 12: 68 dB ACS capability is needed for co-channel co-site inter-sector CLI. It is challenging if SU is the same with current specification.

2.2.4 CLI analysis parameters table

CLI analysis parameters table is same as SI analysis parameters. In our understanding, Digital IC is difficult to be implemented for co-site BS especially in two base stations. This item is included in the following table in brackets.

Table 7: CLI analysis parameters

| Item |  | Parameters | Value |
| --- | --- | --- | --- |
| General assumption |  | BS total Tx power @ DL SB (dBm) |  |
|  |  | SBFD configuration |  |
|  |  | Guard band assumption |  |
|  |  | Rx in-band blocking capability (dBm) |  |
|  |  | Rx ADC dynamic range (dB) |  |
|  |  | BS NF (dB) |  |
| Tx leakage contribution |  | Tx SB ACLR (dB) |  |
|  |  | Sector isolation (dB) |  |
|  |  | [Beam nulling/isolation (dB)] |  |
|  |  | [Digital IC for Tx leakage (dB)] |  |
|  |  | Overall RSIC for Tx leakage (dB) |  |
|  |  | Tx leakage contribution to CLI (dBm) |  |
| Rx path noise contribution | Blocking analysis | Overall interference cancllation for blocking capability (dB) |  |
|  |  | Tx SB interference signal level at Rx path (dBm) |  |
|  | Rx IMD | Rx IIP3 capability (dBm) |  |
|  |  | Rx IM3 contribution (dBm) |  |
|  | Rx ACS | ACS capability (dB) |  |
|  |  | Rx ACS contribution (dBm) |  |
|  | Rx path overall noise contribution (dBm) |  |  |
| Total CLI contribution (dBm) |  |  |  |
| Overall REFSENS degradation (dB) |  |  |  |

Proposal 2: Table 7 can be used for CLI analysis parameters alignment in the discussion..

  - RF Requirement Impact

2.3.1 New RF requirements for SBFD

In RAN4#104bis-e, the following agreements on RF requirement impact were reached [1]. 2.5.1 Potentially impacted RF requirement for SBFD capable gNB.

Our views on the potentially impacted RF requirements are summarized in the following table:

Table 8: New RF requirements for SBFD

| Requirements | Needed? | Comment |
| --- | --- | --- |
| In-channel adjacent subband leakage ratio | yes |  |
| In-channel adjacent subband Blocking+Adjacent subband selectivity | yes | Tx signal received in Rx path leads to both IM3 and ACS to Rx SB. The name of this requirement can be discussed later. |
| Receiver OTA Reference | yes |  |
| Receiver intermodulation | FFS |  |
| Transmitter intermodulation (for FR2 only) | FFS |  |

2.3.2 Existing requirement without impact for SBFD capable gNB

  - For the existing requirements with respect to wanted signal as below, our understanding is summarized in the following table:

Table 9: Existing requirements with respect to wanted signal

|  | Requirements |
| --- | --- |
| Conducted RF requirement | BS output power |
|  | Output power dynamics |
|  | Transmit ON/OFF power |
|  | Transmitted signal quality |
|  | Occupied bandwidth |
|  | Dynamic range |
| Radiated RF requirement | Radiated transmit power |
|  | OTA base station output power |
|  | OTA output power dynamics |
|  | OTA transmitted signal quality |
|  | OTA occupied bandwidth |
|  | OTA dynamic range |

  - Receiver out-of-band blocking and receiver spurious emission requirement in TS38.104 are still applicable gNB capable of SBFD.

Co-location minimum requirements for BS type 1-C and BS type 1-H of out-of-band blocking need further study for SBFD gNBs. The requirement assume a 30 dB coupling loss between interfering transmitter and NR BS receiver and are based on co-location with base stations of the same class, 30dB coupling loss can’t satisfy SBFD isolation requirement.

Observation 13: Co-location minimum requirements of out-of-band blocking need further study for SBFD gNBs.

- Summary

- gNB Self-interference

Observation 1: The Tx leakage and Rx contribution requirements for 1 dB SI target can be divided to -105 dBm each if each of them contributes to half of the noise.

Observation 2: The overall RSIC requirement for Tx leakage should be at least 148 dB for WA SBFD BS.

Observation 3: The RSIC requirement for Rx blocking issue should be 89 dB for WA SBFD BS.

Observation 4: If IM3 contribution is -108 dBm for WA BS SBFD Rx noise, IIP3 should be at least-10.5 dBm in whole operation temperature range, which is very challenging.

Observation 5: If Rx blocking level is assumed to be the current WA BS requirement, ADC dynamic range can cover the whole signal level including IBB blocking and the Rx wanted signal.

Observation 6: AGC is not need to be analysed for BS Rx path because ADC dynamic is not a problem.

Observation 7: 65 dB ACS capability is needed for WA BS. It is challenging if SU is the same with current specification.

Proposal 1: SI analysis parameters is specified in Table4 .

Co-channel inter-cell co-site inter-sector gNB-to-gNB inter-subband CLI

Observation 8: The Tx leakage and Rx contribution requirements to the CLI can be divided to -108 dBm each if each of them contributes to half of noise.

Observation 9: The interference cancellation requirement for Tx leakage issue should be 151 dB for co-channel co-site inter-sector CLI, which is very challenging.

Observation 10: The interference cancellation requirement for Rx blocking issue should be 92 dB for co-channel co-site inter-sector CLI.

Observation 11: If IM3 contribution is -108 dBm for co-channel co-site inter-sector CLI Rx noise, IIP3 should be at least-9 dBm in whole operation temperature range, which is very challenging.

Observation 12: 68 dB ACS capability is needed for co-channel co-site inter-sector CLI. It is challenging if SU is the same with current specification.

Proposal 2: Table 7 can be used for CLI analysis parameters alignment in the discussion..

- RF Requirement Impact

  - New RF requirements for SBFD

Proposal 3: New RF requirements for SBFD are proposed in Table8 .

  - Existing requirement without impact for SBFD capable gNB

Proposal 4: Existing requirements without change are proposed in Table9 .

Observation 13: Co-location minimum requirements of out-of-band blocking need further study for SBFD gNBs.

Reference

[1] R4-2217464, “WF on SBFD feasibility study and RF impact: BS aspects”，RAN4#104bis-e