3GPP TSG-RAN WG4 Meeting #105                              	R4-2218575

Toulouse, France, 14th Nov. – 18th Nov., 2022

Source:	Xiaomi

Title:	Discussion on RRM requirements for FR2 HST CA

Agenda Item:	8.12.5.2

Document for:	Discussion

# 1 Introduction

In RAN4#104-bis-e meeting, RAN4 has some initial discussion on RRM impact due to FR2 HST CA operation, and the related WF was approved in [1]. In this contribution, we would like to discuss the RRM requirements for FR2 HST CA and provide our proposals.

- Discussion

- Impacted RRM requirement for CA Agreement: - For NSCC_SSB for CSSFoutside_gap,i, Rel-17 fixed requirement is applied to both FR1 and FR2 HST if CA operation is applied. Way forward: Open issue needs further discussion: - For Rel-18 FR2 PC6, by introducing CA operation, RRM impacts are expected for: - Measurement period for Inter-frequency measurement - L1-RSRP measurement requirements - FFS whether the follow requirements shall be considered: - PSS/SSS detection, Time index detection for Intra-frequency and inter-frequency measurements - IDLE mode in inter-frequency measurement requirements - Beam failure recovery on SCell - TCI state switch in SCell. - SCell activation and deactivation delay - FFS whether deactivated SCell and corresponding requirements shall be considered.

In Rel-18 FR2 HST, only intra-band CA scenario is supported. According to the principle defined in Rel15, if there is at least one active serving cell on one FR2 band, the timing of the active serving cell can be applied to other neighbour cell of the same band. Thus, UE is not required to detect PSS/SSS and SSB index for the neighbour cell in intra-band CA scenario.

Proposal 1: RAN4 not to define PSS/SSS detection and SSB index detection requirement for intra-band CA scenario in FR2 HST.

Regarding whether deactivated SCell and corresponding requirements should be considered in FR2 HST CA scenario, since in FR2 HST scenarios, only CPE UE (PC6) is considered, the power consumption is not a critical issue. Thus, we think the SCell would not be activated/deactivated dynamically, and the corresponding requirements for deactivated SCell are not needed.

Proposal 2: RAN4 not to define requirements of deactivated SCell for intra-band CA scenario in FR2 HST.

Proposal 3: RAN4 not to define SCell activation/deactivation delay requirements for intra-band CA scenario in FR2 HST.

In Rel-17, the requirements of PSS/SSS detection and intra-/inter-frequency measurement are enhanced by a configurable scaling factor under the case of non-DRX and DRX cycle ≤ 80ms, and the FR2 HST enhancement is applied to SMTC periodicity ≤ 40ms. From our perspective, similar enhancement on PCell measurement in R17 FR2 HST could be reused for active SCell measurement in Rel-18 FR2 HST enhancement.

Table 9.2.5.1-11: Time period for PSS/SSS detection when [highSpeedMeasFlagFR2-r17] is configured, (Frequency range FR2) when SMTC period <= 40ms

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80ms | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms< DRX cycle≤ 320ms | ceil(1.5 x Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if [highSpeedMeasFlagFR2-r17 = set1] or M1 = 18 if [highSpeedMeasFlagFR2-r17 = set2] NOTE 3: Mpss/sss_sync_w/o_gaps =24. |  |

Proposal 4: For active SCell measurement requirement, the enhanced requirement defined in R17 FR2 HST can be reused in Rel-18 FR2 HST.

In Rel-17 FR2 HST, the BFD requirement was enhanced by a configurable scaling factor under the case of non-DRX and DRX cycle ≤ 80ms, while CBD requirement has not been enhanced. According to TS38.306, the beam failure recovery on SCell is an optional feature as shown in below, RAN4 need to decide whether the train roof-mounted high-power devices (FR2 HST UE) support the beam failure recovery on SCell. If yes, RAN4 need to define the enhanced requirement for BFD and the enhanced BFD requirement defined in Rel-17 FR2 HST can be reused.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxNumberSCellBFR-r16 Defines the maximum number of SCells configured for SCell beam failure recovery simultaneously. The UE indicating support of this also indicates the capabilities of maxNumberCSI-RS-BFD, maxNumberSSB-BFD and maxNumberCSI-RS-SSB-CBD. | Band | No | N/A | N/A |

Table 8.5.2.2-3: Evaluation period TEvaluate_BFD_SSB for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P N Note2) TSSB) |
| DRX cycle ≤ 80ms | Max(50, Ceil(7.5 P N Note2) Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. Note 2: scaling factor N=2 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N=6 when highSpeedMeasFlagFR2-r17 is configured to set2.ctor N=6 when [highSpeedMeasFlagFR2-r17] is configured to [set2]. |  |

Observation 1: RAN4 needs to decide whether the train roof-mounted high-power devices (FR2 HST UE) support the beam failure recovery on SCell.

Proposal 5: The enhanced BFD requirement defined in Rel-17 FR2 HST can be reused in Rel-18 FR2 HST if the FR2 HST UE supports the beam failure recovery on SCell.

- Conclusion

In this contribution, we discuss the RRM requirements for intra-band CA in FR2 HST and provide our proposals as follows:

Proposal 1: RAN4 not to define PSS/SSS detection and SSB index detection requirement for intra-band CA scenario in FR2 HST.

Proposal 2: RAN4 not to define requirements of deactivated SCell for intra-band CA scenario in FR2 HST.

Proposal 3: RAN4 not to define SCell activation/deactivation delay requirements for intra-band CA scenario in FR2 HST.

Proposal 4: For active SCell measurement requirement, the enhanced requirement defined in R17 FR2 HST can be reused in Rel-18 FR2 HST.

Observation 1: RAN4 needs to decide whether the train roof-mounted high-power devices (FR2 HST UE) support the beam failure recovery on SCell.

Proposal 5: The enhanced BFD requirement defined in Rel-17 FR2 HST can be reused in Rel-18 FR2 HST if the FR2 HST UE supports the beam failure recovery on SCell.

- Reference

[1]	R4-2217255,	WF on other RRM core requirement impacts for FR2 HST enhancement, Nokia, Nokia Shanghai Bell