3GPP TSG-RAN WG4 Meeting # 105      		R4-2218656

Toulouse, France, Nov 14 – Nov 18, 2022

Source:	CMCC

Title:	Discussion on test cases for RLM/BFD measurement relaxation

Agenda Item:	5.3

Document for:	Discussion

- Introduction

In RAN4#104-e meeting, we discussed the details about test setup for each test case, a way forward was agreed in [1]. In this contribution, we further discuss the test design for multiple RSs.

- Discussion

Two issues about multiple RSs are still open so far, we list them as below for information.

Issue 2-1-2: Test set up on the number of RSs Agreement: - Choose one test case with multiple RS configured - Revisit the above agreement if needed when the maintenance part for multiple RS is concluded. - For the rest test cases, configure the single RS. - Configure multiple RS for either TC 13 or TC 16. The conclusion will be made in RAN4#104b meeting. Issue 2-2-1: RLM OOS test – exit relaxation mode during T2 or T3 Agreement: - Option 2: UE is expected to exit relaxation mode during T3 - SNR2 is set as same as SNR1 which is higher than the entering threshold. - UE is allowed to send OOS indication based relaxed RLM evaluation during T3. - The length of D1 is calculated based on relaxed RLM evaluation period. - Option 2 does not preclude multiple RS configurations. Option 2 is only applied the first configured RS.

RAN4 has not decided which TC will be configured with multiple RSs. Two candidates are TC 13 and TC 16, both are BFD test case.

| Case |  | Test case |  | TC# |
| --- | --- | --- | --- | --- |
| FR2 SSB | EN-DC | Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX= 40 ms | A.5.5.5.X | TC13 |
| FR2 CSI-RS | NR-SA | Beam Failure Detection and Link Recovery Test for FR2 Pcell configured with CSI-RS-based BFD and LR in DRX= 40 ms | A.7.5.5.X | TC16 |

We think it is feasible to configure 2 BFD-RSs for both TC13 and TC16 from a technical perspective. Since TC16 has already been configured with low mobility criterion, to avoid a too complicated test, TC13 has more supporters in last meeting, we are also fine with TC13 to achieve a quick conclusion.

Proposal 1: Configure 2 BFD-RSs in q0 for TC13.

Before discussing the SNR setting for the second BSD-RS, we would like to first review the entering and exiting conditions:

Agreement in RAN4#103-e meeting: - For entering condition, - good serving cell quality criterion is fulfilled when the radio link quality is better than the threshold (Qin + X dB) for at least one resource in the set of resources for RLM/BFD. Agreement in RAN4#104-e meeting: - For exiting condition, - The UE is no longer allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met: - The UE sends out-of sync indications to the higher layers, - The timer T310 is running. - No DRX is used - The UE is no longer allowed to relax BFD measurements and apply the relaxed link recovery procedures provided that at least one of the following conditions is met: - The timer beamFailureDetectionTimer is running. - No DRX is used

In order to verify UE correct behavior for both entering and exiting, we propose to define the SNR for 2RSs as shown in Figure 1:

Figure 1. SNR settings for multiple RSs BFD test case

For entering procedure (T1), only the first RS meets the (Qin+XdB) condition. The design is to verify UE can enter the power saving even if only one RS’s quality is above the threshold. The SNR for the second RS should below the Qin+XdB, we propose to use legacy SNR2 here, which is -3dB.

For the relaxed measurement procedure (T2), we design that only the first RS is above Qout_LR. In this case, UE will not send beam failure indication or trigger beamFailureDetectionTimer, UE still can work in power saving mode. The SNR for the second RS should below the Qout_LR, we propose to use legacy SNR3, which is -12dB.

For the exiting procedure (T3), all the RSs should below Qout LR, the SNR value can keep -12dB.

Proposal 2: For the SNR setting of the second RS:

- SNR1 below the Qin+XdB，for example -3dB in T1

- SNR2 and SNR 3 below the Qout_LR，for example -12dB in T2 and T3

We also provide the related CR at R4-2218657.

- Conclusion

In this contribution, we give our consideration of test configuration on multiple RSs test case, the proposals are provided as follows:

Proposal 1: Configure 2 BFD-RSs in q0 for TC 13.

Proposal 2: For the SNR setting of the second RS:

- SNR1 below the Qin+XdB，for example -3dB in T1

- SNR2 and SNR3 below the Qout_LR，for example -12dB in T2 and T3

- References

R4-2214474, WF on RLM/BFD relaxation for UE Power Saving enhancements.

R4-2210613, WF on RLM/BFD relaxation for UE Power Saving enhancements.